| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0284 |
| --- | --- |

| Title: | Non-SCE4: Verification of on Weighted Prediction Based Color Gamut Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution verifies performance of weighted prediction based method proposed by Qualcomm for color gamut scalability. In average in SCE4 tests the Luma BD-rate gain 5,2% (for 10 bits base layer) and 4,8% (for 8 bits base layer) is confirmed.

# Introduction

In order to achieve the same effect gain-offset model for color gamut scalability shows in verified contribution HEVC weighted prediction is applied. The only modification compare to HEVC design is new flag in PPS.  This flag indicates the case when weighted prediction is applied for inter-layer texture prediction (in order to achieve color gamut scalability), but not temporal references.

# Test results

Tests were performed using special video sequences distributed in SCE4. Spatial scalability ratio is 2. All intra and random access configurations were tested. Table 1 summaries test results. In 5,2% (for 10 bits base layer) and 4,8% (for 8 bits base layer) is confirmed.

Table 1. The summary test results (SCE4 content)

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -6,5% | -5,2% | -8,5% | -6,0% | -4,7% | -9,0% |
| Overall (Test vs Ref) | -6,5% | -5,2% | -8,5% | -6,0% | -4,7% | -9,0% |
| Overall (Test vs single layer) | 18,6% | 22,7% | 17,9% | 22,0% | 25,8% | 20,0% |
| Overall (Ref vs single layer) | 26,8% | 29,4% | 28,9% | 29,8% | 32,1% | 32,1% |
| EL only (Test vs Ref) | -11,8% | -10,4% | -13,6% | -11,3% | -10,0% | -14,2% |
| Enc Time[%] | 99,5% |  |  | 99,7% |  |  |
| Dec Time[%] | 101,7% |  |  | 110,6% |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -4,0% | -1,6% | -5,3% | -3,5% | -1,1% | -4,9% |
| Overall (Test vs Ref) | -4,0% | -1,6% | -5,3% | -3,5% | -1,1% | -4,9% |
| Overall (Test vs single layer) | 27,1% | 33,3% | 25,8% | 29,0% | 35,0% | 27,2% |
| Overall (Ref vs single layer) | 32,3% | 35,5% | 33,0% | 33,8% | 36,4% | 33,9% |
| EL only (Test vs Ref) | -7,0% | -4,4% | -8,1% | -6,5% | -3,8% | -7,5% |
| Enc Time[%] | 134,9% |  |  | 135,0% |  |  |
| Dec Time[%] | 104,7% |  |  | 112,6% |  |  |
|  | -5,2% | -3,4% | -6,9% | -4,8% | -2,9% | -6,9% |

# Conclusion

Samsung confirms the performance of weighted prediction based method proposed by Qualcomm for color gamut scalability. In average in SCE4 tests the Luma BD-rate gain 5,2% (for 10 bits base layer) and 4,8% (for 8 bits base layer) is confirmed.