| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0285 |
| --- | --- |

| Title: | non-SCE3: Verification for simplified design of sharpening inter-layer filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution verifies performance and complexity assessment test for simplified version sharpening inter-layer filter presented by Huawei in SCE3. Average Luma BD-rate gain 1,2%, 0,7% and 0,4% is confirmed in spatial scalability ×2, ×1.5 and SNR scalability test correspondently. Measured encoding and decoding time compare to anchor is 105% and 116% correspondently.

# Introduction

Simplifications of [1] verified in this report mainly affect processing of Chroma component by sharpening filter.

The multipass processing of inter-layer reference frame is replaced one pass which includes all processes performed with few lines delay. So this part of simplification is just implementation issue.

# The summary of test results

Test were performed according to SCE3 description for both ×2 and ×1,5 spatial scalability and SNR scalability tests. All intra, random access, low-delay-B and low-delay-P tests were performed. Results we obtained perfectly match with proponent’s data. Performance is summarized in the Table 1 below.

Table 1. Performance test results summary.

# Conclusion

Samsung confirms test results reported in SCE3 for sharpening inter-layer filter. Average Luma BD-rate gain 1,2%, 0,7% and 0,4% is confirmed in spatial scalability ×2, ×1.5 and SNR scalability test correspondently.

# References

[1]M. Sychev, V. Anisimovskiy, S. Ikonin, “Inter-layer prediction modes based on base layer sharpness filter”, JCTVC-N0070, Vienna, Austria, July 2013.