| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0288 |
| --- | --- |

| Title: | Proposal on Main 4:2:2 10 profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Shun-ichi Sekiguchi Hiroharu Sakate 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution studies profile specification adopted in RExt DAM and proposes a few modifications to the specification of “Main 4:2:2 10” profile. First of all, this document provides a result of investigation on the coding performance of RExt coding tools, which have been adopted during RExt activity, for YUV4:2:2 10 bit sequences using the latest HM software. From this investigation, it has been confirmed that the RExt coding tools have small impact on coding efficiency for natural YUV4:2:2 10 bit source. This contribution also proposes a few modifications to the description of “Main 4:2:2 10” profile specified in DAM, based on the investigation results and the proposal at the last meeting as described in JCTVC-N0312.

# Introduction

A RExt profile supporting YUV4:2:2 10bit format was proposed in JCTVC-N0312[1]. The proposed profile extends “Main 10” profile to support 4:2:2 chroma sampling format without any additional coding tools relative to version1 specification. As a result of discussions at the last meeting, the RExt DAM adopted “Main 4:2:2 10” profile and “Main 4:2:2 12” profile with allowing to use all RExt coding tools.

In this contribution, we will investigate the coding performance of the RExt coding tools, which have been adopted during RExt activity, in order to clarify performance impact of these tools for natural YUV4:2:2 10 bit video sources. We also propose modifications to the description of “Main 4:2:2 10” profile specified in DAM, based on the investigation results and the proposal at the last meeting as described in JCTVC-N0312.

# Simulation results

The performance evaluation of the following test sets is conducted under the RExt common test conditions(CTC)[2]. “Anchor” means version 1 specification that disables the RExt coding tools. “Test 1” is an encoding configuration turns on all screen contents specific tools relative to version 1 specification. “Test 2” is to evaluate an impact of the extension to transform accuracy relative to version 1 specification. Test sequences defined YUV4:2:2 test sets in the RExt CTC are used in this experiment.

Table 1. Encoder configurations related to the RExt coding tools

| syntax | Test sets |  |  |
| --- | --- | --- | --- |
|  | Anchor | Test 1 | Test 2 |
| transform_skip_rotation_enabled_flag | 0 | 1 | 0 |
| transform_skip_context_enabled_flag | 0 | 1 | 0 |
| intra_block_copy_enabled_flag | 0 | 1 | 0 |
| residual_dpcm_intra_enabled_flag | 0 | 1 | 0 |
| residual_dpcm_inter_enabled_flag | 0 | 1 | 0 |
| extended_precision_processing_flag | 0 | 0 | 1 |
| intra_smoothing_disabled_flag | 0 | 1 | 0 |
| log2_max_transform_skip_size_minus2 | 0 | 3 | 0 |

Table 2 and Table 3 show the coding performance of “Test 1” and “Test 2” compared with the anchor, respectively. Table 2 shows that the screen contents specific tools have small impact on coding performance without “EBUGraphics” but have significant impact on encoding complexity. It is noted that “EBUGraphics” is not a natural video source and is a sequence consisted of moving CG graphics and scrolling text content over still picture background. Table 3 indicates that extending accuracy of transform operation does not give coding efficiency improvements for Main/High-tier and achieves only 0.2% gain for Super-High-tier.

Even though this document does not provide experimental evidences on other RExt profiles in DAM than Main 4:2:2 10, it might be worth to do this kind of investigation to define RExt profiles targeting natural video source.

Table 2. BD bitrate savings of “Test 1”

Table 3. BD bitrate savings of “Test 2”

# Proposed modifications to Main 4:2:2 10 profile

Based on the simulation results in section 2 and the proposal made in JCTVC-N0312 at the last meeting, we propose the following modifications to the description of “Main 4:2:2 10” profile in RExt DAM. The proposed changes are highlighted in yellow.

A.3.5 Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 and Main 4:4:4 12 profiles

Bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 and Main 4:4:4 12 profiles shall obey the following constraints:

- In bitstreams conforming to the Main 12, Main 4:2:2 10 or Main 4:2:2 12  profile, SPSs shall have chroma_format_idc constrained as follows:

–	In bitstreams conforming to the Main 12 profile, SPSs shall have chroma_format_idc equal to 0 or 1 only.

–	In bitstreams conforming to the Main 4:2:2 10 or Main 4:2:2 12 profile, SPSs shall have chroma_format_idc equal to 0, 1 or 2 only.

- SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 constrained as follows:

–	In bitstreams conforming to the Main 4:2:2 10 or Main 4:4:4 10 profile, SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

–	In bitstreams conforming to the Main 12, Main 4:2:2 12, or Main 4:4:4 12 profile, SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 in the range of 0 to 4, inclusive.

- In bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 and Main 4:4:4 12 profiles,

- SPSs shall have separate_colour_plane_flag, when present, equal to 0 only.

- SPSs shall have extended_precision_processing_flag, when present, equal to 0 only.

- CtbLog2SizeY shall be in the range of 4 to 6, inclusive.

- When a PPS has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a PPS has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

- general_level_idc shall not be equal to 255 (which indicates level 8.5).

- The level constraints specified for the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 or Main 4:4:4 12 profile in in subclause Error: Reference source not found, as applicable, shall be fulfilled.

Bitstreams conforming to the Main 4:2:2 10 profile shall obey the following constraints:

- transform_skip_rotation_enabled_flag, when present, shall be equal to 0.

- transform_skip_context_enabled_flag, when present, shall be equal to 0.

- intra_block_copy_enabled_flag, when present, shall be equal to 0.

- residual_dpcm_intra_enabled_flag, when present, shall be equal to 0.

- residual_dpcm_inter_enabled_flag, when present, shall be equal to 0.

- extended_precision_processing_flag, when present, shall be equal to 0.

- intra_smoothing_disabled_flag, when present, shall be equal to 0.

- log2_max_transform_skip_size_minus2, when present, shall be equal to 0.

Conformance of a bitstream to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 or Main 4:4:4 12 profile is indicated by general_profile_idc being equal to 4 or general_profile_compatibility_flag[ 4 ] being equal to 1 with the following additional indications as applicable:

- Conformance of the bitstream to the Main 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 1, and general_max_420chroma_constraint_flag equal to 1.

- Conformance of the bitstream to the Main 4:2:2 10 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 1, general_max_422chroma_constraint_flag equal to 1, and general_max_420chroma_constraint_flag equal to 1.

- Conformance of the bitstream to the Main 4:2:2 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 1, and general_max_420chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:4:4 10 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 1, general_max_422chroma_constraint_flag equal to 0, and general_max_420chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:4:4 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 0, and general_max_420chroma_constraint_flag equal to 0.

All other combinations of general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_constraint_flag, and general_max_420chroma_constraint_flag with general_profile_idc equal to 4 or general_profile_compatibility_flag[ 4 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. Such combinations shall not be present in bitstreams conforming to this specification. However, decoders conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 or Main 4:4:4 12 profile shall allow other combinations as specified below in this subclause to occur in the bitstream.

Decoders conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 or Main 4:4:4 12 profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which all of the following conditions apply:

- One or both of the following conditions apply:

–	general_profile_idc is equal to 1 or 2, general_profile_compatibility_flag[ 1 ] is equal to 1, general_profile_compatibility_flag[ 2 ] is equal to 1.

–	general_profile_idc is equal to 4 or general_profile_compatibility_flag[ 4 ] is equal to 1, and any of the following conditions apply:

–	The decoder conforms to the Main 12 profile, general_max_12bit_constraint_flag is equal to 1, and general_max_420chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:2:2 10 profile, general_max_10bit_constraint_flag is equal to 1, and general_max_422chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:2:2 12 profile, general_max_12bit_constraint_flag is equal to 1, and general_max_422chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:4:4 10 profile and general_max_10bit_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:2:2 12 profile and general_max_12bit_constraint_flag is equal to 1.

- general_level_idc is not equal to 255 and represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.

# Level definition

On profile-specific level definition, we propose to only modify the maximum bitrate and CPB size for professional use case. The detail of the proposed definition is shown as follows. It is noted that this contribution only proposes the definition relevant to “Main 4:2:2 10” profile and does not claim to change to the specific values highlighted in blue on the other profiles.

Table A-3– Specification of CpbBrVclFactor, CpbBrNalFactor, FormatCapabilityFactor, and MinCrScaleFactor

| Profile | CpbBrVclFactor | CpbBrNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main | 1000 | 1100 | 1.500 | 1.0 |
| Main 10 | 1000 | 1100 | 1.875 | 1.0 |
| Main 12 | 1000 | 1100 | 2.250 | 1.0 |
| Main 4:2:2:10 | 1600 | 1920 | 2.500 | 0.5 |
| Main 4:2:2 12 | 1600 | 1920 | 3.000 | 0.5 |
| Main 4:4:4 10 | 1600 | 1920 | 3.750 | 0.5 |
| Main 4:4:4 12 | 1600 | 1920 | 4.500 | 0.5 |

# Conclusions

This contribution has proposed a few modifications to the specification of “Main 4:2:2 10” profile in RExt DAM, based on technical investigation on the coding performance of RExt coding tools and the proposal made in JCTVC-N0312 at the last Vienna meeting. We recommend adopting the proposed modifications into study text of DAM.

# References

- S. Sekiguchi, A. Minezawa, H. Sakate and K. Kazui, “A proposal on HEVC 4:2:2 profile,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N0312, 14th Meeting: Vienna, AT, July, 2013.

- D. Flynn, K. Sharman and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N1006, 14th Meeting: Vienna, AT, July, 2013.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).