| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0295 |
| --- | --- |

| Title: | RCE2: Cross-check of Test C2 JCTVC-O0065 Entropy Coding Compression Efficiency for High Bit Depths |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the results and the implementation of RCE2 Test C2 JCTVC-O0065.

# Introduction

JCTVC-O0065 presents test C.2 of RCE2, a scheme modifying the initial Rice parameter based on already codec coefficients. This contribution presents the cross-check report of JCTVC-O0065.

# Experimental results

The JCTVC-O0065 software provided by the proponents has been used to generate the results and compare with results generated with HM-12.0+RExt-4.1 under RCE2 common test conditions [1]. Decoding time was obtained using homogeneous cluster. It was confirmed that for the test cases, the BD-rate results exactly match to the results provided by the proponents.

Table 1 Results for AHG8 lossless coding

|  | AI Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 4.57 | 4.62 | 5.57 | 5.64 | 2.27 | 2.27 | 11.14 | 11.24 | 1.1% | 1.1% | 0.3% | 2.1% |
| Class B | 2.24 | 2.24 | 2.26 | 2.26 | 2.08 | 2.08 | 2.44 | 2.44 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 7.88 | 8.45 | 9.5 | 10.72 | 5.23 | 5.46 | 14.73 | 18.24 | 6.8% | 9.1% | 1.4% | 19.2% |
| Animation RGB 444 | 2.48 | 2.48 | 2.5 | 2.52 | 2.15 | 2.15 | 3.05 | 3.05 | 0.1% | 0.1% | 0.0% | 0.2% |
| SC YUV 444 | 11.09 | 11.48 | 13.2 | 14.03 | 7.87 | 7.95 | 19.32 | 21.96 | 3.3% | 4.8% | 0.6% | 12.0% |
| Animation YUV 444 | 3.00 | 3.00 | 3.2 | 3.17 | 2.57 | 2.57 | 3.93 | 3.93 | 0.2% | 0.1% | 0.0% | 0.4% |
| RangeExt | 1.92 | 1.97 | 2.38 | 2.42 | 1.46 | 1.52 | 4.37 | 4.39 | 2.7% | 2.2% | 0.4% | 4.2% |
| SC GBR 444 Optional | 20.30 | 24.86 | 24.82 | 32.40 | 9.69 | 11.11 | 36.10 | 48.59 | 18.4% | 20.7% | 12.8% | 25.7% |
| SC YUV 444 Optional | 31.85 | 37.19 | 34.39 | 40.59 | 22.30 | 25.26 | 51.97 | 62.34 | 14.4% | 14.6% | 11.7% | 16.6% |
| Enc Time[%] |  |  | 102% |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  | 100% |  |  |  |  |  |  |  |  |  |

|  | RA Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.66 | 8.67 | 36.22 | 36.63 | 3.01 | 3.01 | 90.48 | 91.78 | 0.1% | 0.6% | 0.0% | 1.4% |
| Class B | 2.60 | 2.60 | 2.60 | 2.60 | 2.57 | 2.57 | 2.64 | 2.64 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 53.71 | 55.31 | 117.01 | 132.09 | 19.48 | 19.70 | 225.68 | 270.78 | 2.9% | 7.3% | 1.1% | 19.2% |
| Animation RGB 444 | 3.58 | 3.59 | 3.60 | 3.60 | 3.43 | 3.43 | 3.70 | 3.70 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 73.79 | 74.86 | 157.08 | 167.74 | 24.42 | 24.52 | 287.13 | 326.27 | 1.4% | 4.0% | 0.4% | 12.0% |
| Animation YUV 444 | 3.8 | 3.84 | 4.48 | 4.48 | 2.75 | 2.76 | 5.70 | 5.70 | 0.1% | 0.0% | 0.0% | 0.1% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.5 | 1.5 | 1.59 | 4.41 | 4.43 | 1.9% | 1.6% | 0.0% | 4.1% |
| SC GBR 444 Optional | 42.0 | 47.7 | 254.1 | 337.0 | 11.5 | 12.85 | 395.52 | 516.31 | 12.0% | 20.2% | 10.9% | 26.2% |
| SC YUV 444 Optional | 101.1 | 113.6 | 314.9 | 376.6 | 30.5 | 33.83 | 531.53 | 644.54 | 11.0% | 14.2% | 10.0% | 17.5% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |  |  |  |  |  |  |

|  | LB Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.9 | 8.9 | 57.8 | 58.1 | 3.0 | 3.0 | 169.5 | 170.6 | 0.1% | 0.3% | 0.0% | 0.6% |
| Class B | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 62.2 | 63.4 | 355.6 | 421.1 | 20.5 | 20.7 | 1618.0 | 2001.2 | 1.9% | 6.5% | 0.9% | 19.1% |
| Animation RGB 444 | 3.6 | 3.6 | 3.6 | 3.6 | 3.4 | 3.4 | 3.7 | 3.7 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 85.1 | 85.8 | 469.0 | 517.8 | 25.7 | 25.7 | 2105.0 | 2397.8 | 0.8% | 3.7% | 0.3% | 12.2% |
| Animation YUV 444 | 4 | 3.9 | 4.5 | 4.5 | 2.8 | 2.8 | 5.8 | 5.8 | 0.1% | 0.0% | 0.0% | 0.1% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.5 | 1.5 | 1.6 | 4.4 | 4.4 | 1.9% | 1.6% | 0.0% | 4.1% |
| SC GBR 444 Optional | 45 | 50.7 | 1010.8 | 1350.7 | 11.6 | 13.0 | 2253.7 | 2998.9 | 11.1% | 20.6% | 10.6% | 26.3% |
| SC YUV 444 Optional | 117.1 | 130.4 | 1139.4 | 1339.8 | 31.0 | 34.4 | 2208.6 | 2526.7 | 10.2% | 13.9% | 9.8% | 19.2% |
| Enc Time[%] | 103% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |  |  |  |  |  |  |

Table 2 Results of Setting 1 for AHG8 lossy coding

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.4% | -0.3% | -0.5% | -0.5% | -0.5% | -0.7% | -0.7% | -0.7% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.3% | -1.3% | -1.3% | -2.7% | -2.5% | -2.5% | -4.1% | -4.3% | -4.3% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.2% | -0.2% | -0.2% | -0.6% | -0.5% | -0.5% | -1.2% | -1.1% | -1.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -5.4% | -5.1% | -5.0% | -8.7% | -8.4% | -8.8% | -12.2% | -12.9% | -13.0% |
| SC(444) YUV Optional | -1.0% | -1.1% | -1.0% | -2.6% | -2.4% | -2.3% | -5.1% | -5.0% | -5.6% |
| Enc Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.1% | 0.0% | -0.3% | -0.2% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.0% | -1.1% | -1.1% | -2.2% | -2.1% | -2.1% |  |  |  |
| Animation RGB 444 | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | 0.0% | -0.1% | 0.0% | -0.2% | -0.3% | -0.3% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -5.1% | -4.8% | -4.7% | -8.3% | -8.2% | -8.4% |  |  |  |
| SC(444) YUV Optional | -0.7% | -0.7% | -0.8% | -2.1% | -2.0% | -2.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | -0.1% | -0.2% | -0.3% | -0.1% |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% |  |  |  |
| SC RGB 444 | -0.6% | -0.6% | -0.8% | -1.6% | -1.6% | -1.7% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -4.3% | -4.3% | -4.1% | -8.1% | -8.2% | -8.6% |  |  |  |
| SC(444) YUV Optional | -0.7% | -0.8% | -0.7% | -2.0% | -1.9% | -2.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 3 Results for AHG18 lossy coding - 6 bit HM forward transform

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -7.0% | -6.7% | -6.7% |
| 14-bit | -18.4% | -18.1% | -18.0% |
| 16-bit | -28.2% | -28.0% | -28.1% |
| Enc Time[%] | 88% |  |  |
| Dec Time[%] | 89% |  |  |

Table 4 Results of Setting 1 for AHG18 lossy coding – high precision forward transform

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -7.0% | -7.0% | -7.0% |
| 14-bit | -18.4% | -18.1% | -18.1% |
| 16-bit | -28.1% | -27.6% | -27.6% |
| Enc Time[%] | 91% |  |  |
| Dec Time[%] | 91% |  |  |

# References

- C. Rosewarne, J. Sole, K. Sharman, and S.-H Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization and update methods”, JCTVC-N1122, Vienna, Austria, Jul. 2013.

# Patent rights declaration(s)