| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0296 |
| --- | --- |

| Title: | AHG5: Cross-check of JCTVC-O0102 for block size restriction of intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the results of JCTVC-O0102.

# Introduction

JCTVC-O0102 presents a method to limit IntraBC to CU with 16x16 and 8x8 sizes. The cross-checkers verified the R-D results (table 3 and 4) of JCTVC-O0102.

# Experimental results

The JCTVC-O0102 software provided by the proponents has been used to generate the results and compare with results generated with HM-12.0+RExt-4.1 under RCE2 common test conditions [1]. Encoding time and decoding time were obtained using heterogeneous cluster. It was confirmed that for the test case, the BD-rate results exactly match to the results provided by the proponents.

Table 1 Results for AHG8 lossless coding

|  | AI Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 4.57 | 4.57 | 5.57 | 5.57 | 2.27 | 2.27 | 11.14 | 11.14 | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 2.24 | 2.24 | 2.26 | 2.26 | 2.08 | 2.08 | 2.44 | 2.44 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 7.88 | 7.88 | 9.5 | 9.52 | 5.23 | 5.23 | 14.73 | 14.73 | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation RGB 444 | 2.48 | 2.48 | 2.5 | 2.52 | 2.15 | 2.15 | 3.05 | 3.05 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 11.09 | 11.09 | 13.2 | 13.22 | 7.87 | 7.87 | 19.32 | 19.32 | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation YUV 444 | 3.00 | 3.00 | 3.2 | 3.16 | 2.57 | 2.57 | 3.93 | 3.93 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 1.92 | 1.92 | 2.38 | 2.38 | 1.46 | 1.46 | 4.37 | 4.37 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 20.30 | 20.30 | 24.82 | 24.82 | 9.69 | 9.69 | 36.10 | 36.10 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 Optional | 31.85 | 31.85 | 34.39 | 34.40 | 22.30 | 22.30 | 51.97 | 51.98 | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  |  | 95% |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  | 100% |  |  |  |  |  |  |  |  |  |

|  | RA Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.66 | 8.66 | 36.22 | 36.24 | 3.01 | 3.01 | 90.48 | 90.48 | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 2.60 | 2.60 | 2.60 | 2.60 | 2.57 | 2.57 | 2.64 | 2.64 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 53.71 | 53.72 | 117.01 | 117.02 | 19.48 | 19.48 | 225.68 | 225.70 | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation RGB 444 | 3.58 | 3.58 | 3.60 | 3.60 | 3.43 | 3.43 | 3.70 | 3.70 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 73.79 | 73.84 | 157.08 | 157.21 | 24.42 | 24.42 | 287.13 | 287.15 | 0.1% | 0.1% | 0.0% | 0.7% |
| Animation YUV 444 | 3.8 | 3.84 | 4.48 | 4.48 | 2.75 | 2.75 | 5.70 | 5.70 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.5 | 1.5 | 1.53 | 4.41 | 4.41 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 42.0 | 41.9 | 254.1 | 253.7 | 11.5 | 11.45 | 395.52 | 395.75 | 0.0% | -0.1% | -0.4% | 0.1% |
| SC YUV 444 Optional | 101.1 | 101.1 | 314.9 | 315.7 | 30.5 | 30.46 | 531.53 | 531.67 | 0.1% | 0.2% | 0.0% | 0.6% |
| Enc Time[%] | 94% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 91% |  |  |  |  |  |  |  |  |  |  |  |

|  | LB Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.9 | 8.9 | 57.8 | 57.8 | 3.0 | 3.0 | 169.5 | 169.5 | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 62.2 | 62.2 | 355.6 | 355.6 | 20.5 | 20.5 | 1618.0 | 1617.9 | 0.0% | 0.0% | -0.1% | 0.1% |
| Animation RGB 444 | 3.6 | 3.6 | 3.6 | 3.6 | 3.4 | 3.4 | 3.7 | 3.7 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 85.1 | 85.1 | 469.0 | 469.0 | 25.7 | 25.7 | 2105.0 | 2105.1 | 0.0% | 0.0% | -0.1% | 0.0% |
| Animation YUV 444 | 4 | 3.9 | 4.5 | 4.5 | 2.8 | 2.8 | 5.8 | 5.8 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.5 | 1.5 | 1.5 | 4.4 | 4.4 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 45 | 45.1 | 1010.8 | 1029.9 | 11.6 | 11.6 | 2253.7 | 2315.2 | 0.0% | 0.7% | -0.6% | 2.7% |
| SC YUV 444 Optional | 117.1 | 117.1 | 1139.4 | 1139.5 | 31.0 | 31.0 | 2208.6 | 2210.4 | 0.0% | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 92% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 84% |  |  |  |  |  |  |  |  |  |  |  |

Table 2 Results of Setting 1 for AHG8 lossy coding

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| SC(444) YUV Optional | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  | 96% |  |  | 95% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC(444) YUV Optional | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 92% |  |  | 92% |  |  |  |  |  |
| Dec Time[%] | 94% |  |  | 91% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.5% | -0.5% | 0.0% | -0.2% | -0.3% |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.3% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.6% | -0.7% | -0.8% | -0.6% | -0.6% | -0.7% |  |  |  |
| Animation YUV 444 | -0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | 0.5% | 0.6% | 0.4% | 0.8% | 0.9% | 0.6% |  |  |  |
| SC(444) YUV Optional | -0.8% | -0.7% | -0.4% | 0.3% | 0.4% | 0.6% |  |  |  |
| Enc Time[%] | 93% |  |  | 93% |  |  |  |  |  |
| Dec Time[%] | 92% |  |  | 87% |  |  |  |  |  |

# References

- C. Rosewarne, J. Sole, K. Sharman, and S.-H Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization and update methods”, JCTVC-N1122, Vienna, Austria, Jul. 2013.

# Patent rights declaration(s)