| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0298 |
| --- | --- |

| Title: | Non-SCE4: Cross-check of JCTVC-O0195 “Picture and region adaptive gain-offset prediction for color space scalability” |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or | Jie Zhao Seung-Hwan Kim | Email: | jzhao@sharplabs.com kimse@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This document reports the cross-check results of JCTVC-O0195 “non-SCE4 Picture and region adaptive gain-offset prediction for color space scalability” by Sony. JCTVC-O195 proposed adding picture and region adaptation to the gain-offset prediction from JCTVC-L0224 for color gamut scalable video coding. We tested the code, and confirmed that the software and result match with those in JCTVC-O0195.

# Introduction

JCTVC-O0195 extended the gain offset color predictor model proposed in JCTVC-L0334 to be picture and region adaptive. For region adaptation, each picture is divided into 1, 4, or 16 rectangular regions. Each region has its own gain and offset. For picture adaptation, the gain of offset is computed at every P-picture in the enhancement layer and applied to the P-picture and the B-pictures prior to the next P-picture in the encoding order. We checked the code and confirmed the code is as described in the document.

# Experimental results

We used the source code provided by proponent. The experimental setup corresponds to the SCE4 description [2].

Table 1 shows the summary RD performance of the tested method compared with the SHM-3.0.1-CE4 anchor in SCE4 common test conditions. The BD rate results matched with those provided by JCTVC-O0195. Below are the detailed results. Since our cluster is heterogeneous, we did not verify coding time.

Table 1. Summary result of JCTVC-O0195

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -7.7% | -6.2% | -9.8% | -6.9% | -5.6% | -10.1% |
| Overall (Test vs Ref) | -7.7% | -6.2% | -9.8% | -6.9% | -5.6% | -10.1% |
| Overall (Test vs single layer) | 17.0% | 21.3% | 16.2% | 20.8% | 24.6% | 18.6% |
| Overall (Ref vs single layer) | 26.8% | 29.4% | 28.9% | 29.8% | 32.1% | 32.1% |
| EL only (Test vs Ref) | -13.9% | -12.3% | -15.8% | -13.1% | -11.7% | -16.1% |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -3.3% | -0.9% | -4.2% | -3.0% | -0.4% | -3.7% |
| Overall (Test vs Ref) | -3.3% | -0.9% | -4.2% | -3.0% | -0.4% | -3.7% |
| Overall (Test vs single layer) | 27.9% | 34.3% | 27.3% | 29.8% | 35.9% | 28.7% |
| Overall (Ref vs single layer) | 32.3% | 35.5% | 33.0% | 33.8% | 36.4% | 33.9% |
| EL only (Test vs Ref) | -5.8% | -3.2% | -6.5% | -5.4% | -2.6% | -5.9% |

# Conclusions

It is confirmed that all RD results generated for the cross-check match the results provided in JCTVC-O0195. Software matches with the document.

# References

- Cheung Auyeung, “Non-SCE4: Picture and region adaptive gain-offset prediction for color space scalability”, JCTVC-O0195, Geneva, October 2013.

[2] A. Segall, P. Bordes, C. Auyeung, X. Li, E. Alshina, A. Duenas, “Description of Core Experiment SCE4: Color Gamut and Bit-Depth Scalability”, JCTVC-N1104, 14th Meeting: Vienna, Austria, AT, July, 2013.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).