| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0309 |
| --- | --- |

| Title: | Cross-check report of RExt: On motion compensation memory bandwidth constraint (JCTVC-O0221) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document is a cross-check report of JCTVC-O0221.

It is reported that the x-check result is identical to the result provided by the proponent.

It is commented that the proposed restriction is concern in terms of encoder complexity, and that considering not only frame-level worst case, but also local (e.g. CTU-level) worst case is useful. In addition, it is also commented that the proposed encoder-side restriction is difficult to ensure the worst case in the real scenario.

# Cross check

In JCTVC-O0221 [1], it is proposed to restrict the number of reference pixels for inter prediction in a frame in order to ensure the maximum bandwidth for RExt profile decoder. An encoder algorithm is implemented on top of RExt 4.1 software to obey the constraint. Also, decoder modification to calculate the worst case bandwidth for a frame is implemented.

The source code is tested in cross-checker environment. It is reported the cross-check result is identical to the proponent result in terms of RD-performance and the bandwidth calculation. The complete result is attached in the document.

# Comments from cross-checker

It is commented that considering implementation complexity for RExt profile is quite useful. However, the encoder complexity should be clarified to obey the proposed constraint since frame-level optimization is necessary for some architecture. For example, if a frame is encoded with several tiles and each tile is encoded independently, the proposed frame-level constraint is difficult to follow without communication among the independent cores (or, harder restriction is required for such encoder architecture).

From the decoder perspective, it is commented that considering local (e.g. CTU-level) worst case is also important because it may cause pipeline stall.

Also, it is asserted that an encoder side restriction can be violated by careless encoders. Thus, it is concerned if the proposed restriction is effective in the real scenario. It is noted that the memory bandwidth constraint is introduced in the version 1 spec as decoder side restriction in order to mandate the worst case number for every encoders.

# Reference

- JCTVC-O0221, “RExt: On motion compensation memory bandwidth constraint”

- JCTVC-N1006, “Common test conditions and software reference configurations for HEVC range extensions”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).