| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0313 |
| --- | --- |

| Title: | Cross-check of JCTVC-O0208 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of proposal JCTVC-O0208 by Qualcomm. The simulation result for all configurations were reported by Qualcomm and tested by Canon. The cross check result confirms the simulation result reported by Qualcomm.

# Cross Check Procedure

This is a cross-check of proposal JCTVC-O0208 [1].

The source code for JCTVC-O0208 was provided by Samsung. Canon compiled the source code using GCC 4.1.2 and the simulation was performed on a Linux cluster under Range extensions common test conditions and AHG18 lossy test conditions [2] [3]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code was studied and the following observations were made:

- A ‘previous Rice parameter’ variable is introduced to hold the Rice parameter from the previous sub-block.

- If the value of the previous Rice parameter is less than four, then the usual coding process for coef_abs_level_greater1_flag and coef_abs_level_greater2_flag is applied for the current sub-block.

- If the value of the previous Rice parameter is four to more, then coding of the coef_abs_level_greater1_flag and coef_abs_level_greater2_flag is skipped. In this case, the ‘baseLevel’ variable is set to 1 and the remaining coefficient magnitude is coded only using coef_abs_level_remaining.

These observations are consistent with the algorithm description for method 1 in [1].

# Coding Performance Measurement

Table 1 and Table 2 provide the cross-check results for JCTVC-O0208 under the Range extensions common test conditions and the AHG18 conditions, respectively (as defined in [2] and [3]). The simulation results for all cases match the provided results.

Note: Anchor and tested simulations were performed on different clusters, therefore run-time comparison is not valid for these reported results.

For the AHG18 conditions, the Range extensions backward compatibility for ticket #1026 was disabled by setting RExt__BACKWARDS_COMPATIBILITY_HM_TICKET_1026 = 0 in TypeDef.h.

Table 1. JCTVC-O0208 Range extensions common test conditions simulation result.

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 2. JCTVC-O0208 AHG18 simulation result – HM forward transform and ticket #1026 backward compatibility disabled.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | 0.6% | 0.7% | 0.7% |
| 14-bit | 0.2% | 0.3% | 0.3% |
| 16-bit | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 97% |  |  |

# Conclusion

This cross-check includes results for JCTVC-O0208 provided by Qualcomm. The provided software matches the description in the contribution. The cross-check result confirms the simulation result reported by Qualcomm.

# References

- J. Sole, R. Joshi, M. Karczewicz, “AhG5 and AhG18: Bypass of the coefficient level flags for higher throughput,” JCTVC-O0208, 15th JCT-VC Meeting, Geneva, CH, October 2013.

- D. Flynn, K. Sharman, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-N1006, 14th JCT-VC Meeting, Vienna, AT, July 2013.

- C. Rosewarne, J. Sole, K. Sharman, S. Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation and update methods”, JCTVC-N1122, 14th JCT-VC Meeting: Vienna, AT, July 2013.