| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0314 |
| --- | --- |

| Title: | Cross-check of JCTVC-O0259 on supporting higher bit-depth with transform skip only changes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunil Lee Chanyul Park Chanyul Kim | Email: | sunil.lee@samsung.com cy1205.park@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of JCTVC-O0259 on supporting higher bit-depth with transform skip changes only. The source code provided by the proponent was reviewed and it was confirmed that the implementation was consistent with the algorithm described in the contribution document. The coding performance is independently evaluated by Samsung and the results match those provided by the proponent.

# Introduction

This contribution reports the cross-verification results of JCTVC-O0259 on supporting higher bit-depth with transform skip changes only [1]. In this method, it is proposed that higher bit depth be supported by changing the transform skip processing path only, rather than requiring transforms capable of processing a larger data range when compared to HEVC version 1. Samsung reviewed the source code and confirmed that the implementation was consistent with the algorithm described in the contribution document. Samsung compiled the source code and did the simulation on a linux cluster under the AHG18 common test condition defined for RCE2 [2].

# Test results

The cross-verification results of JCTVC-O0259 on supporting higher bit-depth with transform skip changes only are summarized in Table 1, and they match those provided by the proponent.

Table 1. Cross-verification results of JCTVC-O0259

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | 0.6% | 0.9% | 0.9% |
| 14-bit | 1.1% | 2.3% | 2.6% |
| 16-bit | 0.6% | 0.4% | 0.4% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusion

This contribution reports the cross-verification results of JCTVC-O0259 on supporting higher bit-depth with transform skip changes only. It was confirmed that the implementation was consistent with the algorithm description and the test results matched those provided by the proponent.

# References

- K. Misra, S.-H. Kim, “AHG18: On Supporting Higher Bit Depth With Transform Skip Only Changes,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0259, Oct. 2013.

- C. Rosewarne, J. Sole, K. Sharman, S.-H Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation and update methods,” Document of Joint Collaborative Team on Video Coding, JCTVC-N1122, Jul. 2013.