| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0315 |
| --- | --- |

| Title: | AhG5: Cross-check for prediction of displacement vector in intra block copying |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E.Alshina, A.Alshin 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: | + 82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This report verifies the performance of displacement vector prediction in intra block copy proposed by Qualcomm. BD-rate gain up to 5.4% has been confirmed. TBA

# Introduction

In [1] the modification of displacement vector (DV) signalling in intra block copy (IntraBC) was proposed. So-called “method 1” from [1] uses a prediction for the vector. The left or above DV is selected as predictor and the resulting displacement vector difference (DVD) is coded. In order to avoid extra line buffer to store the above MV, only the above MV inside current CTU is allowed to be used as the predictor.

There is a flag indicating selection between predictor. This flag is coded only if there are two different DV predictions available (both from left and above blocks).

This flag is encoded as CABAC ctx coded bin. First extra ctx model added to the design is used for these purposes.

Two flags (for vertical and horizontal components) indicate whether the DVD is zero. They also encoded as CABAC cxt coded bins (2 more ctxs are added). When DVD is not zero, exponential-Golomb codes with order 3 are used to encode the remaining absolute level of the DVD.

Additional flag (CABAC by-pass) is used to code DVD sign.

In total 3 extra ctx models are added to the design. In current IntraBC implementation MVD syntax is re-used for Intra MV coding. In verified contribution separate module in introduced for IntraBC DV coding.

# Test results

In this section, experimental results are presented for the proposed DV signaling methods. The methods are implemented in HM12.0_Rext4.1 and simulated under the RCE3 common test conditions [2]. The performance is compared to the anchor for lossless as well as lossy configuration in terms of BD-rates.

TBU

- Results of method 1 for lossless (Table 1) and lossy (Table 2);

Table 1. Lossless results for method 1

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.6% | 0.5% | 0.5% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.5% | 0.4% | 0.4% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.6% | 0.4% | 0.5% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 1.0% | 0.9% | 1.1% |
| SC YUV 444 Optional | 1.9% | 1.6% | 0.7% |
| Enc Time[%] | 98% | 98% | 97% |
| Dec Time[%] | 98% | 99% | 98% |

Table 2. Lossy results for method 1

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.3% | -1.2% | -1.2% | -0.9% | -1.0% | -0.9% | -0.7% | -0.7% | -0.7% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.8% | -1.8% | -1.9% | -1.4% | -1.5% | -1.5% | -1.2% | -1.2% | -1.1% |
| Animation RGB 444 | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| SC YUV 444 | -2.8% | -2.8% | -2.9% | -2.2% | -2.2% | -2.3% | -1.7% | -1.6% | -1.7% |
| Animation YUV 444 | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -5.4% | -5.6% | -5.6% | -5.1% | -5.1% | -5.1% | -4.5% | -4.2% | -4.2% |
| SC(444) YUV Optional | -4.8% | -4.9% | -4.9% | -4.0% | -4.1% | -4.2% | -3.3% | -3.4% | -3.3% |
| Enc Time[%] | 98% |  |  | 99% |  |  | 98% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 97% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -1.1% | -1.0% | -0.8% | -0.8% | -0.7% |  |  |  |
| Class B | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.4% | -1.4% | -1.5% | -1.1% | -1.1% | -1.1% |  |  |  |
| Animation RGB 444 | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
| SC YUV 444 | -2.4% | -2.4% | -2.3% | -1.7% | -1.7% | -1.7% |  |  |  |
| Animation YUV 444 | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -5.1% | -5.1% | -5.2% | -4.6% | -4.7% | -4.6% |  |  |  |
| SC(444) YUV Optional | -4.8% | -4.8% | -5.0% | -4.1% | -4.0% | -4.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.8% | -1.4% | -1.2% | -0.6% | -0.8% | -0.8% |  |  |  |
| Class B | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.8% | -0.7% | -0.7% | -0.6% | -0.4% | -0.4% |  |  |  |
| Animation RGB 444 | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC YUV 444 | -2.4% | -2.3% | -2.5% | -1.8% | -1.6% | -1.8% |  |  |  |
| Animation YUV 444 | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -3.6% | -3.3% | -3.4% | -3.6% | -3.3% | -3.3% |  |  |  |
| SC(444) YUV Optional | -4.7% | -4.3% | -4.7% | -3.2% | -2.7% | -3.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |

# References

Samsung verifies performance of prediction method for Intra block copy displacement vector. It seems that the most part of the gain can be achieved with smaller modification of existing IntraBC design.

TBA

# References

[1] C. Pang, J. Sole, L. Guo, M. Karczewicz, and R. Joshi, “AhG5: Displacement vector signaling for intra block copying”, JCTVC-O0154, Geneva, Switzerland, October 2013.

[2] A. Saxena, D. Kwon, M. Naccari, and C. Pang, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction Techniques”, JCTVC-N1123, Vienna, AT, August 2013.