| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0317 |
| --- | --- |

| Title: | Cross check for JCTVC-O0178: Explicit signalling for intra RDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for the proposal described in Document JCTVC-O0178. All the results released by the proponent are tested and it is reported that all the test points are confirmed. Analysis of the source code reveals perfect matching between the algorithm and the implementation.

# Introduction

Document JCTVC-O0178 proposes to signal the Residual DPCM (RDPCM) mode for intra prediction both in lossy and lossless coding. In the current draft of the HEVC-RExt specification text the prediction direction for RDPCM in intra coding is set equal to this direction used for block-based prediction. What JCTVC-O0178 proposes is to use the same signalling mechanism adopted for inter coding where the RDPCM prediction direction is explicitly signalled including the case when no RDPCM is needed. This cross-check report presents the experimental results as well as the source code inspection in the next sections.

# Experimental results

The proposed explicit signalling for intra RDPCM has been tested over lossy and lossless coding using the RCE3 common test conditions described in Document JCTVC-N1123. The cross-checker has run the provided software under the same coding conditions and results included in JCTVC-O0178 are confirmed. The cross-check results are listed in the following ?? and ??.

Table 1: Experiment results for explicit signaling intra RDPCM and lossless coding.

Table 2: Experiment results for explicit signaling intra RDPCM and lossy coding.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.6% | -0.3% | -0.5% | -0.6% | -0.3% | -0.4% | -0.5% | -0.3% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.9% | -0.9% | -0.9% | -1.0% | -1.0% | -1.0% | -1.0% | -1.0% | -1.0% |
| Animation RGB 444 | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| SC YUV 444 | -0.8% | -0.6% | -0.6% | -0.9% | -0.7% | -0.6% | -0.9% | -0.7% | -0.7% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -1.1% | -1.1% | -1.0% | -1.4% | -1.5% | -1.4% | -1.8% | -1.6% | -1.6% |
| SC(444) YUV Optional | -1.1% | -0.9% | -0.6% | -1.1% | -1.2% | -0.9% | -1.3% | -1.5% | -1.5% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.3% | -0.3% | -0.3% | -0.3% | -0.2% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.5% | -0.6% | -0.6% | -0.6% | -0.7% | -0.8% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.5% | -0.5% | -0.5% | -0.5% | -0.5% | -0.5% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -0.8% | -0.9% | -0.9% | -1.1% | -1.0% | -1.1% |  |  |  |
| SC(444) YUV Optional | -1.0% | -1.0% | -1.2% | -1.1% | -1.2% | -1.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | -0.3% | -0.1% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.3% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC RGB 444 | 0.3% | 0.2% | 0.1% | -0.1% | -0.1% | -0.3% |  |  |  |
| Animation RGB 444 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.4% | -0.4% | -0.5% | -0.2% | -0.2% | -0.4% |  |  |  |
| Animation YUV 444 | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | 0.4% | 0.3% | 0.1% | -0.7% | -0.7% | -0.9% |  |  |  |
| SC(444) YUV Optional | -0.6% | -0.4% | -0.5% | -0.4% | -0.4% | -0.6% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |  |  |  |

# Source code inspection

The proponent has implemented the proposed explicit signalling of intra RDPCM using the HM-RExt software Version 4.1. In the following, the performed source analysis is detailed.

All changes relative to the proposal are marked in the source with the following macros:

- INTRA_RDPCM_SIG

- INTRA_RDPCM_SIG_LS

The proponent has modified the following C++ classes of the HM-RExt software:

- TComDataCU: Support has been added to store the intra RDPCM mode for each TU. In particular analogous arrays and functions as for the inter coding case have been replicated for intra coding.

- TEncEntropy, TEncCAVLC, TEncSbac: Support to encode the intra RDPCM flags is added in the same way for inter coding.

- TDecEntropy, TDecCAVLC, TDecSbac: Support to parse the intra RDPCM flags is added in the same way for inter coding.

- TEncSearch: In this class, all the members functions which perform RDO search for intra coding have been modified to search for the best intra RDPCM mode and then store it during RQT recursion.

- TDecCU: The decoding for intra coded TU is modified to account for the signalled intra RDPCM direction.

The source code analysis performed by the cross-checker reveals perfect matching between the algorithmic description and the provided implementation.

# Text changes inspection

Not provided at the time of cross-check report writing.