| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0321 |
| --- | --- |

| Title: | HEVC Interlaced: Application Software Proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zineb AGYO (z.agyo@ateme.com), Jérôme VIERON (j.vieron@ateme.com), Jean Marc THIESSE (jm.thiesse@ateme.com), Pierre LARBIER (p.larbier@ateme.com) |  |  |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution presents an application software proposal for HEVC interlaced coding where both interlaced coding and display are handled. It is presented as a patch that can be applied to the HM 12.0.

As interlaced coding [1] is now included in the latest HM, this application software adds pre-processing and post-processing units to the encoder and the decoder in order to handle all display matters related to interlaced coding, this includes 4:2:2 chroma sampling format, chroma planes shift, 3:2 Pull-down and correct field related signalization.

# Software proposal

## Pre-processing and Post-processing:

- Support of 4:2:2 chroma format input

Most interlaced content have naturally 4:2:2 chroma subsampling. Thus, the proposed software handles 4:2:2 interleaved chroma format inputs using an additional parameter “ChromaFormatIdc” in the configuration file. Both YUYV and UYVY are supported.

- 4:2:2 to 4:2:0 conversion

Since the encoder accept input only in 4:2:0 planar formats, it is therefore necessary to convert 4:2:2 contents to 4:2:0 before encoding. For this purpose, a 16-taps resampling filter described in [2] is used. It is based on Kaiser windowed half-band FIR filtering.

Moreover, when decoded content shall be displayed on a screen, it must be converted back to 4:2:2 chroma subsampling. In order to simulate the final output displayed on the screen, this software performs the opposite conversion using the same resize filter. An additional output at the encoder and the decoder contains the decoded video in 4:2:2 interleaved chroma format.

- Chroma planes shift

When using field based coding, it is necessary to handle the chroma right in order to avoid chroma coding artifacts [3] that could seriously demean the visual quality.

Both top and bottom fields chroma samples should be aligned before encoding. The related chroma displacements are signaled in the VUI syntax elements “chroma_sample_loc_type_top_field” and “chroma_sample_loc_type_bottom_field”, in order, at the decoding side, to realign the chroma samples in a proper way.

The same Kaiser windowed resize filter [2] is used to shift the chroma samples prior to encoding and to realign it in the original position after decoding.

This tool can be enabled using the parameter “ChromaShift” while setting the parameters “ChromaSampleLocTypeTopField” and “ChromaSampleLocTypeBottomField” to the right values.

- 3:2 Pull-down

We can generate 3:2 Pull-down content from progressive inputs using SEI syntax element “pic_struct”. This application is typically useful when 24 fps content needs to be converted to 29.97 fps. The user can choose between three patterns in the configuration file:

Pattern 1: A, Bt|Bb, Bt|Cb, Ct|Db, Dt|Db

Pattern 2: At|Ab, At|Bb, Bt|Cb, Ct|Cb, D

Pattern 3: At|Ab, At|Bb, Bt|Cb, Ct|Db, Dt|Db

Where A, B, C and D are 4 frames and t, b refers to top or bottom.

## Configuration and outputs:

Some new parameters have been added to the encoder and are all specified in the attached configuration file example. At the decoder side, the user can chose to write an additional output that correspond to what would be displayed on a screen, i.e. in 4:2:2 chroma sampling format, with fields duplicated to generate 3:2 Pull-down… etc. This option is enabled by setting “ReconDisplayedFile” to the desired path at the encoder and the decoder input.

# Conclusion

This application software proposal provides a complete processing chain for HEVC interlaced coding in order to get as close as possible to a realistic application case. This application should help the assessment of HEVC interlaced coding especially when a visual approach is considered.

Accompanying this contribution is the patch that should be applied to the version 12.0 of the HEVC reference software, plus a configuration file example of interlaced coding.

# References

[1] Zineb AGYO, Jérôme VIERON, Jean-Marc THIESSE, “HEVC Software modifications for field based coding”, M30319, August 2013, Vienna, Austria.

[2] Gary SULLIVAN, “Color format downconversion for test sequence generation”, Error: Reference source not found, September 2003, San Diego, USA.

[3] Jérôme VIERON, Jean-Marc THIESSE, Pierre LARBIER, Antoine DEPREUX, “HEVC interlaced coding assessment and chroma issue consideration”, JCTVC-K0216, October 2012, Shanghai, China.

# Patent rights declaration(s)

ATEME does not have any current or pending patent rights relating to the technology described in this contribution.