| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0332_r1 |
| --- | --- |

| Title: | UHD test sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Authors: | Rajitha Weerakkody Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 rajitha.weerakkody@bbc.co.uk matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This contribution presents UHD (3840 × 2160) source material produced by BBC R&D which is available for non commercial use, including HEVC testing and verification. The set consists of the material captured using Sony F55 camera and is available in 4:2:0 format (Rec. 709).

# UHD test sequences

All sequences are captured using Sony F55 camera in XAVC mode (4096 × 2160, 10-bit, 4:2:2). Captured sequences are decoded and cropped to 3840 × 2160 using ffmpeg. Then they are chroma sub-sampled to 4:2:0 using the filters in the sgi2yuv library (as available in JCT-VC repository). All sequences are provided in below specification:

Resolution			: 3840 × 2160

Chorma subsampling	: YUV 4:2:0

Frame rate			: 50 fps

Number of frames	: 500

Screenshots are shown in Figure 1, along with the rate-distortion plots for the comparative coding performance of HEVC (HM-12.0) and AVC (JM 18.5). For the test, the input bit depth is 8-bit and the random access (RA) configuration has been used.

| ID | Name | Screenshot | HM12.0 (RA_Main) Vs JM18.5 (RA_HE) |
| --- | --- | --- | --- |
| A | Book |  |  |
| B | Park And Buildings |  |  |
| C | Vehicles |  |  |
| D | Calendar And Plants |  |  |
| E | Men And Plants |  |  |

Figure 1: Details of submitted UHD sequences.

The analysis of the spatial and temporal indices (SI and TI, [1]) for all presented sequences is shown in Figure 2. The sequence IDs as in Figure 1 are used in the below plot.

Figure 2: SI-TI analysis of the new UHD sequences.

# Copyright and Use Information

The video sequences provided above and all intellectual property rights therein remain the property of the BBC.

The BBC is making available the video sequences for use under the Creative Commons Attribution-NonCommercial 3.0 licence.

You are free to use, share (to copy, distribute and transmit) or remix (to adapt) the BBC video sequences, provided that:

- No-commercial- you may not use these video sequences for commercial purposes; and

- Attribution- you attribute the work to the BBC by indicating that the video sequences (or elements thereof) were produced by the BBC.  Your attribution must not be in any way that suggests that the BBC endorses you or your use of the video sequences.

# References

- ITU-T Recommendation P.910, “Subjective video quality assessment methods for multimedia applications”, 2008.