| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0340 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0243 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Summary of contributions on random access and layer switching structures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution provides a summary of the contributions in the agenda category “random access and layer switching structures”, section 6.4.3 of the JCT-VC meeting notes.

It is asserted that the reviewed contributions are independent of each other, except for JCTVC-O0220/Proposal 1/Alternative 2 and JCTVC-O0139/Proposal 3. In addition, it is asserted that JCTVC-O0149/Proposal 3 relates to JCTVC-O0266 (which is allocated into the HRD agenda category).

# Summary of the Contributions

JCTVC-O0220 MV-HEVC/SHVC HLS: On first decodable CRA picture of the layer-wise startup [J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

The contribution has two proposals:

Proposal 1:

Problem: NoRaslOutputFlag of the first decodable CRA picture on an enhancement layer is not set. Consequently, the associated leading pictures are not correctly processed.

Proposal 1, alternative 1:

NAL unit type of the first decodable CRA picture is changed to BLA_W_LP, BLA_W_RADL, or BLA_N_LP.

Proposal 1, alternative 2:

NoRaslOutputFlag is set to equal to 1 when the current picture is an IRAP picture, LayerInitialisedFlag[k] = 0, and LayerInitialsedFlag[refLayerId]=1 for all values of refLayerId equal to RefLayerId[k][j], where j is in the range of 0 to NumDirectRefLayers[k]-1, inclusive. In this solution, LayerInitialisedFlag[k] shall be set equal to 1 after setting NoRaslOutputFlag to 1.

Proposal 2:

Problem: MV-HEVC/SHVC currently creates unavailable reference pictures for the first picture of a layer after NoClrasOutputFlag has been set. However, all pictures of a layer are marked as “unused for reference” at the start of decoding an IRAP picture with NoRaslOutputFlag equal to 1. Thus, reference pictures of RASL pictures associated with the IRAP picture are not available.

Proposal: Invoke the decoding process for generating unavailable reference pictures (subclause F.8.1.3) again when the current picture is the IRAP picture with NoRaslOutputFlag equal to 1.

JCTVC-O0139 MV-HEVC/SHVC HLS: Layer-wise initialization and parameter set activation [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

The contribution has five proposals:

Proposal 1:

Problem: A base-layer CRA picture does not invoke the layer-wise start-up process (like a BLA picture does) when HandleCraAsBlaFlag is equal to 1 for the CRA picture.

Proposal: Invoke the layer-wise start-up process for a base-layer CRA pictrure with HandleCraAsBlaFlag equal to 1.

Proposal 2:

Problem: MV-HEVC/SHVC includes the following constraint:

An activated SPS RBSP for a particular nuh_layer_id value shall remain active for a sequence of pictures in decoding order with that nuh_layer_id value starting from a LIP picture having that nuh_layer_id value, inclusive, until either the next LIP picture with that nuh_layer_id value, exclusive, or the end of the CVS, whichever is earlier. (A LIP picture is defined as a picture that is an IRAP picture with NoRaslOutputFlag equal to 1 or that is contained in an initial IRAP access unit.)

However, when a LIP picture activates a new layer SPS and there are RASL pictures associated with a LIP picture, the RASL picture may use reference pictures that used a different SPS in their decoding process.

Proposal:

The SPS syntax elements pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, bit_depth_chroma_minus8, and chroma_format_idc shall remain unchanged within a sequence of activated SPS RBSPs, in their activation order, from any activated SPS RBSP until the end of the bitstream or up to but excluding an SPS RBSP that is activated within the next access unit in which at least one of the following conditions is true:

-  The access unit includes a picture for each nuh_layer_id value in TargetDecLayerIdList and each picture in the access unit is an IDR picture.

- The access unit includes an IRAP picture with nuh_layer_id equal to 0 for which NoClrasOutputFlag is equal to 1.

Proposal 3: This proposal seems aligned with JCTVC-O0220 proposal 1, alternative 2. The yellow-highlighted text is proposed to be added:

When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and one of the following conditions is true, LayerInitialisedFlag[ nuh_layer_id ] is set equal to 1:

–	nuh_layer_id is equal to 0.

–	LayerInitialisedFlag[ nuh_layer_id ] is equal to 0 and LayerInitialisedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to RefLayerId[ nuh_layer_id ][ j ], where j is in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] – 1, inclusive.

Proposal 4:

Problem: The definition of LIP picture does not cover the case where reference layer(s) of an IRAP picture have not been initialized yet.

Proposal:

layer initialisation picture (LIP): A picture that is an IRAP picture with NoRaslOutputFlag equal to 1 or that is contained in an initial IRAP access unit, of which LayerInitialisedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to RefLayerId[ nuh_layer_id ][ j ], where j is in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] – 1, inclusive.

Proposal 5:

Problem: It is asserted that if cross_layer_irap_aligned_flag is equal to 1 and two pictures having no dependency on each other in an access unit have different nal_unit_type values, the POC value alignment cannot be guaranteed.

Related specification text excerpts include:

cross_layer_irap_aligned_flag equal to 1 specifies that IRAP pictures in the CVS are cross-layer aligned, i.e. when a picture pictureA of a layer layerA in an access unit is an IRAP picture, each picture pictureB in the same access unit that belongs to a direct reference layer of layerA or that belongs to a layer for which layerA is a direct reference layer of that layer is an IRAP picture and the VCL NAL units of pictureB have the same value of nal_unit_type as that of pictureA.

When cross_layer_irap_aligned_flag is equal to 0, it is a requirement of bitstream conformance that num_extra_slice_header_bits shall be greater than or equal to 1.

Proposal 5, alternative 1:

cross_layer_irap_aligned_flag equal to 1 specifies that IRAP pictures in the CVS are cross-layer aligned, i.e. when a picture pictureA of a layer layerA in an access unit is an IRAP picture, each picture pictureB in the same access unit have the same value of nal_unit_type as that of pictureA. cross_layer_irap_aligned_flag equal to 0 specifies that the above restriction may or may not apply.

Proposal 5, alternative 2:

poc_reset_flag equal to 1 specifies that the derived picture order count for the current picture is equal to 0. poc_reset_flag equal to 0 specifies that the derived picture order count for the current picture may or may not be equal to 0. It is a requirement of bitstream conformance that when cross_layer_irap_aligned_flag is equal to 1, the value of poc_reset_flag shall be equal to 0. When not present, the value of poc_reset_flag is inferred to be equal to 0.

JCTVC-O0149 MV-HEVC/SHVC HLS: on splicing and layer-wise start-up of the decoding [M. M. Hannuksela (Nokia)]

The contribution has three proposals:

Proposal 1:

Problem: When the layer-wise start-up process is invoked, earlier enhancement-layer pictures remain marked as “used for reference”, even though they are never used as reference pictures, occupy picture storage buffers in the DPB, and may cause ambiguities in choosing a reference picture if they happen to have the same POC as pictures that are decoded subsequently.

Proposal: A base-layer IRAP picture that initiates the layer-wise start-up process (i.e. has NoClrasOutputFlag equal to 1) causes marking of all pictures in the DPB as “unused for reference”.

Proposal 2:

Problem: A base-layer IDR picture cannot invoke the layer-wise start-up process, which would be desired when a spliced CVS starts with a base-layer IDR picture in the base layer but not in all layers.

Proposal: A new slice_reserved_flag is taken into use to indicate if a base-layer IDR picture initiates the layer-wise start-up process.

Proposal 3:

Problem: When a spliced CVS contains an IRAP picture in the base layer but not in all layers, a splicer probably would like to set NoOutputOfPriorPicsFlag for all layers equal to no_output_of_prior_pics_flag of the base layer in the spliced CVS, i.e. the splicer should be able to control if pictures at any layer from the CVS are output before the splice point.

Proposal: A base-layer picture that initiates the layer-wise start-up process causes NoOutputOfPriorPicsFlags of all layers to be set equal to no_output_of_prior_pics_flag of the base layer picture.

Note: It is remarked that contribution JCTVC-O0266/JCT3V-F0090 relates to NoOutputOfPriorPicsFlag and its handling in the DPB.

JCTVC-O0212 MV-HEVC/SHVC HLS: On CL-RAS pictures [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

Proposal 1:

Motivation: Enable middle-boxes to exclude CL-RAS pictures from the forwarded stream.

Proposal: Specify nal_unit_type values CL_RAS_N and CL_RAS_R for CL-RAS pictures.

Proposal 2:

Motivation: Enable HRD operation and parameters for bitstreams where CL-RAS pictures are not present for IRAP pictures with NoClrasOutputFlag equal to 1.

Proposal: The proposal contains the following parts:

- A variable NoClrasPicPresentFlag, which can be set externally, is used to control whether CL-RAS pictures are present in the bitstream for an initial IRAP AU.

- The picture timing SEI message is appended to contain alternative “cross-layer” CPB removal delay and DPB delay for the case that CL-RAS pictures are not present for an initial IRAP AU.

- A cross-layer HRD parameters SEI message is proposed. It contains bit-rate and CPB size combinations for a certain POC range intended to be used in HRD operation when NoClrasPicPresentFlag is equal to1.

- When NoClrasPicPresentFlag is equal to 1, the HRD process is changed to use the alternative “cross-layer” CPB removal delay and DPB delay as well as the bitrate and CPB size of the cross-layer HRD parameters SEI message.