| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0341 |
| --- | --- |

| Title: | Cross-check for JCTVC-O0066: Unifying DPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for the proposal described in Document JCTVC-O0066. All the results released by the proponent are tested and it is reported that all the test points are confirmed. Analysis of the source code and text changes reveals perfect matching between the algorithm and the implementation.

# Introduction

JCTVC-O0066 proposes design unification for Residual DCPM whereby discrepancies between intra and inter RDPCM as well as coefficient rotation issues are addressed. In this cross-check report only the RDPCM design unification is analysed and in particular the so-called Coefficient DPCM is considered. CDPCM aligns the intra RDPCM design to the inter one where the DPCM prediction, at the encoder side, is applied over residuals after transform skip operation is performed. At the decoder, the coefficient reconstruction is applied before the shift down operation related to inverse transform skip. Given the higher precision at which coefficient prediction and reconstruction is performed, JCTVC-O0066 also proposes a clipping operation to keep the input of inverse transform skip module in 16-bit precision. The experimental results, source code and text changes analysis are reported in the following sections.

# Experimental results

The proposed CDPCM has been tested over lossy and lossless coding using the RCE3 common test conditions described in Document JCTVC-N1123. For lossless coding, since intra and inter RDPCM design is identical (transform and quantisation are bypassed) the aim of the experiment is to show no difference with respect to the anchor values, i.e. no influence of the proposed clipping on the particular data set tested (internal bit depth is set equal to the input one which never exceeds 16-bit precision). The cross-checker has run the provided software under the same coding conditions and results included in JCTVC-O0066 and the results are listed in Table 1 and Table 2.

Table 1: Experimental results for lossless coding.

Table 2: Experimental results for lossy coding.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | 0.0% | -0.1% | -0.3% | -0.1% | -0.2% | -0.3% | -0.2% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.3% | -0.5% | -0.4% | -0.5% | -0.6% | -0.5% | -0.7% | -0.8% | -0.8% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.4% | 0.1% | -0.1% | -0.4% | -0.1% | -0.3% | -0.6% | -0.4% | -0.5% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -1.1% | -1.3% | -1.5% | -0.9% | -1.2% | -1.1% | -1.2% | -1.7% | -1.6% |
| SC(444) YUV Optional | -0.4% | -0.4% | -0.4% | -0.7% | -0.6% | -0.5% | -0.9% | -0.9% | -0.8% |
| Enc Time[%] | 107% |  |  | 107% |  |  | 108% |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  | 104% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -0.4% | -0.5% | -0.6% | -0.5% | -0.6% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.3% | -1.3% | -1.3% | -1.5% | -1.6% | -1.7% |  |  |  |
| Animation RGB 444 | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| SC YUV 444 | -1.2% | -0.6% | -0.8% | -1.2% | -0.9% | -1.1% |  |  |  |
| Animation YUV 444 | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -1.5% | -1.7% | -1.8% | -1.4% | -1.7% | -1.6% |  |  |  |
| SC(444) YUV Optional | -1.2% | -1.3% | -1.3% | -1.6% | -1.6% | -1.5% |  |  |  |
| Enc Time[%] | 111% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.2% | -0.4% | -0.5% | -0.4% | -0.6% |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.9% | -1.0% | -1.0% | -1.7% | -1.7% | -1.7% |  |  |  |
| Animation RGB 444 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC YUV 444 | -1.1% | -0.3% | -0.7% | -1.2% | -0.4% | -0.8% |  |  |  |
| Animation YUV 444 | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -3.2% | -3.2% | -3.2% | -4.2% | -4.4% | -4.7% |  |  |  |
| SC(444) YUV Optional | -3.1% | -3.0% | -3.2% | -3.3% | -2.9% | -3.1% |  |  |  |
| Enc Time[%] | 109% |  |  | 109% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

# Source code inspection

The proponent has implemented the proposed explicit signalling of intra RDPCM using the HM-RExt software Version 4.1. In the following, the performed source analysis is detailed.

All changes relative to the proposal (i.e. CDPCM and clipping) are marked in the source with the following macros:

- RExt__UNIFIED_DPCM

- RExt__UNIFIED_DPCM__INVERSE_PATH_CLIPPING

In particular, the macro RExt__UNIFIED_DPCM can assume three values and for CDPCM proposal is set to 2. The proponent has modified the TComTrQuant class in the source code. In particular the following member functions of this class related to the cross-checked part are modified:

- transformNxN

- invTransformNxN

- xQuant

- xRateDistOptQuant

- xDeQuant

The proponent has modified the execution flow by selecting the best RDPCM/CDPCM mode inside the transformNxN function rather than inside the xQuantInterRdpcm as it is in HM-RExt-4.1. The CDPCM processing is therefore now carried out in the xQuant and xRateDistOptQuant functions at the encoder and in the xDeQuant function at the decoder. The source code analysis performed by the cross-checker reveals perfect matching between the algorithmic description and the provided implementation.

# Text changes inspection

The proponent provided the text changes for the CDPCM part in the JCTVC-O0066 document. The text changes are consistent with what proposed in JCTVC-O0066 and with also the text fixes proposed in JCTVC-O0133.