| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0296 |
| --- | --- |

| Title: | Non-RCE2: Cross-Check of JCTVC-O0327 Rice parameter initialization for higher bit depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo Rajan Joshi | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the technical description of the results of JCTVC-O0327.

# Introduction

A method for Rice parameter initialization is proposed in JCTVC-O0327 which includes 3 modifications:

1) Rice parameter is initialized based on bit depth (BD) information as follows

If (extended_precision_processing_flag)

{

if (transform_skip_flag || cu_transquant_bypass_flag)  uiGoRiceParam = BD-8;

else   uiGoRiceParam = (BD-8) >>1;

}

else  uiGoRiceParam = 0;

2)

If (extended_precision_processing_flag)

{

if (absCoeff[idx] > 3* (1<<uiGoRiceParam))

uiGoRiceParam = min <UInt> (uiGoRiceParam+1, 16)

}

else

{

if (absCoeff[idx] > 3* (1<<uiGoRiceParam))

uiGoRiceParam = min <UInt> (uiGoRiceParam+1, 4)

}

The software provided by the proponents has been inspected by the cross-checker and it is confirmed that the implementation matches the technical descriptions.

# Experimental results

The JCTVC-O0102 software provided by the proponents has been used to generate the results and compare with results generated with HM-12.0+RExt-4.1 under RCE2 common test conditions [1]. Encoding time and decoding time were obtained using heterogeneous cluster. It was confirmed that for the test case, the BD-rate results exactly match to the results provided by the proponents.

Table 1 Results for AHG18 test

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -6.8% | -6.4% | -6.4% |
| 14-bit | -18.4% | -18.2% | -18.1% |
| 16-bit | -28.2% | -28.1% | -28.2% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 85% |  |  |

Extra results with the proposed method combined with RCE2-A1 are also verified.

Table 2 Results combined with RCE2-A1 for AHG18 test

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -6.6% | -6.2% | -6.2% |
| 14-bit | -18.4% | -18.1% | -18.1% |
| 16-bit | -28.2% | -28.0% | -28.1% |
| Enc Time[%] | 88% |  |  |
| Dec Time[%] | 87% |  |  |

# References

- C. Rosewarne, J. Sole, K. Sharman, and S.-H Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization and update methods”, JCTVC-N1122, Vienna, Austria, Jul. 2013.

# Patent rights declaration(s)