| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0345 |
| --- | --- |

| Title: | BoG Report on single loop decoding and key pictures in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Kemal Ugur | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | BoG chair |  |  |

_____________________________

# Abstract

The BoG met on 24 October 2013 at 11:30 to discuss single loop decoding and key picture concepts for SHVC. The proposals were analyzed with respect to the following points:

- Coding efficiency and complexity characteristics.

- Profiling considerations for SNR scalability and single loop operation

- Burden on multi-loop encoder/decoder that doesn’t operate in single loop fashion

# Participants

K. Ugur (chairing), K. Rapaka, X. Xiu, C. Feldmann, M. Wien, A. Tourapis, O. Nakagami

# Single loop related SCE2 contributions

## JCTVC-O0145, [SCE2] Key picture concept and single loop decoding Christian Feldmann, Mathias Wien (RWTH Aachen University)

Coding efficiency: 	1.2% loss EL+BL (RA) and 6.7% loss BL (RA) (key picture period of 8)

1.3% loss EL+BL (LDB) and 15.5% loss BL (LDB) (key picture of 4)

Illustration of key picture concept for the random access configuration

Illustration of key picture concept for the Low Delay B configuration

Remark: Two reasons for the loss for BL-only decoding: EL drift, CIP is enabled.

Remark: Reporting worst case RD measurements for BL only decoding with key pictures was requested for better understanding of the impact of drift.

Remark: If SNR, always do single loop. Other option, make it switchable.

Remark: The drift could be controlled by more advanced mode decision algorithms. The current implementation does not include joint optimization.

Remark: A BL decoder should be capable of outputting unfiltered samples for inter layer prediction in the EL.

Remark: The current design has a low level change for TMVP derivation. It was remarked that this can be fixed with high level solutions (e.g. disable TMVP for BL).

Remark: The subjective results need to be checked because of the drift. Cross checker mentioned that he has not noticed any artifacts.

Remark: The common test conditions might not be well describing the practical use of SNR scalability (maybe reducing the QP offset between BL and EL).

Remark: The loss for low delay is higher, not desirable for video conferencing.

## JCTVC-O0162, SCE2: Single-loop decoding based SNR scalability for SHVC, X. Xiu, Y. Ye, Y.-W. He, Y. He (InterDigital)

Summary: Base layer decoding is not changed. If EL refers to BL, then base layer motion is used with EL reference pictures to generate the inter layer prediction.

Coding efficiency: 	2.6% loss EL+BL (RA) and 2.0% loss BL (RA) (BL loss is for CIP)

2.9% loss EL+BL (LDB) and 2.0% loss BL (LDB) (BL loss is for CIP)

Remark: Encoder has to mandate usage of CIP for BL, and EL has to parse BL motion (similar to other proposal)

Remark: EL decoder needs to access BL residual. If BL residual is not used the coding efficiency loss drops to 11% more.

Remark: A BL decoder should be capable of outputting unfiltered samples for inter layer prediction.

Remark: Reference prediction structures should be the same for BL and EL.

Remark: This method is pure single loop (i.e. key pictures are multi loop for the other single loop proposal).

General remark (1.1 & 1.2): If one needs to get the benefit of single loop one needs to go a bit down in the codec design (e.g. don’t reconstruct INTER CUs in BL but only reconstruct INTRA CUs). It was remarked that it is a design choice and in order to get the benefit of single loop one could decide to invest.

# Key picture related SCE2 contribution

## JCTVC-O0227,  SCE2 : On key pictures in SHVC, K. Rapaka, J. Chen, V. Seregin, Y.-K. Wang, M. Karczewicz (Qualcomm)

Utilizes key picture concept for coding efficiency increase

Coding efficiency: 	3.1% gain EL+BL (RA) and 3.9% loss BL (RA) (key picture period of 8)

3.3% gain EL+BL (LDB) and 3.3% loss BL (LDB) (key picture of 4)

Remark: TMVP issue is fixed with a reference picture list modification which is considered not problematic.

Remark: This proposal seems cleaner from high level syntax point of view, the main question is do we get the gain for EL plus BL and lose in BL.

The proponent suggested the main use case is reduction of overall bitrate with the functionality of extracting a still good base layer.

Remark: For non-key pictures for EL decoding some BL pictures could be removed from DPB (potentially save some complexity).

Remark: Now BL decoder needs to communicate with EL decoder, might be an additional implementation burden. Additionally change to support this functionality is only high level reference picture management.

Remark: In the current implementation of key pictures, the temporal scalability is required as TemporalId is used for deciding key picture or not.