| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0348 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0246 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Summary of Contributions on HRD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of the contributions in the agenda section 6.4.7, “Hypothetical Reference Decoder” of the JCT-VC meeting notes document at the Geneva October 2013 meeting.

# Summary of the Contributions Related to HRD at Geneva October 2013 Meeting

Proposals on DPB Parameters Signaling and Operation

It is asserted that proposals in this section do not conflict with each other. The VPS signaling of max DPB size in O0136 is competitor to DPB parameters signaling in O0217.

| Document | Proposal Information |
| --- | --- |
| JCTVC-O0136 MV-HEVC/SHVC HLS: Signalling decoded picture buffer size (Samsung) | Motivation: Signalling VPS DPB parameters are for session negotiation. Proposal Summary: Three alternative syntax to indicate the maximum decoded buffer sizes in VPS Proposal Details: Syntax 1: 1a) maximum DPB size signalling for each operation point (for each layer set for each temporal sub-layer). 1b) Also maximum DPB size for each layer set for each temporal sub-layer for each representation format can be optionally signalled based on flag. Flag is signaled for each layer set for each temporal sub-layer. Syntax 2: 2a) same as 1a) 2b) Also maximum DPB size for each layer set for each temporal sub-layer for each layer is optionally signaled based on flag. Flag is signaled for each layer set for each temporal sub-layer Syntax 3: 3) Explicitly selecting the level of DPB signaling (hybrid of option 1&2). Additional u(2) idc to signal if one or both of 1b and 2b is signaled in addition to 1a. Additional Comments: -No DPB operation based on signalled parameters is defined. -The signaling of DPB parameters in |
| JCTVC-O0217 MV-HEVC/SHVC HLS: Sub-DPB based DPB operations (Qualcomm, Nokia, Sharp) | Motivation: Signalling of sub-DPB parameters in VPS, DPB operation based on sub-DPBs Proposal: The DPB is partitioned into several sub-DPBs, and each sub-DPB is managed independently. VPS signaling of DPB parameters is proposed with a common signalling scheme that can be applied to both modes of sub-DPB operation – layer-specific DPB and resolution-specific DPB. In the layer-specific DPB mode, each layer has its own sub-DPB, while in the resolution-specific DPB mode, all layers that have the same spatial resolution, bit depth, and color format share the same sub-DPB. DPB operation based on signaled VPS parameters is proposed. Proposal Details: Operation point & Output layer sets: It is proposed that an operation point be associated with a set of target output layers and a temporal ID. A modification in the definition for operation points is proposed, along with a new definition for output layer sets. - operation point: A bitstream that is created from another bitstream by operation of the sub-bitstream extraction process with the another bitstream, a target highest TemporalId, and a target layer identifier list as inputs, and that is associated with a set of target output layers. - output layer set: A layer set that is associated with a set of target output layers. VPS signaling of DPB parameters: -max_vps_dec_pic_buffering_minus1 is signaled for each output layer set for temporal sub-layers (optionally based on flag), for each sub-dpb. -max_vps_num_reorder_pics, max_vps_latency_increase_plus1 is signaled for each output layer for temporal sub-layers (optionally based on flag). DPB operation: -For target output layers signalled DPB parameters from VPS are selected. And variables maxNumReorderPics, maxLatencyIncreasePlus1, maxLatencyPictures, maxDecPicBufferingMinus1 are derived as follows: -if CVS conforms to profiles specified in Annex A DPB parameters from active SPS are used -if CVS conforms to Annex G/ H VPS DPB parameters are used. The derived variables are used for bumping. Sub-DPB mode: Two modes of sub-DPB operations are presented: -layer-specific sub-DPB mode : separate sub-DPB for each layer, -resolution-specific sub-DPB mode (involves resolution/chroma format/bit-depth specific sub-DPB operation (referred as resolution–specific mode): In the resolution-specific sub-DPB model, all the pictures that have the same spatial resolution, colour format and bit depth share the same sub-DPB. The number of sub-DPBs for each output layer set, and association of each layer with a sub-DPB is derived. Pros and cons of each mode of operation are discussed. Group is requested to discuss and choose one of the modes of operation that is applied for both SHVC and MV-HEVC. Additional Comments: -Proposal is specific to operation of DPB only, and that it is independent of, and not conflicting with, the multi-layer HRD operation (with layer-specific CPB model). |

Other Proposals Related to DPB:

It is asserted that proposals in this section relate to modification of DPB operation and are largely independent of each other except as noted below:

-O0149 and O0266 both have text which relate to output and removal of pictures from DPB and some careful editing would be needed for their integration.

| Document | Proposal Information |
| --- | --- |
| JCTVC-O0149 MV-HEVC/SHVC HLS: on splicing and layer-wise start-up of the decoding (Nokia) | Motivation: When a spliced CVS contains an IRAP picture in the base layer but not in all layers, a splicer probably would like to set NoOutputOfPriorPicsFlag for all layers equal to no_output_of_prior_pics_flag of the base layer in the spliced CVS, i.e. the splicer should be able to control if pictures at any layer from the CVS are output before the splice point. Proposal Summary: It is proposed that a base-layer picture that initiates the layer-wise start-up process causes NoOutputOfPriorPicsFlags of all layers to be set equal to no_output_of_prior_pics_flag of the base layer picture. Proposal Details (Text): If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0 that is not picture 0, the following ordered steps are applied: 1. The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows: – If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag). – Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag. NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case. – Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag. 2. The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows: – If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0. – Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause F.13.5.2.4 and the DPB fullness is set equal to 0. – Otherwise, if nuh_layer_id of the current picture is greater than 0 and NoClrasOutputFlag is equal to 1 for picture with nuh_layer_id equal to 0 in the access unit containing the current picture, the following ordered steps are applied: – NoOutputOfPriorPicsFlag of the current picture is set equal to NoOutputOfPriorPicsFlag of the picture with nuh_layer_id equal to 0 in the access unit containing the current picture. – The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows: – If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0. - Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause F.13.5.2.4 and the DPB fullness is set equal to 0. |
| JCTVC-O0266 MV-HEVC/SHVC HLS: On flushing of decoded pictures from the DPB based on NoOutputOfPriorPicsFlag (Qualcomm) | Motivation: The process defined for flushing pictures from DPB in SHVC WD3 and MV-HEVC WD5 have the following shortcomings: - The flushing behavior is not aligned for the two types of decoder conformance: output order conformance and output timing conformance. - For output timing conformance decoders, the flushing is invoked for each layer picture that is not the first picture of the layer in the bitstream and that has NoRaslOutputFlag equal to 1, and when invoked, it flushes all decoded pictures of that layer in the DPB. - For output order conformance decoders, the flushing is only invoked for the base layer picture that is not the first picture in the bitstream and that has NoRaslOutputFlag equal to 1, and when invoked, it flushes all decoded pictures of all layers in the DPB. - In a bitstream with two layers, when a different resolution is activated at the EL at a Layer Initialisation Picture (LIP) that is an IRAP picture and that does not belong to an IRAP AU (and the resolution of the base layer cannot change at this AU as the base layer picture in an non-IRAP AU cannot be an IRAP picture), layer specific flushing of pictures may be needed. Here only pictures from the EL but not from the BL need to be flushed. This is currently not possible for output order conformance. - In a bitstream with two layers, let there be an access unit where the BL picture is an IDR picture and the EL picture is a non-IRAP picture, and the resolution of the BL picture is updated, while the resolution of the EL picture is not updated. In this case, flushing should be performed only for the pictures from the BL, while the EL pictures should not be flushed. Again this is currently not possible for output order conformance. Proposal Summary: It is proposed that the flushing of pictures be made layer specific for both types of decoder conformances. The flushing process is enabled to occur at each IRAP picture with NoRaslOutputFlag equal to 1, and at each LIP picture, instead of to occur at only each IRAP picture that has NoRaslOutputFlag equal to 1 and nuh_layer_id equal to 0. The value of NoOutputOfPriorPicsFlag is derived for all IRAP pictures with NoRaslOutputFlag equal to 1 and inferred, based on the value of NoClRasOutputFlag, for all non-IRAP pictures that are LIP. Proposal Details (Text): C.3.2 Removal of pictures from the DPB The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of access unit n (containing the current picture) and proceeds as follows: – The decoding process for RPS as specified in subclause Error: Reference source not found is invoked. – When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 a LIP (or an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0) that is not picture 0, the following ordered steps are applied: 1. The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows: – If the current picture is a CRA picture with NoRaslOutputFlag equal to 1, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag). – Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS of the current picture is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture with the same layer ID as the current picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag. NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case. – Otherwise, if the current picture is an IRAP picture, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag. – Otherwise if the current picture (which is a LIP) is not an IRAP picture and if NoClrasOutputFlag associated with the IRAP picture nuh_layer_id equal to 0 in the current access unit is equal to 1, NoOutputOfPriorPicsFlag is set equal to 1. – Otherwise, NoOutputOfPriorPicsFlag is set equal to 0. 2. The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD, such that when the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers that are in the DPB and that contain pictures having the same value of nuh_layer_id as that of the current picture, are emptied without output of the pictures they contain, and the DPB fullness is decremented by the number of pictures removed set equal to 0. – When both of the following conditions are true for any pictures k in the DPB, all such pictures k in the DPB are removed from the DPB: – picture k is marked as "unused for reference" – picture k has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e. DpbOutputTime[ k ] is less than or equal to CpbRemovalTime( m ) – For each picture that is removed from the DPB, the DPB fullness is decremented by one. - F.13.5.2.2 Output and removal of pictures from the DPB The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the current picture is removed from the CPB and proceeds as follows: The decoding process for RPS as specified in subclause Error: Reference source not found is invoked. – If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 a LIP (or an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0) that is not picture 0, the following ordered steps are applied: 1. The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows: – If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag). – Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS of the current picture is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture with the same layer ID as the current picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag. NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case. – Otherwise, if the current picture is an IRAP picture, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag. – Otherwise if the current picture (which is a LIP) is not an IRAP picture and if NoClrasOutputFlag associated with the IRAP picture nuh_layer_id equal to 0 in the current access unit is equal to 1, NoOutputOfPriorPicsFlag is set equal to 1. – Otherwise, NoOutputOfPriorPicsFlag is set equal to 0. 2. The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows: – If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers that are in the DPB and that contain pictures having nuh_layer_id equal to that of the current picture are emptied without output of the pictures they contain, and the DPB fullness is decremented by the number of pictures removed. set equal to 0. – Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause Error: Reference source not found, and the DPB fullness is set equal to 0. |

Proposals on Multi-layer HRD/ ultra-low delay HRD:

It is asserted that proposals in this section have some interaction with each other:

-O0164 and O0234 signal CPB parameters differently for multi-layer HRD management. O0164 signals hrd_parameters for bitstream partitions in VPS vui. O0234 signals hrd parameters per layer in VPS or a SEI.

-Bitstream partitions signaling and CPB operation in O0234 is defined to support ultra-low delay HRD. A DPB operation change for ultra-low delay HRD is defined in O0188.

| Document | Proposal Information |
| --- | --- |
| JCTVC-O0164 MV-HEVC/SHVC HLS / JCT-VC AHG20: Multi-layer HRD operation (Nokia) | Motivation: To supports use cases of ultra-low delay operation, conveying layers on different logical transmission channels, and adding or removing layers at re-distribution points. Proposal Summary: The contribution proposes that the coded picture buffer (CPB) can be divided into bitstream partition buffers (BPBs), which operate similarly to the CPB but handle bitstream partitions, such as layers. It is proposed to enable indication of CPB parameters separately for each BPB Proposal Details: Following is proposed: - The term bitstream partition is defined. The bitstream can be partitioned into one or more bitstream partitions. The decoding of bitstream partition 0 (the base bitstream partition) is independent of other bitstream partitions. For example, the base layer (and the NAL units associated with the base layer) may be the base bitstream partition, while bitstream partition 1 may consist of the remaining bitstream excluding the base bitstream partition. base bitstream partition: A bitstream partion that is also a conforming bitstream itself. bitstream partition: A sequence of bits, in the form of a NAL unit stream or a byte stream, that is a subset of a bitstream according to a partitioning. - In addition to HRD parameters for the entire bitstream, HRD parameters can (but need not) be given for bitstream partitions. When the HRD parameters are provided for bitstream partitions: - The hypothetical scheduling and coded picture buffering operate for each bitstream partition. - The conformance of the bitstream is tested for bitstream partition based HRD operation. vps_vui( ){; Descriptor. …. num_bitstream_partitions_minus1; ue(v). if( num_bitstream_partitions_minus1 ). vps_vui_bsp_hrd( ). …. }. num_bitstream_partitions_minus1 equal to 0 specifies no bitstream-partition-specific HRD parameters are present. num_bitstream_partitions_minus1 plus 1 specifies the number of bitstream partitions for which HRD parameters are specified in the bitstream. vps_vui_bsp_hrd( ){; Descriptor. vps_num_bsp_hrd_parameters_minus1; ue(v). for( i = 0; i <= vps_num_bsp_hrd_parameters_minus1; i++ ) {. if( i > 0 ). bsp_cprms_present_flag[ i ]; u(1). hrd_parameters( bsp_cprms_present_flag[ i ], vps_max_sub_layers_minus1 ). }. num_bsp_sched_combinations_minus1; ue(v). for( i = 0; i <= num_bsp_sched_combinations_minus1; i++ ) {. du_based_bpb_sync_flag[ i ]; u(1). for( j = 0; j <= num_bitstream_partitions_minus1; j++ ) {. bsp_comb_hrd_idx[ i ][ j ]; ue(v). bsp_comb_sched_idx[ i ][ j ]; ue(v). }. }. for( i = 0; i <= num_bitstream_partitions_minus1; i++ ) {. num_bsp_layers_minus1[ i ]; ue(v). for( j = 0; j <= num_bsp_layers_minus1[ i ]; j ++ ). bsp_layer_idx[ i ][ j ]; ue(v). }. }. vps_num_bsp_hrd_parameters_minus1 plus 1 specifies the number of hrd_parameters( ) syntax structures present within the vps_vui_bsp_hrd( ) syntax structure. bsp_cprms_present_flag[ i ] equal to 1 specifies that the HRD parameters that are common for all sub-layers are present in the i-th hrd_parameters( ) syntax structure in the vps_vui_bsp_hrd( ) syntax structure. bsp_cprms_present_flag[ i ] equal to 0 specifies that the HRD parameters that are common for all sub-layers are not present in the i-th hrd_parameters( ) syntax structure in the vps_vui_bsp_hrd( ) syntax structure and are derived to be the same as the ( i − 1 )-th hrd_parameters( ) syntax structure in the in the vps_vui_bsp_hrd( ) syntax structure. bsp_cprms_present_flag[ 0 ] is inferred to be equal to 1. num_bsp_sched_combinations_minus1 plus 1 specifies the number of combinations of delivery schedules and hrd_parameters( ) specified for bitstream partitions. du_based_bpb_sync_flag[ i ] affects the CPB operation when the i-th combination of delivery schedules and hrd_parameters( ) is used. bsp_comb_hrd_idx[ i ][ j ] specifies the index of hrd_parameters( ) within the vps_vui_bsp_hrd( ) syntax structure used in the i-th combination. bsp_comb_sched_idx[ i ][ j ] specifies the index of delivery schedule within the hrd_parameters( ) syntax structure with the index bsp_comb_hrd_idx[ i ][ j ] that is used in the i-th combination. num_bsp_layers_minus1[ i ] plus 1 specifies the number of layers in the i-th bitstream partition. bsp_layer_idx[ i ][ j ] specifies the j-th layer index in the i-th bitstream partition. - A bitstream demultiplexer can be present to partition the bitstream into bitstream partitions. However, the bitstream partitions may also be considered to be directly provided by the systems layer. A new Demultiplexing process for deriving a bitstream partition is defined. This process takes as input a layer identifier list and number of layer identifiers in the layer identifier list and outputs a bitstream partition. - DUs are processed causally according to their prediction dependencies, i.e. - When the cbr_flag is equal to 0, the initial arrival time of a DU into BPB 1 is limited not to start earlier than the final arrival time of any DU or picture it predicts from. - When the cbr_flag is equal to 1, a non-zero initial arrival time for DU 0 is defined for BPB 1 (and higher) and it is a requirement for bitstream conformance (and particularly to the initial arrival delay) that the initial arrival time of a DU into BPB 1 shall not start earlier than the final arrival time of any DU or picture it predicts from. - A VPS VUI flag, du_based_bpb_sync_flag, can be used to indicate whether the final arrival time of pictures or DUs are considered above. A final arrival time of pictures suits conventional (picture-based) HRD operation, while a final arrival time of DUs suits ultra-low delay operation with layered scalability (JCTVC-N0290). A new bitstream partition initial arrival time SEI message is defined and used during CPB operation - The buffering period and picture timing information for bitstream partitions is conveyed using the buffering period, picture timing and decoding unit information SEI messages included in scalable nesting SEI messages for which bitstream_subset_flag is equal to 0. It is specified that when these SEI messages are contained in the scalable nesting SEI message with bitstream_subset_flag equal to 0, they apply to bitstream partitions consisting of the layers indicated by the scalable nesting SEI message. |
| JCTVC-O0234 Multilayer HRD Management (Arrisi, Sony) | Motivation: Layer based buffer flow management (CPB bit rate, CPB size, CPB management, DPB management) for a multilayer operation is investigated. Proposal Summary: The key issues of layer based buffer flow management include: - Layer based CPB buffer flow model - Syntax and semantics for signaling layer based HRD management - Scope of buffering period and picture timing SEI messages for cross layer timing and synchronization. - System level STD operation and issues to support both layer based CPB management multi-layer HRD model Proposal Details: Two methods (VPS extension or a new SEI) for signalling layer specific CPB parameters are defined. VPS extension to signal layer specific CPB parameters: A sub-CPB is associated with each layer. hrd_parameters can be signaled for each layer in the layer set. sub_cpb_parameters_table () {; Descriptor. …. for( i=1; i <= vps_num_layer_sets_minus1; i++) {. sub_cpb_info_present_flag[ i ]; u(1). if(sub_cpb_info_present_flag[ i ]) {. for(k=0; k < vps_num_hrd_parameters && i != hrd_layer_set_idx[k] ; k++). for( j=1; j <= vps_max_layer_id; j++) {. if( layer_id_included_flag[ i ][ j ] ) {. hrd_parameters( cprm_present_flag[ k ], vps_max_sub_layers_minus1 ). }. }. }. }. …. }. sub_cpb_info_present_flag[ i ]: sub_cpb_info_present_flag[ i ] = 1 indicates CPB parameters for each layer in the layer set i are given. sub_cpb_info_present_flag[ i ] = 0 indicates CPB parameters for each layer are not present. SEI Signaling of layer specific CPB parameters: multilayer_cpb_info( payloadSize ) {; C; Descriptor. active_vps_id; 5; u(4). for( i = 1; i <= vps_max_layer_id; i++ ) {. layer_cpb_info_present_flag[i]; 5; u(1). if(layer_cpb_info_present_flag[i]) {. layer_max_bitrate_value[ i ] layer_average_bitrate_value[i]; 5 5; u(16) u(16). layer_max_cpb_size_value[i]; 5; u(16). }. }. }. active_vps_id: identifies an active VPS that contains the layer information. The value of active_vps_id shall be equal to the value of video_parameter_set_id of the active VPS for the VCL NAL units of the access unit containing the SEI message. layer_cpb_info_present_flag[i]: specifies whether the CPB parameter information about layer index I is included. layer_max_bit_rate_value[ i ]: specifies the maximum bit rate value of layer index i. layer_avg_bit_rate_value[ i ]: specifies average bit rate value of layer index i. layer_max_cpb_size_value[i]: specifies maximum cpb size value of layer index i. Scope of Buffering Period and Picture Timing SEI Messages For BP & PT SEI messages two possible solutions are envisioned: First, because of the strict output timing requirements it is proposed to have a single removal time and accordingly a single PT SEI message to be associated with all the layers. BP SEI messages can be however associated with each layer for sub_CPB buffer management. A second possibility is to have both BP and PT SEI messages to be associated with each layer. Additional Notes: CPB operation text based on signaled hrd parameters is not defined. |
| JCTVC-O0188 AHG20: Ultra Low Delay for SHVC, MV-HEVC and 3D-HEVC (Fraunhofer HHI) | Motivation: The current bitstream order in SHVC, MV-HEVC and 3D-HEVC prohibits ultra-low delay applications. Proposal Summary: This contribution proposes to enable ultra-low delay operation with a corresponding bitstream order and decoding picture buffer operation and discusses implications of the CPB layout on legacy devices. Proposal Details: -A VPS du_interleaving_enabled_flag indicating du interleaving across layers is defined. -Bitstream order is restricted based on the VPS flag. A change is proposed to enable inter-layer prediction from DUs with nuh_layer_id equal to nuhLayerIdA before all DUs with nuh_layer_id equal to nuhLayerIdA are decoded. Text: - Current decoded picture marking and storage The process specified in this subclause happens instantaneously at the CPB removal time of anythe last decoding unit m of picture n, CpbRemovalTime[ n ]. - If du_interleaving_enabled_flag is equal to 1 then the following applies. - If decoding unit m is the first decoding unit of the current decoded picture, the current decoded picture is stored in the DPB in an empty picture storage buffer picStorBufA, the DPB fullness is incremented by one, and the current picture is marked as "used for short-term reference". - Otherwise, the current decoded picture is stored in the DPB in the in picture buffer storage picStorBufA. - Otherwise (du_interleaving_enabled_flag is equal to 0) and decoding unit m is the last decoding unit of picture n, Tthe current decoded picture is stored in the DPB in an empty picture storage buffer, the DPB fullness is incremented by one, and the current picture is marked as "used for short-term reference". |