| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0350 |
| --- | --- |

| Title: | Cross check for JCTVC-O0193: Unification of processing order of RDPCM (Method 2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for the proposal described in Document JCTVC-O0193. All the results released by the proponent are tested and it is reported that all the test points are confirmed. Analysis of the source code and text changes reveals perfect matching between the algorithm and the implementation.

# Introduction

JCTVC-O0193 addresses some discrepancies between the text and the software for Residual DPCM (RDPCM). In particular, two methods are proposed and in the report only method 2 is cross-checked. Method 2 aligns the intra RDPCM design towards the inter one, i.e. residuals reconstruction at the decoder is done before the scaling operated by transform skip. Moreover, also residual rotation is harmonised between intra and inter by performing it always after RDPCM is completed. The experimental results and source code analysis are reported in the following sections.

# Experimental results

The proposed RDPCM design unification has been tested for lossy coding according to the RCE3 common test conditions described in JCTVC-N1123. The cross-checker has run the provided software under the same coding conditions and the results are listed in Table 1. All the test points are confirmed.

Table 1: Experimental results for lossy coding.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | 0.3% | 0.0% | 0.1% | 0.2% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | 0.1% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% | -0.3% | -0.2% | -0.2% |
| SC(444) YUV Optional | 0.2% | 0.1% | 0.0% | 0.3% | 0.2% | -0.1% | 0.2% | 0.1% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.1% | -0.1% | -0.1% | -0.2% | -0.3% | -0.2% |  |  |  |
| Animation RGB 444 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -0.2% | -0.3% | -0.3% | -0.1% | -0.2% | -0.2% |  |  |  |
| SC(444) YUV Optional | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.2% | 0.0% | 0.0% | 0.3% | 0.2% | 0.1% |  |  |  |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 0.2% |  |  |  |
| SC RGB 444 | 0.6% | 0.5% | 0.6% | 0.4% | 0.4% | 0.3% |  |  |  |
| Animation RGB 444 | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC YUV 444 | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| Animation YUV 444 | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | 0.7% | 0.7% | 0.4% | 0.1% | 0.2% | -0.1% |  |  |  |
| SC(444) YUV Optional | -0.6% | -0.3% | -0.4% | 0.1% | 0.3% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Source code inspection

The proponent has implemented the proposed explicit signalling of intra RDPCM using the HM-RExt software Version 4.1. In the following, the performed source analysis is detailed.

The source changes are marked with the following macros:

- DECOUPLE_TS_RR

- INTER_ENC_TS_RDPCM_RR_DEC_RR_RDPCM_TS

- INTRA_ENC_TS_RDPCM_RR_DEC_RR_RDPCM_TS

In particular the transformNxN, invTransformNxN, xTransformSkip, xITransformSkip, xQuantInterRdpcm functions are modified to account for the aforementioned design alignments. The source analysis reveals perfect matching between the algorithmic description of method 2 and implementation.

# Text changes inspection

Not provided at the time of cross-check report writing.