| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0351 |
| --- | --- |

| Title: | AHG5: Sample masking for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema, Miska Hannuksela, Kemal Ugur Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a possibility for “masking” areas in blocks predicted with intra block copy mode and substituting all the sample values in a masked area with a single sample value. It is argued this kind of operation helps coding efficiency by being able to provide finer prediction granularity for intra block copy with no significant complexity implications. It is reported that the proposed approach provides objective bitrate reductions of -1.3 %, -1.6 % and -0.4 % for All Intra SC YUV 444, SC RGB 444 and Class F categories, respectively. In other categories the average of reported bitrate differences is 0.0 %.

# Introduction

It was observed that the current intra block copy operates occasionally inefficiently due to its fixed size. E.g. it can copy significant residue together with the actually desired content. This kind of behaviour is illustrated in Figure 1 below where part of the next letter “a” gets copied together with the letter “c” to the predicted block.

Figure 1: Example of intra block copy

# Proposal

It is proposed to signal a mask for the CUs coded with intra block copy mode. The mask is covering a square area extending from one of the borders of the CU. Different allowed mask types are depicted in the figure below.

Figure 2: Example of masks on different borders of the prediction block

The height of the masks for modes 1 and 3 and the width of the masks for modes 2 and 4 can vary from 1 pixel to 3 pixels. This parameter is signalled together with the position of the mask using bypass bins. For chrominance blocks the signalled width of the mask area is scaled according to the component scale parameter. Whether masking is applied or not is signalled for each intra block copy CU with arithmetically coded bin using a single context.

All the sample values in the masked area are substituted with a single sample value obtained from the middle sample of the inner boundary of the mask. The black samples in Figure 2 illustrate the location of the sample which value is used in the substitution process.

# Results

Results below are obtained with a simple encoder search algorithm where a SAD based cost of the luminance intra block copy mode is evaluated for all the available masking modes after the intra block search is performed for a CU.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.4% | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.6% | -1.4% | -1.5% | -1.5% | -1.4% | -1.4% | -1.4% | -1.3% | -1.3% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -1.3% | -1.1% | -1.2% | -1.3% | -1.1% | -1.2% | -1.3% | -1.1% | -1.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101 % |  |  | 101 % |  |  | 101 % |  |  |
| Dec Time[%] | 100 % |  |  | 100 % |  |  | 100 % |  |  |

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).