| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0352 |
| --- | --- |

| Title: | BoG report on Range Extensions topics |  |  |
| --- | --- | --- | --- |
| Status: | BoG report |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D. Flynn C. Rosewarne | Email: | dflynn@blackberry.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

A break out group was held between Friday the 25th and Thursday the 31st of October 2013 to review input documents relating to the Range Extensions.  Specifically, topics on RDPCM, Intra block copying, memory bandwidth minimization, inter-component decorrelation, transform coefficient coding, intra prediction methods, alpha channel support, lossless coding, screen content coding, evaluations of range extensions software coding performance, specification text refinements and source video test material were examined.  In addition, core experiments were planned and reviews of the current common coding conditions and the specification text of adopted proposals were conducted.

This report conveys a number of recommendations from the BoG and are summarised in section 1.  Throughout the report, the expression "Recommend:" is used to express the consensus in the group when a positive action should be taken.

- Recommendations

The following recommendations are made by the AHG:

- On RDPCM:

  - Rotation in the same place for all methods, as per current spec, [superseded by later recommendation]

  - Unify the ordering of inverse transform and inverse RDPCM operations for inter and intra, and to pick one of the two possible orderings of inverse transform and inverse RDPCM:

    - invTS → invRDPCM

    - invRDPCM → invTS

  - Clean up software implementation of RDPCM (Various contributions have been provided, leave to software co-ordinator's discretion as to how).

- Disable Rotation for non-Intra (ie, Inter and IntraBC) blocks with a skipped transform [O0186]

- RCE2, change the value of the maximum rice parameter chosen for the RCE.A1 adoption, as current value (9) is best only for 16 bit, while (7) may be a more appropriate balanced choice.

- Spec issues:

  - Fix an incompatibility with version 1, caused by an intra filtering process being disabled conditional on cu_transquant_bypass_flag.  It is proposed to do this only if the SPS level intra_rdpcm_enable_flag is equal to 1.

- On IntraBC:

  - Enable Inter RDPCM (whatever method) for intraBC (ie, match the software) [O0185]

  - Rename the flags intra and inter RDPCM enable flags to not be so confusing [O0185]

  - Only use a single context for the intra_bc_flag (ie, no up/left contextualisation) [O0073]

  - Define the CIP behaviour for intraBC blocks: an IntraBC block should be regarded as available for determination of Intra prediction reference samples, and an encoder should avoid referencing reconstructed pixels from Inter blocks when encoding an IntraBC block. [O0155]

  - Modify the fast encoder mode decision for IntraBC. [O0245 (2)].

  - Not to change the current intended intraBC reference sample restriction on the neighbouring CTU, but that there should be some investigation into the cost of various restriction sizes. [discussion of O0074]

  - Spec should be clearer in specifying vector limits.

  - BV coding should use a delta BV based on the previously encoded BV in the CTU (resets at the start of each CTU) [O0122]

  - Always use DST for 4x4 Luma TBs with IntraBC [O0183]

  - Adopt spec fixes on what samples may be used as reference for intraBC [O0183]

    - Don't reference anything outside current CTU + Left CTU,

    - Don't reference anything outside current Slice/Tile,

    - Don't reference anything within the current CU being reconstructed.

  - Prohibit IntraBC from referencing samples outside the current picture [discussion of O0183]

Two new methods (JCTVC-O0351 and JCTVC-O0205) are presented for the first time that improve the performance of intra block copying.  Further study of these methods is strongly encouraged, possibly in the context of a CE.

If a CE is to be performed, the following methods would be of interest to test the benefits of finer granularity prediction techniques along with padding and vector range restrictions:

- Anchor: with current restrictions

- Test: all unavailable samples pre-set to default value (mid grey), overlap search restriction relaxed

  - Well defined behaviour (Can't be violated) [JCTVC-O0074]

- Test padding: with relaxed vector restrictions for overlap [JCTVC-O0157]

  - Performance improvement

- Test 2NxN, Nx2N [JCTVC-O0205]

  - Performance improvement

- Test TU process [no method currently available: maybe if (overlap) {force TU split}] [MS/O0183]

- Test NxNPU at smallest CU level (follow current split modes for Intra, and each PU has a BV) [QC]

- Test masking [JCTVC-O0351]

- Possible test: relax search in other directions (ie, non-overlapping).

- Left search area vs Cost [Comment]

- Combinations of the above, as appropriate.

- It is also recommended for a CE to study palette coding.  This could be in the context of the above CE, or an alternative.

# RDPCM

All contributions were presented prior to discussion of which method(s) should be recommended.

  - Discussion

For background, the current software and specification behaviours are:

For transquant bypass blocks,

sw/spec:	Entropy decoding → rotation → invRDPCM → residual

For transform-skip blocks,

Intra/IntraBC:

sw/spec:	Entropy decoding → invQuant → invTS + rotation → invRDPCM → residual

Inter:

spec:	Entropy decoding → invQuant → invTS + rotation → invRDPCM → residual

sw:	Entropy decoding → invQuant → invRDPCM → invTS → rotation → residual

The following items were discussed:

- Rotation in the same place for all methods, as per current spec:

entropyDecode → invQuant → invRotate → ...

- Spec currently says: invTS → invRDPCM

- All proposals suggest the ordering of operations in 2 should be the same for inter and intra

- Ordering:

  - invTS → invRDPCM ("RDPCM")

    - Suggest this is simpler in the current spec

    - May have a lower implementation impact since it is a post-processing block, rather than affecting invQuant.

  - invRDPCM → invTS ("CDPCM")

    - Has a shorter encoder critical path (but this is two trivial shifts)

    - Results from O0193 suggest marginally better than (a) (ignoring RDOQ)

    - Operates at a higher bitdepth than (a): but the worst case is intra transquant bypass and the same for both schemes due to the position of the clipping.

- If (a), although could be with (b) but is untested, one may wish to additionally disable rotation in certain circumstances.  See (JCTVC-O0186 [not reviewed here] and JCTVC-O0171).

- Clip input to invXDPCM?

  - Comment: costs of larger intermediates may not be worse than clipping.

Comment: (b) changes behaviour from v1.

Recommend: Do (1), Do (3), Do one of (4), Think about (5).

## Contributions

JCTVC-O0066 AHG5: Unifying DPCM [K. Sharman, N. Saunders, J. Gamei (Sony)]

At the previous meeting, a number of proposals were accepted that in effect provided RPDCM coding for inter and intra (and intra-block-copy) prediction modes, for transform-skipped lossy and transquant-bypass lossless processing modes. In addition, an independent rotation function for transform-skip and transquant-bypass were introduced. The resulting algorithm differed for inter and intra processing. This proposal examines the current algorithm, indicates the relative merits of the differences, and then describes two methods by which the algorithm can be unified.

BD-rate changes for the CDPCM-only method versus the anchor (HM12.0_RExt4.1) are reported to range from 0.0% (for non-screen-content) to -4.1% (for low-delay high tier GBR screen-content).

Inconsistent between intra and inter:

- Rotation was restricted to 4x4, whereas other RDPCM modifications are extended to larger sizes.  This was intentional at the previous meeting (due to possible losses at larger blocks and decoder cost).

- RDOQ can be more easily applied if rotation occurs after RDPCM (encoder) -- because the processing order ends up being the same as the reverse scan of the entropy encoder.

Critical path length on the encoder: is sample serial in the direction of the prediction, the paths are unequal for the intra (residuals) and inter (coefficients) path.

Clipping of accumulator in DPCM:  Currently unclipped, requiring 16+5 bit.

Comment: instead of truncating or clipping, input of inverse RDPCM stage should be clipped instead (which isn't the minimal bitwidth but doesn't cause parallelization problems), Ie accumulator is then bitdepth + 1 + 5.

Comment: RDPCM can be expressed as a transform, adding clipping in the accumulation loop would prevent that.

Multiple proposals:

Unify to make inter behave like intra:

- Rotation occurs after RDPCM

Unify

Results:  Shows some gains for both methods since it allows the application of RDOQ for all DPCM TUs

Lossless results are identical.

Remarked: Could be possible to change inter scan order based on DPCM mode for possible hardware savings.

The authors are favourable to either method, but prefer the version with a shorter feedback path in the encoder.

Side point: Encoder issues

Fast search is enabled in HM for intra, where a three stage intra mode decision is performed.  First to find candidate angles (using minimal SAD cost after a hadamard transform).  Then test the five best angles without RQT, then compute the final RQT for the best angle.

JCTVC-O0341 Cross-check for JCTVC-O0066: Unifying DPCM [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-O0134 On residual DPCM design unification for lossy coding in HEVC-Rext [M. Naccari, M. Mrak (BBC)]

Residual DPCM (RDPCM) is adopted for inter and intra coding in the HEVC-RExt specification text Draft 4. Both in intra and inter coding, RDPCM performs reconstruction of residuals after inverse quantisation but inter RDPCM reconstructs the residuals before the inverse shift operated when transform skip flag is equal to one while intra RDPCM performs the reconstruction after the shift down. This documents analyses a design unification approach and reports both on the performance as well as on the required text changes. Under RCE3 common test conditions it is reported that the proposed design unification does not bring losses when compared to the anchors and, for some sequence classes, BD-rate gains are registered.

This contribution is similar to JCTVC-O0066, but does not touch aspects relating to rotation.  It proposes  unifying the design towards the inter design.  Rotation stages are untouched and therefore doesn't resolve the RDOQ issue highlighted in JCT|VC-C0066.  It switches the order of transform and RDPCM in intra such that RDPCM occurs after the inverse transform.

Results: minor coding benefits are seen, but mostly no effect.

JCTVC-O0310 Cross-check of JCTVC-O0134 on residual DPCM design unification [S. Lee, C. Park, C. Kim (Samsung)] [late] [miss]

JCTVC-O0171 Order of rotation, inter RDPCM and transform-skip [R. Joshi, J. Kang, C. Pang, J. Sole, M. Karczewicz (Qualcomm)]

It is asserted that there is a mismatch between the HEVC Range Extensions text specification draft 4 and the test model software HM12.0-RExt-4.1. For lossy coding of blocks using inter RDPCM (inter and intra block copy), on the decoder side, the order of operation in software is entropy decoding, inverse quantization, inverse RDPCM, inverse transform-skip and rotation, whereas in the text specification the order of operations is entropy decoding, rotation, inverse quantization, inverse transform-skip and inverse RDPCM. Four different options are presented to reconcile the differences between text specification and software. Each option results in no gain or small losses for most classes. For SC GBR (optional) and SC YUV (optional) classes for lowdelay configuration, some BD-rate gains are observed.

Does not touch RDOQ in any of the following four options:

- Use spec (N1005_v3) (about 0.3% loss for SCC)

- Do opposite of JCTVC-O0134, ie, Inter RDPCM is at residual precision (cf. O0134, slight losses compared to anchor (at most ~0.1%))

- Use spec + disable rotation for inter RDPCM (between option 1 and option 2)

- Use spec + disable rotation for inter and intra bc. (about the same as option 2).

Comment: favours spec style behaviour since it is simpler to add extra functional units rather than modify previous ones, and would favour option 4.

Comment: Option 4 would have rotation sometimes on sometimes off.

NB, rotation is enabled/disabled in as an SPS parameter.  Decoder pipelines must support it being absent.

JCTVC-O0333 Cross-check of option 4 of JCTVC-O0171 [B. Li, J. Xu (Microsoft)] [late]

JCTVC-O0338 Cross-check of option 3 of JCTVC-O0171 [Y. H. Tan (I2R)] [late] [miss]

JCTVC-O0185 RDPCM operation unification and cleanup [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

Current HM-12.0+RExt-4.1 has two different RDPCM modes. One RDPCM mode is used for intra in which the residue is refined after final scaling and the other is used for inter, in which the residue is refined before final scaling (with higher precision). This document proposes to unify these two cases that the residue is always refined after final scaling, as what the current spec does. And the current spec and the software does not match each other on inter RDPCM and intra RDPCM. This document also proposes to make the spec on RDPCM clearer.

The suggested RDPCM method is identical to option 1 of O0171 (Make Inter behave as Intra).

IntraBC, which behaves as an inter tool coding wise, has application of RDPCM controlled by residual_dpcm_intra_enabled_flag.  The spec also omits RDPCM of IntraBC.

Suggests:

- to allow inter RDPCM for intra BC and use inter_enabled flag.

  - Assuming that it is enabled, disabling it for:

    - intra, losses 0.4%, 1.2%, 1% for Classes F, SCC RGB, SCC YUV.

    - inter, losses 0.2%, 0.8%, 0.4%

    - lossless, intra: 1.4%, inter: 0.8%

    - NB, losses worse in optional SCC sequences

- rename  the enable flags (since the "inter" flag will also be need for intra pictures):

  - Intra RDPCM => "implicit RDPCM"

  - Inter RDPCM (with direction signalling) => "explicit RDPCM"

NB, JCTVC-O0170, JCTVC-O0192(point 2) makes the same proposal regarding the flag behaviour (although they do not contain the editorial renaming).

Considering the above two suggestions only:

Recommend: Enable inter RDPCM (whatever method) for intraBC (ie, match the software)

Recommend: rename the flags to not be intra/inter based.

This contribution also includes a software clean up to remove nearly six hundred lines of code.

Recommend: Some form of clean up of software is performed.

JCTVC-O0291 Cross-check of JCTVC-O0185 RDPCM operation unification [P. Lai, S. Liu (MediaTek)] [late]

JCTVC-O0193 AHG5: Unification of processing order of RDPCM [P. Lai, S. Liu, S. Lei (MediaTek)]

Currently in the Range Extension reference software, lossy RDPCM methods for Intra and Inter blocks have different processing orders with respect to transform-skip and residue rotation. Furthermore, the Range Extension text specification only captures one of the processing orders, namely the one for lossy-RDPCM Intra. This document presents methods to unify the processing order of lossy RDPCM in Intra and Inter cases, in particular when residue rotation is involved. Simulation results and modifications to the text specification are provided.

Two methods:

- Same as JCTVC-O171 option 1.  Ie, make inter behave as intra.

- Same as part of JCTVC-C0066 (missing the RDOQ part), performs slightly better than option 1, but worse than C0066 due to the lack of RDOQ for that mode.

JCTVC-O0324 Cross-check of Unification of processing order of RDPCM (JCTVC-O0193) [B. Li, J. Xu (Microsoft)] [late]

# Block Copying

## General comments

Comment: It has been observed during testing of intra block copy that there have been instances where characters are incorrectly predicted.  For example, a number 5 being reconstructed as a number 6.  This effect was only noticed on single frames -- the temporal nature of the encoding tended to correct the error.  However, if a single frame were frozen for display in a video conferencing situation, this could be problematic.

## Context modelling

JCTVC-O0073 AhG5: On context modelling simplification for Intra_bc_flag coding [E.Alshina, A.Alshin (Samsung)]

Contribution present performance test results for Intra Block Copy signalling simplification. In current implementation context for this flag is predicted from above and left. 3 context models are used in total. If both predictions from left and above are not used then 2 context models are removed from design and lien buffer for Intra Block Copy flag storage is not needed. The performance change in loss-less test is 0,0% for all test classes (for one class even 0,2% gain is observed). In lossy test configuration average performance change is 0,0% for all 3 colour components.

Intra BC adds a syntax element intra_bc_flag that uses context models based on up and left.

Proposal is to remove the up and left selectors (ie removes a line buffer) and use a single context.

Minor performance loss:

lossless: nothing

lossy: no significant loss

Recommend: reduce intra_bc_flag to a single context model (no prediction)

JCTVC-O0113 AHG5: On context modelling for intra block copy mode [C. Rosewarne, M. Maeda (Canon)]

Not presented, same as 0073 (recommended for adoption).

JCTVC-O0158 AhG5: Context derivation method for intra_bc_flag coding [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

Intra Block Copying (Intra BC) was adopted at the 14th JCT-VC meeting as an intra coding tool in the HEVC Range Extensions draft specification. Intra BC codes the intra_bc_flag to signal whether the method is applied for the CU. The context derivation for this flag needs a line buffer. Two methods are proposed in this contribution to remove the line buffer for the intra_bc_flag.

Two methods are proposed:

- Prediction is only within the current CTU

- Same as JCTVC-O0073

The second method has been recommended for adoption in the notes of O0073.

JCTVC-O0286 Cross-check of JCTVC-O0158 method 2 [J. Xu (Sony)] [late]

## Constrained intra-prediction

JCTVC-O0155 AhG5: Constrained intra prediction for intra block copying [C. Pang, J. Chen, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

Intra Block Copying was adopted as an intra coding tool in the HEVC Range Extensions draft specification. Consequently, the existing constrained intra prediction (CIP) method does not provide the intended benefit. In this document, the CIP in HEVC Range Extensions is addressed. Three CIP methods are proposed to enable the CIP functionality.

NB, there maybe some overlap with JCTVC-O0232 and JCTVC-O0183.

Three methods are provided,

- CIP => disable intraBC in inter slices

- CIP => treat intraBC as inter, it can reference anything, but intra can't reference it.

- CIP => intraBC and intra can only reference each other

Performance: 2 > 3 > 1

Outright disabling of intraBC seemed to be excessive (and the least performing option): Discount (1).

Comment: While option (2) may be higher performing than (3), it restricts an encoder form conducting its normal intra search process that it would use in intra pictures when operating gradual decoder refresh in an inter picture.

|  | Lossless RA |  | Lossy LB |  |  |
| --- | --- | --- | --- | --- | --- |
|  | (2) | (3) | (2) | (3) |  |
| Class F | 3 | 3 | 4 | 3 |  |
| (RGB 4:4:4) SCC | 15 | 13 | 17 | 9 |  |
| (YCbCr 4:4:4) SCC | 15 | 13 | 15 | 8 |  |
| (RGB 4:4:4) SCC Opt | 33 | 29 | 32 | 14 |  |
| (YCbCr 4:4:4) SCC Opt | 37 | 34 | 35 | 17 |  |

Comment: The test methodology does not represent how CIP is actually used.

NB: JCTVC-O0232 also suggests (2) and an encoder-side solution

NB: JCTVC-O0183 also suggests options (1) and (3)

Comment: Proponent of O0183 prefers (3)

Comment: Proponent of O0232 prefers (2)

Recommend: pick one of (2) or (3) after evaluating a test performed during the meeting.

Test:

- LowDelayB: pick of column of CTUs per picture and force "intra"

- Consider also use dithered pattern instead.

(Further discussion after the above experiment was conducted)

An experiment has been performed to encode video using a pseudo-intra refresh with a scrolling column of CTUs progressing from left to right in a low-delay B configuration.  The methods (2) and (3) are tested, with the results below using (2) as a reference, and (3) as the tested configuration.  Method (3) is shown to have a 4.2% BD-rate advantage over method (2), when averaging the class F and SC YUV 444 results.

Comment: It should be noted that the CTU size of 64x64 is much larger than more traditional macroblock based approaches in systems such as AVC.

Recommend: option (3), where an IntraBC block should be regarded as available for determination of Intra prediction reference samples, and an encoder should avoid referencing reconstructed pixels from Inter blocks when encoding an IntraBC block.

## Encoder search

JCTVC-O0245 AHG5: Fast encoding using early skipping of Intra block copy (IntraBC) search [D.-K. Kwon, M. Budagavi (TI)]

In this contribution, two three early skipping decision methods for Intra block copy (IntraBC) search are proposed to reduce encoding complexity. In the first method, for 8x8 and 16x16 CUs, IntraBC mode search is skipped when regular intra mode cost at the same CU depth is smaller than a threshold dependent on QP. In the second method, for 8x8 CUs, when a parent 16x16 CU is not IntraBC mode and 8x8 CU activity is smaller than a threshold, then IntraBC mode search is performed using only 1-D search points in the second method or is skipped in the third method. A summary of reported results is provided below:

•	Under RCE2/RCE3 lossy common conditions: The combination of both methods Method 1 and Method 3 is reported to reduce encoder run-time for IntraBC from around 140142% to 122% (AI), 116% to 102% (RA), 112113% to 101% (LDB). The corresponding average BD-Rate results are reported to be in the range of 0.0% to 0.8% (AI), 0.0% to 0.5% (RA), −0.2% to 0.64% (LDB). For optional screen content sequences the BD-Rate results are reported to be in the range of 0.31% to 2.87%. (Negative BD-Rate numbers mean bit-rate saving, positive BD-Rate numbers mean bit-rate increase).

•	Under RCE2/RCE3 lossless common conditions: The combination of both methods Method 1 and Method 3 is reported to reduce encoder run-time for IntraBC from 155% to 131% (AI), 112% to 103% (RA), 110% to 103% (LDB). The corresponding total bit-rate results are reported to be −0.6% to −0.8% (AI), −0.1% to −0.3% (RA), −0.1% to −0.3% (LDB). For optional screen content sequences the total bit-rate results are reported to be in the range of −1.0% to −1.8%. (Negative bit-rate numbers mean bit-rate increase).

Three methods are described:

- If intra cost is less than lambda dependent threshold, skip checking of intraBC.

- Method (1) + Method (3), except method (3) is modified to perform two 1D searches instead of skipping.

- Method (1) + If 16x16 CU is not selected as intra BC and activity for the 8x8 CU is less than a bitdepth dependent threshold, then skip intra BC search for this CU.

Runtime results are shown below, the anchor has intraBC disabled.

|  | AI |  |  |  | RA |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Lossless | MT | HT | SHT | Lossless | MT | HT | Lossless | MT | HT |
| HM | 155 | 142 | 139 | 137 | 112 | 116 | 115 | 110 | 113 | 113 |
| Methd1 | 149 | 128 | 130 | 131 | 107 | 102 | 102 | 106 | 102 | 102 |
| Methd2 | 135 | 123 | 124 | 124 | 104 | 101 | 102 | 104 | 102 | 102 |
| Methd3 | 131 | 122 | 123 | 123 | 103 | 102 | 102 | 103 | 101 | 102 |

Method 2 has 0.8–1.2% loss in lossless conditions for class F, otherwise the losses are lower.  For lossy, the typical losses are ~0.3%.

Method 3 has similar lossless losses, but larger non-lossless losses of ~0.5%.

Comment: what if the search area is reduced to 1 CTU

Comment: most of the benefit in inter seems to come from method 1

Recommend: adopt method 2

JCTVC-O0289 A cross-check report for JCTVC-O0245: Method 1+Method 3 [G. Jin, A. Saxena, F. Fernandes] [late] [miss]

JCTVC-O0329 Crosscheck of JCTVC-O0245: Test 1 in Fast encoding using early skipping of Intra block copy (IntraBC) search [C. Pang (Qualcomm)] [late] [miss]

JCTVC-O0156 AhG5: Fast encoder search and search region restriction for intra block copying [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

Intra Block Copying (Intra BC) was adopted as an intra coding tool in the HEVC Range Extensions draft specification. To reduce complexity from encoder search for Intra BC, a fast search method is proposed in this document. In addition, the search region is normatively restricted to the current and left CTU (as the current setting in the HM12.0_Rext4.1). The reported speed-up of the fast encoder is 17% on average with a performance penalty of about 0.5% loss for SC sequences and about 1% loss for optional sequences.

Proposed method:

- Search only for 16x16 and 8x8 (current fast mode)

- 16x16 has 1D search only (current fast mode)

- 8x8 1D and 2D search (current fast mode)

- new: 1D search when no neighbours have intraBC

- new: if RD cost less than some threshold terminate search early

- new: if component being searched has more unique values than a threshold (6?), don't search

|  | AI |  |  |  | RA |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Lossless | MT | HT | SHT | Lossless | MT | HT | Lossless | MT | HT |
| HM | 155 | 142 | 139 | 137 | 112 | 116 | 115 | 110 | 113 | 113 |
| O0156 | 130 | 121 | 122 | 123 | 103 | 100 | 101 | 103 | 102 | 102 |

The results show losses under lossless conditions of 0.4 – 0.2%, and ~0.5% for lossy conditions.

The colour detection process seems to offer a minor benefit.

JCTVC-O0303 AHG5: Crosscheck of JCTVC-O0156 on fast encoder search and search region restriction for intra block copying [D.-K. Kwon (TI)] [late]

When compared to Method2 of 245, this method is slightly faster but has slightly more loss.

## Block sizes and res

JCTVC-O0102 AHG5: Block size restriction of intra block copy [S. Lee, C. Park, E. Alshina, C. Kim (Samsung)]

In this contribution, it is proposed to specify the maximum CU size in which intra block copy is enabled in HEVC range extension text to make the design clean and to avoid the redundant signalling of flags for intra block copy. It is also suggested to enable intra block copy up to 16x16 CU in the common test condition, considering the trade-off between the coding gain and complexity.

This is a coding efficiency proposal, that allows the encoding of the intrabc flag for large CU sizes (ie, specifies a minimum depth at which CU can use intraBC).

The overhead isn't large.

Comment: what if a restriction was static? Eg, prohibit 64x64, does that make the decoder simpler?

This might be sensible, but recommend no action otherwise.

JCTVC-O0296 AHG5: Cross-check of JCTVC-O0102 for block size restriction of intra block copy [L. Guo (Qualcomm)] [late] [miss]

JCTVC-O0074 AhG5: Intra block copy within one LCU [E.Alshina, A.Alshin, S.Lee (Samsung)]

Contribution presents performance and complexity of intra block copy depending on intra motion vector range. If intra block copy is allowed within only current LCU then BD-rate gain from this tool shown for screen content is 19.7% in all-intra, 15.9% in random access and low-delay tests. This is 71-72% of intra block copy gain shown in default configuration (27.4% in all-intra, 22.3% in random access and low-delay tests) in lossy test. In lossless test situation is very similar 18,2% (from 25,0%) in all-intra test and 14,4% (21,0%) in MC test IntraBC gain is preserved if search range is restricted for one LCU. The restriction of intra block copy within current LCU only is highly preferable for HW implementation.

The current CTU search area is the current and previous CTU.

This contribution suggests to restrict this to 1CTU plus 4 samples above or to the left.

Previously, N0254 and N0256 restricted the search area to 1CTU plus 4 samples to the left.  This version has a marginally better result

This contribution also suggests that unavailable samples are filled with mid-grey.  However, this is not tested.

Comment: Dealing with the upper four lines can be problematic for implementations.  Since the four lines above may be partially deblocked.

Comment: This same point is also made in O0232.

Proponent does not object to not including the four lines above.

|  | I | RA | LB |
| --- | --- | --- | --- |
| IntraBC: | 17.3 | 14 | 9.1 |
| 1CTU+4x4 | 10.8 | 8.4 | 5.5 |
| 1CTU+4+Pad | 8.0 | 6.3 | 4.6 |

The restriction seems to severely reduce the efficacy of the intra BC tool.

Comment: there may be a trade-off between the use of 2*CTU and CTU+4

There was a strong preference to restricting the buffer size, with preference expressed for CTU+4.

Recommend: to leave the restriction as is intended currently, and to form a CE to investigate the trade-offs on the middle ground.

Recommend: spec should be clearer in specifying the vector limits.

JCTVC-O0328 Crosscheck of JCTVC-O0074: Intra block copy within one LCU [C. Pang (Qualcomm)] [late] [miss]

## Padding methods

### Discussion

There are several padding issues:

- If the reference region overlaps the current CU

- If the reference region exceeds the 2*CTU search area

- If the samples are unavailable due to CIP

- If the samples are unavailable due to a picture/slice/tile boundary

Solutions:

- Pad bottom right [O0112/O0157]: solves overlap and provides performance gain

  - This doesn't link overlapping with RQT decisions.

  - If 4x4 PUs, padding degenerates into a prefill with a horizontal blit of the left hand outside PU edge (actual implementation details may vary).  This would also solve the issue with CIP conditionality for the edge samples.

- Use default value for unavailable samples [untested]: solves all

- Use a TU based loop for reconstruction: possibly reduces overlap issue, may provide performance gain.

  - However, this means that the overlap amount and RQT are linked.

- Define bitstream conformance to prohibit illegal vectors.

Comment: is this for having well-defined behaviour or is it for a coding gain

Comment: why don't we have 4x4 PUs at the moment?  Enabling 4x4 PUs would be more consistent with the current design for intra and inter.  The intra BC reconstruction loop could then be performed exactly in the same way as Intra.

Comment: Start with one solution for padding, perhaps the O0157 which has now been tested with O0122.

Comment: It is expected that there will be a CE to study other IntraBC methods, some of which provide significant gains.  Perhaps padding should be examined in that context, especially to determine what benefit it has if the PU shape is changed.

Recommend: consider a CE.

### Contributions

JCTVC-O0112 AHG5: Extension of intra block copy [S. Lee, E. Alshina, C. Kim (Samsung)]

In this contribution, a method to extend intra block copy is proposed. When the current CU to be predicted overlaps the reference CU, the prediction samples in the overlapped region are generated by copying the available samples from either vertical or horizontal direction. This reference sample generation method extends the range of valid motion vectors in intra block copy, and provides the coding gain of -0.8% in AI-MT, -0.6% in RA-MT, and -1.1% in LDB-MT for SC YUV 444, respectively. In the proposed method, intra block copy is further extended by applying the motion vector prediction from left CU coded in intra block copy mode. When this motion vector prediction is combined with the reference sample generation, the coding gain is -2.5% in AI-MT, -1.8% in RA-MT, and -2.0% in LDB-MT for SC YUV 444, respectively.

If an overlap occurs, the overlapped region is edge extended from the longest available edge.

|  | AVG(Class F;SC YUV 444) |  |  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI-LL | RA-LL | LD-LL | AI-MT | RA-MT | LD-MT |
| Gains | 0.4 | 0.2 | 0.2 | 0.6 | 0.4 | 0.5 |
| + BV prediction |  |  |  | 1.5 | 1.1 | 1 |

Question: is the padding limited to the bottom left of the reference block, or can it be padded on the top right to allow the use of more vertically negative vectors (where a non-overlapping region may access non-available samples).

A: limited to top-right.

Comment: Padding when combined with BV prediction doesn't seem to offer higher gains than some of the BV offsetting techniques

Comment: What happens with a zero BV?

Proponent believes that doing something else would be better (BV offsetting)

JCTVC-O0307 Cross check for JCTVC-O0112: Extension of intra block copy [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-O0157 AhG5: Intra block copying with padding [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm), T. Lin, S. Wang (Tongji University)]

Intra Block Copying (Intra BC) was adopted as an intra coding tool in the HEVC Range Extensions draft specification. For the CUs with Intra BC mode, all the samples in reference block need to be reconstructed (available). In this document, the intra block copying method is extended to the situation where the reference block is overlapped with current CU, and the reference block is completed by padding. The performance gains for the AhG8 test conditions are reported to be average BD-rate reduction of 0.8%, 0.9%, 3.2%, 1.9%, respectively for RGB SC, YUV SC, optional RGB SC, optional YUV SC sequences in AI HE Main-Tier configurations.

Unlike O0112, this proposal pads only in the horizontal direction.  For the zero vector case, it essentially becomes a horizontal intra prediction.

Comment: how does this interact with CIP, or the picture edge?

An availability check is performed, and unavailable samples replaced with (1 << bitdepth -1)

It probably doesn't interact with CIP.

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| O0157 | 0.7 | 0.5 | 0.7 |
| O0157 + 1CTU+4pel restriction | 1.1 | 0.9 | 0.8 |
| O0157 + O0154(1) DV coding | 2.6 | 2.1 | 2.1 |

Comment: Is the padding process too complicated? It requires a more complex block copy operation, ie it isn't as simple as a line copy, it now require a line fetch + possible blit to replace padded pixels + write.

It might not be so different from inter which has to do a constant replacement after an availability check

Comment: However the inter process is more complicated due to the filtering operations required.

Comment: It would be interesting to see what the distribution of overlaps is, are they generally large or tiny?

Comment: Currently the block based copy is very simple, would prefer to keep more CTU storage for reference samples rather than introduce a right-side padding process.

Following the valid vector range restriction discussion, where it was recommended to take no action at this time and keep the current 2*CTU area:

Following the adoption of O0122 (intraBC BV prediction), the results of this method were updated and shown to be additive:

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| O0157 | 0.7 | 0.5 | 0.7 |
| O0122 | 1.2 | 1 | 0.9 |
| O0122 + O0157 | 1.8 | 1.4 | 1.4 |

Comment: perhaps the best course of action is to place this into a CE that evaluates other proposed methods (such as using a smaller PU size or changing the TU reconstruction process) which provide substantial gains.

Comment: there is still an unresolved issue of illegal vectors, from referencing unavailable samples.

Comment: an obvious solution is to replace unavailable samples with a default value.  Although, the default value may be quite poor for prediction.

JCTVC-O0311 Cross-check of JCTVC-O0157 [J. Xu (Sony)] [late] [miss]

## Block vector coding

### Discussion

|  | AVG(Class F;SC YUV 444) |  |  | HEVC MVD | IntraMV |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT | Syntax Changes | Predictor | Comments |
| Current Spec | 0 | 0 | 0 | N | (0,0) |  |
| O0122 | 1.2 | 1 | 0.9 | N | variable | memory for 1BV, reset on CTB boundary |
| O0123 | 1.1 | 0.8 | 0.8 | N | - | 2 "+" |
| O0154(1) | 2.1 | 1.7 | 1.6 | Y | variable | conditional flag to select predictor, 3 cxts; memory for 15 DVs (already exists for decoding inter), no reset on CTB boundary. With bypass for zero difference |
| O0154(1') | 1.9 | 1.7 | 1.5 | Y | variable | unconditional flag to select predictor, otherwise as above. |
| O0154(2) | 1 | 0.7 | 0.9 | Y | variable |  |
| O0167(1) | 0.9 | 0.6 | 0.6 | N | - | 2 "+" |
| O0167(2) | 1.4 | 0.9 | 1.1 | N | - | 2 "+"; logic |
| O0277(1) | 0.8 | 0.7 | 0.3 | Y | (0,0) | ensure max search range; fixed lengh code |
| O0277(2) | 1.1 | 0.9 | 0.9 | Y | (0,0) | ensure max search range; 288 entied for huffman table |
| O0232(1) | 0.2 | 0.1 | 0.1 | N | - | conditionally add offset to vector (y += S) |
| O0232(2) | 0.7 | 0.6 | 0.6 | N | - | conditionally add offset to vector (x += S) |
| O0232(3) | 0.5 | 0.4 | 0.4 | N | - | unconditionally add offset to vector (x += S) |
| 112 | .6 | .5 | .6 |  |  |  |
| 112^167(2) | 1.8 | 1.3 | 1.4 |  |  |  |
| 157 | 0.7 | 0.5 | 0.7 |  |  |  |
| 157&0122 | 1.8 | 1.4 | 1.4 |  |  |  |
| 157&154 | 2.6 | 2.1 | 2.1 |  |  | Maybe incompatible with 167 |
| 157+1CTU | 1.1 | 0.9 | 0.8 |  |  |  |

154: Addressed concerns by making the flag that was conditionally decoded

157+154(1):

| 157&154 | 2.6 | 2.1 | 2.1 |  |  | Padding process, |
| --- | --- | --- | --- | --- | --- | --- |

After much discussion. The consensus of the group is to recommend O0122.

### Contributions

JCTVC-O0122 AHG5: Vector prediction for Intra Block Copy [G. Laroche, T. Poirier, C. Gisquet (Canon)]

This contribution presents a vector prediction scheme for an Intra Block Copy method. In the adopted Intra block copy method, the block predictor is signalled with a vector displacement relatively to the position of the current CU. In the current method, the vector is directly coded with the Inter motion vector residual method. This contribution proposes to predict the vector by the last decoded vector of the current CTB. This modification is only a modification of the decoding process and needs to store only one vector as predictor. In average, it was reported that the proposed method gives1.0% in average for lossy configurations. For the lossless configurations, it is reported that the proposed method gives additional gains for sequences where the Intra Block Copy mode is selected.

1 Method: Differentially encode vectors from the most recent encoded vector within the CTU.

NB, intervening non-intrabc CU's within the CTU are ignored.

The predictor is reset to 0 at the start of each CTU.

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| Gains | 1.2 | 1 | 0.9 |

Comment: sparseness of the CTU can lead to large gaps between predictor and CU.

Comment: requires a very small state change, with reasonable results

Comment: Seems sensible, probably adopt something like this after reviewing everything else

- JCTVC-O0326 AhG5: Cross-check of Motion Prediction for Intra Block Copy in JCTVC-O0122 [W.-S. Kim (Qualcomm)] [late]

JCTVC-O0123 AHG5: Vector transformation for Intra Block Copy [C. Gisquet, G. Laroche, P. Onno (Canon)]

During the previous JCTVC meeting in Geneva, several proposals around the Intra Block copy method were made, using various methods to encode the displacement vector. It was however preferred to reuse the MVD entropy coding. In this proposal is presented a simple transformation of the vector components to better suit the properties of the entropy coder. It is reported that the technique yields bitrate savings for lossless coding of SCC classes from 0.1% to 0.6% on AI Main, 0.1% to 0.5% on RA Main, and 0.0% to 0.5% on LDB Main. Gains of up to respectively 2.4%, 2.0% and 2.0% for lossy coding of respectively AI, RA and LDB are also reported.

Proponents analysed the distribution of vectors to determine the offsets (2N).

Comment: should the offset be sequence dependent?

Comment: This method exploits that some block vectors are not legal due to overlaps.  Other methods proposed handle the overlap is proposed in other documents (by filling in the missing areas)

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| Gains | 1.1 | 0.8 | 0.8 |

Lossless results show no change (0.1%).

The contribution also specifies normative restrictions on the vectors.

Question: are there parsing dependencies with x and y?

A: No, they are parsed as normal and then interpreted

NB: a similar method is presented in JCTVC-O0183

JCTVC-O0154 AhG5: Displacement vector signalling for intra block copying [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

Intra Block Copying was adopted as an intra coding tool in the HEVC Range Extensions draft specification. Currently, the displacement vector (DV) is signalled by sharing the motion vector difference coding method in HEVC. In this document, two DV signalling methods are proposed. The first method uses a vector predictor, while the second one does not. Reported experimental results demonstrate that up to 5.4% and 2.6% bit-rate reductions are obtained, respectively.

This proposal describes two methods:

- Prediction occurs only within the CTU. Uses a real BVD using above or left, using a flag to encode direction, and expgolomb codes the magnitude + sign flag + flag to signal if BV is 0 to skip coding of BVD.

(About 1% better than previous, but with parsing dependency)

Requires 15 vectors to be stored for prediction (8vertical + 8horizontal)

- Same as method (1) without the predictor.

(slightly worse than JCTVC-O0122, although with marginally less state)

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| Method (1) | 2.1 | 1.7 | 1.6 |
| Method (2) | 1 | 0.7 | 0.9 |

Comment: would like to avoid the parsing dependency which isn't a trivial check.

Crosschecker JCTVC-O0315:

Comment: (1) flag is only encoded if there are two predictions and if they differ.  Parser needs to perform the comparison operation, complicating the process.  Adds three contexts, although this is not a problem given the gain.

The proponents have provided additional results during the meeting to examine the behaviour of removing the parsing dependency.

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| Modified (1) | 1.9 | 1.7 | 1.5 |

Comment: doesn't see benefit to changing the BVD coding, when other methods are offering nearly as much.  Especially when IntraBC gain is higher.  It would be much better to reuse the existing logic.

JCTVC-O0167 AHG5 : Intra Motion Vector Coding [C. Park, S. Lee, C. Kim (Samsung)]

Current intra motion compensation technique called as intra block copy includes no MV predictor and thus the redundancy among intra MVs has not been exploited yet. In order to exploit the redundancy and improve coding efficiency with no side information, four methods based on structural offset removal and unary sign to dual sign conversion are proposed with and without intra MV information of previously decoded left-side coding unit.

NB, original proposal had four methods, two have been removed.

NB, sign coding is currently using bypasss

Two methods, using the MVD structure as currently exists and is coded.

- Offset the vectors so that the x vector zero is the right hand edge of the block (Similar to JCTVC-O0123)

(slightly worse than 123)

- Adds an offset and then uses sign flag as LSB of the vector.  This has a side-effect of making the CABAC coded gt_1, gt_2 flags affect more vectors.  No change to parsing process.

(better than 123)

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| Method (1) | 0.9 | 0.6 | 0.6 |
| Method (2) | 1.4 | 0.9 | 1.1 |

Comment (2) has slightly more logic than 123.

Comment: Performs as well as the JVCVC-O0122 method using a predictor for BVD, without the need for one, ie no extra state.

JCTVC-O0300 Cross-check of JCTVC-O0167 on AHG5:Intra Motion Vector Coding [S.-H. Kim (Sharp)] [late]

JCTVC-O0244 RExt: Signalling Motion Vector Range for Intra Block Copying [L. Guo, C. Pang, J. Chen, J. Sole, R. Joshi, M. Karczewicz, Y.-K. Wang (Qualcomm)]

This document proposes to add a new VUI syntax element to indicate a restriction on the range of the horizontal component of the motion vector for Intra Block Copying. It is asserted that such signalling can help the codec better allocate memory and optimize pipelining.

IntraBC introduces a need to copy non-deblocked pixels.  It might be possible to optimize a decoder pipeline by knowing the latency required before deblocking can be performed, or reduce the amount of memory required.

Comment: You need to handle the worst case.  Is a full CTU really that large for storage optimization?

Comment: Do people ever fill out VUI? Correctly?

Comment: Could BV length restrictions be put in levels?

JCTVC-O0277 RExt: On Intra Block Copy mode [G. Jin, A. Saxena, F. Fernandes (Samsung)] [late]

Improved Intra Block Copy mode and 2-D Intra Block Copy mode were proposed in JCTVC-N0254 and JCTVC-N0256 respectively. Intra Block Copy mode can significantly improve the coding performance for screen content. For coding the motion vectors in Intra Block copy, Exponential-Golomb code was used. However, Exponential-Golomb code is not optimal for coding such motion vectors. In this contribution, a simplified fixed length code instead of Exponential-Golomb code is first presented. Next, a Huffman code to further improve the performance is also presented. Moreover, one of the possible reasons why in JCTVC-N0256 Exponential-Golomb code achieves a 1% coding gain is also explained. It is asserted that applying the fixed length code in lossless setting leads to average bit-rate savings of 0.3 % for All Intra, 0.2% for Random Access, and 0.1% for Low Delay-B settings for both YUV444 and RGB444 screen content coding sequences. For lossy setting, the gains are 1% gains for All Intra Main-tier, 0.9% for Random Access Main-tier and 1.2% for Low delay Main-tier of YUV444 screen content. For the Huffman coding based technique, the gain for All Intra Main-tier SC YUV444 is 1.5%.

Two Methods to replace the current exp-golomb coding of BV MVD

- Use fixed length code.

- Use huffman code

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| Method (1) | 0.8 | 0.7 | 0.3 |
| Method (2) | 1.1 | 0.9 | 0.9 |

Comment: both methods limit the maximum search range.

Comment: huffman table has 288 entries.  Perhaps not so interesting.

Comment: could also change the current exp-golomb order (other proposals used 4)

Comment: these statistics were gathered using the fast search, would you expect them to be the same with that disabled?

A: they are similar (an internal test was performed on an earlier HM release).

JCTVC-O0305 Crosscheck of JCTVC-O0277 on intra block copy motion vector coding [D.-K. Kwon (TI)] [late] [miss]

JCTVC-O0232 On intra block copying in RExt [J. Xu, A. Tabatabai, O. Nakagami (Sony)]

Intra block copy (IntraBC) is studied from implementation point of view in this contribution. For the clean-up and bug-fix, several modifications are proposed.

- It is proposed to clarify the BvIntra range restriction, especially for the vertical component.

- It is proposed to modify MV coding of IntraBC.

- It is proposed to change the interaction of IntraBC and constrained_intra_pred_flag.

Considering aspect 2:

Three methods, none affect parsing:

- if (X < S), Y += H

- if (Y < S), X += S

- X,Y += S.  This method does not require a condition check.

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| Method (1) | 0.2 | 0.1 | 0.1 |
| Method (2) | 0.7 | 0.6 | 0.6 |
| Method (3) | 0.5 | 0.4 | 0.4 |

Aspects (1) and (3) have been handled during the presentation of other contributions

JCTVC-O0297 Cross-check of JCTVC-O0232 on Intra Block Copying in Rext [L. Guo (Qualcomm)] [late] [miss]

## Composite

JCTVC-O0183 On Intra BC mode [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

This contribution proposes following modifications for Intra BC mode. 1) to use DST for luma 4x4 blocks of Intra BC CU; 2) to use inter-like deblocking for Intra BC CU; 3) constraints of block vectors for Intra BC CU; 4) modifications on block vector coding; 5) constraints when using Intra BC when constraint intra is enabled.

Transform use for 4x4 IntraBC

In the current design, intra blocks use a DST for 4x4 luma.  However, IntraBC blocks which are able to appear in intra slices introduce a requirement for a 4x4 DCT.

This contribution shows that switching to a DST for the 4x4 luma TU of IntraBC blocks (for all slice types) yields a ~0.1% loss.

Comment: while this may make things conceptually simpler, is there any need to actually make a change?

Comment: previously some intra prediction modes used a DST or DCT dependent upon the mode, and we switched everything to DST.  Why wouldn't this be done in this case too?

There was a concern that one would be switching from DST to DCT on a TU basis.  In this case it happens at an 8x8 CU level which is a slightly lower rate.

There was discussion about the consistency of the design, and that there are negligible losses when making such a change.

Recommend: adopt

Use inter deblocking for the intraBC CU.

It is proposed to use the inter deblocking process for intraBC blocks (even in intra slices) since the behaviour of intraBC may be more like inter residual.

Comment: is this going in the opposite direction to the previous comment?

Comment: If the inter deblocking process is used, is the block vector difference being used?

A: Yes.

Comment: has the vector length been adjusted (as BVs are integer)?

Comment: Maybe it would be better just to use the inter filter strength rather than full complexity.

Comment: Do lower bitrates get a lower gain?

A: Unknown

Recommend further study

Constraint of block vectors for Intra BC

The current spec just limits the horizontal vector component magnitude to 0..128 for syntax parsing purposes.  Additional constraints should be defined to implement the intentions of the IntraBC adoption at the last meeting.

The original intent of the adoption at the previous meeting was that the available pixels for an IntraBC PU consist of the current CTU and the CTU to the left.  However, the specification maximum limit is 128 samples.

A discussion ensued on whether the specification should limit the maximum horizontal vector length to 128 or to the extent of the neighbouring CTU.

Comment: Why not have a block size that is limited to the buffer space required for 64x64, ie the horizontal search area increases as the CTU size decreases?

Comment: Having it in CTUs allows simpler pipeline management.  It can also interact with deblocking.

Comment: It rather depends on how memory is used for the reference samples and the architecture design.

It was suggested that a fixed 128 limit might be useful, although it should not increase by a factor of four for each division of the maximum CTU size.

Returning to the contribution, it suggests various constraints on pixels that can be used for intra BC:

- Samples may only come from the current CTU or the left CTU

- Samples may only come from the same slice (ie, the CTU on the left must be in the same slice/tile)

- As yet not reconstructed samples (ie, the reference samples may not overlap the current CU).

Recommend: adoption of bug fixes (1), (2), (3)

An additional constraint was discussed that prohibit the referencing of samples outside of the current picture.  Two obvious alternatives were discussed: to replace unavailable samples with a default value, or to edge extend the picture.  It was remarked that the choice of default value could significantly affect the use of blocks that exceed the picture boundary; since the test material has a significant amount of white or black backgrounds.

Recommend: prohibit the referencing of samples outside the current picture.

Reconstruction order in the TU loop.

The contribution suggests that the current intra behaviour for reconstruction does not apply in the case of IntraBC.  It would be possible to define the prediction and reconstruction process on a per TU basis, this would allow a greater degree of overlap between the prediction area and the current CU area, without resorting to the need for padding.

Comment: It was enquired as to what the minimal useful vector difference would be.  Is there a large benefit to a four column overlap in an 8x8 CU for example.

Recommend further study and coding results of such a method would be highly desirable to evaluate the merits of padding.

BC vectors coding

This proposal discusses as one aspect BV coding with two methods:

- Check MVx or MVy, and add CU sized offset

Similar to 167(1)

0.7	0.5	0.5

- Check MVy and add a fixed offset of 8

Similar to O0232(2), although that uses CU sized offset

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| Method (1) | 0.7 | 0.5 | 0.5 |
| Method (2) |  |  |  |

Intra BC constraints for CIP

- Method 1 is equiv. to O0155 option 3

- Method 2 is equiv. to O0155 option 1

The proponent prefers method 1.

JCTVC-O0306 A cross-check report for JCTVC-O0183 [A. Saxena, E. Alshina, F. Fernandes (Samsung)] [late]

JCTVC-O0323 Crosscheck of JCTVC-O0183 (On IntraBC mode) Section 2.4 about BC vector coding [Z. Ma, J. Ye, H. Yu (Huawei)] [late]

## Uncategorised

JCTVC-O0170 AHG8: Use of inter RDPCM for blocks using intra block copy mode [R. Joshi, C. Pang, L. Guo, J. Sole, M. Karczewicz (Qualcomm)]

The topics covered by this contribution have been covered earlier in notes.

JCTVC-O0325 Cross-check of Use of inter RDPCM for blocks using intra block copy mode (JCTVC-O0170) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-O0205 AHG8: Pseudo-PU-based Intra Block Copy [C.-C. Chen, T.-S. Chang, R.-L. Liao, W.-H. Peng, H.-M. Hang (NCTU/ITRI), C.-L. Lin, F.-D. Jou (ITRI)] [late]

This contribution introduces a line-based intra block copy (IntraBC) mode that splits a CU of size 2Nx2N equally into multiple block partitions, termed as lines, of size Mx2N or 2NxM, where M represents a power-of-two integer and is smaller than 2N. For each line, pixels are predicted in the same way as the original IntraBC except that 1) the block size is of Mx2N or 2NxM and 2) blocks of size Mx2N and 2NxM are restricted to only perform horizontal search and vertical search, respectively. Therefore, there are 2N/M intra MVs per CU needed to be searched and signalled. Experiments are conducted using the HM-12.0+RExt-4.1 software and the RCE3 common test conditions to evaluate three variants (M=1, 4, and N) of this mode in the All-Intra configuration. The search range is limited to one left CTU and part of the current CTU. The average BD-rate reduction provided from the line-based IntraBC mode for screen contents (Class F/SC RGB 444/SC YUV444) is reported as follows:

- M=1: (MT) 3.0/10.6/7.5, (HT) 2.7/10.7/8.3, (SHT) 2.4/10.6/8.9, (Avg. Enc.) 114%, (Avg. Dec.) 97%;

- M=4: (MT) 2.2/7.0/6.8, (HT) 1.9/6.4/6.4, (SHT) 1.5/6.0/6.1, (Avg. Enc.) 113%, (Avg. Dec.) 94%;

- M=N: (MT) 2.5/7.0/6.7, (HT) 2.2/6.4/6.4, (SHT) 1.9/5.9/6.1, (Avg. Enc.) 114%, (Avg. Dec.) 94%.

This proposal introduces two new partition modes for intra BC: Mx2N, 2NxM, and conducts evaluations with M=1, 4, N.  Ie, in extreme when M=1, it permits row or column copying and reconstruction.  This would create cu_size ÷ M prediction partitions.  Each PU is signals a BV without any prediction (ie, as per the current IntraBC design).

Since single column access may be as expensive as retrieving multiple columns for some implementations, a value of M equal to 4 has been proposed as an alternative.  NB, M=4, is actually equal to N for 8x8 CUs.

Mode estimation is still on 16x16 and 8x8 CUs only.

Comment: Does the ability to split a CU into multiple PUs affect the selection of the 16x16 and 8x8 CUs? Ie, in one sense is it acting as a MVD predictor, where a 16x16 CU with a 2NxN partition is more efficient in signalling than four 8x8 CUs?

A: No analysis of CU sizes has been performed.

All three configurations had similar coding times.  This might be somewhat expected as the same number of comparisons are being performed.

Average BD-rate gains of classes F and SCC YUV 444 relative to the current RCE3 conditions are shown below:

|  |  |  |  | AVG(Class F;SC YUV 444) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | AI-MT |  | RA-MT | LD-MT |
| M = 1 |  |  |  | 5.2 |  | 4.3 | 3.2 |
| M = N |  |  |  | 4.6 |  | 3.9 | 3.1 |

Unfortunately results are not cross-checked.  Software is available upon request.

Comment: seems interesting.

Comment: column based prediction would be very tricky.  any value of M<4 would be tricky.

Comment: how does this compare to intra BC 4x4?

A: going to 4x4 adds gains.

Comment: this might integrate well into the specification.  Perhaps more so than the current design.

No text available.

Recommend further study.

JCTVC-O0186 On residual rotation for Inter and Intra BC modes [X. Peng, B. Li, J. Xu (Microsoft)]

This document reports results of disabling residual rotation (applied to transform skipped blocks) in inter CUs and Intra BC CUs and proposes to remove the residual rotation process for non-Intra CUs.

No performance difference is observed for disabling rotation in Inter CUs (some negligible losses (0.1) for both lossy and lossless).

When disabling rotation for both types of blocks, similar results are seen.

NB, considering when inter RDPCM is enabled, RDOQ can't be applied.  When the ordering of rotation is fixed, RDOQ can be applied and benefits are seen.

(The following discussion was chaired by C. Rosewarne)

Comment: can the RDOQ be done in the reverse manner?  If this was done, context modelling couldn't be done.

Comment: But RDOQ does have a complexity increase associated with it.

Comment: separate flag for intra and inter rotation. (in the same way as the explicit and implicit flags)

Comment: extra flags don't simplify anything.

Comment: O0171 option 4 is the same as this proposal, but results slightly different because sw+text harmonisation was included. For LB getting 1.4%loss for the video conferencing sequence.

Comment: Disabling for inter is breaking harmonisation intra+inter on the decoder.

Comment: Inverse rotation to be put before the inverse quant and after entropy decoding.

Comment: For the RDPCM case, the most energy is at the top of the block. After rotation, the energy is moved to the bottom/right of the block, which is less suitable for the entropy coder.

Comment: Experiments on RDPCM and rotation, rotation was causing degradation of the residual coding.

Recommend: Adopt.

JCTVC-O0053 RExt: On transform selection for Intra-BlockCopy blocks [A. Saxena, E. Alshina, F. Fernandes (Samsung)]

See 183 method 2.1 (recommendation as per O0183 method 2.1)

JCTVC-O0315 AhG5: Cross-check for prediction of displacement vector in intra block copying [E. Alshina, A. Alshin] [late]

JCTVC-O0316 AhG5: Cross-check for constrain intra design for Intra block copy [E. Alshina, A. Alshin] [late]

JCTVC-O0351 Masking method [late] [XXX]

This contribution proposes a possibility for “masking” areas in blocks predicted with intra block copy mode and substituting all the sample values in a masked area with a single sample value. It is argued this kind of operation helps coding efficiency by being able to provide finer prediction granularity for intra block copy with no significant complexity implications. It is reported that the proposed approach provides objective bitrate reductions of -1.3 %, -1.4 % and -0.4 % for All Intra SC YUV 444, SC RGB 444 and Class F categories, respectively. In other categories the average of reported bitrate differences is 0.0 %.

Proposing optional masking of up to three row/column samples in a block, on one of the four edges.

Masked samples are discarded and replaced with a value determined from the middle of the inside edge of the masked region from the prediction.

Mask is determined after the block vector has been determined.  All mask positions are then evaluated.

|  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- |
|  | AI-MT | RA-MT | LD-MT |
| Gains | 0.9 | 0.9 | 0.8 |

For chroma, the same mask for luma is used.  For 4:2:0 the mask size is scaled.

About 25% of intra block copy CUs are masked.

This is still CU based.

Comment: what happens if we move to TU based method?  It should still be doable, although does risk the reference sample used for filling not existing.

Comment: how would this interact with the 2NxN?  It may mitigate some of these modes.

This is performed at all CU sizes.  The mask size does not change.

Recommend further study.

Comment: if there is texture, will there be possible blocking issues?  In theory, yes, but in practice possibly not.

# Memory bandwidth minimization (4)

JCTVC-O0088 AhG5: Memory Bandwidth Reduction for HEVC Rext [W.-S. Kim, V. Seregin, J. Sole, M. Karczewicz (Qualcomm)]

HEVC Range Extensions 4:4:4 potential applications include consumer applications, so the complexity increase due to the 4:4:4 processing has to be taken into account. The scope of this contribution is the memory bandwidth increase for 4:4:4 motion estimation and compensation, which is around 37% more than the 4:2:0 case for 4×2 memory blocks. Specifically, in this contribution, restriction of bi-directional prediction for 8×8 prediction unit is studied. This modification increases 4:4:4 bandwidth by 8%, as opposed to the current 37% with respect to 4:2:0. It is reported that the impact on coding performance is 0.8% and 0.6% luma BD-rate loss for RA and LB, respectively, in YUV 4:4:4.

Proposes to restrict bipred on 8x8 CTUs, for a reduction in 20% worst case cf. the current 4:4:4 design with a 1% BD-rate penalty.

Comment: Are we at risk from having 4:4:4 luma perform worse than 4:2:0?

Comment: Should we expect 4:4:4 to have a higher cost burden than 4:2:0?  Currently

Do we currently have a 4:4:4

Memory

Comment: How do you measure the memory bandwidth, is this really the worst case?  This is a theoretical investigation, the same as for 4:2:0.

Comment: which model of memory layout is used?  It was assumed that there were three independent colour planes.

Comment: what is the worst case loss? Given that this is a worst case bandwidth reduction and an average performance penalty?

|  | AVG-MT |  |  | AVG-HT |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD | AI | RA | LD |
| (Class F;SC YUV 4:4:4) |  | 1.4 | 1.1 |  | 1.7 | 1.6 |
| RGB 4:4:4 |  | 2.4 | 2.0 |  | 3.1 | 2.5 |

Comment: these are still average over a sequence, how does this vary on a per frame basis which could significantly affect rate control?

Comment: this is only intended for 4:4:4? But what about having to decode 4:2:2?

JCTVC-O0318 Cross check of Memory Bandwidth Reduction for HEVC RExt (JCTVC-O0088) [G. Laroche (Canon)] [late]

JCTVC-O0221 RExt: On motion compensation memory bandwidth constraint [M. Zhou (Broadcom)]

Motion compensation memory bandwidth has increasingly become a bottleneck in chip designs as video fidelity increase in the HEVC range extension. It is proposed to reduce the worst case luma reference block size from the current 28800 to 22080 per 64x64 region in Main tier HEVC range extension profiles. The constraint is expressed in terms of the total luma reference block size per picture, with the luma reference block size for an intra prediction unit defined as 0, and for an inter prediction unit of size nPbW x nPbH defined as (nPbW + 7)*(nPbH + 7) if it is a uni-directional prediction unit, and as 2*(nPbW + 7)*(nPbH + 7) if it is a bi-directional prediction unit. Experimental results show that the proposed constraint can reduce the worst case memory bandwidth by about 23% without causing quality loss.

Advocates against disabling 8x8 bi-prediction, due to compatibility with v1, and performance reasons.

Proposes to prohibit evil bitstreams where all 8x8 Inter CUs are bi-predicted.

The constraint is defined by counting the area of reference samples used for luma motion compensation within a picture.  A limit based on picture area in CTUs is multiplied by a magic number.  The magic number has been chosen such that there is a limit of 80% of the CUs are 8x8 bipred PUs.

An example encoder method is presented that examines maintains a running cost and disables certain prediction modes if the cost increases too quickly.

On common conditions there is no reported loss (except for a couple of sequences).

Some sequences (1 at MT, and 4 at HT, for both LB and RA configurations) violate the constraint.  For example, the sequence EBUKidsSocker (4:2:2) at QP 22 in the common conditions exceeded this limit 25% of the time.

Comment: Should the limit scale with chroma format?

Comment: As currently described, would a 4:2:0 bitstream be able to violate the constraint due to the counting process being performed on Luma samples.  A: Yes, this needs to be modified to calculate taking Chroma into account.

Comment: This sequence seems to be, at QP 22, using 40Mbit/sec, which is very high for a consumer application.

This proposal only suggests applying such a limit to the main tier and not the high tier.

Comment: Is the unevenness of having a long run of peak memory bandwidth okay?  As this won't happen at a CTU level.  Should this be per CTU row?

A: This is a problem.

Comment: We have previously avoided techniques such as this in HEVC (such as the MV limit per macroblock pair of AVC).  Is this going to cause encoder grief?

Comment: what happens to tiles and slices? how does this apply to tiles?  A: this applies to a picture, it is just the same as sorting out rate control if tiles are encoded in parallel.

Comment: for hardware encoders (for both methods) it may be problematic for 4:2:0 to behave differently from 4:4:4 due to mode choices being made based on luma.

Comment: Cross-fades need bi-prediction, could this be a problem for it?  But all the blocks may not all be 8x8.

Comment: does this unduly affect lower resolution videos?  Should it be level dependent instead?

JCTVC-O0309 Cross-check report of RExt: On motion compensation memory bandwidth constraint (JCTVC-O0221) [O. Nakagami, T. Suzuki (Sony)] [late]

Cross-checker has some concerns:

- This is per frame, which can cause problems for an encoder

- Worst case reduction is more useful for decoders than average reduction

- The restriction is very easy to violate.  A lazy encoder can easily cause a problem.

# RCE1 related (inter-component decorrelation) (4)

JCTVC-O0150 Non-RCE1: Extended Adaptive Inter-Component Prediction [A. Khairat, T. Nguyen, M. Siekmann, D. Marpe (Fraunhofer HHI)]

This document presents an extension of the inter-component prediction (ICP) scheme in RCE1. In the proposed extension, the second chroma residual block can be predicted either from the corresponding luma or the corresponding first chroma residual block. For that purpose, a flag is transmitted in the bit stream indicating whether the corresponding luma or the first chroma residual block is employed for ICP. Experimental results show an improvement in terms of BD-rate for Y'CbCr sequences relative to a version without ICP of about to −1.8% on average for the common test conditions all-intra configuration, -0.7% for the random access configuration, and -0.3% for the low delay configuration.

The newly adopted adaptive inter-component prediction method (from RCE1) uses luma to predict both chroma components.  This proposal suggests using luma or the first chroma component as the predictor of the second component.

This yields a 0.4, 0.2 and 0.1 % BD-Rate improvement for luma with a 10% runtime increase.  The presenter asserts that the software is not well optimized.  However, RGB loses 0.2, 0.9 and 1.1 relative to the RCE1 experiment on which this is based.

The cross-checker reported that, based on partial results, the runtime increase for intra is 20% and 10% for intra and low-delay B respectively.

This should be enabled on a PPS basis, so assuming a correctly setup encoder this loss can be avoided.

Comment: did you try other colour spaces?  No.

Comment: Does this cause a problem for decoders?

It should be the same incremental cost as the RCE1 approach.

It was noted that the main benefit is found (when examining the results relative to the RCE1 adoption) in the VenueVue sequence which seems to somewhat skew the results

Comment: have you tried a fast mode estimator for this?  Perhaps there are some better tricks than a full R-D decision.

Comment: how about bringing it back with RCE3 results (particularly looking at the animation category), perhaps also with a possible fast mode estimator.

JCTVC-O0320 Cross checking of Non-RCE1: Extended Adaptive Inter-Component Prediction [W. Pu (Qualcomm)] [late]

Only partial results are available.  A code review and partial results match.

JCTVC-O0263 Non-RCE1: Inter colour-component residual coefficients prediction [K. Kawamura, S. Naito (KDDI)]

This contribution proposes the inter colour-component residual coefficient prediction for HEVC Range Extensions. In the range extensions’ core experiments 1 (RCE1), inter-colour component residual prediction is studied. This proposal presents an improvement of RCE1 Experiment 2. In the proposed method, inter component prediction is performed before inverse transforms. Compared to Experiment 2, dependent point in a decoding pipeline becomes forward. No syntax change is required to the Experiment 2. For common test conditions of RGB sequences, the average BD-rates of the method are 16.5%, 13.0%, and 12.3% for AI, RA, and LDB main tiers, respectively. For YUV sequences, the average BD-rates are 1.2%, 0.4%, and 0.2% for AI, RA, LDB main tiers, respectively.

This contribution suggests to move the prediction stage in the adaptive inter-component prediction (RCE1) process to occur prior to inverse transform (in the transform domain) rather than after inverse transform (in the spatial domain).  This is purportedly to increase parallelism, and yields a 1.3, 0.8 and 0.2% (AI/RA/LB MT) loss relative to the adopted RCE1 (experiment 2) method.

Comment: Have difference in Luma and Chroma QPs been taken into account?  This is mitigated to some extend by occurring after inverse transform.

There are two limitations with such an approach, the mismatch in transform type for 4x4 intra DCT vs DST for luma vs chroma, and transform skipping is independent of colour component.

Various options are presented to address these complications:

- Encoder can choose not to use the ILCP mode when this happens.

- Remove ILCP for 4x4 Intra TUs.

- Use a DST for 4x4 Intra chroma.

A test was performed to examine various combinations of these solutions.  It was found that option (3) is a little worse than the anchor, and both (2) and (1) are much worse.  The contributor recommends the least performing method, (1), since it has no syntax changes.  It was noted that while a DST to fix the problematic cases performs marginally worse than the anchor, if the anchor is modified to also use a DST, the performance was approximately the same.

Comment: What happens when the bitdepths are different between luma and chroma?  This shouldn't be affected

The contributor remarked that it was very awkward to implement this method in the specification.

Comment: In an encoder transform and quantisation are often very well connected, integrating this process may be difficult at that point.  In addition there is a larger storage requirement to store the inverse quantized residual, although it was commented that the worst case storage requirements are likely to be no worse due to transquant bypass.

Comment: The adopted design has been around for a while and has now been adopted, is this an actual problem that needs to be solved?

Comment: on RCE2 sequences (RGB) this method does not perform well.

JCTVC-O0319 Cross checking of Non-RCE1: Inter colour-component residual coefficients prediction [W. Pu (Qualcomm)] [late]

# RCE2 related (transform coefficient coding) (4)

JCTVC-O0129 Non-RCE2 and AHG18: Increase in the maximum value of Rice parameter for high bit-depth support [S. Lee, J. Min, E. Alshina, C. Kim (Samsung)]

The topics in this contribution have already been covered during the discussion of RCE2.

JCTVC-O0312 Cross-check of JCTVC-O0129 [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-O0327 Non-RCE2: Rice parameter initialization for higher bit depth coding [S.-H. Kim, M. Kiran (Sharp)] [late]

In this contribution, a simple Rice parameter initialization method based on bit depth information is introduced for higher bit depth coding. In addition, maximum rice parameter is extended up to 16. The coding gain of the proposed method under AHG18 test condition is -6.8% for 12-bit, -18.4% for 14-bit, and -28.2% for 16-bit internal bit-depth, respectively.

Currently a golomb-rice code is used to code coeff_abs_level_remaining using a rice parameter.  This parameter is set to zero at the start of each TU.

Previous methods have:

- Use a constant value

- Explicitly signal the value, requiring a syntax change

- Initialise based on the previous coded value, requires state to be kept

This contribution proposes two methods to initialise the parameter:

- as a function of bitdepth

  - if transform is skipped: BD−8

  - otherwise, (BD−8)>>1

- an extension of the first method for when extended precision processing is enabled, a more complex function based upon QP is used.

RCE2 adopted method (A1) that initialised the value to zero for each TU, and the parameter value is inherited between 4x4 coefficient groups.

This contribution proposes a purportedly simpler method that initialises to a constant value for each subblock instead and remove the inheritance.

From RCE2: Method D1 was ~3% higher performing than A1.  This method is 0.3% worse than D1.

Comment: there should be some caution interpreting these results, since the SVT material is quite noisy.

History on D1: D1 came from a merger of C1 and C2.  Using a simple system using QP/-6, using QP instead of bitdepth -- due to the IBDI like case.

Comment: there is little variation in the SVT data, and doesn't need to have any local adaptation.

Comment: this data is perhaps mostly transform skip,

See what happens with TS disabled, high precision transform, and also when coding with IBDI and using high QPs.

D1 was not accepted due to confusion about various settings and is proposed to be reexamined in a CE, which should examine a much larger set of input data and configurations.

Comment: We tried using a magic constant dependent on bitdepth, which performed well on some sequences.

Comment: when A1 was considered, scope was left for small changes on that method.

Possible CE.

Possible BoG on CE to discuss the details of testing.

- JCTVC-O0344 Non-RCE2: Cross-Check of JCTVC-O0327 Rice parameter initialization for higher bit depth coding [L. Guo, R. Joshi (Qualcomm)] [late] [miss]

# RCE3 related (intra prediction methods) (8)

JCTVC-O0181 Non-RCE3: Implicit derivation for adaptively turning filtering off in intra prediction [J. Kang, R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

This contribution proposes implicit derivation methods for adaptively turning off a set of filters used for intra prediction. These include the interpolation filter, intra smoothing filters and the horizontal and vertical gradient filters. In the proposed schemes, the decision is made based solely on the transform_skip_flag for a TU or in conjunction with the reference samples used for intra prediction. Simulation results reportedly show that the proposed scheme achieves coding gains in YUV and RGB screen content sequence classes and incurs no loss in natural videos and RExt sequences.

Two methods are presented. (RCE A.3)

- .Transform skip flag and a filtering operation of the reference samples (complexity comparable to MDIS, same filter, if any filtered result is greater than a constant threshold).  This test is performed on a TU level.

(Lossless) NB, the RGB performance was better by ~0.5%

- Disabling all filters for intra for transform skipped blocks

|  | AVG(Class F;SC YUV 444) |  |  | AVG(Class F;SC YUV 444) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI-LL | RA-LL | LD-LL | AI-MT | RA-MT | LD-MT |
| Method (1) | 0.3 | 0.2 | 0.1 | 0.8 | 0.7 | 0.5 |
| Method (2) |  |  |  | 0.9 | 0.6 | 0.6 |

Comment: what happens if this algorithm is applied to lossless?  A: There would be some loss, perhaps 0.5–0.6%

Comment: for an encoder this can be nasty, as an encoder may have to generate multiple predictions.  One can typically pre-generate the predictions.  Is this something that needs to be performed on a localised basis?

Comment: does this work in any framework other than full RD optimisation?

Comment: Is (2) worse for an encoder?  One typically finds a prediction first and then works out later if transform skip should be used.  This requires a re-computation if transform skip would be chosen.

Comment: for chroma, the two chroma components are independent.  Which implies that one component could be filtered while the other is not.  Which is a bit of a problem if the chroma data is interleaved.

Comment: why not insert a CU level flag to control the filtering and leave it as an encoder problem?  Remember that this was done for transquant bypass in version 1, rather than using qp=0.

JCTVC-O0292 Non-RCE3: Cross-check of JCTVC-O0181 on Implicit derivation for adaptively turning filtering off in intra prediction, Method 2 [P.Lai, S. Liu (Mediatek)] [late] [miss]

JCTVC-O0308 A cross-check report for JCTVC-O0181 [A. Saxena, F. Fernandes (Samsung)] [late]

JCTVC-O0147 RExt: Proposed changes in the horizontal and vertical gradient filtering control for intra prediction [M. Zhou (Broadcom)]

In the current HEVC range extension design, the horizontal and vertical gradient filtering in intra prediction is conditioned on the cu_transquant_bypass_flag while it is unconditionally turned on in the HEVC version 1 design. This leads to a backward comptibility issue beween the HEVC range extension and the HEVC version 1 profiles. To reslove the problem, it is proposed to condition the intra horizontal and vertical gradient filtering on intra_smoothing_disabled_flag in the HEVC range extension. It is also recommended to rename intra_smoothing_disabled_flag to more properly refect the filtering process controlled by this flag after the proposed changes.

Currently the spec contains [8.4.4.2.6] (NB, for H and V):

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0, nTbS is less than 32 and cu_transquant_bypass_flag is equal to 0, the following filtering applies with x = 0, y = 0..nTbS − 1:

However this diverges with the version 1.  It needs some form of guard to enable compatibility with version 1.  This contribution suggests to change the semantics of the intra_smoothing_disabled_flag to enable/disable this filtering process as well.

The HM currently enables this behaviour using the intra_rdpcm_enable_flag.  Alternatively an additional control flag could be added at the PPS level.

The opinion in the room was to follow the software rather than add a new control flag, as the disabling of the filtering process was added due to SDIP, which is equivalent to Intra RDPCM.

Recommend: adopt

JCTVC-O0087 Non-RCE3: Unified lossless residual coding [Y. H. Tan, C. Yeo (I2R)]

This contribution studies a residue coding scheme that is applied uniformly to all residues, independent of coding mode (inter/intra) or colour components. In the proposed scheme, a flag is used to signal whether DPCM is used at the residue coding stage; if so, another flag is used to signal if the prediction used in DPCM is in the horizontal or vertical direction. The proposed scheme extends the coding scheme for inter DPCM to intra coding, unifying lossless residue coding for inter and intra coding. It is asserted that, with the residue prediction scheme used for intra coding, the proposed method brings average gains of 2%, 0.7% and 0.5% for All-Intra, Random-Access and Low-DelayB lossless test configurations.

Proposes: to allow directional rdpcm for intra, that is chained with the current intra rdpcm.  Ie, at the decoder, first undo the optional directional rdpcm, then undo the implicit intra rdpcm, then carry on.

Minor losses in lossy classF at higher tiers, but minor gains in some screen content classes ~0.2%

Comment: if intra prediction directional is diagonal, this directional rpdcm is still optional.

Signalled on a TU basis

Comment: how does this perform against the previous proposal?  Eg, class F 0.6 -> 2%

Comment: encoding times for the lossless case seem to be 10% more?  The cross-checkers were getting a lower runtime.

Comment: is there an interaction with the search process?  There is currently a defficiency in the search.  RDPCM is taken into account during the prediction process, but inter RDPCM doesn't.  O0178 is getting lower gains than presented here, but is that an effect of the search?

Comment: are there statistics for the rdpcm direction usage?

- JCTVC-O0339 Crosscheck result of JCTVC-O0087 Non-RCE3: Unified lossless residual coding [J. Kang] [late]

JCTVC-O0178 Explicit signalling for intra RDPCM [J. Kang, R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

This contribution proposes explicit signalling of intra RDPCM to align it with inter RDPCM. The existing inter RDPCM signalling scheme to signal the RDPCM mode ((DPCM_OFF, DPCM_VER, and DPCM_HOR) is reused. Simulation results are provided that reportedly show that the proposed scheme achieves coding gains in all configurations for Class F, SC YUV, and SC RGB test sequences without incurring any loss for natural videos and RExt video sequences.

Last time there was a proposal (N0100) to use explicit rdpcm for intra in lossless conditions.  However it was showing some losses in one category.

This contribution proposes to replace the current intra rdpcm with an explicit rpdcm for all intra blocks, ie all rdpcm modes become explicit.  Separate contexts used are used for Intra and IntraBC|Inter.

|  | Lossless |  |  |  | MT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LL-AI | LL-RA | LL-LB |  | MT-AI | MT-RA | MT-LB |
| Class F |  | 0.6 | 0.3 | 0.3 |  |  |  |  |
| Class B |  | 1.4 | 0.2 | 0.2 |  |  |  |  |
| (RGB 4:4:4) SCC |  | 1.1 | 0.3 | 0.1 |  |  |  |  |
| (RGB 4:4:4) Anim |  | 1.6 | 0.1 | 0 |  |  |  |  |
| (YCbCr 4:4:4) SCC |  | 0.6 | 0.2 | 0 |  |  |  |  |
| (YCbCr 4:4:4) Anim |  | 1.1 | 0.3 | -0.1 |  |  |  |  |
| RExt |  | XXX | 0.4 | 0.2 |  |  |  |  |
| AVG(Class F;SC YUV 444) |  |  |  |  |  | 0.8 | 0.5 | 0.2 |

A graph (derived from decoder statistics) showing the use of DPCM mode versus intra prediction angle shows that there is a range of angles that benefit from rdpcm.

Comment: this is different from the current design, and requires more intra searches.

If the current implicit intra derivation method is still used, ie not increasing encoder complexity, although using a slightly wider set of angles, there are losses compared to the current design:

0.7	0.7	0.6	(AI mt/ht/sht)

Using the current implicit angles a ~0.2 additional loss would be expected.

Unsure if this only applies for the directional modes.

Comment: decode side this doesn't have any significant implication, but the encoder could be more difficult.

Comment: this has lower gain than the previous proposal in lossless, but slightly better for lossy.

Question: should the range of implicit intra modes be extended?  There were gains previously.

Interest was expressed in this option.  However, updated results would be needed.

Comment (XC): during the RD search, the encoder does not include the transform skip or rdpcm when doing the preselection of angles (See slides of O0066).

JCTVC-O0317 Cross check for JCTVC-O0178: Explicit signalling for intra RDPCM [M. Naccari, M. Mrak (BBC)] [late]

# Alpha channel (1)

JCTVC-O0132 AHG5/AHG9: On support for alpha channel in HEVC [M. Naccari, M. Mrak (BBC)]

Alpha channel signals are used in professional (studio) video coding applications and their usage may become popular also for frame composition at the receiver side. Briefly, the alpha channel complements the information contained in the main bitstream by providing, for each image pixel, its degree of transparency. At the 13th meeting in Incheon it was decided that alpha channel will be supported in the Range Extensions of HEVC using auxiliary pictures. Moreover, at the same meeting it was also decided that the depth information used in 3D video coding will be also transmitted using the auxiliary pictures. At the 14th meeting in Vienna, contribution JCTVC-N0063/JCTVC-E0049 proposed a general framework to transmit auxiliary data as an additional scalability dimension. This contribution builds on the framework described in JCTVC-N0063/JCTVC-E0049 and proposes high level syntax changes to support functionalities relative to the use of alpha channel in frame composition. The parameters and data required for alpha channel manipulation are provided using a SEI message.

An SEI message that provides the alpha_opaque_value and alpha_transparent_value from AVC.  In addition it provides a thresholding option for conversion of the alpha channel to a bitmap.

Comment: Could we have a method of profiling auxiliary pictures?  It would provide some means of providing a guarantee on what is required to decode them.

Comment: Is thresholding particularly important?  Have experiments been performed on these degradations?

Yes, using an SD resolution signer (for overlay on HD at a lower resolution) some alpha channels using a "binary mask" were encoded as monochrome using the RExt software, and it was found to cause a low-pass filtering effect on sharp edges.

Does that affect composition?  Yes, there are some pixels that get misclassified, and the chroma-key background could be observed.  What was the alpha channel rate?

Comment: could a mid-grey value be used instead?

Comment: what is the range of the threshold.

Comment: the SD overlay would be scaled, so the alpha channel is going to get filtered.

Comment: if up-conversion of the alpha channel is being performed, should the filter be specified?

Comment: If the issue is that the chroma-key background was green and some pixels were showing, shouldn't it be rekeyed with a more suitable background?

Revisit

# Lossless and screen content coding related contributions (10)

JCTVC-O0070 Fixed MPM set in intra mode coding for screen contents [X. Xu, S. Liu (MediaTek), J. Min, S. Lee, E. Alshina, C. Kim (Samsung)]

This contribution is a follow-up document of JCTVC-N0231. It presents an intra mode coding method for screen content. 3 MPMs (Most Probable Mode) are set fixed without referring intra modes of neighbouring prediction units. 3 variations of such fixed MPM strategy are described in this document. The proposed method provides performance gains of 0.3%~0.5% for both screen content sequences with RGB and YUV formats for AI lossless testing conditions. For AI lossy testing conditions, gains of 0.8 ~1.0 % are observed for screen content sequences with RGB and YUV formats. Additional flag in SPS can be employed to indicate whether proposed derivation method will be used or not.

N0231 used a fixed MPM for intra mode coding, one of horizontal, vertical or diagonal are selected.

Two methods are proposed, both include an SPS flag to enable the feature:

- horizontal / vertical / 18 [N0231]

- horizontal / vertical / DC

- horizontal / vertical / planar

For lossless test conditions, 0.3–0.5% saving

Lossy, AI: 0.5% / 0.6% / 0.6% (1,2,3) gain on avg of Class F and SC YUV 444.  At high tier the results diminish by ~0.2% and a further ~0.1% for SHT.

However, the performance on natural video and animation is nearly a 1% loss, although V3 > V2 > V1, hence the recommendation for a flag.

Comment: this puts a very strong bias to these modes in mode decision, have the pictures been examined?

Comment: how should an encoder handle mixed content?

Comment: generally it is nicer if an encoder doesn't have to determine if it is encoding screen content or not.  Having tools that perform well on one content type but are detrimental to another, implies a larger burden to an encoder to correctly choose the mode of operation depending upon the content type.  For the other SCC-type tools, they are relatively benign in offering zero performance improvement.

JCTVC-O0304 Crosscheck of JCTVC-O0070 on fixed MPM set in intra mode coding for screen contents [D.-K. Kwon (TI)] [late]

JCTVC-O0083 HW complexity analysis on screen content coding tools [T. Suzuki (Sony)] [late]

TBP

JCTVC-O0085 AHG5/AHG8: Improvement of MV-HEVC-based RGB coding for screen contents [A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami (Mitsubishi)]

In the last meeting, a picture-based RGB coding scheme using MV-HEVC was proposed. This scheme encodes each colour plane as 4:0:0 format view video and employs inter-view prediction to exploit inter-colour redundancy. This contribution proposes a modification scheme of MV-HEVC-based RGB coding. The proposed scheme introduces intra block copying instead of motion search based inter-colour prediction. The inter-colour prediction of the proposed scheme only permits to refer a collocated block in the reference colour plane. Coding performance of the proposed scheme has been evaluated on top of HTM8.0 under AHG8 recommendation configurations. It has been confirmed that the proposed scheme achieves 18.0%-20.4%, 14.9%-17.2% and 11.4%-14.7% average BD-rate improvement relative to MV-HEVC (HTM8.0) for AI, RA and LDB configurations, respectively.

This proposal suggests to extend a capability of MV-HEVC to the HEVC base codec for RGB coding.  Where a block from one component may use as a prediction of the reconstruction of a block from another component.

All components predict from G

In MV-HEVC all components have been configured to use the same QP.

Comment: inter-plane decoding dependency was previously adopted in as the outcome of an RCE at this meeting

Comment: this is essentially the same thing as LM-Chroma, but without the parameter derivation process.

Comment: these results are for screen content only.  and the results

Comment: coding three components independently does change the RDO search in the encoder.

Comment: The aim of this contribution seems to be unclear, it is either suggesting to implement an inter-component prediction scheme in the current RExt design, or is it suggesting to remove coding tools from RExt

Comment: is 8bit RGB sufficient? As this is what MV-HEVC provides

Comment: previous contributions showed large losses for natural video content encoded in RGB, is this still the case?

Comment: should MV-HEVC

JCTVC-O0357 Screen content coding using 2-D dictionary mode [W. Zhu (BJUT), J. Xu (Microsoft), W. Ding (BJUT)] [late]

[This document is an informative document]

This contribution presents a 2-D dictionary coding mode for coding screen content, which is based on the intra coding framework of HEVC. The proposed scheme first hashes each intra 8x8 coding unit (CU), then find a matching block based on the generated hashing value in the 2-D dictionary. Experimental results show that for SCC YUV 444, on average 23.8%, 23.2% and 22.2% bit-saving can be achieved for AI-MT, AI-HT and AI-SHT. The numbers are 21.6% and 21.1% for RA-MT and RA-HMT; 20.1% and 20.2% for LB-MT and LB-HMT. More gains can be obtained when both 2-D and 1-D dictionary methods are used. For example, the bit-saving numbers for SCC YUV 444 are 29.2%, 27.2% and 24.5% for AI-SHT, RA-HMT and LB-HMT.

Due to the 2D nature of video, the previously proposed 1D dictionary encoder that converts a 2D area to a 1D string by vertical scanning, the ..... This proposal suggests a 2D dictionary method instead.

Unlike the previous 1D method, where if a substring could not be matched, a replacement would be explicitly coded.  This method offers no such alternative.

This method only looks for a single CU sized match in the dictionary.  No sub area calculation is performed, using the whole picture as a dictionary.  The dictionary is reset at each picture boundary.

For some blocks, instead of coding a distance into the dictionary, a hash is used instead, implying that the decoder must also implement the hash method rather than it just being a simple substring lookup.

The dictionary is dynamically updated, after each CU is reconstructed prior to deblocking.  The hash is kept for each 8x8 sub-region of all pixel positions in the dictionary buffer (which is the full picture).

Comment: that is very large.  Especially as deblocked samples are used.

Comment: previously the dictionary seemed to be intolerant to noise due to the way the hashing at the encoder is performed for the search process.  The same method is used in this contribution.

Using the RCE3 anchor which has intra block copy enabled additional gains may be found.

|  | AI-MT | RA-MT | LD-MT |
| --- | --- | --- | --- |
| AVG(Class F;SC YUV 444) | 14.1 | 12.5 | 11.4 |
| RGB | 22.5 | 20.6 | 20.1 |

When the 2D and 1D dictionaries are combined, further gains are found in the SCC YUV and RGB classes of ~3–5%.  Even further gains are found in the optional sequences.

Comment: In the optional sequences, does the encoder actually select the 2D mode?

Comment: It would be good to see different dictionary sizes being demonstrated.

Comment: the current whole picture dictionary will interact badly with slices and tiles.

The proponent believes this should be investigated in a CE.

Comment: how does this integrate with the palette? A. not tested

Comment: if there was going to be a CE that looks at combinations, maybe this could be included.

Comment: this could be included of a study into the trade-offs of the intrabc search range.

Comment: do you see any major difference for the lossless mode between this method and the previous 1D method.

Comment: there is no cross-check.

Comment: what is the hashing burden for a real encoder?  This might happen earlier in the mode decision process to allow an early skip on motion search.

Comment: How big is the hash table?  It has the same number of entries as the picture size.  These tests using a 12-bit hash (generated from CIC).

Comment: perhaps Intra BC can be used for left CTU and this for all other areas (with reference in external RAM)?

Comment: however, that would double the write bandwidth and add an extra set of reads.

Comment: could the dictionary be preserved between pictures?

Comment: this could also be investigated in an AHG that also studies the software.

## Palettes

### Discussion

Comment: does this take away anything from intra block copy?

In RCE1 inter-component de-correlation has been adopted.  Will that interact with these palette methods.

The gain of these techniques are good, they are greater than the modification to intra block copy vector coding adopted at this meeting.

It was suggested that a CE would allow the committee to fully evaluate the methods presented here, in the context of the current codec design (taking into account the current adoptions).

A CE should also study any overlap with intra BC

Comment: from a previous CE study (prior to the adoption of methods such as rdpcm), Palette coding gains were similar to Intra BC for YCbCr, .

Comment: a CE should also investigate if the modifications to parsing which now explicitly integrates copying directions (in the case of O0182) which were previously predicted in the reconstruction phase.

Comment: the range of test sequences has expanded since the previous meeting, and more gains are showed now that there is more content.

Comment: Given the method of palette quantisation different visual distortion may be introduced.  Has this been checked subjectively?

Comment: As an aside, when looking at spatial quantisation of transform skip for block sizes larger than 4x4, care needs to be taken to avoid visual artefacts.

Recommend: a CE to study the palette coding.

### Contributions

JCTVC-O0182 AHG8: Major-colour-based screen content coding [X. Guo, B. Li, J. Xu, Y. Lu, S. Li (Microsoft)]

This contribution proposes a major-colour-based method for screen content coding, which has similar concept with previously investigated palette-based methods. In proposed method, new colour classification and colour index coding are used to keep the complexity comparable with the current HEVC for both encoder and decoder. Experiments show that, compared with HM-12.0+RExt-4.1 anchor, 9.5%, 12% and 13.7% bit-rate saving is observed on SC YUV 444 sequences for AI-Main, AI-High and AI-Super-High, respectively.

Histogram of colours in a block to generate a palette of four major colours (other colours may be encoded using an escape method).  Entropy coding of the palette values by various methods: (a) a single value per line, (b) copy from the above line, (c) copy from above or left or send new value.  The index map coding is shared between all three components, but each component has its own codebook.

The 4:4:4 and 4:2:0 processing is different.  In 4:4:4 the "prediction"/copy mode is shared between all three components, however. in 4:2:0 and 4:2:2 there are two sets, one for luma and one for the pair of chroma components.

Comment: Regarding the discussion on quantization, an encoder is free to not perform quantization of the source block to major colours.  Additionally, any sample values that are not in the palette may be escape coded after being quantized with the current QP.

Three methods were implemented and tested using the RCE3 test conditions (any gains reported are above those of intra block copy):

- Major colours, no escape coding and tested for all CU sizes.

Minor losses (~0.1%) observed on natural video (and CG animated)

Runtime 1--2% increase (encoder)

- Same as method (1), except: not performed on 8x8 and the per-pixel left-copy has been removed

Performs about 1–3% worse than (1)

No increase in encoder times

- No prediction/copy mechanism, just uses normal coefficient coding.  Uses all CU sizes and allows RQT with a forced transform skip (although the flag is not removed).  Palette is still sent once per CU.

Runtime: 2--4% increase encoding time

Results:

|  | Lossless | Lossy MT |  |  |
| --- | --- | --- | --- | --- |
|  | AI-LL | AI-MT | RA-MT | LD-MT |
| Method 1: AVG(Class F;SC YUV 444) | 6.4 | 6.4 | 5.3 | 3.7 |
| Method 1: RGB SCC 4:4:4 | 21 | 18.3 | 16.1 | 14.0 |
| Method 2: AVG(Class F;SC YUV 444) | 5.15 | 5.1 | 4.2 | 3.0 |
| Method 2: RGB SCC 4:4:4 | 18 | 15.0 | 13.4 | 11.4 |
| Method 3: AVG(Class F;SC YUV 444) | 1.6 | 2.1 | 1.3 | 1 |
| Method 3: RGB SCC 4:4:4 | 8.7 | 3.5 | 3.4 | 2.4 |

JCTVC-O0302 Crosscheck of JCTVC-O0182 [S.-T. Hsiang (MediaTek)] [late]

JCTVC-O0218 Evaluation of Palette Mode Coding on HM-12.0+RExt-4.1 [L. Guo, M. Karczewicz, J. Sole, R. Joshi (Qualcomm)]

In the 14th JCTVC meeting, contribution JCTVC-N0249 proposed a palette-based coding for screen contents. In this contribution, the method described in N0249 has been implemented on top of the latest Range Extension software HM-12.0+RExt-4.1 and the performance has been provided.

This method of palette coding consists of a single palette containing a maximum of three triplets

Uses a CU based mode, with minimum operation size of 8x8.  Entropy coding always uses a horizontal scan.  Individual indices are coded using run-length coding, where a run of coefficients either has a single index value or is copied from the previous row.

This coding mode uses the palette as a prediction of the current block, allowing a residual to be transmitted.

The residual coding process is the current HEVC design, allowing the residual to be coded using either a transform or transform skip using the normal RQT decomposition and encoder decision process.

Comment: this involves two block sized parsing operations.  However, the number of bins may stay the same as a better prediction reduces the residual energy.

Comment: what is the gain from coding the residual as opposed to skipping them?

A: both were tested, the presented method was found to be better, but the difference was minor

Comment: How to handle 4:2:0.

The handling of chroma in non-4:4:4 formats is via direct subsampling of the index map.

Results:

|  | Lossless |  |  | MT |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI-LL | RA-LL | LD-LL | AI-MT | RA-MT | LD-MT |
| Class F;SC YUV 444 | 5.8 | 4.8 | 3.8 | 2.6 | 2.3 | 1.8 |
| RGB | 17.4 | 12.7 | 10.3 | 13.8 | 13.2 | 10.3 |

In the lossless configurations, the class F performance is ~0%

In the lossy two-class average resules, class F AI: 0.2 gain becomes in LB, 0.2 loss)

Gains seem to be higher at higher tiers.  It was also observed that the performance of class F is

Comment: Palette formation is conducted using a histogram and populating the palette with the most common tripplets.  In the lossy case the source data is quantized prior to forming the histogram.

The runtimes show an increase in runtime of ~5–15% (AI) (except for the AI lossless case, which may be due to cluster issues).

JCTVC-O0299 Cross-check of JCTVC-O0218 [J. Xu (Sony)] [late]

## Syntax optimizations

JCTVC-O0228 AHG9: Signalling lossless slices [S.-T. Hsiang, Y.-W. Huang, S. Lei (MediaTek)]

This contribution proposes the high level syntax modifications to support signalling a lossless representation for an entire slice. As a result, the use of losslessly coding a whole slice can be compactly signaled by a syntax flag. The irrelevant syntax information such as the SAO parameters, quantization parameters, and deblocking filter parameters can be explicitly removed from the bitstreams. It further enables us to exclude any tools and resources that are only utilized in the lossy coding mode when it is indicated the entire slice is losslessly coded. For example, we can reduce the number of the modeling contexts for entropy coding syntax significant_coeff_flag from 44 to 2 for losslessly coding a slice, leading to potential power consumption savings.

This contribution suggests two syntax optimizations:

To provide a flag in the slice header to signal that all CUs use transquant bypass.  This allows SAO syntax to be skipped per CTU and to infer the cu_transquant_bypass_flag.  Additionally, any slice parameters relating to deblocking and QP offset are skipped.  Furthermore, if delta qp is enabled, the syntax elements in the TU are skipped.

Bitrate savings for lossless encoding are negligible.  This contribution is presented as some form of simplification.

Comment: Currently SAO can be disabled in the slice header.

Comment: Does the lossless case really need these?

Comment: What flexibility does having this at the slice level actually provide over what is already present?

Comment: This doesn't really simplify the design as it introduces extra conditions, that don't really seem to bring general benefit.

# Other (7)

JCTVC-O0238 Sign coding for transform skipped blocks [J. Wang, D. He, N. Hu, D. Flynn (BlackBerry)]

The HEVC video codec provides a mechanism for transform skipping. It is observed that in the case of transform skipping, the assumption that the probability of a quantized coefficient’s sign being positive or negative is 0.5 does not necessarily hold. This proposal describes a sign coding scheme to address the biased sign distribution for individual transform skipping blocks.

(The discussion of this contribution was chaired by C. Rosewarne)

Distributions of 70-80% were measured in class F sequences.

current scheme uses bypass coding.

Proposed to use an FSM to code the sign bits: Bypass for first sign, check if next sign is equal. If so, use a context. Otherwise, use bypass coding for the remaining sign bits.

Lossy: Class F 0.2% gains in AI (all tiers).

Lossless: Class F 1%, SC YUV 444 1%, 0.6%, 0.4% (AI,RA,LB).

Observed slightly higher gains when Intra_BC was disabled for lossy. So Intra_BC tends to make the sign distribution more uniform. For lossless disabling Intra_BC made little difference.

Comment: Sequential contexts has throughput implications for CABAC.

Comment: Similar to the significance context approach where runs of context bins are coded.

Comment: You could try using just one context because the distribution appears quite stable.

Comment: For runs of low magnitude coefficients, signs can become significant.

Comment: SC YUV 444 results should be interpreted with caution due to the dodgy sequence (but another comment that this outlier may not be quite so severe in this case).

Comment: 15 context bins consecutive per sub-block is the worst case.

The proponent notes that an upper limit, similar to the high thorughput binarisation could be used.

Comment: Technique is disabled for intra block copy blocks.

Comment: Interaction with palette coding could be interesting to see.

Recommend further study.

JCTVC-O0330 Crosscheck of JCTVC-O0238: Sign coding for transform skipped blocks [C. Pang (Qualcomm)] [late] [miss]

JCTVC-O0230 Chroma Deblocking [Alexis Tourapis (Apple), David Flynn (BlackBerry), David Singer (Apple)] [late]

In JCTVC-N0292 and JCTVC-O0044 a method was presented that allows for finer control of the quantization process of chroma information down to the CU level. However, currently the HEVC specification takes in consideration only the PPS level chroma QP offsets, as well as the CU luma QP parameters for the determination of the chroma components deblocking strength for the purposes of in-loop filtering. This contribution proposes to, assuming JCTVC-O0044 is adopted, allow the consideration of the additional CU level quantization information during the deblocking process.

(The discussion of this contribution was chaired by C. Rosewarne)

Chroma QP deblocking currently only considers PPS QP adjustments and ignores slice-level adjustments.

Slice-level QP not added, but the CU-level QP adjustment is proposed to be added.

No subjective testing yet, but no PSNR impact.

Only chroma adjusted.

Slice level enable flag.

Gives extra level of flexibility to control chroma deblocking strength.

Proponents assert that no complexity impact to change filter strength at CU level.

Slice level QP not added in v1 design, not proposed to be added here.

Comment: Maybe the slice-level control of this feature is not necessary.

Comment: Complexity seems not an issue.

Recommend further study.

JCTVC-O0276 Cross-check of JCTVC-O0044 on RExt: CU-adaptive chroma QP offsets [J. Xu, A. Tabatabai(Sony)] [miss]

JCTVC-O0089 AhG5: Deblocking Filter in 4:4:4 Chroma Format [W.-S. Kim, W. Pu, J. Sole, M. Karczewicz (Qualcomm)]

In this contribution, methods are proposed to improve the performance of deblocking filter of chroma components in 4:4:4 format. In the first method, the luma deblocking filter is applied to the chroma components. In the second method, the luma deblocking filter is applied to the chroma components without the strong filter. In the third method, the chroma filter is used when the boundary strength is larger than 0. It is also proposed to signal deblocking control parameters separately for chroma components. Experimental results report visual quality improvements as well as coding efficiency improvements of 0.2/2.2/2.2 and 0.4/2.2/1.8 (Y/U/V BD-rate (%)) for RA and LB Main-tier in Method 1, 0.2/1.9/1.9 and 0.4/1.9/1.7 in Method 2, and 0.1/0.5/0.5 and 0.0/0.3/0.4 in Method 3, respectively.

This proposal is identical to JCTVC-N0263.

Three methods are presented:

- Apply luma filter to luma components

|  | Rext MT |  |  | HT |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| AI YCbCr | 0.0% | -1.1% | -1.2% | 0.0% | -0.6% | -0.5% |
| AI RGB | 0.0% | -0.3% | 0.2% | 0.0% | -0.9% | -0.3% |
| RA YCbCr | -0.2% | -2.9% | -2.3% | -0.1% | -1.8% | -1.3% |
| RA RGB | -0.2% | -1.6% | -2.2% | -0.2% | -1.3% | -1.8% |
| LB YCbCr | -0.2% | -2.6% | -0.8% | -0.2% | -1.3% | 0.2% |
| LB RGB | -0.7% | -1.8% | -2.6% | -0.4% | -1.5% | -1.9% |

- As per (1), but do not use the strong filter

|  | Rext MT |  |  | HT |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| AI YCbCr | 0.0% | -0.7% | -0.7% | 0.0% | -0.5% | -0.3% |
| AI RGB | 0.0% | -0.3% | 0.3% | 0.0% | -0.9% | -0.3% |
| RA YCbCr | -0.1% | -2.3% | -1.7% | -0.1% | -1.5% | -1.0% |
| RA RGB | -0.2% | -1.5% | -2.1% | -0.2% | -1.2% | -1.7% |
| LB YCbCr | -0.1% | -2.2% | -0.9% | -0.2% | -1.3% | -0.1% |
| LB RGB | -0.6% | -1.6% | -2.5% | -0.4% | -1.4% | -1.9% |

- Extend the chroma filtering in inter-coded chroma blocks. Ie, Apply the chroma filter when bS > 0, currently only intra-coded chroma block boundaries are filtered (when bS=2).

|  | Rext MT |  |  | HT |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| RA YCbCr | -0.1% | -0.9% | -0.7% | -0.1% | -0.3% | 0.1% |
| RA RGB | -0.1% | -0.1% | -0.4% | 0.0% | 0.3% | 0.5% |
| LB YCbCr | -0.1% | -1.1% | -0.8% | -0.1% | -0.1% | 0.1% |
| LB RGB | 0.1% | 0.3% | 0.0% | 0.2% | 0.7% | 1.0% |

NB, the performance seems to reduce as QP decreases.

Additionally, it is recommended to provide separate deblocking filter control parameters for chroma, for both components independently (beta_offset_div2, tc_offset_div2).  Although beta_offset is not required for method (3).

The contributor suggests method (3) is used.

A subjective viewing session was held to examine the performance of method (1).  There were multiple concerns expressed about the test setup, where the monitor has not been correctly configured prior to running the subjective test (It is scaling 1080p → 2560x1600, contrast and brightness are at 100%, colourspace may not be correct.

The test was conducted using a dual stimulus simple preference vote, examining QPs 32 and 37.

|  | Score (votes) |  |  |
| --- | --- | --- | --- |
|  | HM | O0089 | Preference |
| (RGB) EBURainFruits | 7 | 8 | 53% |
| (RGB) DucksAndLegs | 4 | 11 | 73% |
| (RGB) EBULupoCandlelight | 7 | 7 | 50% |
| (YCbCr) EBURainFruits | 4 | 11 | 73% |

Comment: based on the conditions of the test, are we able to determine if there is any subjective improvement.

Comment: a participant in the subjective test said that at these high QPs there were worse artefacts than deblocking.

Comment: Could the subjective viewing be repeated for method (3)? (And with a correctly set up monitor).

Informal subjective viewing by the contributor suggested that method (3) shows little subjective difference, but the ability to control the offset parameters may be useful.

Comment: Would method (3) be applied to both RGB and YCbCr?  An issue with (3) is that controlling the offset just affects the filter strength unconditionally where chroma does not perform a test at the block boundaries.  So for RGB there would be no test at the block boundary and it would be worse.

Comment: could method (1) be turned on/off at the picture level?  However, the signalling of RGB vs YCbCr is at the sequence level.

Comment: did you test content that has lots of water or fire or sky?  The tested content may not have these characteristics.

Comment: has 4:2:2 been examined?  No.

Comment: When encoding RGB would one want to use a lower QP?

Comment: For YCbCr, can this be solved with a negative Qp offset?  This can now be done at the block level too.

Considering the extra control over chroma deblocking parameters:

- Comment: there should be some demonstration of the benefit to this

- Comment: from experience, it is useful to vary the parameters.  At high bitrates for example it can be sometimes be useful to reduce the amount of filtering independently of the luma.

- Comment: from the discussion earlier, there was a suggestion to include the slice level QP offset in the chroma deblocking calculation.  If this were to be the case, it would be a good idea to also add extra control over the parameters.

Considering method (3): This method is quite simple, are we in a hurry since it is quite simple.  There is an offer to kindly arrange for high end consumer monitors at the next meeting.  Consultation with the organisers of previous deblocking tests.

Considering methods (1) or (2), that use luma filtering on chroma components:  This performs well for RGB, but isn't the prime focus, however it also has a decoder cost burden.

Recommend: Study in AHG, and for the AHG to study software and the effects of changing the chroma deblocking parameters.  The AHG should also evaluate the subjective merits of this and collect more evidence.

Comment: Compared to the issues present in HEVC v1 deblocking in Geneva (Jan 2013), the issues presented here seem less significant.

JCTVC-O0279 AHG5: Crosscheck of JCTVC-O0089 on deblocking filter in 4:4:4 chroma format [D.-K. Kwon (TI)] [late]

# RExt coding performance (5)

JCTVC-O0101 HDR coding results [C. Fogg (Harmonic)]

Results from preliminary tests exploring combinations of bitdepths, profiles, colour space, and Electro-Optic Transfer Function (EOTF) indicate unmodified HEVC version 1 Main12 is well suited for coding High Dynamic Range (HDR), large colour volume digital 1080p 24 frames/sec cinematography sequences. Main12 YDzDx PQ movie clips “Tears of Steel” (from Roosendaal & Hubert (CC) Blender Foundation | mango.blender.org) and “Oblivion” (from Universal Pictures) derived from 16-bit master source video originally captured from the Sony F65 camera are used in this study. In the “Oblivion” clip, coding performance of Main12 YDzDx-colour PQ-EOTF video has similar coding efficiency to Main10 Rec. 2020-colour-transformed mapped to the same PQ-EOTF. RExt12, which adds the screen content coding tools adopted so far, measured a BD-Rate reduction of -3% in the Qp 7,12,17,22 range of Oblivion, while BD-Rate increased by +0.11% in the “mid-Qp” range of 22,27,32,37. In a cursory test, additional AHG 18 precision tools (extended_precision_processing_flag=1) being studied in AHG18 showed only small benefits in the tested sequence range of this study (Qp 7 to 37), though are expected to yield greater gains below Qp=7. Future sequences will be tested across a wide range of Qp and a set of practical tool/profile combinations.

JCTVC-O0366 Cross-check Report for JCTVC-O0101 on High Dynamic Range Video Coding Results [XXX]

[Not uploaded yet]

This contribution examines what elements of the range extensions contribute to the reported gain of 2.7%.  Various software encoder options are tested.

Encoding the 4:2:0 12-bit sequence, "Oblivion".

Anchor: HEVC version1 CTC

|  | BD-Rate gain @ QP range |  |
| --- | --- | --- |
|  | 22 to 7 | −3 to −18 |
| 1: Anchor + FULL_NBIT + HIGHBITDEPTH | 2.4 | 51.4 |
| 2: (1) + high precision forward transform: | 2.5 | 51.4 |
| 3: (2) + extended precision | 2.6 | 54.6 |
| 4: (3) + SCC tools | 2.7 | 54.6 |

The conclusion is that the FULL_NBIT brings the majority of the gain.

Comment: have the common conditions been tested? no.

The common conditions have not been tested recently, however, it is not expected to have such a significant effect at 10-bit on noisy content.

Comment: looking at the per-frame results may be useful, as the 2.6% may be affected by the hierarchical GOP.

The contributor suggests enabling FULL_NBIT=1.

JCTVC-O0148 RExt: Testing results on 12-bit 4:2:0 Traffic Sequence [M. Zhou, P. Chen (Broadcom)]

This document reports test results on 12-bit 4:2:0 Traffic sequence which is converted from 12-bit 4:4:4 RGB Traffic sequence. Based on the range extension common testing condition QP settings, an average loss of 0.4% in AI, 0.2% in RA and 0.02% in LB of Main tier, and 0.3% in AI, 0.2% in RA and 0.01% in LB of High tier was observed by disabling all the range extension tools in the range extension software HM-12.0+RExt-4.1, along with a reduction in runtime cost.

This is a supporting document for the profiling discussion.

Comment: Would a different partitioning of tools be beneficial?

JCTVC-O0184 Comparison of Compression Performance of HEVC Range Extensions Draft 4 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

This contribution is a study of the relative objective (i.e. PSNR-based) compression performance of HEVC RExt Profile and AVC High 4:4:4 Predictive Profile. It builds upon the prior work reported in JCTVC-G399 JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, JCTVC-M0329, and updating the results by using the latest available reference software (JM-18.5 and HM-12.0+RExt-4.1) and the RExt test sequences.

Results are presented for BD-Rate, also a PSNRavg is used (6*Y + U + V)/8

Same coding structure

RExt conditions and RCE3 sequences for SCC

Gains are shown.

|  | RExt CTC YCbCr 4:4:4 |  |  | RCE3 CTC (avg F+444) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Y/G | Cb/B | Cr/R |
| AI/MT | 21.6 | 13.5 | 15.4 |  |  |  |
| AI/HT | 17.6 | 14.7 | 15.8 |  |  |  |
| AI/SHT | 13.3 | 13.8 | 14.2 |  |  |  |
| RA/MT | 35.1 | 32.3 | 27.4 |  |  |  |
| RA/HT | 29.4 | 32.1 | 25.5 |  |  |  |
| LD/MT | 39.8 | 45.6 | 48.4 |  |  |  |
| LD/HT | 32.8 | 39.8 | 41.2 |  |  |  |

Comment: is it possible at lower QPs that AVC will out perform when using CAVLC.  Examination of the RD-curves shows that the two plots are not converging particularly quickly at the high rates.

In SCC,

an interesting effect occurred, where the anchor for one screen content sequence produces a worse result for QP 17 than QP 22.  When the anchor bitrates are adjusted to match HM, the following benefits are observed:

XXX table.

JCTVC-O0269 AhG8: Using qp=0, 4, 8, 12 for visually lossless coding experiments and results from SC coding by packed pixel pseudo-2D-matching integrated with HM12.0RE4.0 [Tao Lin, Xianyi Chen, Peijun Zhang, Liping Zhao (Tongji Univ.)]

This document proposes to use QPs 0, 4, 8, 12 in visually lossless screen content coding experiments. This document also reports the coding results of 30 screen content test sequences by both HM-12+RExt-4.0 and HM-12+RE-4.0+packed pixel Pseudo-2D-matching technique of JCTVC-L0303. The coding results report that the PSNR using QPs 0, 4, 8, 12 is between 50dB to +inf dB.

This proposal is not about suggesting what constitutes visually lossless results, but suggests extending the current common test conditions to use lower QPs for screen content.

Is QP 0 useful?  It conveys bits from the transform.  However, the exact implementation of the encoder's quantizer can vary results significantly.

Comment: has QP 12 been determined to not be visually lossless using the current sequences?

Comment: what would be learnt from this new data?

JCTVC-O0337 Caution required when evaluating the performance of screen content sequences [D. Flynn, D. He, G. Martin-Cocher (Blackberry)] [late]

(Discussion chaired by C. Rosewarne)

Generally RDOQ offers a performance benefit that is fairly independent of QP. On the 'sc_video_conferencing_doc_sharing' test sequences, RDOQ is reducing performance at three of the five QP points. The coding efficiency of this sequence varied greatly in experiments, much more so than other mandatory screen content test sequences. This unstable performance calls into question the PSNR vs bit-rate curve resulting from measuring this test sequence.

Study of particular encoded frames revealed that entire frames were static, with skipped blocks and then fast scrolling of the application, followed by many more essentially static frames.

An artefact that resembles tearing was observed (240 pel MV), but was not tearing in the capture process, but a result of the way the window is painted in the application. A second region had a 540pel MV.

A diagram of the mode choices reveals that some frames consist entirely of skipped CTUs. Frame 39 shows regions with skipped CTUs when RDOQ is disabled, and when RDOQ is enabled, an entire line of CTUs becomes skipped. Other frames showed similar behaviour, with large regions of the picture either skipped or coded. This led to large differences in the BD-rate.

To encode one of the blocks, a very large MV needs to be coded. Yet an intra-predicted block can also be chosen. Once one block has the large MVD, then subsequent blocks can skip using the predicted MV.

The conclusion is that the encoder is getting caught in a local minima.

This contribution recommends that either the HM is modified to prevent this behaviour, or to remove this test sequence from the test set.

Comments: Have you considered adding hierarchical motion compensation?

More complicated mode decisions could be applied but the increased sw complexity could be an issue.  Testing skip can be one approach.

Comment: Testing for very low residual could be an improved.

Comment: RDOQ behaviour could also be contributing to this issue (possible implementation issue may exist).

Comment: I think any screen content sequence has quite different statistics from traditional test sequences. One suggested solution is more encoder improvement to address this case.

Comment: Another two options: Keep the sequence for all-intra or can be remove repeated frames.

Comment: Removing the repeated pictures does not really address the unstable encoder behaviour.

Comment: We have many skipped CTUs in the screen content test sequences. We have a fast skip mode that is not currently enabled, we could evaluate this encoder tool.

Comment: A frame-level test is another option although this is a bit different to the HM encoder architecture.

Recommended: Take some action regarding this test sequence.

The information contained in this email message and any attachments may be confidential and may also be the subject to legal professional privilege. If you are not the intended recipient, any use, interference with, disclosure or copying of this material is unauthorised and prohibited. If you have received this email in error, please immediately advise the sender by return email and delete the information from your system.

# Source video test material (10)

JCTVC-O0069 AHG5 and AHG18: 16-bit Test Sequences for Range Extensions [K. Sharman, N. Saunders, J. Gamei (Sony)]

This contribution presents 16-bit source material for use in development of 16-bit profiles. Three test sequences are proposed and benchmarks are provided for those sequences using the HM12.0_RExt4.1 test model.

This contribution provides camera captured 16-bit HDR content in linear-light.

Recommended for the high bitdepth studies.

JCTVC-O0172 AHG22: Sequences with medical mixed content and medical visual content with bit depth beyond 10 bits for HEVC development [P. Amon, A. Hutter, U.-E. Martin, B. Heigl (Siemens)]

This contribution proposes test sequences from the medical domain for the development of HEVC, especially for the specification of range extensions. The proposed set contains medical mixed content (i.e., 8-bit 2D+t RGB image data) and medical visual content with bit depth beyond 10 bits (i.e., 12-bit and 16-bit monochrome 3D volume image data). The adoption of this material as test sequences for the further development of HEVC range extensions is proposed.

Suggests 16bit monochrome content and also provides CT scans at 12-bit, which may be useful for the common conditions (12-bit), and useful for the high bitdepth testing at 16-bit.

Comment: If using lossy, don't want one error from one segment to propagate into another area. So is temporal prediction OK for this content?

JCTVC-O0222 New test sequences for Screen Content coding [Alexis Tourapis, David Singer, Krasimir Kolarov (Apple)] [late]

(Discussion chaired by C. Rosewarne)

eight new mixed content test sequences 4:4:4 8-bit  at 1920x1080 or 2560x1440. All RGB.

available from next week once some copyright notices are added.

Attempted to create realistic screen content material. tried compressing and combining different frame-rate material together.

Sequences include video playback, animation, static regions. Some animated desktop widgets are also included. 24fps. One sequences includes a residual error signal (including an amplified version) being displayed and a terminal with a semi-transparent background and motion (e.g. scrolling text).

Sequences include a 'mission control' effect (animated desktop effects such as zoom) where windows are moving (including those with animation to video playing).

Comment: top-right corner (amplified residual) would be really difficult to code.

We hope to provide per-frame results to help understand the behaviour of the encoder on these sequences.

JCTVC-O0256 AHG8: New 4:4:4 test sequences with screen content [H. Yu, W. Wang, X. Wang, J. Ye, Z. Ma (Huawei)]

To support further development of HEVC extensions, new 4:4:4 test sequences are proposed in this contribution. These new sequences are in the format of 1920x1080 at 60fps with 8 bits per component and consist of computer screen content seen from typical computer applications. They were produced by saving the desktop pixel data from memory directly to files, as opposed to capturing computer DVI RGB output. The original RGB sequences as well as their YUV version, colour format conversion information, and copyright statement, have been uploaded to the folder /testsequences/FrExt-candidate-sequences/screen_content/O0256 at ftp.tnt.uni-hannover.de.

JCTVC-O0258 AHG8: Results from coding 4:4:4 screen content sequences [H. Yu, Z. Ma, J. Ye, X. Wang, W. Wang (Huawei)] [late]

This contribution presents the latest investigation results on screen content coding. In the investigation, the coding performance of HM12.0_RExt4.1 was studied and evaluated against the coding results from two modified versions of HM12.0_RExt4.1 where additional coding tools had been added to the base software. Based on the outcome of our investigation, we would like to suggest further development of HEVC extensions for coding screen content.

This contribution also suggests adding an extra column in the results that:

- Currently we calculate the relative gain from the per-encoder averages of the ratios.

- Suggests taking mean of the per-sequence relative gains

Comment: we have not generally been looking at the compression ratios when evaluating the lossless results, using instead the average bitrate savings.  It was noted that on the results presented that the two metrics seem to be well correlated.

JCTVC-O0268 AhG8: New 4:4:4 screen content test sequence [Tao Lin, Kailun Zhou (Tongji Univ.)]

Twist tunnel replaced with a colour version at 720p 30fps. This is the only non-animation 3D computer graphics. The proponent asserts that the tearing issue is also fixed.

(Discussion chaired by C. Rosewarne)

Comment: these sequences somehow appear to have variable frame-rate based on the apparent motion, is it intended?

Some frames were dropped after capture because the graphics application introduced tearing.

The Direct3D API was used to send frames, so application tearing within frames is not expected.

Comment: Duplicated frames were removed. Repeated frames would keep smoother playback.

JCTVC-O0332 UHD test sequences [R. Weerakkody, M. Naccari, M. Mrak (BBC)] [late]

UHD test sequences are provided , captured using a Sony F55 camera with 4:2:0 chroma format. Slight compression was applied using XAVC (10-bit 4:2:2 cropped to 3840x2160 50fps and 4:2:0 sub-sampling with 10s length).

(Discussion chaired by C. Rosewarne)

Goal was to produce content with large motion activity.

'Book' shows a book with a fan blowing the pages.

'ParksAndBuildings' taken on a windy day, so motion from trees and flags, panning motion too. this may include some aliasing artefacts from the downsampling.

'Vehicles' shows moving vehicles.

'Calendar and Plants' shows a rotation motion.

'Men and Plants' shows people handling plants and moving.

Encoded each with JM-18.5 and HM-12.0. CTC QPs used and some lower (e.g. 17) and higher QPs (e.g. 42).

ITU P.910 spatial and temporal indices are calculated for each test sequence, showing a metric of encoding difficulty for intra and inter coding tools. B hardest to encode for intra and inter according to this metric.

The metric has limitations such as not accounting for translation motion very well, so some scores may be misleading.

Comment: the 'no commercial' clause may be an issue for some people.

JCTVC-O0354 Selected medical imaging sequences for HEVC development [Didier Nicholson, Piotr Pawałowski, Jean-Marie Moureaux] [late]

Presenter not available.

- JCTVC-O0360 Proposed additional UHD test sequences [Pierrick Philippe] [late]

Presenter not available.

# Common test conditions

A discussion of the common test conditions was held.  The common test conditions document will include the test conditions for both natural video and screen content test configurations to reduce the number of documents that must be referenced for test configuration information.  An initial selection of test material updates was performed, and is recommended to be reviewed after the meeting.  Test material in the non-screen content classes has generally been reduced in length to aid simulation times.

# Core Experiments

The following core experiments are proposed:

## RCE1 — High throughput / bitdepth [R. Joshi, K. Sharman]

Two tests categories: Category (A) examines methods that use bypass coding to avoid CABAC coding syntax elements.  Category (B) examines methods that pre-align the BAC interval and combinations with (A).

There were some concerns that this CE may be repeating an investigation into high throughput that was conducted for version 1.  However, it is reminded that the coding rates can be far higher in RExt for very high bitdepths.

## RCE2 — Rice parameter initialization [C. Rosewarne]

Suggests two tests to examine the methods from O0239 and O0327 that alter the Rice parameter initialization behaviour.

## RCE3 — intra block copy [J. Sole]

Four test categories, examining methods and combinations:

- Search area and performance

- Block partitioning: Nx2N|2NxN, TU, NxN, Masking

- Padding: Mid-grey, Horizontal padding (two methods), H|V padding. (search area to be same for all methods)

- Combinations.

Some combinations may not be so useful to specify before knowing what the performance of the methods are.  However, some useful combinations are likely to involve the search area and the partitioning or padding.

## RCE4 — palette [L. Guo, X. Guo]

Three tests, to examine two palette methods (O0182, O0218)

Question: how will the palette overlap with the intraBC?  It would be good to see how it behaves with intra block copy with extra splits to 4x4.

Looking at the pictures as well to address any concerns about spatial quantisation.

# RExt text improvements (3)

JCTVC-O0133 Text fixes for inter RDPCM in HEVC-Rext [M. Naccari, M. Mrak (BBC)]

The document provides raises specification issues for inter RDPCM:

- The parsing for the flags inter_rdpcm_flag and inter_rpdcm_dir_flag should be identical for both lossy and lossless coding. In the current version of the text both flags get parsed only for lossy coding

- The flags inter_rdpcm_flag and inter_rpdcm_dir_flag should be signalled for each TU and on a per component basis. In the current version of the text the component index cIdx is missing while parsing both flags

- Clause 8.6.5 (“8.6.5 Residual modification process for blocks using a transform bypass”) should be invoked for both lossy and lossless coding in Section 8.5.4.3. Instead in the text is only invoked for lossy coding

- Clause 8.6.5 is missing the TS scaling operated after residuals reconstruction for the lossy case

Recommend: Fix the bugs reported in (1), (2), (3).

However, (4) is no longer pertinent given adoptions at this meeting.

JCTVC-O0192 AHG5/AHG6/AHG9: Text changes for HEVC Range Extensions [P. Lai, S. Liu, S. Lei (MediaTek)]

This contribution proposes refinements and modifications to the specification behaviour:

- Aspects relating to RDPCM (identical to O0133 points 1,2,3)

- Extending RDPCM to IntraBC (also recommended earlier in the BoG)

- Applying a PPS level flag to force transquantbypass for all CUs. (No action taken in earlier discussions)

# AOB

Raise: How to do the profiling for 4:2:2, 4:2:0 vs 4:4:4 given the different profile requirements

Raise: High precision weighted prediction adoption, is it enabled/disabled in 4:2:2