| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0362 |
| --- | --- |

| Title: | Cross-check report of 'Non-RCE2: Rice parameter initialization for higher bit depth coding' (JCTVC-O0327) by Sharp |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of a method 2 in proposal JCTVC-O0327 by Sharp combined with RCE2 Test A1 by Qualcomm. The method 2 alters the initialisation of the Rice parameter to depend on the bit depth and QP when extended precision processing is enabled. The simulation result for all configurations were reported by Sharp and tested by Canon. The cross check result confirms the simulation result reported by Sharp.

# Cross check procedure

This is a cross-check of method 2 of proposal JCTVC-O0327 [1].

The source code for JCTVC-O0327 was provided by Sharp. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under the AHG18 conditions [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code was analysed and the following observations were made:

- Simulation results for the AHG8 lossy and lossless conditions were obtained with the ‘RExt__HIGH_BIT_DEPTH_SUPPORT’ macro set to ‘1’ (in other simulations this macro is only set to ‘1’ for the AHG18 conditions).

- The maximum Rice-parameter is 9.

- The ‘no reset’ and ‘fast update’ methods of RCE2 Test A1 for the Rice parameter is included (both for extended precision processing enabled and disabled). Refer to [3] for more details. The ‘no ‘reset’ results in the Rice parameter for sub-blocks other than the first processed sub-block being initialised based on the final Rice parameter value from the previous processed sub-block. The first processed sub-block in the TU is initialised to 0.

- When extended precision processing is enabled, for transform skip and transquant bypass blocks, the Rice parameter of the first processed sub-block is initialised as follows:

  - Max (BD - 8 – (QP < 6(12-BD) ? 0 : QP>>3+BD–12), 0);

- When extended precision processing is enabled, for transformed blocks, the Rice parameter for the first processed sub-block is initialised as follows:

  - Max ((BD-8) >>1 -( QP < 6*(12-BD) ? 0 : QP>>3+BD-12 ), 0);

These observations are consistent with the description in [1].

# Coding performance measurement

Table 1 provide the cross-check results for JCTVC-O0327 method 2 under AHG18 conditions, as defined in [2].

Note that as the modifications are only applicable when extended precision processing is enabled, simulations under common test conditions and AHG8 conditions are not performed.

Table 1. JCTVC-O0327 method 2 AHG18 simulation result.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -6.6% | -6.2% | -6.2% |
| 14-bit | -18.4% | -18.1% | -18.1% |
| 16-bit | -28.2% | -28.0% | -28.1% |
| Enc Time[%] | 90% |  |  |
| Dec Time[%] | 88% |  |  |

# Conclusion

This cross-check includes results for JCTVC-O0327 method 2 provided by Sharp. The provided software matches the description in the contribution. The cross-check result confirms the simulation result reported by Sharp.

# References

- S.-H. Kim, M. Kiran “Non-RCE2: Rice parameter initialization for higher bit depth coding,” JCTVC-O0327, 15th JCT-VC meeting, Geneva, CH, Oct. 2013.

- C. Rosewarne, J. Sole, K. Sharman, S. Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation and update methods”, JCTVC-N1122, 14th JCT-VC Meeting: Vienna, AT, July 2013.

- J. Sole, M. Karczewicz, “RCE2 Test A1: Simplified update of the coefficient level Rice parameter,” JCTVC-O0206, 15th JCT-VC Meeting, Geneva, CH, July 2013.