| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0366 |
| --- | --- |

| Title: | Cross-check Report for JCTVC-O0101 on High Dynamic Range Video Coding Results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Peng Yin, Taoran Lu, 432 Lakeside Dr. Sunnyvale CA 94085 USA | Email: | pyin@dolby.com |
| Source: | Dolby Laboratories Inc. |  |  |

_____________________________

# Abstract

This document is the cross-check report on JCTVC-O0101 High Dynamic Range Video Coding Results. The experimental data provided by the proponent are confirmed. Regarding to the tests of comparing Main 12 VS (extended precision setting) and Main 12 V1 setting for software HM-12.0+RExt-4.1 in the contribution, the crosscheckers did further study on analyzing where the gain comes. The results show that for sequence Oblivion XYZ YDzDx PQ EOTF 12b, most of the coding gain comes from the encoder mode decision through the macro FULL_NBIT in the software which computes distortion in full N bits instead of using only MSB 8 bits in the encoder mode decision.

# Crosscheck Results

JCTVC-O0101 [1] presented some preliminary compressing results using HEVC Rext software on 12-bit 4:2:0 YDzDx PQ. The experimental data are confirmed by the crosscheckers.

Regarding to the tests of comparing Main 12 VS (extended precision setting) and Main 12 V1 setting for software HM-12.0+RExt-4.1 in the contribution, the crosscheckers did further study on analyzing where the gain comes from for the sequence Oblivion XYZ YDzDx PQ EOTF 12b.

Main 12 VS setting enabled the following macro:

#define RExt__HIGH_BIT_DEPTH_SUPPORT            1
#define RExt__HIGH_PRECISION_FORWARD_TRANSFORM              1

In addition,

#if RExt__HIGH_BIT_DEPTH_SUPPORT

#define FULL_NBIT                                                              1 ///< When enabled, use distortion measure derived from all bits of source data, otherwise discard (bitDepth - 8) least-significant bits of distortion

#else

#define FULL_NBIT                                                              0 ///< When enabled, use distortion measure derived from all bits of source data, otherwise discard (bitDepth - 8) least-significant bits of distortion

#endif

The above macro shows in Main 12 V1 setting, FULL_NBIT is set to 0, while in Main 12 VS setting, FULL_NBIT is set to 1.

In the configuration for Main 12 VS setting, ExtendedPrecision is set to 1. In addition, all SCC related tools are also enabled which does not exist in Main 12 V1.

In the crosscheck, several tests are performed to turn on the encoder optimization and coding tools one by one and analyze where the gain comes from.

Reference is Main 12 V1 (macro FULL_NBIT is turned off).

Test 1 is to compare Main 12 V1 but turn on macro FULL_NBIT with Main 12 V1.

Test 2 is to compare Main 12 VS but set EPP=0 and disable all SCC tools with Main 12 V1.

Test 2 is to compare Main 12 VS but set EPP=1 and disable all SCC tools with Main 12 V1.

Test 4 is to compare Main 12 VS but set EPP=1 and enable all SCC tools with Main 12 V1.

Table 1 shows the setting in HM-12.0+RExt-4.1 for reference and Test 1, 2 and 3.

Table 1 Software setting for reference and Test 1, 2 and 3

|  | Reference (Main 12 V1) | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| FULL_NBIT | 0 | 1 | 1 | 1 | 1 |
| RExt__HIGH_BIT_DEPTH_SUPPORT | 0 | 1 | 1 | 1 | 1 |
| RExt__HIGH_PRECISION_FORWARD_TRANSFORM | 0 | 0 | 1 | 1 | 1 |
| ExtendedPrecision | 0 | 0 | 0 | 1 | 1 |
| SCC Coding Tools | No | no | no | no | yes |

Table 2 showed the BDrate for Oblivion XYZ YDzDx PQ EOTF 12b. The simulations show the most gain, 2.43% out of 2.66% comes from enabling an encoder only setting FULL_NBIT.

Table 2 BDRate for 3 performed tests on sequence Oblivion XYZ YDzDx PQ EOTF 12b

| BDRate | Test 1 (encoder only) | Test 2 (encoder only) | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| QP (7, 12, 17, 22) | -2.43% | -2.47% | -2.60% | -2.66% |
| QP (22, 27, 32, 37) | -0.07% | 0.00% | -0.09% | 0.11% |

| BDRate | Test 1 (encoder only) | Test 2 (encoder only) | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| QP (-18 -13 -8 -3 ) | -51.38% | -51.43% | -54.55% | -54.55% |
| QP (2 7 12 17) | -6.30% | -6.39% | -6.75% | -6.92% |

# References

[1] B. Mandel, C. Fogg, J. Helman, “High Dynamic Range video Coding Results”, JCTVC-O0101, 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013

# Patent rights declaration(s)

Dolby Laboratories Inc. do not have any current or pending patent rights relating to the technology described in this contribution.