| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0367 M31720 |
| --- | --- |

| Title: | Report on Subjective Viewing Test for Deblocking Filter in 4:4:4 Chroma Format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim | Tel: Email: | 1-858-845-8893 wooshikk@qti.qualcomm.com |
| Source: |  |  |  |

_____________________________

# Abstract

This is a report on informal subjective viewing test for the chroma deblocking filter proposal for 4:4:4 video coding, JCTVC-O0089, that was held during the 15th JCT-VC meeting on October 26, 2013. The description of the test is given and the results are reported. The goal of informal subjective viewing test was to determine how the deblocking filter proposal affects the subjective quality.

# Introduction

High Efficiency Video Coding (HEVC) version 1 [1] has been developed mainly targeting video in 4:2:0 chroma format, while its Range Extension (RExt) work considers video in 4:2:2 and 4:4:4 formats as well [2]. In case of 4:4:4, test sequences in both YCbCr and RGB color spaces are included in the common test conditions [3].

In JCTVC-N0263 [4], the blocking artifact in chroma components is reported and it is proposed to apply stronger filtering for chroma components in case of 4:4:4 coding. The same methods are proposed in JCTVC-O0089 [5].

In this document, the subjective test to evaluate the chroma deblock filtering proposal in JCTVC-O0089 is reported, which was performed during 15th JCT-VC meeting.

# Test Description

The Method 1 in JCTVC-O0089 is tested. HM12.0_RExt4.1 is used to generate the anchor bitstreams and the proposed method is also implemented on top of it. The common test condition defined in [3] is followed and no adjustment of deblock control parameter is applied.

Below are the information regarding the sequences and coding condition used for the test.

- EBURainFruits, RGB, LB, QP32

- DucksAndLegs, RGB, LB, QP37

- EBULupoCandlelight, RGB, LB, QP32

- EBURainFruits, YUV 4:4:4, LB, QP37

15 experts (no one from proponents’ party) participated the test. For each session one subject participated the test.

In each session for each sequence the anchor and the proposed method was assigned to A and B in random order. For each sequence the viewing was done in A-B-A-B order followed by voting time. Each subject selected either A or B, which looked better. One subject did not vote for the third sequence, so the total number of vote for the third sequence is 14 instead of 15.

The test facility was provided from ITU-T. The session was held in Room T072 in ITU-T building. Dell 30” PC monitor was used for the viewing. Videoplayer was used to display decoded raw file and each sequence was played in real time with its original frame rate. As the display does not support 10 bit, the decoded output is converted to 8 bit by discarding two least significant bits.

# Experimental Results

Below table summarizes the test result.

Table 1. Summary result of the test.

|  | Sequence | HM | O0089 | % |
| --- | --- | --- | --- | --- |
| RGB | EBURainFruits | 7 | 8 | 53 |
|  | DucksAndLegs | 4 | 11 | 73 |
|  | EBULupoCandlelight | 7 | 7 | 50 |
| YUV | EBURainFruits | 4 | 11 | 73 |

This result shows there were preference to JCTVC-O0089 for two sequences and almost no preference for the other two sequences.

# Conclusions

In this document, the subjective test to evaluate the chroma deblock filtering proposal in JCTVC-O0089 is reported, which was performed during 15th JCT-VC meeting.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 10,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1003, Jan. 2013.

- D. Flynn, J. Sole, and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1005, Jul. 2013.

- D. Flynn, K. Sharman, and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1006, Jul. 2013.

- W.-S. Kim, W. Pu, J. Sole, and M. Karczewicz, “AhG5: Deblocking Filter in 4:4:4 Chroma Format,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0263, Jul. 2013.

- W.-S. Kim, W. Pu, J. Sole, and M. Karczewicz, “AhG5: Deblocking Filter in 4:4:4 Chroma Format,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-O0089, Oct. 2013.