| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O1006 |
| --- | --- |

| Title: | Common test conditions and software reference configurations for HEVC range extensions |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | David Flynn, Karl Sharman, Chris Rosewarne | Email: | dflynn@blackberry.com, karl.sharman@eu.sony.com, chris.rosewarne@cisra.canon.com.au |
| Source: | JCT-VC |  |  |

_____________________________

# Abstract

This document defines common test conditions and software reference configurations to be used in the development of HEVC Range extensions conducted between the 15th and the 16th JCT-VC meetings.  These common test conditions are also recommended for use in technical contributions to the 16th JCT-VC meeting, if applicable.

# Introduction

Common test conditions are defined for conducting experiments in an environment where outcome of the experiments can be compared.

This document defines three configurations, three encoding formats and three quantization parameter (QP) ranges, as shown in Table 1.

Table 1. Configurations, encoding formats and QP ranges.

| Configurations: | - All intra, - Random access, - Low-delay B. |
| --- | --- |
| Encoding formats: | - R’G’B’ 4:4:4, - Y’CbCr 4:4:4, - Y’CbCr 4:2:2. |
| QP ranges: | - Main tier (MT), - High tier (HT), - Super-high tier (SHT). |

Two sets of test sequences are defined, covering ‘natural content coding’ and ‘screen content coding’ cases.

Excel templates are provided for reporting the results of experiments.

HM-12.1+RExt-5.0.1 software (based upon HM-12.1) will include Range extension support and is expected to be used for experiments.

Notes:

Experiments investigating R’G’B’ coding tools should also provide results using Y’CbCr 4:4:4.

## Configurations

Configuration files are provided in the cfg/ folder of the common software package (available at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/HM-range-extensions).  There are three encoder configurations files provided as follows:

- All intra: encoder_intra_main_rext.cfg

- Random access: encoder_randomaccess_main_rext.cfg

- Low-delay B: encoder_lowdelay_main_rext.cfg

When the random access configuration is used, the period for intra-predicted frames for each test sequence needs to be specified, as follows:

- IntraPeriod: Specify the intra refresh period in the random access configuration. The intra refresh period is dependent on the frame rate of the test sequence.  A value 16 shall be used for test sequences with a frame rate equal to 20fps, 24 for 24fps, 32 for 30fps, 48 for 50fps, and 64 for 60fps.

To ensure decoded frame checking, the following setting must be applied to the encoder and the decoder for all configurations:

- SEIDecodedPictureHash: Must be set to 1.

## Encoding formats

The encoding format is the chroma format of a test sequence and the colour space (only RGB and YCbCr are considered).  The following setting is applied to the encoder for each encoding format:

- InputChromaFormat: Specifies the chroma format of the test sequence. Use “420”, “422” and “444” for the 4:2:0, the 4:2:2 and the 4:4:4 chroma formats, respectively.

For test sequences using the RGB colour space, the following settings are applied to the encoder:

- InputColourSpaceConvert: Must be set to ‘RGBtoGBR’.

- SNRInternalColourSpace: Should be set to 1.

- OutputInternalColourSpace: Should be set to 0.

For test sequences using the RGB colour space, the decoder should be configured with:

- OutputColourSpaceConvert: Should be set to ‘GBRtoRGB’.

Notes:

The settings for SNRInternalColourSpace and OutputInternalColourSpace are optional since they will not affect the coding performance.

## QP ranges

Table 2 defines three QP ranges.  Each QP range includes four QP values.  For each configuration, each test sequence is encoded using each QP value for a given QP range.  The QP values are used for encoding intra-predicted frames and the configuration files further define QP offsets to be applied for inter-predicted frames.

Table 2 QP ranges.

| QP range | QP |
| --- | --- |
| Main tier (MT) | 22, 27, 32, 37 |
| High tier (HT) | 17, 22, 27, 32 |
| Super high tier (SHT) | 12, 17, 22, 27 |

The following setting is applied to the encoder:

- QP: Specify the quantization parameter to be applied (using values as defined in Table 2).

## Test sequences

Test sequences are available from the following location:

- ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/ (please contact the JCT-VC chairs for login information).

Note that test sequences from the EBU (i.e. with the string ‘EBU’ included in the test sequence name) are password protected.  The password can be obtained by registering at the following location:

- http://tech.ebu.ch/testsequences/uhd-1_public_form

Annex A includes md5sums for the test sequences in the common test conditions.

The following parameters are used to configure the encoder for a given test sequence:

- InputFile: Specifies the file name and path of a given test sequence.

- FrameRate: Specifies the frame rate of a given test sequence.

- SourceWidth: Specifies the width of a given test sequence.

- SourceHeight: Specifies the height of a given test sequence.

- FramesToBeEncoded: Specifies the encoded frame count for a given sequence.

- InputBitDepth: Specifies the bit-depth of a given test sequence.

- InternalBitDepth: Specifies the internal bit-depth of operation for the encoder for a given test sequence.

Notes:

When the encoder operates in 10-bit internal bit-depth (InternalBitDepth=10) for an 8-bit source, each 8-bit source sample x is converted prior to encoding to a 10-bit value 4*x. This behavior is built into the reference encoder and no external conversion program is required.

(Although not applicable with the current test conditions, when the encoder operates in 8-bit internal bit-depth (InternalBitDepth=8) for a 10-bit source, each 10-bit source sample x is converted prior to encoding to an 8-bit value (x+2) / 4 clipped to the [0,255] range.)

The encoded frame count may be less than the total frame count for a given test sequence.  In particular, the encoded frame count may also be less than the length of 10 seconds that is recommended for subjective viewing according to ITU-R Recommendation BT.500.  Reduction of the encoded frame count has only been performed where the statistics of the frame data are stable throughout the test sequence.

# AHG5: Natural content coding conditions

The test sequences listed in section 2.1 are to be used, with the encoding format of each test sequence configured in accordance with section 1.2.

The configurations as described in section 1.1 and the QP ranges as described in section 1.3 are to be used.

Results are to be reported using the following template: JCTVC-O1006_AHG5.xlsm

## Test sequences

Table 3 defines the set of test sequences to be used for natural content coding (AHG5).  The encoded frame count, the input bit-depth and the encoding format are specified.  The encoded frame count may be less than the frame count of the test sequence.

For each test sequence, the internal bit-depth is configured to be the same as the input bit-depth, except for VenueVu in RGB 4:4:4, which uses an internal bit-depth of 10-bits (even though the input bit-depth is 8-bits).

Configuration files for each test sequence with each encoding format are provided in the cfg/per-sequence/ folder.  These configuration files specify the parameters listed in section 1.4.

Table 3 Test sequences

| Class / Encoding format | Input bit-depth | Frame rate | Test sequence filename | Encoded frame count |
| --- | --- | --- | --- | --- |
| RGB 4:4:4 | 12 | 30 | Traffic_2560x1600_30_12bit_444.rgb | 150 |
|  | 10 | 24 | Kimono1_1920x1080_24_10bit_444.rgb | 240 |
|  | 10 | 50 | EBULupoCandlelight_1920x1080_50_10bit_444.rgb | 250 |
|  | 10 | 50 | EBURainFruits_1920x1080_50_10bit_444.rgb | 250 |
|  | 8 | 30 | VenueVu_1920x1080_30_8bit_444.rgb | 150 |
|  | 10 | 30 | DucksAndLegs_1920x1080_30_10bit_444.rgb | 150 |
|  | 10 | 50 | OldTownCross_1920x1080_50_10bit_444.rgb | 250 |
|  | 10 | 24 | ParkScene_1920x1080_24_10bit_444.rgb | 240 |
| YCbCr 4:4:4 | 10 | 30 | Traffic_2560x1600_30_10bit_444_crop.yuv | 150 |
|  | 10 | 24 | Kimono1_1920x1080_24_10bit_444.yuv | 240 |
|  | 10 | 50 | EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 250 |
|  | 10 | 50 | EBURainFruits_1920x1080_50_10bit_444.yuv | 250 |
|  | 10 | 30 | VenueVu_1920x1080_30_10bit_444.yuv | 150 |
|  | 10 | 60 | BirdsInCage_1920x1080_60_10bit_444.yuv | 300 |
|  | 10 | 24 | CrowdRun_1920x1080_50_10bit_444.yuv | 250 |
| YCbCr 4:2:2 | 10 | 30 | Traffic_2560x1600_30_10bit_422_crop.yuv | 150 |
|  | 10 | 24 | Kimono1_1920x1080_24_10bit_422.yuv | 240 |
|  | 10 | 50 | EBUHorse_1920x1080_50_10bit_422.yuv | 250 |
|  | 10 | 50 | EBUGraphics_1920x1080_50_10bit_422.yuv | 250 |
|  | 10 | 50 | EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 250 |
|  | 10 | 50 | EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 250 |
|  | 10 | 50 | Seeking_1920x1080_50_10bit_422.yuv | 250 |

# AHG8: Screen content coding conditions

## Lossy screen content coding conditions

The test sequences listed in section 3.3 are to be used, with the encoding format of each test sequence configured in accordance with section 1.2.

The configurations as described in section 1.1 and the QP ranges as described in section 1.3 are to be used.

Results are to be reported using the following template: JCTVC-O1006_AHG8_lossy_r1.xlsm

## Lossless screen content coding conditions

The test sequences listed in section 3.3 are to be used, with the encoding format of each test sequence configured in accordance with section 1.2.

The configurations as described in section 1.1 are to be used, with the following parameters applied to configure the encoder for lossless coding:

- QP: Must be set to 0.

- TransquantBypassEnableFlag: Must be set to 1.

- CUTransquantBypassFlagForce: Must be set to 1.

- IntraReferenceSmoothing: Must be set to 0.

- CostMode: Must be set to ‘lossless’.

Results are to be reported using the following template: JCTVC-O1006_AHG8_lossless_r1.xlsm

## Test sequences

For each test sequence, the following setting is applied:

- InternalBitDepth: Set equal to the input bit-depth for a given test sequence.

Table 4 lists test sequences to be used for both lossless and lossy coding conditions for screen content coding.

Table 4. Screen content coding test sequences.

| Class | Input bit-depth | Frame rate | Encoding format | Test sequence filename | Encoded frame count |
| --- | --- | --- | --- | --- | --- |
| Class F | 8-bit | 50 | YCbCr 4:2:0 | BasketballDrillText_832x480_50.yuv | 500 |
|  | 8-bit | 30 | YCbCr 4:2:0 | ChinaSpeed_1024x768_30.yuv | 500 |
|  | 8-bit | 30 | YCbCr 4:2:0 | SlideEditing_1280x720_30.yuv | 300 |
|  | 8-bit | 20 | YCbCr 4:2:0 | SlideShow_1280x720_20.yuv | 500 |
| Class B | 8-bit | 24 | YCbCr 4:2:0 | Kimono1_1920x1080_24.yuv | 240 |
|  | 8-bit | 24 | YCbCr 4:2:0 | ParkScene_1920x1080_24.yuv | 240 |
| RGB 4:4:4 SC | 8-bit | 60 | RGB 4:4:4 | sc_map_1280x720_60_8bit.rgb | 600 |
|  | 8-bit | 30 | RGB 4:4:4 | sc_web_browsing_1280x720_30_8bit_300_rgb.rgb | 300 |
|  | 8-bit | 60 | RGB 4:4:4 | sc_wordEditing_1280x720_60_8bit.rgb | 600 |
|  | 8-bit | 20 | RGB 4:4:4 | sc_SlideShow_1280x720_20_8bit_500.rgb | 500 |
|  | 8-bit | 60 | RGB 4:4:4 | sc_programming_1280x720_60_8bit.rgb | 600 |
|  | 8-bit | 20 | RGB 4:4:4 | sc_ppt_doc_xls_1920x1080_20_8bit_200_rgb.rgb | 200 |
| RGB 4:4:4 Animation | 8-bit | 30 | RGB 4:4:4 | VenueVu_1920x1080_30_8bit_444.rgb | 150 |
|  | 8-bit | 30 | RGB 4:4:4 | sc_robot_1280x720_30_8bit_300.rgb | 300 |
|  | 8-bit | 30 | RGB 4:4:4 | sc_viking_1280x720_30_8bit_300.rgb | 300 |
| YCbCr 4:4:4 SC | 8-bit | 60 | YCbCr 4:4:4 | sc_map_1280x720_60_8bit_444.yuv | 600 |
|  | 8-bit | 30 | YCbCr 4:4:4 | sc_web_browsing_1280x720_30_8bit_300_444_r1.yuv | 300 |
|  | 8-bit | 60 | YCbCr 4:4:4 | sc_wordEditing_1280x720_60_8bit_444.yuv | 600 |
|  | 8-bit | 20 | YCbCr 4:4:4 | sc_SlideShow_1280x720_20_8bit_500_444.yuv | 500 |
|  | 8-bit | 60 | YCbCr 4:4:4 | sc_programming_1280x720_60_8bit_444.yuv | 600 |
|  | 8-bit | 20 | YCbCr 4:4:4 | sc_ppt_doc_xls_1920x1080_20_8bit_200_444_r1.yuv | 200 |
| YCbCr 4:4:4 Animation | 10-bit | 30 | YCbCr 4:4:4 | VenueVu_1920x1080_30_10bit_444.yuv | 150 |
|  | 8-bit | 30 | YCbCr 4:4:4 | sc_robot_1280x720_30_8bit_300_444.yuv | 300 |
|  | 8-bit | 30 | YCbCr 4:4:4 | sc_viking_1280x720_30_8bit_300_444.yuv | 300 |
| RangeExt | 10-bit | 50 | YCbCr 4:2:2 | EBUHorse_1920x1080_50_10bit_422.yuv | 250 |
|  | 10-bit | 50 | YCbCr 4:2:2 | EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 250 |
|  | 10-bit | 50 | YCbCr 4:4:4 | EBURainFruits_1920x1080_50_10bit_444.yuv | 250 |
|  | 10-bit | 24 | YCbCr 4:4:4 | Kimono1_1920x1080_24_10bit_444.yuv | 120 |
| RGB 4:4:4 SC (Optional) | 8-bit | 20 | RGB 4:4:4 | sc_cad_waveform_1920x1080_20_8bit_200_rgb.rgb | 200 |
|  | 8-bit | 20 | RGB 4:4:4 | sc_pcb_layout_1920x1080_20_8bit_200_rgb.rgb | 200 |
|  | 8-bit | 30 | RGB 4:4:4 | sc_cg_twist_tunnel_1280x720_30_8bit_rgb.rgb | 300 |
| YCbCr 4:4:4 (Optional) | 8-bit | 20 | YCbCr 4:4:4 | sc_cad_waveform_1920x1080_20_8bit_200_444_r1.yuv | 200 |
|  | 8-bit | 20 | YCbCr 4:4:4 | sc_pcb_layout_1920x1080_20_8bit_200_444_r1.yuv | 200 |
|  | 8-bit | 30 | YCbCr 4:4:4 | sc_cg_twist_tunnel_1280x720_30_8bit_300_444_r1.yuv | 300 |

- Annex A. Test sequence md5sums

For each test sequence included in the common test conditions, the ‘md5sum’ value is provided as follows:

Natural content test sequences

2ef18e36431be68c7434b40c4c1047bb  Traffic_2560x1600_30_12bit_444.rgb

437db0d18f414b74e4c62e0485da0716  Kimono1_1920x1080_24_10bit_444.rgb

3835ce632e0a45b8d529cb20dcca8b04  EBULupoCandlelight_1920x1080_50_10bit_444.rgb

20c876a78951a985bf604e050e75ff62  EBURainFruits_1920x1080_50_10bit_444.rgb

8f98fbad7261bcd78f86d8ba8fd4a19b  VenueVu_1920x1080_30_8bit_444.rgb

a6e0c402fd1cc116a72fb2c0c2ffc07a  DucksAndLegs_1920x1080_30_10bit_444.rgb

ffaa261159ba236bb1f4671246201f7f  OldTownCross_1920x1080_50_10bit_444.rgb

612640a857ff95cc7bd3859a4e2b8a4f  ParkScene_1920x1080_24_10bit_444.rgb

5f6f594778d01c8150e9df877ed760f0  Traffic_2560x1600_30_10bit_444_crop.yuv

afd06b6a7823d854b89277ccee1e2aa8  Kimono1_1920x1080_24_10bit_444.yuv

0943294a422600371c988193e36cc3dc  EBULupoCandlelight_1920x1080_50_10bit_444.yuv

14e011ee5ceb1c8d7a046e2cc0401774  EBURainFruits_1920x1080_50_10bit_444.yuv

6f5d8ac82ff84043603025edd3e9a8eb  VenueVu_1920x1080_30_10bit_444.yuv

f018bed8cdfab52505d9169fe946d9bf  BirdsInCage_1920x1080_60_10bit_444.yuv

e9b443a3adc7538c4d4104f141cce0fb  CrowdRun_1920x1080_50_10bit_444.yuv

423e0e2d2c3ead29c267b349b988f083  Traffic_2560x1600_30_10bit_422_crop.yuv

d8bd4994c1b68392f296a297be3114fa  Kimono1_1920x1080_24_10bit_422.yuv

3bef26469ee0d1a3ae5fc972497f17b9  EBUHorse_1920x1080_50_10bit_422.yuv

1c200a39b4ec8216f2a63c403df4e015  EBUGraphics_1920x1080_50_10bit_422.yuv

b493aa9a920cf8e1490280279432285c  EBUWaterRocksClose_1920x1080_50_10bit_422.yuv

d2f9262feea06b7159ae2a2beb83701b  EBUKidsSoccer_1920x1080_50_10bit_422.yuv

d595003e45041e17f021a3db388381e9  Seeking_1920x1080_50_10bit_422.yuv

Screen content test sequences

480881343081e3f4adf85075a1ed0f08  BasketballDrillText_832x480_50.yuv

9dfcbb87d095987891cf90cc593c5ba9  ChinaSpeed_1024x768_30.yuv

dbf1d0e765ba9f775c5208adb4dc6c0c  SlideEditing_1280x720_30.yuv

d6b28f127b9ddf088eb9db9fc5e611bd  SlideShow_1280x720_20.yuv

4a83005bc719012ac148dd3898e5e4ed  Kimono1_1920x1080_24.yuv

b7ada0912d693304165254177d08343d  ParkScene_1920x1080_24.yuv

d83e96251f40e03bc3d27823ab54cba0  sc_map_1280x720_60_8bit.rgb

aa992e5577c7d70d68c6a0f16697d213  sc_web_browsing_1280x720_30_8bit_300_rgb.rgb

e4bdfef57a01392e5c04c783989f0cea  sc_wordEditing_1280x720_60_8bit.rgb

90bfb5b90419ff16d0490ef919eee3e1  sc_SlideShow_1280x720_20_8bit_500.rgb

dde4b9da34f4b698a9ce984caa4c5034  sc_programming_1280x720_60_8bit.rgb

d638fcc2ab30c95c5c223675a33ad189  sc_ppt_doc_xls_1920x1080_20_8bit_200_rgb.rgb

8f98fbad7261bcd78f86d8ba8fd4a19b  VenueVu_1920x1080_30_8bit_444.rgb

24455b665c4b8b951170ed8e9972d191  sc_robot_1280x720_30_8bit_300.rgb

3f4ee8b1da7f8c378a321139cb5b7456  sc_viking_1280x720_30_8bit_300.rgb

1b844c4ab08a2885aeafa7a473ec3e16  sc_map_1280x720_60_8bit_444.yuv

a6926facd648dc46a8c3dd7ae5ddccf8  sc_web_browsing_1280x720_30_8bit_300_444_r1.yuv

63436dc5c1adca429129daf17c6a58e5  sc_wordEditing_1280x720_60_8bit_444.yuv

f1a721e59ae9b662d6aefffa6fb64f8d  sc_SlideShow_1280x720_20_8bit_500_444.yuv

8ffc06bf2873388fbc20e75d4852a21c  sc_programming_1280x720_60_8bit_444.yuv

e3bcd5597228302fc0fe528f642338c1  sc_ppt_doc_xls_1920x1080_20_8bit_200_444_r1.yuv

6f5d8ac82ff84043603025edd3e9a8eb  VenueVu_1920x1080_30_10bit_444.yuv

15cb0ddd57c9d67bae34e16ce84f5f72  sc_robot_1280x720_30_8bit_300_444.yuv

cca7d59accafc336116553565bc06df2  sc_viking_1280x720_30_8bit_300_444.yuv

3bef26469ee0d1a3ae5fc972497f17b9  EBUHorse_1920x1080_50_10bit_422.yuv

b493aa9a920cf8e1490280279432285c  EBUWaterRocksClose_1920x1080_50_10bit_422.yuv

14e011ee5ceb1c8d7a046e2cc0401774  EBURainFruits_1920x1080_50_10bit_444.yuv

afd06b6a7823d854b89277ccee1e2aa8  Kimono1_1920x1080_24_10bit_444.yuv

419d13950c685bb9df2139ff2b37fb74  sc_cad_waveform_1920x1080_20_8bit_200_rgb.rgb

f513b714d3d59432d5c5f532f6b73a0c  sc_pcb_layout_1920x1080_20_8bit_200_rgb.rgb

d3eb1fcd8995d808bdce33ee3131c8c6  sc_cg_twist_tunnel_1280x720_30_8bit_rgb.rgb

6b57b8e56b89dbcf6e9cf1ae16fae3d6  sc_cad_waveform_1920x1080_20_8bit_200_444_r1.yuv

12720e1d2c5b571b8b9b7ca30e5b247b  sc_pcb_layout_1920x1080_20_8bit_200_444_r1.yuv

885602f7bca5d9293aa4f11d1a0121a3  sc_cg_twist_tunnel_1280x720_30_8bit_300_444_r1.yuv