| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-O1122 |
| --- | --- |

| Title: | HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Chris Rosewarne Marta Karczewicz Karl Sharman S.-H. Kim | Email: | chris.rosewarne@cisra.canon.com.au martak@qti.qualcomm.com karl.sharman@eu.sony.com kimse@sharplabs.com |
| Source: | CE co-ordinators |  |  |

_____________________________

# Abstract

This document is a description of the HEVC Range Extensions Core Experiment 2 on Rice parameter initialization methods.

# Introduction

This document defines HEVC Range Extensions Core Experiment 2 (RCE2) on Rice parameter initialization methods, to be performed for the upcoming January 2014 JCT-VC meeting.

# Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| J. Sole L. Guo M. Karczewicz | joels@qti.qualcomm.com liweig@qti.qualcomm.com martak@qti.qualcomm.com | Qualcomm | X | X |
| K. Sharman N. Saunders | karl.sharman@eu.sony.com nick.saunders@eu.sony.com | Sony Europe | X |  |
| C. Rosewarne | chris.rosewarne@cisra.canon.com.au | Canon |  |  |
| S. H. Kim M. Kiran | kimse@sharplabs.com misrak @sharplabs.com | Sharp Labs | X |  |
| E. Alshina S. Lee | elena_a.alshina@samsung.com sunil.lee@samsung.com | Samsung |  | X |
| S. Liu Y.-W. Huang S.-T. Hsiang X. Zhang P. Lai | shan.liu@mediatek.com yuwen.huang@mediatek shih-ta.hsiang@mediatek Xianguo.zhang@mediatek.com polin.lai@mediatek.com | Mediatek |  |  |
| A. Saxena G. Jin F. Fernandes | asaxena@sta.samsung.com guoxin.jin@sta.samsung.com ffernandes@sta.samsung.com | Samsung Telecom America |  |  |
| M. Budagavi D.-K. Kwon | madhukar@ti.com d-kwon@ti.com | TI |  |  |
| A. Minezawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp | Mitsubishi |  |  |
| K. Chono | chono@ct.jp.nec.com | NEC |  |  |
| C. Auyeung | cheung.auyeung@am.sony.com | Sony USA |  |  |

# Tools to be tested

Note: Significant change to proposals require a non-CE contribution.

## Rice parameter initialization methods

Test A.1 (JCTVC-O0239, Qualcomm & Sony)

4x4 sub-blocks are divided into different categories “sbType” depending on the transform skip flag.  Statistic statCoeff is maintained for each sbType, and is updated using the remaining level coefficient of each sub-block. This statistic is used to calculate the initial Rice parameter for the next sub-block.  Extra test for further splitting sub-blocks based on luma / chroma flag will also be conducted to evaluate the performance change with two extra counters.

Test A.2 (JCTVC-O0327, Sharp)

Rice parameter is initialized based on the existing information such as bit depth and quantization parameter. In addition, this method may be tested with the existing ‘no reset’ aspect of the Rice parameter update.

This method may be tested with the existing ‘no reset’ aspect of the Rice parameter update.

# Cross-checks

Table 1 specifies the cross-checker(s) for each subtest.

| Subtest | Cross-checker(s) |
| --- | --- |
| A.1 | E. Alshina (Samsung), C. Rosewarne (Canon) |
| A.2 | L. Guo (Qualcomm) |

Table 1: Cross-checks

# Test conditions

## Screen content coding testing (lossy and lossless)

The SCC lossy and SCC lossless conditions as defined in [1] are to be used.

## AHG18 testing (lossy high bit-rate)

The AHG18 test conditions as defined in [2] are to be used, with the following additions:

- Testing with the high precision forward transform is mandatory.

- Produce a set of test results with transform skip disabled and a set with transform skip enabled.

- Testing to be performed using ‘all intra’, ‘random access’ and ‘low-delay B’ configurations.

## Subtest test conditions

Table 2 specifies the test conditions are used for each subtest.

| Subtest | SCC (lossy) | SCC (lossless) | AHG18 lossy |
| --- | --- | --- | --- |
| A.1, A.2 | Yes | Yes | Yes |

Table 2: Coding conditions to be tested for each subtest

# Evaluation of CE Results

Results of the CE will be evaluated on the basis of bit-rate and complexity. Complexity assessment is performed on the base of hardware and encoding/decoding complexity.

## Complexity assessment

To measure software run times, range extension software with the provided configurations and the proposals implemented on range extension software shall be used.  The computational time must be measured for each test sequence and test case for both anchor and proposals.  Relative computational time calculated against the anchor must be presented.

Additional evaluation of the HW and SW complexity of the proposed tools for both encoder and decoder is encouraged.

# Timeline

- November 22nd 2013: CE description finalized and uploaded.

- November 22nd 2013: Test sequences and test conditions are finalized with AHG8.

- Meeting end + 2 weeks: November 15th 2013 (tentative): Range extension reference software available.

- Range extension software availability date + 2 weeks: Cross-check begins: Proponents for tests provide software, draft specification text and results to CE participants.

- January 3rd 2014: Input documents and summary report uploaded.

# References

- D. Flynn, K. Sharman, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-O1006, Geneva, CH, Oct. 2013.

- R. Joshi, K. Sharman, “HEVC Range Extensions Core Experiment 1 (RCE1): High throughput,” JCTVC-O1121, Geneva, CH, July 2013.