| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O1124r1 |
| --- | --- |

| Title: | HEVC Range Extensions Core Experiment 4 (RCE 4): Palette Coding For Screen Content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Liwei Guo Xun Guo Ankur Saxena | Email: | liweig@qti.qualcomm.com xun.guo@microsoft.com asaxena@sta.samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document is a description of the HEVC Range Extensions Core Experiment 4 on palette coding methods.

# Introduction

This document defines HEVC Range Extensions Core Experiment 4 (RCE 4) on palette coding methods for screen content, to be performed for the upcoming January 2014 JCT-VC meeting.

# Participants List

| Participant | Email address | Affiliation | P |
| --- | --- | --- | --- |
| L. Guo J. Sole M. Karczewicz W. Pu | liweig@qti.qualcomm.com joels@qti.qualcomm.com martak@qti.qualcomm.com wpu@qti.qualcomm.com | Qualcomm | X |
| X. Guo B. Li J. Xu | xun.guo@microsoft.com libin@microsoft.com jzxu@microsoft.com | Microsoft | X |
| A. Saxena G. Jin F. Fernandes S. Lee C. Park | asaxena@sta.samsung.com guoxin.jin@sta.samsung.com ffernandes@sta.samsung.com sunil.lee@samsung.com cy1205.park@samsung.com | Samsung |  |
| P. Lai S. Liu Y.-W. Huang S.-T Hsiang X. Zhang Y. Sun | polin.lai@mediatek.com shan.liu@mediatek.com yuwen.huang@mediatek shih-ta.hsiang@mediatek.com Xianguo.zhang@mediatek.com YC.Sun@mediatek.com | MediaTek |  |
| Chris Rosewarne P. Onno G. Laroche C. Gisquet | Chris.Rosewarne@cisra.canon.com.au Patrice.Onno@crf.canon.fr Guillaume.LAROCHE@crf.canon.fr Christophe.Gisquet@crf.canon.fr | Canon |  |
| D.-K. Kwon M. Budagavi | d-kwon@ti.com madhukar@ti.com | TI |  |
| A. Minezawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp | Mitsubishi |  |
| X. Xiu | Xiaoyu.Xiu@InterDigital.com | InterDigital |  |
| K. Ugur J. Lainema D. Bugdayci | Kemal.Ugur@nokia.com Jani.Lainema@nokia.com Ext-Done.Bugdayci@nokia.com | Nokia |  |
| J. Xu C. Auyeung K. Sato | jun.xu@am.sony.com Cheung.Auyeung@am.sony.com Kazushi.Sato@jp.sony.com | Sony |  |
| K. Chono | chono@ct.jp.nec.com | NEC |  |
| H. Yu Z. Ma X. Wang | haoping.yu@huawei.com zhan.ma@huawei.com xian.w@huawei.com | Huawei USA |  |
| X. Wang T. Ji | xiaofenwang@blackberry.com tiji@blackberry.com | Blackberry |  |
| J. Zhu K. Kimihiko | zhujianqing@cn.fujitsu.com Kazui.Kimihiko@jp.fujitsu.com | Fujitsu |  |
| W.-H. Peng C.-C. Chen F.-D. Jou C.-L. Lin | wpeng@cs.nctu.edu.tw cheerchen.cs98g@g2.nctu.edu.tw fandi@itri.org.tw chunlung@itri.org.tw | NCTU/ITRI |  |

# Tested Methods

## Test 1 (O0182, Microsoft)

In this test, the major-color-based screen coding algorithm will be tested, which has the similar concept with previously investigated palette-based method. In specific, pixels in a CU are classified into different major color sets and converted into color indices. Major color number, values and the color indices and escape pixels are then transmitted for each CU. Line mode prediction is used to exploit the correlations among color indices including horizontal mode, vertical mode and normal mode. In horizontal mode, the whole pixel line has the same index value. In vertical mode, the current pixel line is exactly the same with the above line. And in normal mode, the color index will be predicted one by one. The color indices from different color components are generated separately but coupled together for the prediction. The methods described in JCTVC-O0182 will be tested. The method without escape pixels will also be tested. Extra test includes extending the method to NxN PU level.

## Test 2 (O0218, Qualcomm)

In this test, palette mode in JCTVC-O0218 will be tested. For each CU, palettes are first transmitted. The pixels in the CU can be converted into palette indices and be predicted with “run” mode or “copy from top” mode. The prediction errors are encoded with HEVC residue coding. The method without residue coding will also be tested. Extra test includes extending the method to NxN PU level.

# Software

HM-12.1+RExt-5.1 will be used as the reference software for this CE.

# Test Conditions

AHG8 test sequences and test conditions defined in JCTVC-O1006 [3] are used to evaluate the tested methods. Both lossless and lossy coding are mandatory tests in this CE. Md5sum check needs to be enabled in both tests (SEIDecodedPictureHash=1).

# Evaluation of CE Results

Results of the CE will be evaluated on the basis of bit rate and complexity. Complexity assessment is performed on the base of hardware and encoding/decoding complexity.

## Complexity assessment

To measure software run time, range extension software with the anchor configuration (for lossless coding the default frame level lossless coding is turned on) and the proposals implemented on range extension software shall be used. The computational time must be measured for each test sequence and test case for both anchor and proposals. Relative computational time calculated against the anchor must be presented.

In addition to the simulation time, the following two metrics shall be provided to measure the decoding implementation complexity.

- Number of operations to decode one sample

- Throughput (e.g. number of residual samples or pixels which can be decoded in parallel)

Additional evaluation of the HW and SW complexity of the proposed tools for both encoder and decoder is encouraged.

# Cross-check Assignment

| Test | Proponent | Cross-checker |
| --- | --- | --- |
| 1 | Microsoft (xun.guo@microsoft.com) | Qualcomm(liweig@qti.qualcomm.com) Huawei(zhan.ma@huawei.com) |
| 2 | Qualcomm (liweig@qti.qualcomm.com) | Huawei(zhan.ma@huawei.com) Samsung(sunil.lee@samsung.com) |

# Timeline

- Nov. 22, 2013 (3 weeks after the meeting): Test sequences and test conditions are finalized.

- Dec. 4, 2013: Range extension reference software HM-12.1+RExt-5.1 is released.

- Dec. 18, 2013 (2 weeks after Range extension software release): Cross-check begins: Proponents provide software, draft of contribution text and results to CE participants.

- Jan. 3, 2014: RCE contributions uploaded.

- Jan. 6, 2014: RCE report uploaded.

# References

[1] L. Guo, M. Karczewicz, J. Sole, and R. Joshi, “Evaluation of Palette Mode Coding on HM-12.0+RExt-4.1”, JCTVC-O0218, Geneva, Switzerland, Oct. 2013.

[2] X. Guo, B. Li, J. Xu, Y. Lu, S. Li, and F. Wu, “AHG8: Major-color-based screen content coding”, JCTVC-O0182, Geneva, Switzerland, Oct. 2013.

[3] D. Flynn, C. Rosewarne, and K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-O1006, Geneva, Switzerland, Oct. 2013.