| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0010 |
| --- | --- |

| Title: | Ad hoc group report: Multi-layer picture order count derivation (AHG10) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC ad hoc group on multi-layer picture order count derivation, including a report of the teleconferences held and the progress made during the interim period since the preceding meeting. (The reader of this report is assumed to be familiar with various jargon and abbreviations used in JCT-VC; recent meeting reports and the draft specification text for HEVC and its SHVC and MV-HEVC extensions can be consulted for definitions of terms.)

The AHG held six two-hour teleconferences with substantial discussions of the issues. 10–20 people participated in each call – each of which was announced with at least 7 days notice. The calls were held at different times of day to ensure reasonable opportunities for participation from different time zones. It was generally agreed that the current draft design seemed basically "broken".The discussions primarily focused on two proposals: a scheme proposed by Nokia (now revised as JCTVC-P0056 / JCT3V-G0042) and a different scheme proposed by Qualcomm and ZTE (now revised as JCTVC-P0041 / JCT3V-G0031). The proposals for each of these schemes were refined during the interim period, with multiple uploaded revisions.

Pending further input, the AHG concluded that the Qualcomm/ZTE scheme of JCTVC-P0041 / JCT3V-G0031 seems the most mature as the basis for moving forward. It has more mature text, has no significant identified problems, and is asserted to have a resilience advantage. It would be desirable to have that resilience assertion more confidently assessed; however, no significant problem seems to exist in that scheme, so its selection is recommended by the AHG unless additional input indicates otherwise.

The AHG noted that software verification has not been conducted to really show the operation of the scheme in practice with important test cases. Follow-up on this issue is needed, as much as possible, to increase our confidence that the scheme has all details worked out and would not have any problems in important use cases.

Further study is also needed to determine what is needed for cross-codec multi-layer operation.

Additional new contributions are noted to be relevant, including the following:

- JCTVC-P0067 / JCT3V-G0045 MV-HEVC/SHVC HLS: Comments on POC alignment [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

- JCTVC-P0260 / JCT3V-G0224 MV-HEVC/SHVC HLS: Additional information on the POC design in JCTVC-P0041/JCT3V-G0031 [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)] [late-registered; not yet available as of preparation of this report]

# Mandates and communication methods

The JCT-VC ad hoc group on multi-layer picture order count derivation had the following mandates:

- Study picture order count (POC) operation for HEVC layered coding, in regard to loss resilience, random access, step-wise layer transitions.

- Identify problems associated with POC derivation.

- Solicit and study candidate solutions for identified problems with POC derivation.

- Hold teleconference discussions (pre-announced on the JCT-VC reflector with at least 7 days notice) to facilitate the work of the AHG.

The reflector used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

Email discussions for this AHG were also copied to the JCT-3V reflector:
jct-3v@lists.rwth-aachen.de, as multi-layer POC derivation affects layering structures based on view layers and depth layers as well as spatial and SNR scalability layers. For subscription to that list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-3v.

# Work of the AHG

## Kick-off and teleconference planning

A kick-off message was sent by the chair on 7 November 2013. It included an announcement of two planned teleconferences. The chair suggested to use the first teleconference to review a couple of the key proposals that were submitted to the recent meeting, and to clarify the issues involved in evaluating them.

According to the summary document JCTVC-O0343 and the BoG report JCTVC-O0349, two key proposals to discuss were JCTVC-O0176 and JCTVC-O0275 / JCT3V-F0092v2. It was therefore suggested (by email on Tuesday 5 November) to focus on those two contributions in the first teleconference call.

## First teleconference

### General

The first teleconference was held at 14:00 Seattle time on Thursday 14 November 2013 with a duration of 1.5 hours. The teleconference was hosted through Microsoft Lync, which allows either dial-in (through a variety of local phone numbers world-wide) or online access (with shared document viewing). Approximately 20 people attended the teleconference (with a peak of approximately 18 people on the call at once). The document sharing capability was used during the teleconference to display the presented contribution documents and the meeting notes as they were being recorded.

A preliminary report of the first teleconference and planned topics for discussion for the second teleconference were sent by email on Tuesday 19 November (Seattle time).

### Discussion of current draft specification for multi-layer POC derivation

A design choice to support non-aligned IRAP pictures was made at the Vienna meetings of JCT-VC and JCT-3V in July/August 2013.

In the Vienna output draft specifications for the SHVC and MV-HEVC extensions of HEVC, POC values are sometimes reset and are thus modified dynamically.

It was remarked that there are several problems with that drafted method, which include the following:

- How to detect and recover from a loss of the reset indication. At the Vienna meeting, it was remarked that the drafted method has loss resilience implications (when the picture that causes the POC reset is lost), and further study of that aspect was encouraged.

- Difficulty determining persistence for POC-based persistence specifications for aspects like SEI messages. An approach to deal with this has not been defined, although a participant remarked that this may be basically easy to solve – e.g., we could specify that persistence does not cross past a POC reset position.

- Another problem, as illustrated in Fig. 2 of document JCTVC-O0176v3, is that a version 1 decoder does not recognize the reset indication, which can result in a situation for which there appears to be no appropriate value for the POC LSBs to send in the base layer because the appropriate LSB values to send should be different depending on whether the POC has been reset or not.

In the AHG discussion, it was agreed that the current design seemed basically "broken" because of these issues, and so some action was needed. It was remarked that three types of fixes had basically been proposed. These included a substantially different scheme proposed in JCTVC-O0275 / JCT3V-F0092, another substantially different scheme proposed in JCTVC-O0176, and some proposals to make "patches" to the existing scheme while basically not modifying it as significantly.

### Discussion of JCTVC-O0275 / JCT3V-F0092 (by M. M. Hannuksela / Nokia)

In response to some issues raised at the recent meeting, the contributor of JCTVC-O0275 / JCT3V-F0092 had uploaded a new "-v3" version of the contribution after the meeting – approximately one day before the teleconference. (This would likely have been registered as an input for the subsequent meeting, but the document registration web site had not yet been configured for the subsequent meeting.) The contributor suggested reviewing this latest version of the contribution, and there was no objection.

The contributor asserted that this proposal presents a design that enables non-aligned IRAP pictures across layers, is error-resilient, does not change the POC values of the base layer, and keeps the POC values static during their persistence. For a more detailed summary of this proposal, please see the abstract in the contribution document (which is not duplicated here for brevity).

In this proposal, POC values remain static for any picture, without further modification. However, EL pictures have two POCs: an ordinary POC and a "Layer POC".

The normal POC LSB is sent and must be cross-layer aligned for its values within each AU.

An additional Layer-POC LSB syntax element is also sent when the base layer had an IDR, and can also be sent when the base layer did not have an IDR. The POC MSB is reset upon the occurrence of a base-layer IDR picture. The only form of reset is a global reset from the base layer that marks all pictures as "unused for reference" and is a flushing point in output order (NoClrasOutputFlag, caused by external means or IDR with a new flag in the extended SH or a BLA).

For a given pair of pictures within a CVS, the difference between their ordinary POCs and between their Layer POCs will be equal.

EL RPSs use the layer-wise POC for picture identification.

Sec. 3 (of the -v3 version of the contribution) discusses evaluation criteria discussed at the Geneva meeting in BoG report JCTVC-O0349/JCT3V-F0248, and generally asserts that it fulfils the evaluation criteria favorably. (The asserted shortcomings of the -v2 version were asserted to be fixed by the -v3 version.)

It was asked whether the proposal effectively supports the case of uncoordinated simulcast multiplexing, and the contributor asserted that it does.

### Discussion of JCTVC-O0176 (by R. Sjöberg, J. Samuelsson, and R. Yu / Ericsson)

In this contribution, a mechanism is proposed that is reported to ensure that the POC values of all pictures of each access unit are the same. The contribution targets specifically the layerwise start-up use case where IRAP pictures and non-IRAP pictures are mixed in the same access unit. For a more detailed summary of this proposal, please see the abstract in the contribution document (which is not duplicated here for brevity).

Version 3 of contribution JCTVC-O0176 was discussed.

Each picture has only one POC value (in contrast with the JCTVC-O0275 / JCT3V-F0092 scheme described above, for which each picture has two POC values). That POC value is the same as the v1 base layer POC, and is reset when the base layer has an IDR.

POC LSBs are signalled for IDR pictures in ELs.

It was remarked that a lost base-layer IDR picture would be a problem in this scheme, although the contributor suggested that this case may not be so important (as the loss of a base-layer IDR might have severe consequences in typical expected use cases anyway).

The proposal included a constraint that a base layer picture must be present in the AU whenever there is an EL IDR picture in the AU (and that base layer picture cannot be a RASL, RADL, or sub-layer non-reference).

The proposal also included a constraint that no BLA pictures are allowed with layer ID greater than zero. One mentioned use case for this scenario was a splice point where the pictures in all layers are CRA pictures, which could hypothetically be changed to BLA pictures when splicing. Also, simulcast streams was another mentioned scenario.

Uncoordinated simulcast is not fully enabled by the proposed scheme, w.r.t. IDR pictures in the base layer.

This proposal seemed to have somewhat lesser flexibility and generality than the JCTVC-O0275 / JCT3V-F0092 proposal.

It was suggested that not requiring POC alignment outside of a dependency tree might be a way to deal with that.

It was remarked that removing that requirement might cause other problems such as an AU boundary detection problem.

### Other discussions in first teleconference

It was suggested that an SEI message could help with the loss resilience aspect that is discussed as the first problem described in section 2.2.2 above.

Contributions JCTVC-O0213 / JCT3V-F0073 (by A. K. Ramasubramonian, Y.-K. Wang, Y. Chen, and Hendry / Qualcomm, and M. Li and P. Wu / ZTE) and JCTVC-O0140 / JCT3V-F0053 (by B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, and C. Kim / Samsung) were reported to have a POC MSB reset that also addresses the third problem discussed in section 2.2.2 above.

Further discussion of these topics in the first teleconference was deferred in order to avoid running past the planned ending time for the teleconference.

## Second teleconference

The second teleconference was held at 0800 Seattle local time on Thursday, 21 November 2013 with a duration of 1.5 hours. The teleconference was hosted through Microsoft Lync, as with the previous call. Approximately 20 people attended the teleconference (with a peak of approximately 16 people on the call at once). The document sharing capability was used during the teleconference to display the presented contribution document and the meeting notes as they were being recorded.

In email on Tuesday 19 November, the chair had suggested that the second teleconference should continue the discussion of topics raised in the first teleconference, including:

- Consideration of JCTVC-O0213 / JCT3V-F0073 and JCTVC-O0140 / JCT3V-F0053,

- Revisiting any issues with the proposed schemes in JCTVC-O0275 / JCT3V-F0092 and JCTVC-O0176 that may have been identified in further study since the first teleconference,

- Discussing any other recent input that may need consideration by the AHG.

Due to lack of time for further discussion, only contribution JCTVC-O0213 / JCT3V-F0073 was actually discussed in depth during the second teleconference (along with some comments on its relationship with the previously-discussed JCTVC-O0213 / JCT3V-F0073).

A preliminary report of the second teleconference and planned topics for discussion for the third teleconference were sent by email shortly after the second teleconference (on Thursday 21 November, Seattle time).

### Discussion of JCTVC-O0213 / JCT3V-F0073 (by A. K. Ramasubramonian, Y.-K. Wang, Y. Chen, and Hendry / Qualcomm, and M. Li and P. Wu / ZTE)

This contribution proposed modifications to the signalling and derivation of picture order count in SHVC and MV-HEVC. It proposed that two flags indicating reset of POC LSB and POC MSB be signalled in the slice header, in place of the one flag that is currently signalled for the POC reset (so that either MSBs-only is reset or both MSBs and LSBs are reset).

A new version of this contribution had been submitted about a day and a half before the teleconference, and that latest version was discussed in the teleconference.

Additionally, a "POC reset information" SEI message was proposed in the first version of the contribution to help recover the POC derivation process when pictures containing POC reset information are lost. In the second and third versions of the proposal, the proposal to send additional SEI messages to provide loss robustness was replaced by an approach involving a "POC resetting period". The resetting period is based on a "POC resetting period ID". Each non-IRAP picture that belongs to an access unit that contains at least one IRAP picture must be the start of a POC resetting period in the layer containing the non-IRAP picture. In that access unit, each picture would be the start of a POC resetting period in the layer containing the picture. POC resetting and update of POC values of same-layer pictures in the DPB are applied only for the first picture within each POC resetting period. For derivation of the delta POC value used for updating the POC values of the same-layer pictures in the DPB as well as for derivation of the POC MSBs of the POC value of the current picture, a POC LSBs value is conditionally signalled in the slice segment header. A loss of a picture at the start of a POC resetting period can be overcome by having one or more same-layer pictures that follow that picture in decoding order and that are assigned the value 3 of the two-bit indication. The same process for support of missing-collocated-picture scenarios apply for resolving the error resilience issue.

To solve the problem of determining the persistence of SEI messages that have a persistence defined by POC values, the scope of the affected SEI messages was proposed to be restricted to be within a POC resetting period. Some details of this aspect were not discussed in the interest of prioritizing the available discussion time. See section 2.4.3 for further information on this aspect.

It was asked whether a "poorly-designed" encoder could omit the delta information to such an extent that it might construct a "fragile" bitstream that is not adequately robust to losses. It was remarked that some NOTEs or constraints in the specification might help address that issue.

It was asked whether the proposal effectively supports the case of uncoordinated simulcast multiplexing, and the contributor asserted that it does not have a problem with that, since the POC derivation within each layer using this proposal is independent of any information from any other layer.

At any moment within a CVS, each picture only has one POC, and all POC values are cross-layer aligned. However, the POC derived by a version 1 decoder can be different than the POC derived by a layer-aware decoder.

The proponent suggested that the key difference relative to other proposals is the independent derivation of POC values within each layer, and said that this provides loss resilience (or intentional removal of base-layer NAL units from the bitstream, or unconstrained simulcast with layer removal) benefit in the case where another layer is lost.

The proponent also asserted that the proposal would be better for layer-switching behaviour relative to JCTVC-O0275 / JCT3V-F0092 (by M. M. Hannuksela / Nokia) – e.g., if a layer is dropped for a substantial period of time and is then re-added – because the derivation of POC values for an IRAP picture in an EL would depend on the POC information of earlier pictures in that EL for the other scheme. The proponent of the other method said that the cross-layer alignment requirement of the other proposal (for the "regular" POC LSBs) would enable the EL POC value to be derived from the lower layer in that case. The proponent suggested that there may also be a problem with JCTVC-O0275 / JCT3V-F0092 in regard to frequent base-layer IDRs. Further study seemed necessary about this aspect.

The proponent of JCTVC-O0275 / JCT3V-F0092 asked about ARC / DRC downswitching/upswitching in relation to MSB wrapping. It was explained that the POC MSBs would be reset whenever any picture some pictures an AU are IRAP and some are not (e.g., for ARC upswitch). When all pics in an AU are IRAP pictures, the reset is not needed.

Reset of POC LSBs (in addition to MSBs) is only needed for base-layer IDR.

The proponent of JCTVC-O0275 / JCT3V-F0092 asked about the impact of POC resets on determination of output order for pictures in the DPB, as a reset can cause POC values to change – especially since the reset affects only the POC values in the same layer – which seems to affect cross-layer POC alignment regarding previously-decoded AUs when additional AUs are received in which some layers are not present. It was suggested that this might be a problem with the JCTVC-O0213 / JCT3V-F0073 proposal, and further study seemed necessary about this aspect (see notes below regarding additional version of JCTVC-O0213 / JCT3V-F0073 submitted later).

## Third teleconference

The third call was held at 2300 Seattle local time on Monday, 2 December 2013, with a duration of 2 hours (using Lync as previously). Approximately 20 people attended the teleconference (with a peak of approximately 15 people on the call at once). The document sharing capability was used during the teleconference to display the presented contribution document and the meeting notes as they were being recorded.

In email on Thursday 21 November, the chair had suggested that the third teleconference should continue the discussion of topics raised in the first two teleconferences, including:

- Consideration of JCTVC-O0140 / JCT3V-F0053,

- Revisiting any issues with the proposed schemes in JCTVC-O0275 / JCT3V-F0092, JCTVC-O0176, and JCTVC-O0213 / JCT3V-F0073 that may have been identified in further study since the second teleconference,

- Discussing any other recent input that may need consideration by the AHG.

Prior to the third teleconference, new versions of JCTVC-O0140 / JCT3V-F0053 (-v3), JCTVC-O0213 / JCT3V-F0073 (-v4), and JCTVC-O0275 / JCT3V-F0092 (-v4) were uploaded by their contributors, and these new versions were considered during the meeting discussions.

A preliminary report of the third teleconference and planned topics for discussion for the fourth teleconference were sent by email shortly after the teleconference, on Tuesday 3 December (Seattle time).

### Discussion of JCTVC-O0140 / JCT3V-F0053 (by B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, and C. Kim / Samsung)

Prior to the third teleconference, a new -v3 version of this contribution was uploaded, and this is the version that was discussed during the teleconference.

In order to improve the existing POC value alignment method with poc_reset_flag, a modification was proposed regarding POC reset. The modification was to propose replacing poc_reset_flag with poc_msb_reset_flag. The proposal suggested that POC LSB values should be reset only at IDR pictures in base layers.

The proponent of this contribution was not present on the call, except by text messaging, due to technical difficulties, and requested (by text) for the presentation of the contribution to be deferred to the fourth teleconference.

### Discussion of JCTVC-O0213 / JCT3V-F0073 (by A. K. Ramasubramonian, Y.-K. Wang, Y. Chen, and Hendry / Qualcomm, and M. Li and P. Wu / ZTE)

Prior to the third teleconference, a new -v4 version of this contribution was uploaded, and this is the version that was discussed during the teleconference.

In the -v4 of this document, the proposal was further revised to address comments received during the discussion of this document at the 2nd POC conference call on Nov. 21 as well as to fix two bugs that existed in -v3. The updated proposal was summarized as follows:

- Revised the POC update mechanism to update the POC values of earlier pictures of all layers in the DPB at the beginning of each access unit that requires POC reset (before the decoding of the first picture received for the access unit but after parsing and decoding of the slice header information of the first slice of that picture). However, POC derivation is still independent of information from any other layer, because it is required that the delta derived based on in-layer information is the same for all pictures in the access unit.

- To fix the two bugs, revised the condition for signalling of poc_lsb_val, such that it is also sent for base layer IDR pictures with poc_reset_idc equal to 2, and added the definition of FirstPicInLayerDecodedFlag[ i ] also for i equal to 0.

There were also some (generic) updates relating to synchronization with the latest MV-HEVC draft text.

The changes in the new version were asserted not to be major changes.

### Discussion of JCTVC-O0275 / JCT3V-F0092 (by M. M. Hannuksela / Nokia)

Prior to the third teleconference, a new -v4 version of this contribution was uploaded, and this is the version that was discussed during the teleconference.

Version 4 of the proposal includes the following ingredients:

- A "layer tree" is defined to consist of one independent layer (without any reference layers) and all layers that directly or indirectly use the independent layer as reference.

- Slice header extensions optionally contain signalling for a layer-tree POC value, which is intended to be used in the IRAP pictures of the independent layers and may be used, for additional error resilience, in other pictures too.

- A layer-tree POC value, LayerTreePicOrderCntVal is derived for each picture based on the mentioned slice header signalling, if present, and based on PicOrderCntVal differences, otherwise.

- Both the POC value and the layer-tree POC value are derived relative to the POC value and the layer-tree POC, respectively, of the previous TID0 picture on the current layer or any lower layer in the same layer tree on which the decoding of the current picture may depend. The layer-tree POC values of all pictures in the same layer tree in the same access unit are equal. This selection of the previous TID0 picture enables down- and up-switching of the number of layers, because the referenced TID0 picture is not chosen from the same layer, when the current picture is an IRAP picture in a dependent layer (i.e. layer up-switch point).

- The function DiffPicOrderCnt( ) is specified to use LayerTreePicOrderCntVal values.

Possibilities to combine certain aspects of the POC reset approach (JCTVC-O0213/JCT3V-F0073) to this contribution and vice versa are discussed. The following proposals were made:

- PrevPicOrderCnt derivation in the POC reset approach is proposed to be modified to happen among the current layer as well as direct and indirect reference layers. This is asserted to improve loss resilience.

- If it is preferred to include base-layer IDR/BLA picture loss detection means in the slice segment header syntax, enhancement-layer slice headers in this contribution can contain POC resetting information. This signalling shall be used for a picture that is in a different layer tree than the layer tree containing the base layer and which is the first picture, in decoding order, in a layer tree after a base-layer IDR or BLA picture. The signalling may also be used in other pictures for additional error resilience.

POC-related semantics were reported to have been found in four SEI messages:

- recovery point

- progressive refinement start

- region refresh

- structure of pictures information

It was proposed in JCTVC-O0213v4/JCT3V-F0073v4 that the use of the above-mentioned SEI messages would be constrained in scalable bitstreams so that the persistence would not cross a POC resetting period. However, it was suggested that such a constraint would disable the use of these SEI messages in a flexible manner. For example, a region refresh information SEI message of the base layer would not be allowed to span for multiple POC resetting periods, even if the POC reset were caused by an independent layer, e.g. an auxiliary picture layer.

Additionally, pan-scan rectangle, tone mapping information, frame packing arrangement, and display orientation SEI messages reportedly cease to persist when a picture with a greater POC value than that of the current picture are output.

It was remarked that issues relating to multi-layer usage would generally benefit from careful further study.

The contribution reported that it was unclear how non-static POC values are used with feedback-based error recovery. It was remarked that keeping such pictures within the same POC resetting period might suffice, and it was suggested that further study of this may be needed.

The proponent confirmed that with the prior version of the proposal, there could be an issue with layer switching. It was proposed that the derivation of PrevPicOrderCnt be made more robust by deriving it from the previous picture in the same layer or any direct or indirect reference layer (i.e., in a similar manner as prevTid0Pic is derived in this contribution).

In the discussion, it was remarked that hybrid (cross-standard in regard to the BL) scalability may have implications for how signals (e.g. POC resets) would be conveyed along with a BL. It was discussed whether the BL and EL could be mixed into a single conceptual "layer". The notion of potentially-distinct "layer trees" in the hybrid case was discussed.

It was suggested that it may be desirable to allow a single "layer tree" to contain multiple independent layers.

In its Fig. 2, the contribution illustrated an example where the POC reset approach (e.g., per O0213) becomes vulnerable to picture loss of BL pictures. prevPicOrderCnt update was suggested to fix this, relative to O0213, in the event that the basic approach of O0213 would be used. (See item 1 above.)

Vulnerability to loss of IDR/BLA in the BL was discussed, and the contribution noted the relevance of the temporal layer zero SEI message. However, the contribution also suggested that it would be possible to make the syntax more robust to this case without depending on using the SEI message. This was suggested not to be a fundamentally necessary aspect of the modified proposal, since the SEI approach could be applied alternatively. (See item 2 above.)

## Fourth teleconference

The fourth call was held at 1300 Seattle local time on Friday, 13 December 2013, with a duration of 2 hours (using Lync as previously). Approximately 15 people attended the teleconference (with a peak of approximately 14 people on the call at once). The document sharing capability was used during the teleconference to display the presented contribution document and the meeting notes as they were being recorded.

In email on Tuesday 3 December (Seattle time), the chair had suggested that the fourth teleconference should continue the discussion of topics raised in the first three teleconferences, including:

- Consideration of JCTVC-O0140 / JCT3V-F0053 (the discussion of which was deferred in the third teleconference),

- Revisiting any issues with the proposed schemes in JCTVC-O0275 / JCT3V-F0092, JCTVC-O0176, and JCTVC-O0213 / JCT3V-F0073 that may have been identified in further study since the second teleconference,

- Discussing any other recent input that may need consideration by the AHG.

A preliminary report of the fourth teleconference and planned topics for discussion for the fifth teleconference were sent by email shortly after the teleconference, on Friday 13 December.

### Discussion of JCTVC-O0140 / JCT3V-F0053 (by B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, and C. Kim / Samsung)

The contributors of this proposal were unable to attend the fourth teleconference. See prior notes in section 2.4.1. It was commented that reset of only the MSBs may not be sufficient when considering that the LSBs of the base layer IDR pictures are not sent. It was suggested that this proposal has some similarity to that of JCTVC-O0213 / JCT3V-F0073 (followed up by JCTVC-P0041 / JCT3V-G0031) in regard to desiring a layer-independent POC derivation. It suggested that having POC values derived within a layer is important for the following purposes:

- Simulcast (seems true)

- Error robustness (still vulnerable to losses within the current layer, although not so much to losses in lower layers in regard to POC although perhaps not to picture quality; and it was suggested that the vulnerability to losses in lower layers can be kept under control – e.g., not having vulnerability to losses as long as some lower layer pictures within a POC MSB cycle are not lost)

- Hybrid framework (AVC + HEVC) (seems like a potentially valid point, although there may be some way to deal with that)

- Minimizing inter-operations across layers (maybe true, but not necessarily important)

### Discussion of JCTVC-O0275 / JCT3V-F0092 (by M. M. Hannuksela / Nokia)

Prior to the fourth teleconference, a new -v5 version of this contribution was uploaded, and this is the version that was discussed during the teleconference.

Version 5 of the proposal updates the layer tree definition so that multiple independent layers in the same layer tree are enabled. A layer tree index is derived for each layer and it identifies which layer tree the layer belongs to.

Layer tree POCs are aligned within each layer tree.

The layer tree index is derived from the properties of the layers (not indicated by the encoder).

Layers in the same AU would be part of the same layer tree.

Discussed examples:

- Layers 0 and 1 are independent of each other, and layer 2 is dependent on both. Then all are part of the same layer tree.

- Layers 0 and 1 are independent of each other, and those are the only layers. They are then different layer trees. (But each picture would still have an ordinary POC – the values of which would stay aligned.)

- Layers 0 and 1 are independent of each other, and layer 2 is dependent on both, but layer 2 has been removed from the bitstream. They are still part of the same layer tree, unless the VPS is modified, because the VPS still expresses the potential dependency of the removed layer on the layers that remain.

It was remarked that there are some circumstances (e.g., when the base layer has an IDR or BLA or CRA with NoRaslOutputFlag equal to 1) for which the layer tree POC information presence flag in the PPS would need to be required to be equal to 1 to force the layer tree POC to be sent in the SH. This was acknowledged by the proponent as something needing to be fixed/clarified.

### Discussion of JCTVC-P0041 / JCT3V-G0031 (by Hendry, A. K. Ramasubramonian, Y.-K. Wang, Y. Chen / Qualcomm, and M. Li and P. Wu / ZTE)

This contribution was uploaded shortly before the fourth teleconference, as a follow-up on the prior proposal JCTVC-O0213 / JCT3V-F0073.

The main changes compared to the proposal in JCTVC-O0213-v4 are as follows:

- In output order conformant decoders, it is proposed to output all earlier pictures in the DPB upon receiving a POC reset picture.

  - It is asserted that by doing this, the problem raised at the 2nd POC conference call, for the solution in JCTVC-O0213-v3, about possibility of having erroneous order of output of pictures is addressed.

- Revert the timing of decrement of POC of earlier pictures in the DPB to that of described in -v3 of the proposal, that is, POC decrement of earlier pictures in the DPB is done in a layer-specific manner.

  - It is asserted that the combination of outputting all earlier picture in the DPB upon receiving a POC reset picture and decrementing POC of earlier pictures in the DPB only of pictures in the same layer as the current layer addresses the problem raised at the 3rd POC conference call, for the solution in JCTVC-O0213-v4, about incorrect POC value decrement in case of down-switching and up-switching and with picture loss.

- Propose to signal POC MSB information in slice header extension when current picture is a CRA or BLA picture.

  - The signalled POC MSB information is used for derivation of POC MSB when current picture is a CRA picture with NoRaslOutputFlag equal to 0 for any conditions and derivation of previous POC MSB for pictures when POC reset is applied at CRA picture.

  - This provides two asserted benefits:

    - The first is to allow correct derivation of POC in some use-cases such as switching down and up, and pseudo-single-loop-decoding where in the base layer only CRA pictures are used for inter-layer prediction and present.

    - The second is to allow correct derivation of POC for trick-mode with CRA pictures, including changing from CRA-based trick-mode to normal playback mode or reduced speed-up ratio e.g adding TemporalId-zero pictures.

It was suggested that RASL pictures might be a problem in regard to output order (see text near nuh_layer_id semantics), because it was suggested that RASL pictures are not constrained to follow, in output order, the trailing pictures that precede them in decoding order. It was remarked that if this is a problem, one potential way to solve it would be to prohibit strange RASL positions in multi-layer bitstreams.

It was remarked that the revised proposal was a significant change relative to the prior proposal.

### General discussion

At the fourth teleconference, the AHG seemed to be converging on primarily considering the Nokia (JCTVC-O0275 / JCT3V-F0092) and Qualcomm / ZTE (JCTVC-P0041 / JCT3V-G0031) proposals.

Cons of intra-layer derivation (Qualcomm / ZTE) proposal:

- POCs are not static (e.g., for external metadata or SEI or feedback-based loss recovery referencing)

- RASL pictures might have an ordering issue (may not be a significant issue, since it is not clear that the ordering freedom is needed)

Cons of layer-tree derivation (Nokia) proposal:

- The "re-writing" case where all lower layer pictures other than IRAP pictures are intentionally removed might not function properly if there is a long duration between base layer IRAP pictures (could potentially be patched, not an inherent problem in the approach)

- Multiple POCs to keep track of (perhaps a bit confusing to learn how to implement, but not a clearly significant problem)

- Loss of a lower-layer picture could have an impact (may not be a significant issue, as long as some lower layer pictures within a half POC MSB cycle are present)

Hybrid scalability impact?

- Unsure, but intra-layer POC derivation might possibly be preferable since EL does not depend on POC of base layer picture.

## Fifth teleconference

The fifth call was held at 0800 Seattle local time on Friday, 20 December 2013, with a duration of 2 hours (using Lync as previously). Approximately 10 people attended the teleconference (with a peak of approximately 9 people on the call at once). The document sharing capability was used during the teleconference to display the presented contribution document and the meeting notes as they were being recorded.

In email on Friday 13 December (Seattle time), the chair had suggested that the fifth teleconference should focus primarily on the two primary proposals (JCTVC-O0275 / JCT3V-F0092 and JCTVC-P0041 / JCT3V-G0031), identify any remaining issues with them, and see whether a consensus can be reached on recommending adoption of one of them.

A preliminary report of the fifth teleconference and planned topics for discussion for the sixth teleconference were sent by email shortly after the teleconference, on Friday 20 December.

### Discussion of JCTVC-P0041 / JCT3V-G0031 (by Hendry, A. K. Ramasubramonian, Y.-K. Wang, Y. Chen / Qualcomm, and M. Li and P. Wu / ZTE)

New versions (-v4 and -v5) of this contribution were uploaded shortly before the fifth teleconference, as a further follow-up on the prior proposal JCTVC-O0213 / JCT3V-F0073.

In v4 of JCTVC-P0041/JCT3V-G0031, the following changes were made, and the text changes are included in the attachment, with changes marks in relative to the attachment in v3 of JCTVC-P0041/JCT3V-G0031 (and the old change marks are also kept, with different user names):

The new version added a bitstream constraint to disallow a picture that follows a POC-resetting picture in decoding order to precede, in output order, another picture that precedes the POC-resetting picture in decoding order. It was asserted that this would also address the issue raised at the fourth POC conference call regarding output order of RASL pictures of an IRAP picture and the trailing pictures preceding the IRAP picture.

The semantics of the following SEI messages for which some of the semantics depend on POC values, were proposed to be updated to ensure that the SEI messages work with the resetting-based POC design:

- pan-scan rectangle SEI message

- recovery point SEI message

- progressive refinement segment start SEI message

- film grain characteristics SEI message

- tone mapping SEI message

- frame packing SEI message

- display orientation SEI messages

- structure of pictures SEI message

- region refresh SEI message

The new revision also proposed a modification to POC derivation of CRA/BLA pictures to always use the signalled POC MSB, as a bug fix to v3 of JCTVC-P0041/JCT3V-G0031.

For identification of a picture in feedback messages, the revised contribution suggested that, when operating in the context of an SHVC or MV-HEVC profile, in addition to the POC value, the POC-resetting period ID of the latest decoded picture would also be signalled in a feedback message. The encoder could then uniquely identify the previously encoded picture. Upon reception of a feedback message with a POC value and a POC-resetting period ID, when the latest encoded picture is in a different POC-resetting period, it would track back to the signalled POC-resetting period and add back the POC delta value decremented for each new POC-resetting period. No spec text change for this aspect was provided (as this aspect would be outside the scope of the video coding standard).

### Discussion of JCTVC-O0275 / JCT3V-F0092 (by M. M. Hannuksela / Nokia)

New versions (-v6 and -v7) of this contribution were uploaded shortly before the fifth teleconference.

An additional contribution JCTVC-P0043/JCT3V-G0032 on access unit boundary determination (for which the use of POC has been earlier mentioned as one alternative, was also uploaded shortly before the fifth teleconference.

Versions 6 and 7 of the contribution included the following changes relative to version 5:

- The POC MSB cycle is conditionally signalled in the slice segment header extension enabling its use also in base-layer slices (whereas in version 5 the POC MSB cycle was signalled within the slice segment header for enhancement layer slices only). It is asserted that this change enables pruning of the base layer so that it only contains IRAP pictures, for which the POC difference can be greater than MaxPicOrderCntLsb / 2. Such pruning can be used for example when only IRAP pictures of the base layer may be used as references for inter-layer prediction (as indicated by max_tid_il_ref_pics_plus1[ i ][ j ]).

- The layer-tree POC MSB cycle is conditionally signalled in the slice segment header extension for additional error resilience and for enabling independent layer-tree POC derivation for each of multiple independent layers in the same layer tree.

- Inter-layer dependencies in POC and layer-tree POC derivation are constrained further as follows:

  - The POC reference picture prevTid0Pic is selected from direct reference layers (rather than from direct or indirect reference layers as in version 5).

  - A non-IRAP picture from a direct reference layer is not prevTid0Pic, when max_tid_il_ref_pics_plus1[ i ][ j ] indicates that only IRAP pictures of the reference layer may be used as references for inter-layer prediction.

  - In version 5, when a layer tree contained more than one independent layer, TID0 pictures of the independent root layer (with the smallest value of nuh_layer_id among all independent layers of the layer tree) could serve as prevTid0Pic. In version 6, pictures of an independent root layer are no longer candidates for prevTid0Pic (unless the independent root layer is the current layer or a direct reference layer for the current layer).

As a consequence, it was asserted that POC and layer-tree POC derivation have the same or a subset of the reference pictures than what inter-layer prediction has.

It was noted that the general spirit of the changes included a reduction of inter-layer dependencies.

Possibilities to combine certain aspects of the POC reset approach (JCTVC-P0041v3/JCT3V-G0031v3) to this contribution and vice versa are discussed. The following proposals were made:

- Instead of deriving POC values from poc_msb_val in enhancement layers with reference layers, it is proposed to derive them from the TID0 pictures of their reference layers. This is asserted to improve loss resilience, be robust against all kinds of layer switching, and reduce bitrate overhead.

- POC MSB signalling similar to that of JCTVC-P0041v3/JCT3V-G0031v3 has been incorporated in this contribution.

### General discussion

Regarding item 1 above, it was asked whether the discussed behaviour would violate version 1 behaviour expectation – and basically, there would be some problems with this – since a version 1 decoder would see decreasing POCs on consecutive CRA pictures – which is a situation that would require an "end of bitstream" NAL unit or BLA interpretation (NoRaslOutputFlag equal to 1). Such an issue exists in both discussed proposals. A suggestion was simply for the encoder to send more LSBs. It was remarked that the proposed changes relating to this (for each proposal) might help with advanced trick play or use with beyond-generation-1 decoders even if constraints are specified to not allow violation of version 1 behaviour at the base layer.

It was suggested to check whether version 1 disallows having decreasing POC LSBs on consecutive CRA pictures within a CVS, and to consider corrigendum action to disallow this if it does not.

JCTVC-O0275 / JCT3V-F0092 suggested that when using the other proposal, post-processing units would need the same sort of POC update functionality as found within the decoding process. With POC resetting, when a picture is output, it may have a different POC than when it was decoded, and the DPB can simultaneously contain pictures with different POC resetting periods. The other proponent commented that this should not matter, since output of pictures with earlier POC resetting periods would already be performed for any pictures in the DPB when operating for output order conforming behaviour. It was suggested that the POC output with a picture should always be the POC that applied within its associated POC resetting period, and that this would be sufficient, and would be similar to the ordinary need to track output pictures across different CVSs.

It was suggested that the first part of item 3 of the Introduction section of JCTVC-O0275 / JCT3V-F0092 may be mistaken due to a resetting constraint specified in the other proposal. The proponent indicated that further study to confirm this would be done offline (see additional notes below on follow-up).

Item 3 of the Introduction section of JCTVC-O0275 / JCT3V-F0092 also discussed the position of layer down-switching relative to POC resetting points, and asserted that there could be a problem with this. The problem relates to having different types of IRAPs in the same AU. It was commented that the given example would violate a POC range constraint specified in the bitstream conformance constraints in subclause C.4 (applying to pictures that have not been output and pictures that are short-term pictures), and this problem should not be possible when obeying this constraint.

Section 4 of JCTVC-P0041 / JCT3V-G0031, some asserted issues with JCTVC-O0275 / JCT3V-F0092 were asserted to be relevant. The first two of these would seem to require some clarifications and constraints. Regarding AU boundary detection, a loss resilience issue was asserted to exist.

The two proposals have each become more mature, and neither appeared to have a fundamental flaw.

Non-proponents remarked that both schemes seem quite complete and viable at this point.

One non-proponent indicated a slight preference for the Nokia proposal due to the other proposal needing to have different interpretations of POC for the base layer, while another suggested that the Qualcomm proposal had the nice property of not needing to track multiple POCs within the decoding process.

Information on how to handle an AVC base layer was requested.

Maturity of the text was also suggested to be important, and had not really been discussed – although both proposals had proponents with substantial experience of text editing.

## Sixth teleconference

The sixth call was held at 2100 Seattle local time on Thursday, 2 January 2014, with a duration of 2 hours (using Lync as previously). Approximately 10 people attended the teleconference (with a peak of approximately 9 people on the call at once). The document sharing capability was used during the teleconference to display the presented contribution document and the meeting notes as they were being recorded.

In email on Friday 20 December (Seattle time), the chair had suggested that the sixth teleconference should continue to focus primarily on the two primary proposals (JCTVC-O0275 / JCT3V-F0092 and JCTVC-P0041 / JCT3V-G0031), identify any remaining issues with them, and see whether a consensus can be reached on recommending adoption of one of them. Maturity of draft text was suggested to be part of that discussion.

A preliminary report of the sixth teleconference was sent by email shortly after the teleconference, on Thursday 2 January.

Updates of the prior proposals JCTVC-O0275 / JCT3V-F0092 (now revised as JCTVC-P0056 / JCT3V-G0042) and JCTVC-P0041 / JCT3V-G0031 had been uploaded shortly before the call. The changes in each of these, relative to their prior versions, were asserted to be minor.

### Follow-up on prior discussions

As follow-up to discussions in the previous teleconference, the following remarks were made:

- It was confirmed that there is a constraint in Annex C that prevents a problem reported in the previous version of JCTVC-O0275 / JCT3V-F0092.

- It was remarked that there are constraints in version 1 that should prevent the problem of decreasing POC between consecutive CRA pictures within a CVS that was discussed in the previous call. Removal of some pictures from the bitstream in a manner that would cause this to occur would be a violation of the base layer conformance requirements, and it would be the responsibility of a process that does this to not do it if that would produce a non-conforming bitstream for a version 1 decoder.

### Discussion of text issue in JCTVC-P0056 / JCT3V-G0042

An aspect of the text for JCTVC-O0275 / JCT3V-F0092 (now revised as JCTVC-P0056 / JCT3V-G0042) was discussed, that the text requires the decoder to check the content of a VPS to determine whether that content changed or not. It was explained that the intent is that the check should consider the content to be the same even if the VPS ID is different.

The proponent indicated that this could be changed to impose a constraint on the content of the bitstream to set the cross-layer BLA flag to 1 under some circumstances to avoid this need for a decoder check.

### Comments in JCTVC-P0056 / JCT3V-G0042

Section 1 of JCTVC-O0275 / JCT3V-F0092 (now revised as JCTVC-P0056 / JCT3V-G0042) included comments about constraints imposed by the other proposal.

The first two of these issues seemed rather minor, as it was not clear whether the discussed constraints would ever be an issue in a useful configuration. The third was in regard to temporally interleaved independent layers, noting that a POC reset would not be allowed to occur under some circumstances involving RASL or RADL pictures in a usage in which temporal alignment of the IRAP pictures in the different layers cannot apply. The third case seemed the most interesting of the three, but it was not clear that it was really important to a practical use case. It was noted that the presence of a CRA in an enhancement layer does not require POC reset. Overall, it did not seem that this issue was very significant.

Issues four and five were basically issues that had been discussed in prior calls and it seemed could be adequately addressed without significant technical problems.

Section 4 responds to comments made in the other contribution. Its item 3 discussed AU boundary detection and the potential for slice_pic_order_cnt_lsb values to be the same in adjacent pictures.

Its item 4 discusses SEI messages and nesting. It was acknowledged that some scenarios might have somewhat more overhead with the JCTVC-P0056 / JCT3V-G0042 approach because of this.

There was insufficient time to discuss all items in detail; however, the general nature of the discussed issues seemed adequately established.

### General discussion

It was suggested and acknowledged that the text for JCTVC-P0056 / JCT3V-G0042 seemed somewhat less mature than the text for the other proposal, such that some small issues would need correction/clarification.

The distinction in philosophy between the two approaches was summarized as follows:

- Independent derivation of POC in each layer (with resets) versus

- Keeping POC values static for ease of referencing of their values and not modifying the POC of the base layer.

Examples cited by its proponent as potentially advantageous for the resetting approach were "gradual decoder refresh" (GDR, with long strings of non-IRAP pictures) and "video redundancy coding" (VRC, essentially temporal interleaving of simulcast).

The scheme with resets was suggested by its proponent to be more robust to data losses (esp. of base layer IDR pictures). However, the proponent of the other scheme somewhat disputed this – saying that at least the relative POC values should be possible to derive in the GDR case and that as long as the gaps in POC values were not extremely long, the VRC case would also work.

To some extent the Nokia contributor indicated that it was expected that systems would often provide picture boundary detection functionality by external means.

Again, both schemes continued to appear workable and it appeared that either could be used as the basis for moving forward.

The Qualcomm/ZTE approach seems to have better draft text and potentially better resilience in some usage scenarios (although this latter aspect does not seem entirely clear). Although the POC resetting in that scheme may be a bit counter-intuitive for POC value computation and referencing purposes, it seems manageable and does not really seem to cause a technical problem.

Thus, if we must choose one scheme to suggest to move forward with at this point, given our current state of knowledge and the maturity of the proposals, it would seem to be the Qualcomm/ZTE scheme.

# AHG conclusions and recommendations

Pending further input, the AHG concluded that the Qualcomm/ZTE scheme of JCTVC-P0041 / JCT3V-G0031 seems the most mature as the basis for moving forward. It has more mature text, has no significant identified problems, and is asserted to have a resilience advantage. It would be desirable to have that resilience assertion more confidently assessed; however, no significant problem seems to exist in that scheme, so its selection is recommended by the AHG unless additional input indicates otherwise.

The AHG noted that software verification has not been conducted to really show the operation of the scheme in practice with important test cases. Follow-up on this issue is needed, as much as possible, to increase our confidence that the scheme has all details worked out and would not have any problems in important use cases.

Further study is also needed to determine what is needed for cross-codec multi-layer operation.

Additional new contributions are noted to be relevant, including the following:

- JCTVC-P0067 / JCT3V-G0045 MV-HEVC/SHVC HLS: Comments on POC alignment [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

- JCTVC-P0260 / JCT3V-G0224 MV-HEVC/SHVC HLS: Additional information on the POC design in JCTVC-P0041/JCT3V-G0031 [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)] [late-registered; not yet available as of preparation of this report]