| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0012 |
| --- | --- |

| Title: | JCT-VC AHG report: SHVC software development (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Ad-hoc group report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Yong He Do-Kyoung Kwon | Email: | vseregin@qti.qualcomm.com Yong.He@InterDigital.com d-kwon@ti.com |
| Source: | AHG12 chairs |  |  |

_____________________________

# Abstract

This report summarises activities of the AHG12 on SHVC software development between 15th and 16th JCT-VC meetings.

# Introduction

| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 4.0 software (based on HM 12) for experimentation. - Provide software to CEs within two weeks after the meeting. - Generate anchors and template based on common test conditions. - Bring software into alignment with HM 12.1 by the next meeting. - Discuss and identify additional issues related to SHVC software. | V. Seregin (chair), Y. He, D. Kwon (vice-chairs) | N |
| --- | --- | --- |

The current latest software version is SHM-4.1 contains almost all the items adopted last meeting, however some work is still needed on signaling POC LSB in enhancement layer IRAP pictures related to JCTVC-N0065.

SHM software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/

The software issues can be reported using bug tracker https://hevc.hhi.fraunhofer.de/trac/shvc

# Issues

Despite several emails sent to the reflector and private messages sent to the proponents, the following items have not been integrated:

- JCTVC-O0109 (Samsung)

  - Move the flag to VPS VUI (see notes relating to O0199 to clarify).

  - Modify the VPS extension syntax and semantics to replace view_id_len_minus1 with view_id_len, always signal that syntax element, add a constraint that (1<<view_id_len) >= NumViews, and modify view_id_val semantics to infer value of 0 when not present

  - Move the vps_vui_present_flag to precede vps_vui_offset, and make vps_vui_offset conditional on that flag.

  - To change default_one_target_output_layer_flag to a two-bit default_one_target_output_layer_idc, and reserve the values 2 and 3

  - Modify the semantics of profile_ref_minus1[ i ] to replace “shall be less than i” with “shall be less than or equal to i”.

- JCTVC-O0135 (Samsung)

  - Relating to section 6, regarding auxiliary picture ID as part of the definition of the semantics of default_one_target_output_flag, adopt first variant.

- JCTVC-O0164 (Nokia/Sony/Arris)

  - It was suggested to constrain the stalling based on the relative cpb removal times, which must be in decoding order. The "du_based_bpb_sync_flag" is not needed, in view of this. SEI in the highest layer of the layer set or (inclusive "or") VPS VUI is used to carry the parameters (at encoder discretion). SEI in higher layer and SEI in VUI do not need to repeat information available in some lower layer. Shall be after APS SEI and buffering period SEI and before all other SEI of all layers except other HRD related SEI.

- JCTVC-O0137, JCTVC-O0200, JCTVC-O0223  (Samsung/HHI/Qualcomm)

  - The value of nuh_layer_id shall be in the range of 0 to 62. The value of 63 for nuh_layer_id is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 63 for nuh_layer_id in a NAL unit." and specify that vps_max_layers_minus1 shall not be equal to 63, but decoders shall allow that value to appear in the bitstream. Specify that the value 63 is interpreted the same as the value 62 (e.g., MaxLayersMinus1 = Min( 62, vps_max_layers_minus1) and subsequently refer to MaxLayersMinus1 instead of vps_max_layers_minus1).

It is expected to be completed in the future software versions.

# Software versions

Two software versions have been released by AHG12, integration details and performance summary are given in the next subsections. In the document, only HEVC base layer results are provided and AVC base layer data can be found in the accompanying excel tables. Performance results are consistent with the adopted techniques.

## SHM4.0

Software version SHM4.0 based on HM12 was released according to the schedule and the following items have been integrated into this version:

- Ticket #5: fix for align all_ref_layer_active_flag

- Ticket #6: fix for multi-slices coding

- Ticket #7: remove a brace

- Calculation of VPS extension offset

- JCTVC-O0216: encoder constraint for motion field mapping

- JCTVC-O0031: Arbitrary Spatial Ratio (ASR) with filters as documented in tables 2 and 3

- JCTVC-O0215: a VPS flag to specify the phase alignment between layers (zero or center phase shift) for up-sampling process

- JCTVC-O0194: support different bit-depth values for different layers, enable weighted prediction for ILR for color gamut scalability

- JCTVC-O0097: remove FINAL_RPL_CHANGE_N0082

- JCTVC-O0041: auxiliary picture layers

- JCTVC-O0062: signal poc_lsb_not_present_flag for each layer in VPS extension

- JCTVC-O0225: proposal 2 regarding signalling of max_tid_il_ref_pics per layer

- JCTVC-O0223 proposal 2, JCTVC-O0139 proposal 5: removal of IRAP align dependency constraints on poc_reset_flag and move IRAP align flag to VPS VUI

- JCTVC-O0223 proposal 4, JCTVC-O0139 proposal 2: on activation of a new SPS by an enhancement layer non-IRAP picture

Description of the common test conditions and anchor data were released as an output document JCTVC-O1009 “Common SHM test conditions and software reference configurations”.

Table1. SHM4.0 against SHM3.0.1 with CTC

## SHM4.1

The listed below adopted items have been integrated on top of SHM4.0 version, software base was updated to HM12.1 and was released as SHM4.1.

- JCTVC-O0071: JSVM based SHVC downsampler

- JCTVC-N0371: inferring of the scaling list

- JCTVC-O0217: layer-specific DPB

- JCTVC-O0223: add a flag in VPS VUI to indicate cross layer pic type alignment

- JCTVC-O0225: change derivation of NumActiveRefLayerPics to consider max_tid_il_ref_pics

- JCTVC-O0226: Modifications to the VUI indicators of tile and WPP alignment related syntax elements

- JCTVC-O0142: a syntax elements structure for the extensions

- JCTVC-O0149: proposal 1, a base-layer IRAP picture that initiates the layer-wise start-up process (i.e. has NoClrasOutputFlag equal to 1) causes marking of all pictures in the DPB as “unused for reference”.. Proposal 2, a new slice_reserved_flag is taken into use to indicate if a base-layer IDR picture initiates the layer-wise start-up process.

- JCTVC-O0059,  JCTVC-O0092: restrict sharing of SPS and PPS across layers to avoid creating problems during sub-bitstream extraction

- JCTVC-O0179: add flag in rep_format( ) syntax structure to control sending of chroma and bit depth related parameters

- JCTVC-O0094: restrict maximum number of direct and indirect reference layers to be 7

- JCTVC-O0096: modify the SPS syntax for layers with nuh_layer_id > 0 to signal a reference to a rep_format index in the VPS. Add a gating flag in VPS extension to condition the presence of direct dependency type, with a default type signalled

- JCTVC-O0120: option 2, add syntax elements to signal max temporal sub-layers for each layer in the VPS, with a gating flag

- JCTVC-O0153: add a flag in the VPS to specify whether the highest available layer shall be output if the target output layer is not available

- JCTVC-O0118: add visual signal information (video_format, video_full_range_flag, colour_primaries, transfer_characteristics, matrix_coeffs) per layer to the VPS VUI

- JCTVC-O0120: modify inter-layer reference picture list default construction to incorporate max temporal sub-layers per layer syntax elements in VPS extension

- JCTVC-O0098: modify signalling of scaled reference layer offsets to allow signaling of any lower layer, rather than just a direct reference layer, in order to enable alignment of auxiliary pictures

- JCTVC-O0199: adding a flag in VPS VUI for indication of skipping enhancement layer IRAP picture when single_layer_for_non_irap_flag is equal to 1

- JCTVC-O0117: modification of the PicOrderCntVal of prevTid0Pic and modification to the decoding process for reference picture set, to address problems found for cross-layer POC alignment

Table2. SHM4.1 against SHM4.0 with CTC

## SHVC downsampler

According to the decision made last meeting, the informal downsampling tool was updated with JSVM based SHVC downsampling from JCTVC-O0071 which supports arbitrary spatial ratio and can be found as a standalone project in the SHM software package.

The downsampler generates reasonable output only in the scalability ratio range of approx. 1.3x ... 2.2x.

The supported command line and parameters can be found in the software itself and are also summarised in below.

Usage: TAppDownConvert <win> <hin> <in> <wout> <hout> <out> [<t> [<skip> [<frms>]]] [[-phase <args>] ]

win     : input width  (luma samples)

hin     : input height (luma samples)

in      : input file

wout    : output width  (luma samples)

hout    : output height (luma samples)

out     : output file

Optional parameters:

t       : number of temporal downsampling stages (default: 0)

skip    : number of frames to skip at start (default: 0)

frms    : number of frames wanted in output file (default: max)

-phase <in_uv_ph_x> <in_uv_ph_y> <out_uv_ph_x> <out_uv_ph_y>

in_uv_ph_x : input  chroma phase shift in horizontal direction (default:-1)

in_uv_ph_y : input  chroma phase shift in vertical   direction (default: 0)

out_uv_ph_x: output chroma phase shift in horizontal direction (default:-1)

out_uv_ph_y: output chroma phase shift in vertical   direction (default: 0)

# Development plan and recommendations

Continue to develop reference software based on SHM-4.1 and improve its quality.