| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0031 |
| --- | --- |

| Title: | SCE1: Summary Report of Colour Gamut and Bit Depth Scalability |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | CE report input to JCT-VC |  |  |  |
| Purpose: | CE Summary |  |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Yan Ye, Elena Alshina Xiang Li, Seung-Hwan Kim, Alberto Duenas, Kemal Ugur, Kazushi Sato | Emails: | philippe.bordes@technicolor.com, yan.ye@interdigital.com, elena_a.alshina@samsung.com, lxiang@qti.qualcomm.com, kimse@sharplabs.com, alberto@ngcodec.com, kemal.ugur@nokia.com, kazushi.sato@jp.sony.com |  |
| Source: | CE coordinators |  |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in SCE1 on Colour Gamut and Bit Depth Scalability. The evaluated tools and the test conditions are defined in document JCTVC-O1101.

# Introduction

Color Gamut Scalability has been mentioned in the requirements of Scalable Coding Extension of HEVC. It allows addressing the cases the original Enhancement Layer uses a different color gamut than the Base Layer. This can be useful for instance in case of deployment of UHD services compatible with legacy HD devices: HD is using the Rec.709, while UHD is likely to use some of the parameters defined in the Rec.2020.

The general diagram of a scalable video encoder including a prediction tool for color differences between the base layer (BL) and enhancement layer (EL) is shown in Figure 1.

Figure 1: Color Space Scalable Encoder.

# Tests conditions

The color graded sequences are provided by AHG14 [2] and the test conditions are described in SCE1 description [1].

The sequences with the associated license conditions are available at the Hannover FTP site: ftp.tnt.uni-hannover.de/scalable/sequences/CGS with scalability use credentials.

Table 1: List of test sequences.

| Test Sequences | size | fps | Duration | Bit-depth |
| --- | --- | --- | --- | --- |
| BT709_Birthday_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_BirthdayFlashPart1_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_BirthdayFlashPart2_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_Parakeets_1920x1080_50_zerophase_0.9pi.yuv | 1920 x 1080 | 50 | 250 | 8 |
| BT709_TableCar_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_Birthday_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_BirthdayFlashPart1_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_BirthdayFlashPart2_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_Parakeets_1920x1080_50_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 50 | 250 | 10 |
| BT709_TableCar_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_Birthday_3840x2160_60_10bit_zerophase_0.9pi.yuv | 3840 x 2160 | 60 | 300 | 10 |
| BT2020_BirthdayFlashPart1_3840x2160_60_10bit_zerophase_0.9pi.yuv | 3840 x 2160 | 60 | 300 | 10 |
| BT2020_BirthdayFlashPart2_3840x2160_60_10bit_zerophase_0.9pi.yuv | 3840 x 2160 | 60 | 300 | 10 |
| BT2020_Parakeets_3840x2160_50_10bit_zerophase_0.9pi.yuv | 3840 x 2160 | 50 | 250 | 10 |
| BT2020_TableCar_3840x2160_60_10bit_zerophase_0.9pi.yuv | 3840 x 2160 | 60 | 300 | 10 |

Reference software (anchors):

The reference software (anchors) is SHM4.0_irap_2013_11_28. It was built from SHM4.0 and was provided by Technicolor to the SCE1 participants 10 days after the SHM4.0 release. The SHM4.0 allows to enable Weighted-Prediction and bit-depth increase on ILR only with macros O0194_WEIGHTED_PREDICTION_CGS and O0194_DIFFERENT_BITDEPTH_EL_BL respectively.

In SHM4.0_irap_2013_11_28, additional functionalities have been implemented, allowing to insert IRAP with SPS, PPS at regular timing interval. An additional parameter SpsPpsPeriod (or “-SpsPpsPeriod” in the command line argument) allows to set the IRAP periodicity as a multiple of the frame rate (modulo 8 as for IntraPeriod), for RA or AI configurations. For example, “-SpsPpsPeriod 1” will insert a IRAP every 48 frames for 50Hz, every 64 frames for 60Hz, etc…

The common SHVC test conditions (QPs) have been used for AI and RA configurations, 2x scalability. The IRAP periodicity has been set to 1 second, which corresponds to realistic TV broadcast IRAP periodicity value.

Description of the experiments:

Two tests (A and B) have been conducted. Test A includes bit-depth increase between BL (8-bit) and EL (10-bit) and test B uses 10-bit content for BL and EL (Table 2).

Table 2: Test conditions.

|  | Base layer | Enhancement layer |
| --- | --- | --- |
| Test A | 1920 x 1080, BT. 709, 8-bit | 3840 x 2160, BT.2020, 10-bit |
| Test B | 1920 x 1080, BT. 709, 10-bit | 3840 x 2160, BT.2020, 10-bit |

Given remarks of this 15th Meeting report of the JCTVC, at the encoder side, the computation of the LUT shall not induce any frame delay. For instance, it can be computed on a picture-basis or RAP-period-basis. Then, two use cases (UC1 and UC2) have been defined [1]:

- Use case 1: LUT derived from the first picture of the sequence. In this case, we will use the regular SHM4.0 software with one single SPS, PPS at the beginning.

- Use case 2: LUT derived using one or several pictures of the previous RAP period. In this case, the modified software SHM4.0_irap will be used, with regular SPS, PPS insertion.

Table 3 : List of the tests (AI, RA configurations) and corresponding software to use.

|  | Use case 1 | Use case 2 |
| --- | --- | --- |
| Test A | AI, RA with SHM4.0 | AI, RA with SHM4.0_irap |
| Test B | AI, RA with SHM4.0 | AI, RA with SHM4.0_irap |

Two techniques to estimate the LUTs have been proposed in JCTVC-O0159 and in JCTVC-O0161 and are compared in Test 1 and Test 2 respectively. The corresponding contributions are JCTVC-P0128 and JCTVC-P0186 respectively. Both proposals performs color gamut conversion parameters calculation on picture-basis at IRAP only and do not have any parameters update at picture level nor any “look-ahead” for parameters training.

# Summary of Results

The SCE1 coordinators thanks the cross-checkers for their contributions.

## Use case 1

### Test 1

The detailed experiment results are described in JCTVC-P0128. The cross-checking has been provided by Qualcomm (JCTVC-P0143) and Sony (JCTVC-P0234).

Table 4: Results of use case 1, tests 1.A (second column) and 1.B (first column).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -0.9% | -4.1% | -3.3% | -2.2% | -5.1% | -4.1% |
| Overall (Test vs single layer) | 17.7% | 17.8% | 14.6% | 18.8% | 18.4% | 14.9% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -2.3% | -5.6% | -4.2% | -4.7% | -7.5% | -6.1% |
| Overall (Test EL+BL vs single EL+BL) | -21.5% | -21.8% | -24.0% | -21.0% | -21.6% | -24.1% |
| Enc Time[%] | 107.9% |  |  | 108.4% |  |  |
| Dec Time[%] | 101.7% |  |  | 102.5% |  |  |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -2.6% | -4.2% | -5.8% | -3.3% | -4.6% | -5.8% |
| Overall (Test vs single layer) | 24.1% | 28.2% | 19.3% | 24.9% | 29.0% | 20.3% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -4.8% | -6.2% | -7.7% | -6.0% | -7.1% | -8.2% |
| Overall (Test EL+BL vs single EL+BL) | -16.6% | -13.4% | -19.3% | -16.4% | -13.2% | -19.0% |
| Enc Time[%] | 70.7% |  |  | 70.3% |  |  |
| Dec Time[%] | 105.4% |  |  | 102.9% |  |  |

### Test 2

The detailed experiment results are described in JCTVC-P0186. The cross-checking has been provided by Qualcomm (JCTVC-P0144).

Table 5: Results of use case 1, tests 2.A (second column) and 2.B (first column).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -2.9% | -5.5% | -5.0% | -3.4% | -5.8% | -5.1% |
| Overall (Test vs single layer) | 15.3% | 16.1% | 12.7% | 17.3% | 17.5% | 13.9% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -6.0% | -8.5% | -7.4% | -6.9% | -9.2% | -7.9% |
| Overall (Test EL+BL vs single EL+BL) | -23.2% | -22.9% | -25.3% | -22.0% | -22.1% | -24.8% |
| Enc Time[%] | 99.9% |  |  | 79.2% |  |  |
| Dec Time[%] | 100.7% |  |  | 101.1% |  |  |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -3.8% | -5.1% | -7.3% | -4.0% | -5.2% | -6.9% |
| Overall (Test vs single layer) | 22.6% | 27.1% | 17.4% | 24.0% | 28.4% | 18.9% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -6.9% | -7.9% | -10.0% | -7.2% | -8.1% | -9.7% |
| Overall (Test EL+BL vs single EL+BL) | -17.7% | -14.0% | -20.7% | -17.0% | -13.5% | -20.0% |
| Enc Time[%] | 69.7% |  |  | 57.7% |  |  |
| Dec Time[%] | 107.1% |  |  | 107.4% |  |  |

## Use case 2

### Test 1

The detailed experiment results are described in JCTVC-P0128. The cross-checking has been provided by Qualcomm (JCTVC-P0143).

Table 6: Results of use case 2, tests 1.A (second column) and 1.B (first column).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -5.6% | -7.8% | -9.9% | -5.9% | -8.0% | -10.0% |
| Overall (Test vs single layer) | 11.9% | 13.1% | 6.7% | 14.0% | 14.7% | 7.8% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -11.5% | -13.3% | -15.4% | -12.0% | -13.7% | -15.7% |
| Overall (Test EL+BL vs single EL+BL) | -26.3% | -25.7% | -30.4% | -24.9% | -24.7% | -29.9% |
| Enc Time[%] | 96.3% |  |  | 96.7% |  |  |
| Dec Time[%] | 96.2% |  |  | 97.8% |  |  |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -5.1% | -6.1% | -9.7% | -5.1% | -6.1% | -9.3% |
| Overall (Test vs single layer) | 20.9% | 25.7% | 14.4% | 22.5% | 27.1% | 15.9% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.9% |
| EL only (Test vs Ref) | -9.6% | -10.2% | -13.9% | -9.7% | -10.2% | -13.5% |
| Overall (Test EL+BL vs single EL+BL) | -19.3% | -15.5% | -23.4% | -18.4% | -14.8% | -22.6% |
| Enc Time[%] | 67.4% |  |  | 67.0% |  |  |
| Dec Time[%] | 104.7% |  |  | 107.7% |  |  |

### Test 2

The detailed experiment results are described in JCTVC-P0186. The cross-checking has been provided by Qualcomm (JCTVC-P0144) and Samsung (JCTVC-P0248).

Table 7: Results of use case 2, tests 2.A (second column) and 2.B (first column).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -7.8% | -8.9% | -11.6% | -7.8% | -8.9% | -11.5% |
| Overall (Test vs single layer) | 9.2% | 11.8% | 4.7% | 11.8% | 13.6% | 6.1% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -15.2% | -15.9% | -18.6% | -15.2% | -15.9% | -18.4% |
| Overall (Test EL+BL vs single EL+BL) | -28.3% | -26.5% | -31.9% | -26.6% | -25.4% | -31.1% |
| Enc Time[%] | 90.9% |  |  | 93.0% |  |  |
| Dec Time[%] | 98.3% |  |  | 98.6% |  |  |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -6.3% | -6.5% | -11.0% | -6.2% | -6.5% | -10.6% |
| Overall (Test vs single layer) | 19.3% | 25.1% | 12.7% | 21.2% | 26.6% | 14.3% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.9% |
| EL only (Test vs Ref) | -11.7% | -11.4% | -16.0% | -11.5% | -11.4% | -15.4% |
| Overall (Test EL+BL vs single EL+BL) | -20.5% | -15.7% | -24.6% | -19.4% | -15.0% | -23.8% |
| Enc Time[%] | 64.0% |  |  | 65.1% |  |  |
| Dec Time[%] | 106.1% |  |  | 106.3% |  |  |

# References

- P.Bordes, Y.Ye, E.Alshina, X.Li, S.H.Kim, A.Duenas, K.Ugur, K.Sato, “Description of Core Experiment SCE1: Color Gamut and Bit-Depth Scalability,” JCTVC-O1101, October 2013, Geneva, CH.

- A.Segall, A.Duenas, P.Bordes, J. Dong, D.-K. Kwon, X. Li, “AHG report: Colour gamut scalability (AHG14),” JCTVC-O0014, October 2013, Geneva, CH.