| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0033 |
| --- | --- |

| Title: | RCE2: Summary report on HEVC Range Extensions Core Experiment 2 (RCE2) on Rice parameter initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chris Rosewarne Marta Karczewicz Karl Sharman S.-H. Kim | Email: | chris.rosewarne@cisra.canon.com.au martak@qti.qualcomm.com karl.sharman@eu.sony.com kimse@sharplabs.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

A summary of RCE2 on Rice parameter initialization for HEVC Range Extensions is reported. The core experiment consisted of two tests that investigated the impact of different approaches to initializing the Rice parameter for each sub-block in a transformblock.  Performance of the proposed methods was evaluated for lossy and lossless configurations based on the test conditions and sequences of AhG8 and AhG18.

# Document List

## List CE contributions and corresponding cross-checks

### Test 1: Statistics-based Rice parameter initialization

- JCTVC-P0199, RCE2: Results of Test 1 on Rice Parameter Initialization, M. Karczewicz, L. Guo, J. Sole (Qualcomm), K. Sharman, N. Saunders, J. Gamei (Sony)

  - JCTVC-P0219, Cross-check report of ' RCE2: Results of Test 1 on Rice Parameter Initialization' (JCTVC-P0199) by Qualcomm and Sony, C. Rosewarne, M. Maeda (Canon)

  - JCTVC-P0236, RCE2: Verification for Test 1 on Rice Parameter Initialization, E. Alshina (Samsung)

### Test 2: Parameter-based Rice parameter initialization

- JCTVC-P0168, RCE2: Test A2. Rice parameter initialization based on quantization parameter and bit-depth, S.-H. Kim, K. Misra, A. Segall (Sharp)

  - JCTVC-P0210, RCE2: Cross-check of test A2 (JCTVC-P0168), L. Guo (Qualcomm)

## Summary of RCE contributions

| Test | Method | Test | JCTVC Doc# | Crosscheck | JCTVC Doc# |
| --- | --- | --- | --- | --- | --- |
| 1 | Statistic counters used to initialize Rice parameter | Qualcomm and Sony | JCTVC-P0199 | Canon, Samsung | JCTVC-P0219, JCTVC-P0236 |
| 2 | Rice parameter initialized based on paramters including bit-depth and QP | Sharp | JCTVC-P0168 | Qualcomm | JCTVC-P0210 |

## Related non-RCE contribution(s)

- JCTVC-P0075, non-[RCE1, RCE2]: Combination of RCE1 subtests B5 and B6 with RCE2 subtest A1, R. Joshi, L. Guo, J. Sole, M. Karczewicz (Qualcomm)

# Summary of methods

## Test 1

The 4×4 sub-blocks are divided into different categories (“sbType”). For each sub-block, the initial Rice parameter is derived based on previously coded sub-blocks in the same category. The categorization is based on whether the block is a transform-skip block (“isTSFlag”) and whether it is the luma component:

sbType = isLuma*2+ isTSFlag

Stats statCoeff are maintained for each sub-block type (sbType) depending on the absolute coefficient value (uiLevel):

if (uiLevel >=3*(1<<(statCoeff /4)))            statCoeff ++;

else if ((2* uiLevel)<(1<<( statCoeff/4)))        statCoeff --;

This variable is updated at most once per 4×4 sub-block using the value of the first coded coeff_abs_level_remaining of the sub-block. The entries of statCoeff are reset to 0 at the beginning of the slice (like the CABAC context variables).

The value of statCoeff is used to initialize the Rice parameter at the beginning of each 4×4 sub-block as

cRiceParam = Min(maxRicePara, statCoeff/4).

Note: The Rice parameter update method in HM12.0+RExt-4.1 is used for the proposed method (i.e. the adoption of the ‘fast update’ method of JCTVC-O0206 at the 15th JCT-VC meeting is reversed in this proposal).

## Test 2

The Rice parameter (uiGoRiceParam) is initialized at TU level, and the initial Rice parameter is determined based on the existing information such as bit depth and quantization parameter as follows

uiGoRiceParam = TransQuantBypass_flag ? 1 : Max {0, 4 + (BD-8)/2 –QP/6}

If( !Mode_INTRA) uiGoRiceParam /=2

Note from the cross-checker: Besides modifying the Rice parameter initial value calculation, the max Rice parameter is also increased from 7 to 10.

# Summary of results

The proposed methods were implemented on top of HM-12.1-RExt-5.1 and simulated under the AhG8 and AhG18 common test conditions.

## Test 1

Table 1, Table 2 and Table 3 show the simulation results for Test 1 under AHG8 lossy, AHG8 lossless and AHG18 conditions with transform skip enabled, respectively.  Note that no significant differences between the transform skip enabled and transform skip disabled cases were observed under AHG18 conditions.

|  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.5% | -1.2% | -2.0% | -0.5% | -1.0% | -0.5% | -0.9% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.4% | -0.9% | -1.6% | 0.2% | -0.3% | -0.1% | -0.3% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -3.7% | -5.8% | -7.9% | -3.9% | -6.1% | -3.2% | -5.3% |
| YCbCr 4:4:4 SC (Optional) | -1.4% | -2.7% | -4.5% | -1.2% | -2.4% | -1.5% | -2.2% |
| Enc Time[%] | 99.4% | 99.6% | 99.6% | 98.8% | 98.7% | 98.6% | 98.2% |
| Dec Time[%] | 97.6% | 97.3% | 98.2% | 102.1% | 102.3% | 100.8% | 100.3% |

Table 1. RCE2 Test 1 AHG8 lossy simulation result.

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0.7% | -0.5% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -3.0% | -2.8% | -2.7% |
| RGB 4:4:4 Animation | -0.3% | -0.3% | -0.3% |
| YCbCr 4:4:4 SC | -2.6% | -2.0% | -2.4% |
| YCbCr 4:4:4 Animation | -0.2% | -0.1% | -0.2% |
| RangeExt | -1.3% | -0.4% | -0.4% |
| RGB 4:4:4 SC (Optional) | -8.7% | -8.7% | -9.2% |
| YCbCr 4:4:4 SC (Optional) | -6.9% | -6.9% | -7.1% |
| Enc Time[%] | 101% | 98% | 100% |
| Dec Time[%] | 101% | 102% | 100% |

Table 2. RCE2 Test 1 AHG8 lossless simulation result.

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -2.7% | -2.3% | -2.3% | -1.1% | -0.8% | -0.8% | -1.9% | -1.9% | -2.0% | -0.8% | -0.8% | -0.9% |
| RExt RGB |  |  | -1.5% | -1.2% | -1.2% | 0.1% | 0.2% | 0.2% | -1.1% | -1.1% | -1.1% | 0.0% | 0.0% | 0.0% |
| HDR |  |  | 0.2% | 0.4% | 0.3% | 0.3% | 0.3% | 0.3% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| 4K from HDR |  |  | -1.6% | -1.2% | -1.1% | -0.1% | 0.2% | 0.2% |  |  |  |  |  |  |
| HDR 4:0:0 Medical |  |  | -0.6% |  |  | 0.0% |  |  | -0.3% |  |  | 0.0% |  |  |
| 4:0:0 Medical |  |  | 1.6% |  |  | 0.9% |  |  | 1.8% |  |  | 0.5% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -1.1% | -0.7% | -0.7% | 0.0% | 0.1% | 0.0% | -0.5% | -0.4% | -0.4% | 0.0% | 0.1% | 0.1% |
| Synthetic RGB HDR (+4 MSBs) |  |  | -0.3% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| All Classes |  |  | -0.7% | -0.7% | -0.7% | 0.0% | 0.0% | 0.0% | -0.3% | -0.6% | -0.6% | -0.1% | -0.2% | -0.2% |
| Encoding Time % |  |  | 105% |  |  | 95% |  |  | 100% |  |  | 100% |  |  |
| Decoding Time % |  |  | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |

Table 3. RCE2 Test 1 AHG18 simulation result (Transform skip enabled).

## Test 2

Table 4, Table 5 and Table 6 show the simulation results for Test 2 under AHG8 lossy, AHG8 lossless and AHG18 conditions with transform skip enabled, respectively.  Note that no significant differences between the transform skip enabled and transform skip disabled cases were observed under AHG18 conditions.

|  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | 0.0% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | 0.0% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.0% | 0.0% | -0.4% | 0.0% | 0.0% | -0.1% | -0.4% |
| YCbCr 4:4:4 SC (Optional) | 0.0% | -0.1% | -0.6% | 0.0% | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100.1% | 99.8% | 99.3% | 97.8% | 98.0% | 98.8% | 98.7% |
| Dec Time[%] | 95.5% | 95.5% | 95.7% | 100.0% | 100.2% | 98.0% | 97.6% |

Table 4. RCE2 Test 2 AHG8 lossly simulation result.

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 109% | 104% | 105% |
| Dec Time[%] | 101% | 100% | 102% |

Table 5. RCE2 Test 2 AHG8 lossless simulation result.

|  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
|  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT | -2.9% | -3.0% | -3.0% | -0.1% | 0.0% | 0.0% | -0.9% | -0.8% | -0.8% | 0.0% | 0.0% | 0.0% |
| RExt RGB | -1.4% | -1.3% | -1.3% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% |
| HDR | 0.0% | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.1% | 0.0% | 0.0% |
| 4K from HDR | -0.5% | -0.4% | -0.4% | -0.1% | 0.0% | 0.0% |  |  |  |  |  |  |
| HDR 4:0:0 Medical | 0.0% |  |  | 0.4% |  |  | 0.1% |  |  | 0.1% |  |  |
| 4:0:0 Medical | -0.6% |  |  | 0.1% |  |  | -0.1% |  |  | 0.0% |  |  |
| Synthetic RGB HDR (+2 MSBs) | -0.6% | -0.3% | -0.3% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) | 0.4% | 0.4% | 0.4% | 0.3% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) | 1.1% | 0.6% | 0.7% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| All Classes | -0.5% | -0.6% | -0.5% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Encoding Time % | 97% |  |  | 100% |  |  | 92% |  |  | 94% |  |  |
| Decoding Time % | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |

Table 6. RCE2 Test 2 AHG8 simulation result (Transform skip enabled).