| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0034 |
| --- | --- |

| Title: | RCE3: Summary report on HEVC Range Extensions Core Experiment 3 (RCE3) on Intra block copy refinement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | J. Sole E. Alshina D.-K. Kwon W.-H. Peng | Email: | joels@qti.qualcomm.com elena_a.alshina@samsung.com d-kwon@ti.com wpeng@cs.nctu.edu.tw |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

A summary of RCE3 on Intra block copy refinement for HEVC Range Extensions is reported. The core experiment consisted of three subtests that investigated the impact of the search area size on the performance, refinements on the block partitioning, and padding techniques to predict from unavailable samples. Performance of the proposed methods as well as combinations was evaluated for lossy and lossless configurations based on the test conditions and sequences of AhG8.

# Document List

## List CE contributions and corresponding cross-checks

### Test A: Search Area vs. Performance

- JCTVC-P0211, RCE3: intra block copy search range (tests A), E.Alshina, A.Alshin (Samsung), C. Pang, J. Sole, M. Karczewicz (Qualcomm)

  - Results A1 to A4 independently verified by Qualcomm and Samsung

    - Late: 02/01/2014

  - JCTVC-P0233, Cross-check report of 'RCE3: intra block copy search range (tests A)' test A5 (JCTVC-P0211) by Qualcomm and Samsung, C. Rosewarne, M. Maeda (Canon)

### Test B: Block partitioning

- JCTVC-P0176, RCE3: Results of Subtest B.1 on Nx2N/2NxN Intra Block Copy, T.-S. Chang, R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU), C.-L. Lin, F.-D. Jou (ITRI)

  - Late: 02/01/2014

  - JCTVC-P0191, RCE3: Cross-check report of Subtest B.1 on Nx2N/2NxN Intra Block Copy (JCTVC-P0176), Xiaozhong Xu, Shan Liu (MediaTek)

- JCTVC-P0145, RCE3: Subtest B.3 - Intra block copy with NxN PU, C. Pang, J. Sole, L. Guo, M. Karczewicz (Qualcomm)

  - JCTVC-P0188, RCE3: Cross-check report of Subtest B.3 - Intra Block Copy with NxN PU (JCTVC-P0145), R.-L. Liao, T.-S. Chang, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU), C.-L. Lin, F.-D. Jou (ITRI)

- JCTVC-P0055, RCE3: Subtest B.4 - Sample masking for intra block copy, J. Lainema, K. Ugur (Nokia)JCTVC-P0057, AHG 5: Profiles for Range Extensions, K. Sharman, N. Saunders, J. Gamei, T. Suzuki, A. Tabatabai (Sony)

  - JCTVC-P0185, RCE3: Crosscheck report of B.4 on sample masking, J. Xu (Sony)

### Test C: Padding/Unavailable samples pre-set

- JCTVC-P0212, RCE3: Subtest C.1 - preset of un-available for intra block copy samples, E.Alshina, A.Alshin (Samsung)

  - JCTVC-P0148, RCE3: Crosscheck report of RCE3 Subtest C.1, C. Pang (Qualcomm)

- JCTVC-P0146, RCE3: Subtest C.2 - Intra block copy with CU-based padding, C. Pang, J. Sole, L. Guo, M. Karczewicz (Qualcomm)

  - JCTVC-P0105, RCE3: Cross-verification of subtest C.2, S. Lee, C. Kim (Samsung)

- JCTVC-P0147, RCE3: Subtest C.3 - Intra block copy with CTU-based padding, C. Pang, J. Sole, L. Guo, M. Karczewicz (Qualcomm)

  - JCTVC-P0232, Cross-check report of 'RCE3: Subtest C.3 - Intra block copy with CTU-based padding' (JCTVC-P0147) by Qualcomm, C. Rosewarne, M. Maeda (Canon)

- JCTVC-P0053, RCE3: Subtest C.4 - Padding-based generation of unavailable samples in intra block copy, S. Lee, E. Alshina, C. Kim (Samsung)

  - JCTVC-P0088, RCE3: Cross-check on Subtest C.4 - Padding horizontally or vertically, A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)

### Test D: Combinations

- JCTVC-P0189, RCE3: Results of Subtest D.1 on Combining Nx2N/2NxN Intra Block Copy with TU process, C.-C. Chen, R.-L. Liao, T.-S. Chang, W.-H. Peng, H.-M. Hang, C.-L. Lin, F.-D. Jou (NCTU/ITRI)

  - Late: 05/01/2014

  - JCTVC-P0196, RCE3: Cross-verification of subtest D.1, S. Lee, C. Kim (Samsung)

- JCTVC-P0180, RCE3: Results of Subtest D.2 on Nx2N/2NxN/NxN Intra Block Copy, T.-S. Chang, R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU), C.-L. Lin, F.-D. Jou (ITRI)

  - Late: 02/01/2014

  - JCTVC-P0193, RCE3: Cross-check report of Subtest D.2 on Nx2N/2NxN/NxN Intra Block Copy, Xiaozhong Xu, Shan Liu (MediaTek)

- JCTVC-P0055 also presents results of combinations of sample masking (B.4) with subtests B.3, C.2, C.3 and C.4

## Summary of RCE contributions

| Subtest | Method | Test | JCTVC Doc# | Crosscheck | JCTVC Doc# |
| --- | --- | --- | --- | --- | --- |
| A.1-A.4 | Left search area | Samsung & Qualcomm | P0211 | _ | _ |
| A.5 | CTU + 4 columns + 4 rows | Samsung | P0211 | Canon | P0233 |
| B.1 | N×2N/2N×N PU (without TU process) | NCTU/ITRI | P0176 | Mediatek | P0191 |
| B.2 | TU-based process | Microsoft |  | BBC |  |
| B.3 | N×N PU | Qualcomm | P0145 | NCTU/ITRI | P0188 |
| B.4 | Masking | Nokia | P0055 | Sony | P0185 |
| C.1 | Unavailable samples preset | Samsung | P0212 | Qualcomm | P0148 |
| C.2 | Padding CU horizontally | Qualcomm | P0146 | Samsung | P0105 |
| C.3 | Padding CTU horizontally | Qualcomm | P0147 | Canon | P0232 |
| C.4 | Padding hor/vertically | Samsung | P0053 | Mitsubishi | P0088 |
| D.1 | B.1 + B.2 | NCTU/ITRI | P0189 | Samsung | P0196 |
| D.2 | B.1 + B.3 | NCTU/ITRI | P0180 | Mediatek | P0193 |

## Related non-RCE contributions

- JCTVC-P0087, Non-RCE3: On padding/unavailable samples pre-set for intra block copy, A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami (Mitsubishi)

- JCTVC-P0089, Non-RCE3: On unavailable sample preset for intra block copy, C. Park, S. Lee, C. Kim (Samsung)

- JCTVC-P0102, Non-RCE3: On displacement vector coding for intra block copy, C. Park, S. Lee, C. Kim (Samsung)

- JCTVC-P0111, Non-RCE3: On initialization of the block vector predictor for Intra Block Copy, G. Laroche, P. Onno, C. Gisquet, T. Poirier (Canon)

- JCTVC-P0149, Non-RCE3: Block vector signaling for intra block copy, C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)

- JCTVC-P0150, Non-RCE3: Block vector predictor initialization for intra block copy, C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)

- JCTVC-P0151, Non-RCE3: Fast encoder search for RCE3 Subtest B.3, C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)

- JCTVC-P0170, Non-RCE3: simplified difference vector coding for Intra block copy, S.-H. Kim, K. Misra, A. Segall (Sharp)

- JCTVC-P0178, Non-RCE3: Bv coding with default predictor, J. Xu, A. Tabatabai, O. Nakagami(Sony)

- JCTVC-P0190, Non-RCE3: On padding of overlapped area for IntraBC, Kai Zhang, Xiaozhong Xu, Jicheng An, Xianguo Zhang, Shan Liu, Shawmin Lei (MediaTek)

- JCTVC-P0202, Non-RCE3: On Intra block copy, C. Rosewarne, M. Maeda (Canon)

- JCTVC-P0205, Non-RCE3: Intra block copy syntax clean up, Xiaozhong Xu, Kai Zhang, Xianguo Zhang, Shan Liu, Shawmin Lei (MediaTek)

- JCTVC-P0206, Non-RCE3: Improvements on non square intra block copy, Xiaozhong Xu, Shan Liu, Shawmin Lei (MediaTek)

- JCTVC-P0217, Non-RCE3: Initialization of block vector predictor for intra block copy, L. Zhu, J. Xu, G. J. Sullivan, S. Sankuratri, B. A. Kumar, F. Wu (Microsoft)

- JCTVC-P0218, Non-RCE3 subtest B.2: Results and search methods for intra block copying for CU-level block vectors with TU-level prediction processing, L. Zhu, J. Xu, G. J. Sullivan (Microsoft)

- JCTVC-P0238, Non-RCE3: Padding schemes for intra block copy, Yao-Jen Chang, Hua-Lung Tsai, Chun-Lung Lin, Fan-Di Jou (ITRI), Chun-Chi Chen, Ru-Ling Liao, Wen-Hsiao Peng, Hsueh-Ming Hang (NCTU)

# Summary of Results

The proposed methods were implemented on top of HM-12.1-RExt-5.1 and simulated under the AhG8 common test conditions.

## Test A: Search Area vs. Performance

Subtest A.1 to A.4:

- Current CTU + 4 left columns

- Current CTU + 8 left columns

- Current CTU + 16 left columns

- Current CTU + 32 left columns

Subtest A.5:

- Current CTU + 4 left columns + 4 top rows

Figure 1 shows the search area used for the subtests.

Figure 1. Memory required for different intra block copy configurations in subtests A1 to A5

Lossy tests summary

| Test | Samples in memory | Screen content (Class F & YCbCr 4:4:4 SC) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI |  | RA |  | LD |  |
| IntraBC=0 | 4624 | 0,0% | 0% | 0,0% | 0% | 0,0% | 0% |
| A1 | 4624 | -13,0% | 59% | -9,1% | 57% | -5,4% | 55% |
| A5 | 4624* | -13,4% | 60% | -9,4% | 59% | -5,3% | 54% |
| A2 | 4880 | -14,2% | 64% | -9,9% | 62% | -6,0% | 61% |
| A3 | 5392 | -16,2% | 73% | -11,4% | 71% | -6,8% | 69% |
| A4 | 6416 | -18,9% | 85% | -13,4% | 84% | -8,2% | 83% |
| Rext5.1 | 8464 | -22,2% | 100% | -16,0% | 100% | -9,9% | 100% |

Lossless tests summary

| Test | Samples in memory | Screen content (Class F & YCbCr 4:4:4 SC) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI |  | RA |  | LD |  |
| IntraBC=0 | 4624 | 0,0% | 0% | 0,0% | 0% | 0,0% | 0% |
| A1 | 4624 | -6,0% | 61% | -2,0% | 65% | -1,1% | 61% |
| A5 | 4624* | -6,2% | 63% | -2,0% | 65% | -1,1% | 61% |
| A2 | 4880 | -6,5% | 66% | -2,1% | 68% | -1,2% | 67% |
| A3 | 5392 | -7,3% | 74% | -2,4% | 77% | -1,3% | 72% |
| A4 | 6416 | -8,3% | 85% | -2,7% | 87% | -1,5% | 83% |
| Rext5.1 | 8464 | -9,8% | 100% | -3,1% | 100% | -1,8% | 100% |

* deblocking might be performed in CTU-based implementations, thus requiring more memory

## Test B: Block partitioning

### Subtest B.1 - N×2N/2N×N Intra BC (without TU process) [JCTVC-P0176]

Introduces a PU-based IntraBC mode that relaxes the restriction that the basic unit for IntraBC must be 2N×2N. This mode splits CUs of size 2N×2N into 2N×N and N×2N PUs, following the inter split modes, and predicts each PU with IntraBC. This subtest is conducted with no reference to prior PUs within a CU.

### Subtest B.3 - N×N PU [JCTVC-P0145]

IntraBC is extended to N×N PU for the smallest CU. Following current split modes for intra, each PU has a block vector. For each PU, the prior PUs with in a CU can be used as the reference.

### Subtest B.4 - Sample masking [JCTVC-P0055]

A mask for the CUs coded with IntraBC mode is signaled. The mask is covering an area extending from one of the borders of the CU and having a width varying from 1 to 3 sample rows or columns. All the sample values in the masked area are substituted with a single sample value obtained from the middle sample of the inner boundary of the mask. The black samples in figure below illustrate the location of the sample which value is used in the substitution process

Figure. Example of masks on different borders of the prediction block

Results of combinations are presented in section D.

### Lossy Results

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subtest B.1 N×2N/2N×N (w/o TU process) | Class F | -3.9% | -3.6% | -3.2% | -2.9% | -2.7% | -1.5% | -1.5% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -8.8% | -8.8% | -8.7% | -7.2% | -7.1% | -5.4% | -5.7% |
|  | Animation RGB 444 | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | SC YUV 444 | -8.7% | -8.8% | -8.6% | -7.1% | -7.4% | -5.1% | -5.5% |
|  | Animation YUV 444 | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -10.3% | -11.2% | -11.7% | -8.5% | -9.4% | -7.4% | -7.9% |
|  | SC(444) YUV Optional | -12.3% | -13.8% | -14.9% | -10.7% | -12.4% | -9.2% | -10.8% |
|  | Enc Time[%] | 124% | 126% | 127% | 102% | 102% | 102% | 103% |
|  | Dec Time[%] | 99% | 99% | 99% | 99% | 99% | 100% | 100% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subtest B.3 N×N PU | Class F | -3.5% | -3.4% | -3.1% | -2.5% | -2.4% | -1.3% | -1.3% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | SC RGB 444 | -9.2% | -9.8% | -10.0% | -7.8% | -8.1% | -5.5% | -6.2% |
|  | Animation RGB 444 | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | SC YUV 444 | -8.8% | -9.5% | -9.9% | -7.5% | -8.0% | -4.8% | -5.7% |
|  | Animation YUV 444 | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -11.2% | -12.3% | -13.0% | -9.0% | -10.3% | -7.3% | -8.3% |
|  | SC(444) YUV Optional | -13.7% | -15.9% | -17.8% | -11.9% | -14.0% | -10.5% | -12.8% |
|  | Enc Time[%] | 130% | 129% | 128% | 102% | 104% | 102% | 104% |
|  | Dec Time[%] | 98% | 97% | 97% | 96% | 97% | 101% | 101% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subtest B.4 Masking | Class F | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% | -0.1% | -0.2% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -1.3% | -1.3% | -1.3% | -1.0% | -1.0% | -0.9% | -1.0% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -1.3% | -1.2% | -1.2% | -1.1% | -1.2% | -0.8% | -0.9% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -1.6% | -1.8% | -1.9% | -1.2% | -1.5% | -1.2% | -1.1% |
|  | SC(444) YUV Optional | -2.1% | -2.3% | -2.6% | -1.9% | -2.1% | -1.4% | -1.0% |
|  | Enc Time[%] | 100 % | 100 % | 100 % | 99 % | 99 % | 100 % | 100 % |
|  | Dec Time[%] | 99 % | 99 % | 99 % | 100 % | 101 % | 99 % | 100 % |

### Lossless Results

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Subtest | Class F | -1.4% | -4.5% | -0.1% | -0.7% | -2.7% | 0.0% | -0.3% | -0.9% | 0.0% |
| B.1 | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -4.8% | -11.0% | -0.1% | -4.2% | -11.0% | -0.1% | -3.7% | -10.0% | 0.0% |
| N×2N/2N×N | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| (w/o TU process) | SC YUV 444 | -5.4% | -12.2% | -0.3% | -4.4% | -11.9% | -0.2% | -4.0% | -10.7% | -0.1% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optio | -8.2% | -14.3% | -3.9% | -7.2% | -13.1% | -2.8% | -7.5% | -15.4% | -2.7% |
|  | SC(444) YUV Optio | -11.1% | -15.3% | -5.7% | -10.6% | -14.3% | -4.0% | -12.1% | -17.2% | -3.8% |
|  | Enc Time[%] | 140% |  |  | 105% |  |  | 104% |  |  |
|  | Dec Time[%] | 98% |  |  | 99% |  |  | 99% |  |  |

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Subtest | Class F | -1.3% | -4.3% | -0.1% | -0.8% | -2.8% | 0.0% | -0.4% | -1.4% | 0.0% |
| B.3 | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -6.2% | -13.2% | -0.2% | -5.4% | -13.0% | -0.1% | -4.8% | -12.1% | -0.1% |
| N×N PU | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -6.6% | -13.8% | -0.4% | -5.3% | -13.3% | -0.2% | -4.9% | -12.0% | -0.2% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -10.2% | -16.4% | -5.0% | -8.7% | -14.9% | -3.6% | -8.4% | -16.1% | -3.5% |
|  | SC(444) YUV Optional | -13.9% | -18.3% | -6.7% | -13.1% | -18.2% | -4.3% | -13.4% | -19.8% | -4.0% |
|  | Enc Time[%] | 132% |  |  | 104% |  |  | 96% |  |  |
|  | Dec Time[%] | 98% |  |  | 98% |  |  | 103% |  |  |

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Subtest | Class F | -0.1% | -0.3% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| B.4 | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.7% | -1.6% | 0.0% | -0.7% | -1.6% | 0.0% | -0.1% | -0.5% | -1.3% |
| Masking | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -0.7% | -1.7% | -0.1% | -0.4% | -1.7% | 0.8% | -0.1% | -0.5% | -1.4% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -1.3% | -2.4% | -0.4% | -1.1% | -2.2% | -0.2% | -0.2% | -1.0% | -2.0% |
|  | SC(444) YUV Optional | -1.9% | -2.5% | -0.7% | -1.8% | -2.5% | -0.5% | -0.5% | -1.7% | -2.4% |
|  | Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

## Test C: Padding/Unavailable samples pre-set

### Subtest C.1 - All unavailable samples pre-set to default value [JCTVC-P0212]

Currently, IntraBC vector shall refer only to the area which contains only already reconstructed samples, and shall not refer to area in a different slice or tile. Because of this requirement, the reference block cannot overlap with current CU. A pre-set value is set for all not available samples for IntraBC samples with default value (mid grey).

### Subtest C.2 - Padding CU horizontally [JCTVC-P0146]

IntraBC is extended to the situation where the reference block is overlapped with current CU and the reference block is completed by horizontal padding.

### Subtest C.3 - Padding CTU horizontally [JCTVC-P0147]

Similar to subtest C.2, but in this case, this subtest tests the performance of horizontal padding for the entire CTU before it is processed.

### Subtest C.4 - Padding horizontally or vertically [JCTVC-P0053]

When the current CU to be predicted by IntraBC overlaps the reference CU, the prediction samples in the overlapped region are generated by copying the available samples from either vertical or horizontal direction depending on relationship between BVx, BVy (horizontal and vertical components of the block vector, respectively), W, and H (width and height of the current CU, respectively). The proposed reference sample generation method is invoked when the following condition is satisfied:

( –W < BVx ≤ 0 ) && ( –H < BVy ≤ 0 ).

When |BVy| > |BVx|, the reference samples are generated by copying the reference samples of the bottommost row of the above CU. Otherwise, when |BVy| ≤ |BVx|, the reference samples are generated by copying the reference samples of the rightmost column of the left CU.

### Lossy Results

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subtest C.1 Pre-set | Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 0.0% | 0.0% | -0.1% | 0.1% | -0.1% | -0.1% | 0.0% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -0.3% | -0.2% | -0.2% | -0.3% | -0.2% | -0.8% | -1.2% |
|  | SC(444) YUV Optional | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.4% | -0.1% |
|  | Enc Time[%] | 101% | 101% | 101% | 99% | 100% | 99% | 99% |
|  | Dec Time[%] | 100% | 100% | 100% | 99% | 99% | 100% | 99% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subtest C.2 CU Hor | Class F | -0.4% | -0.3% | -0.2% | -0.2% | -0.2% | 0.0% | 0.0% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.4% | -0.4% | -0.4% | -0.5% | -0.6% | -0.4% | -0.5% |
|  | Animation RGB 444 | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | SC YUV 444 | -0.5% | -0.5% | -0.5% | -0.5% | -0.5% | -0.3% | -0.3% |
|  | Animation YUV 444 | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -1.0% | -1.1% | -1.1% | -1.0% | -1.1% | -0.8% | -1.1% |
|  | SC(444) YUV Optional | -1.4% | -1.4% | -1.5% | -1.3% | -1.4% | -1.1% | -0.7% |
|  | Enc Time[%] | 102% | 102% | 102% | 101% | 102% | 100% | 101% |
|  | Dec Time[%] | 98% | 98% | 98% | 101% | 101% | 102% | 102% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subtest C.3 CTU Hor | Class F | -0.3% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.4% | -0.3% | -0.3% | -0.2% | -0.2% | -0.4% | -0.5% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -0.4% | -0.4% | -0.4% | -0.3% | -0.4% | -0.3% | -0.2% |
|  | Animation YUV 444 | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -0.8% | -0.9% | -0.9% | -0.8% | -0.8% | -0.8% | -0.1% |
|  | SC(444) YUV Optional | -1.3% | -1.4% | -1.4% | -1.2% | -1.3% | -1.1% | -1.5% |
|  | Enc Time[%] | 103% | 102% | 102% | 99% | 100% | 102% | 102% |
|  | Dec Time[%] | 99% | 99% | 99% | 100% | 101% | 102% | 102% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subtest C.4 CU Hor or Ver | Class F | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | 0.0% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.7% | -0.7% | -0.7% | -0.4% | -0.4% | -0.5% | -0.6% |
|  | Animation RGB 444 | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | SC YUV 444 | -0.6% | -0.5% | -0.5% | -0.4% | -0.4% | -0.3% | -0.4% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -2.6% | -2.8% | -2.8% | -2.1% | -2.2% | -1.3% | -1.4% |
|  | SC(444) YUV Optional | -1.5% | -1.5% | -1.5% | -1.4% | -1.6% | -1.3% | -1.2% |
|  | Enc Time[%] | 101% | 101% | 100% | 100% | 100% | 100% | 100% |
|  | Dec Time[%] | 100% | 99% | 99% | 99% | 100% | 100% | 99% |

### Lossless Results

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Subtest | Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| C.1 | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Pre-set | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -0.2% | -0.6% | 0.0% | -0.2% | -0.5% | 0.0% | -0.1% | -0.2% | 0.0% |
|  | SC(444) YUV Optional | -0.3% | -0.8% | 0.0% | -0.7% | -2.0% | 0.0% | -0.2% | -0.5% | 0.0% |
|  | Enc Time[%] | 100% |  |  | 98% |  |  | 98% |  |  |
|  | Dec Time[%] | 98% |  |  | 94% |  |  | 95% |  |  |

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Subtest | Class F | -0.5% | -2.0% | 0.0% | -0.5% | -1.9% | 0.0% | -0.5% | -2.1% | 0.0% |
| C.2 | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.3% | -0.8% | 0.0% | -0.2% | -0.8% | 0.0% | -0.2% | -0.7% | 0.0% |
| CU Hor | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -0.3% | -0.8% | -0.1% | -0.3% | -0.8% | 0.0% | -0.2% | -0.7% | 0.0% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -0.7% | -1.3% | -0.2% | -0.7% | -1.3% | -0.2% | -0.6% | -0.8% | -0.2% |
|  | SC(444) YUV Optional | -1.2% | -1.4% | -0.8% | -1.2% | -1.4% | -0.8% | -1.3% | -2.2% | -0.8% |
|  | Enc Time[%] | 96% |  |  | 92% |  |  | 97% |  |  |
|  | Dec Time[%] | 92% |  |  | 101% |  |  | 105% |  |  |

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Subtest | Class F | -0.4% | -1.4% | 0.0% | -0.3% | -0.9% | 0.0% | -0.3% | -1.3% | 0.0% |
| C.3 | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.2% | -0.7% | 0.0% | -0.2% | -0.7% | 0.0% | -0.1% | -0.6% | 0.0% |
| CTU Hor | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -0.2% | -0.6% | -0.1% | 0.0% | -0.6% | 1.4% | -0.2% | -0.6% | 0.0% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -0.7% | -1.3% | -0.1% | -0.7% | -1.3% | -0.1% | -0.7% | -1.1% | -0.1% |
|  | SC(444) YUV Optional | -1.1% | -1.4% | -0.4% | -1.4% | -2.7% | -0.4% | -1.3% | -2.1% | -0.4% |
|  | Enc Time[%] | 96% |  |  | 104% |  |  | 97% |  |  |
|  | Dec Time[%] | 92% |  |  | 101% |  |  | 106% |  |  |

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Subtest | Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| C.4 | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -0.2% | -0.9% | 0.0% | -0.2% | -0.9% | 0.0% | -0.2% | -0.7% | 0.0% |
| CU | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Hor or Ver | SC YUV 444 | -0.2% | -0.8% | 0.0% | -0.2% | -0.8% | 0.0% | -0.2% | -0.7% | 0.0% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -0.7% | -1.3% | -0.1% | -0.7% | -1.3% | -0.1% | -0.4% | -1.1% | 0.1% |
|  | SC(444) YUV Optional | -1.0% | -1.4% | -0.5% | -1.1% | -1.4% | -0.4% | -1.0% | -1.5% | -0.4% |
|  | Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
|  | Dec Time[%] | 101% |  |  | 99% |  |  | 101% |  |  |

## Test D: Combinations

### Lossy Results

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subtest D.1 B1 + B2 (N×2N/2N×N w/ TU process) | Class F | -4.0% | -3.7% | -3.3% | -2.9% | -2.7% | -1.6% | -1.6% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -9.3% | -9.3% | -9.1% | -7.7% | -7.7% | -5.5% | -5.9% |
|  | Animation RGB 444 | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -9.0% | -9.2% | -9.0% | -7.4% | -7.7% | -5.1% | -5.6% |
|  | Animation YUV 444 | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -11.0% | -11.8% | -12.4% | -9.0% | -10.0% | -7.3% | -8.3% |
|  | SC(444) YUV Optional | -13.1% | -14.6% | -15.8% | -11.3% | -13.1 | -10.5% | -12.0% |
|  | Enc Time[%] | 128% | 130% | 131% | 106% | 107% | 104% | 104% |
|  | Dec Time[%] | 100% | 100% | 100% | 99% | 99% | 101% | 101% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subtest D.2.a B1 + B3 ( N×2N/2N×N w/o TU process) (B3 w/o fast NxN) | Class F | -4.7% | -4.5% | -4.0% | -3.3% | -3.2% | -1.9% | -1.9% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -11.1% | -11.5% | -11.5% | -9.4% | -9.5% | -6.9% | -7.5% |
|  | Animation RGB 444 | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | SC YUV 444 | -10.8% | -11.3% | -11.4% | -8.9% | -9.4% | -6.2% | -7.0% |
|  | Animation YUV 444 | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | RangeExt | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -13.5% | -14.7% | -15.4% | -10.9% | -12.3% | -9.5% | -10.5% |
|  | SC(444) YUV Optional | -16.1% | -18.2% | -20.0% | -14.1% | -16.0% | -12.8% | -14.9% |
|  | Enc Time[%] | 158% | 159% | 156% | 109% | 110% | 105% | 106% |
|  | Dec Time[%] | 98% | 98% | 98% | 100% | 100% | 99% | 99% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subtest D.2.b B1 + B3 (N×2N/2N×N w/o TU process) (B3 w/ fast NxN) | Class F | -4.6% | -4.4% | -4.0% | -3.3% | -3.1% | -1.8% | -1.9% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -11.0% | -11.4% | -11.5% | -9.3% | -9.5% | -6.7% | -7.2% |
|  | Animation RGB 444 | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | SC YUV 444 | -10.6% | -11.1% | -11.3% | -8.7% | -9.3% | -6.1% | -6.8% |
|  | Animation YUV 444 | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | RangeExt | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -12.8% | -14.1% | -15.0% | -10.2% | -11.8% | -8.5% | -9.2% |
|  | SC(444) YUV Optional | -15.0% | -17.2% | -19.2% | -12.9% | -15.0% | -10.7% | -12.9% |
|  | Enc Time[%] | 141% | 147% | 149% | 104% | 105% | 105% | 106% |
|  | Dec Time[%] | 99% | 98% | 98% | 99% | 99% | 101% | 101% |

### Lossless Results

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Subtest | Class F | -1.4% | -4.8% | -0.1% | -0.8% | -2.8% | 0.0% | -0.3% | -1.0% | 0.0% |
| D.1 | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -5.1% | -11.2% | -0.1% | -4.2% | -11.1% | -0.1% | -3.9% | -10.3% | -0.1% |
| B1 + B2 | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -5.7% | -12.3% | -0.3% | -4.5% | -12.0% | -0.2% | -4.3% | -10.9% | -0.1% |
| (N×2N/2N×N w/ TU process) | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -8.9% | -15.7% | -4.0% | -7.8% | -14.5% | -2.9% | -8.1% | -16.2% | -2.8% |
|  | SC(444) YUV Optional | -11.9% | -16.9% | -5.8% | -11.4% | -15.6% | -4.1% | -12.8% | -18.3% | -3.9% |
|  | Enc Time[%] | 143% |  |  | 105% |  |  | 105% |  |  |
|  | Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Subtest | Class F | -1.8% | -5.6% | -0.2% | -1.0% | -3.6% | 0.0% | -0.5% | -1.8% | 0.0% |
| D.2.a | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -6.9% | -14.2% | -0.2% | -5.7% | -14.0% | -0.2% | -5.3% | -13.0% | -0.1% |
| B1 + B3 | Animation RGB 444 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| ( N×2N/2N×N w/o TU process) (B3 w/o fast NxN) | SC YUV 444 | -7.5% | -15.2% | -0.5% | -6.0% | -14.8% | -0.3% | -5.5% | -13.4% | -0.2% |
|  | Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -11.4% | -18.6% | -5.9% | -9.9% | -17.0% | -4.4% | -9.9% | -19.0% | -4.3% |
|  | SC(444) YUV Optional | -15.6% | -20.0% | -8.4% | -14.6% | -19.5% | -5.7% | -15.7% | -22.7% | -5.4% |
|  | Enc Time[%] | 179% |  |  | 110% |  |  | 108% |  |  |
|  | Dec Time[%] | 98% |  |  | 99% |  |  | 99% |  |  |

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Subtest | Class F | -1.8% | -5.6% | -0.2% | -1.0% | -3.6% | 0.0% | -0.5% | -1.7% | 0.0% |
| D.2.b | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -6.9% | -14.2% | -0.2% | -5.7% | -14.0% | -0.2% | -5.3% | -13.0% | -0.1% |
| B1 + B3 | Animation RGB 444 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| (N×2N/2N×N w/o TU process) (B3 w/ fast NxN) | SC YUV 444 | -7.5% | -15.2% | -0.5% | -6.0% | -14.8% | -0.3% | -5.5% | -13.3% | -0.2% |
|  | Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -11.4% | -18.5% | -5.9% | -9.8% | -16.9% | -4.3% | -9.9% | -18.9% | -4.2% |
|  | SC(444) YUV Optional | -15.5% | -20.0% | -8.3% | -14.6% | -19.6% | -5.5% | -15.2% | -21.5% | -5.2% |
|  | Enc Time[%] | 178% |  |  | 110% |  |  | 108% |  |  |
|  | Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |

### Combinations with Sample Masking

|  |  | Mandatory |  | Optional |  |  | Gain of RCE3-B.4 (masking) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RCE | Test | RGB SC | YUV SC | RGB SC | YUV SC | Aver. |  |
| B.4 | Mask | -1.3 % | -1.3 % | -1.6 % | -2.1 % | -1.6 % | -1.6 % |
| B.3 | NxN PU | -9.2 % | -8.8 % | -11.2 % | -13.7 % | -10.7 % |  |
| B.3 + B.4 | NxN PU + Mask | -10.1 % | -9.5 % | -12.3 % | -15.3 % | -11.8 % | -1.1 % |
| C.2 | Pad CU hor | -0.4 % | -0.5 % | -1.0 % | -1.4 % | -0.8 % |  |
| C.2 + B.4 | Pad CU hor + Mask | -1.7 % | -1.8 % | -2.6 % | -3.3 % | -2.4 % | -1.5 % |
| C.3 | Pad CTU | -0.3 % | -0.4 % | -0.8 % | -1.3 % | -0.7 % |  |
| C.3 + B.4 | Pad CTU hor + Mask | -1.7 % | -1.8 % | -2.4 % | -3.3 % | -2.3 % | -1.6 % |
| C.4 | Pad CU horver | -0.7 % | -0.6 % | -2.6 % | -1.5 % | -1.3 % |  |
| C.4 + B.4 | Pad CU horver + Mask | -1.9 % | -1.8 % | -4.1 % | -3.4 % | -2.8 % | -1.5 % |

### Comparisons of 2NxN/Nx2N and NxN PUs

|  |  | AI-RExt, Main-tier |  |  |  |  | AI-RExt, Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RCE | Test | Class F | RGB SC | YUV SC | Enc. | Dec. | Class F | RGB SC | YUV SC | Enc. | Dec. |
| B1 | Nx2N/2NxN | -3.9% | -8.8% | -8.7% | 124% | 99% | -3.2% | -8.7% | -8.6% | 127% | 99% |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| D1 | Nx2N/2NxN | -4.0% | -9.3% | -9.0% | 128% | 100% | -3.3% | -9.1% | -9.0% | 131% | 100% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| B3 | NxN | -3.5% | -9.2% | -8.8% | 130% | 98% | -3.1% | -10.0% | -9.9% | 128% | 97% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| D2.a | B1 + B3 | -4.7% | -11.1% | -10.8% | 158% | 98% | -4.0% | -11.5% | -11.4% | 156% | 98% |
|  | (w/o Fast NxN) |  |  |  |  |  |  |  |  |  |  |
| D2.b | B1 + B3 | -4.6% | -11.0% | -10.6% | 141% | 99% | -4.0% | -11.5% | -11.3% | 149% | 98% |
|  | (w/ Fast NxN) |  |  |  |  |  |  |  |  |  |  |
|  |  | RA-RExt, Main-tier |  |  |  |  | RA-RExt, High-tier |  |  |  |  |
| RCE | Test | Class F | RGB SC | YUV SC | Enc. | Dec. | Class F | RGB SC | YUV SC | Enc. | Dec. |
| B1 | Nx2N/2NxN | -2.9% | -7.2% | -7.1% | 102% | 99% | -2.7% | -7.1% | -7.4% | 102% | 99% |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| D1 | Nx2N/2NxN | -2.9% | -7.7% | -7.4% | 106% | 99% | -2.7% | -7.7% | -7.7% | 107% | 99% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| B3 | NxN | -2.5% | -7.8% | -7.5% | 102% | 96% | -2.4% | -8.1% | -8.0% | 104% | 97% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| D2.a | B1 + B3 | -3.3% | -9.4% | -8.9% | 109% | 100% | -3.2% | -9.5% | -9.4% | 110% | 100% |
|  | (w/o Fast NxN) |  |  |  |  |  |  |  |  |  |  |
| D2.b | B1 + B3 | -3.3% | -9.3% | -8.7% | 104% | 99% | -3.1% | -9.5% | -9.3% | 105% | 99% |
|  | (w/ Fast NxN) |  |  |  |  |  |  |  |  |  |  |
|  |  | LB-RExt, Main-tier |  |  |  |  | LB-RExt, High-tier |  |  |  |  |
| RCE | Test | Class F | RGB SC | YUV SC | Enc. | Dec. | Class F | RGB SC | YUV SC | Enc. | Dec. |
| B1 | Nx2N/2NxN | -1.5% | -5.4% | -5.1% | 102% | 100% | -1.5% | -5.7% | -5.5% | 103% | 100% |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| D1 | Nx2N/2NxN | -1.6% | -5.5% | -5.1% | 104% | 101% | -1.6% | -5.9% | -5.6% | 104% | 101% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| B3 | NxN | -1.3% | -5.5% | -4.8% | 102% | 101% | -1.3% | -6.2% | -5.7% | 104% | 101% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| D2.a | B1 + B3 | -1.9% | -6.9% | -6.2% | 105% | 99% | -1.9% | -7.5% | -7.0% | 106% | 99% |
|  | (w/o Fast NxN) |  |  |  |  |  |  |  |  |  |  |
| D2.b | B1 + B3 | -1.8% | -6.7% | -6.1% | 105% | 101% | -1.9% | -7.2% | -6.8% | 106% | 101% |
|  | (w/ Fast NxN) |  |  |  |  |  |  |  |  |  |  |