| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0035 |
| --- | --- |

| Title: | RCE4: Summary report of HEVC Range Extensions Core Experiments 4 (RCE4) on palette coding for screen content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xun Guo Ankur Saxena | Email: | xunguo@microsoft.com asaxena@sta.samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

HEVC Range Extensions Core Experiments 4 (RCE4) was established to investigate the proposals related to palette/major color based screen content coding. As described in RCE4 description JCTVC-O1124, two tested methods are included. This document reports the performance evaluation of the methods.

# Introduction

The tested methods were proposed in Geneva meeting by contributions of JCTVC-O0182 [1] and JCTVC-O0218 [2].

# Document list

## CE contributions

Test 1. Major-color-based screen content coding

- JCTVC-P0108 (JCTVC-O0182), “RCE4: Test 1. Major-color-based screen content coding”, X. Guo. Y. Lu, S. Li (Microsoft)

  - Cross-check: JCTVC-P0226 (Huawei), JCTVC-P0271 (Qualcomm)

Test 2. Palette mode for screen content coding

- JCTVC-P0198 (JCTVC-O0218), “RCE4: Results of Test 2 on Palette Mode for Screen Content Coding”, L. Guo, W. Pu, M. Karczewicz, J. Sole, R. Joshi, F. Zou (Qualcomm)

  - Cross-check: JCTVC-P0225 (Huawei), JCTVC-P0104 (Samsung)

## Non-CE contributions

| Document No. | Title | Proponents |
| --- | --- | --- |
| JCTVC-P0090 | RCE4 related: Advanced copy mode for palette coding | ITRI |
| JCTVC-P0091 | Non-RCE4 : On palette update for palette coding | Samsung |
| JCTVC-P0093 | Non-RCE4: Cross-CU major color index prediction | MediaTek |
| JCTVC-P0094 | Non-RCE4: Inter-component major color table sharing | MediaTek |
| JCTVC-P0095 | Non-RCE4: Removal of syntax redundancy in RCE4 Test2 | MediaTek |
| JCTVC-P0096 | Non-RCE4: Major color table propagation through non-palette CUs | MediaTek |
| JCTVC-P0098 | Non-RCE4: Four-neighbor major color index prediction | MediaTek |
| JCTVC-P0101 | Non-RCE4: Removal of syntax redundancy in RCE4 Test1 | MediaTek |
| JCTVC-P0103 | Non-RCE4 : On run-length coding for palette coding | Samsung |
| JCTVC-P0113 | Non-RCE4: Run coding for Palette mode | Canon |
| JCTVC-P0114 | Non-RCE4: Palette Prediction for Palette mode | Canon |
| JCTVC-P0115 | Non-RCE4: Transition copy mode for Palette mode | Canon |
| JCTVC-P0116 | Non-RCE4: On Palette prediction modes coding | Canon |
| JCTVC-P0117 | Non-RCE4: combined coding of run and index for RCE4 Test2 | Canon |
| JCTVC-P0119 | Non-RCE4: combination of improvements for Palette mode | Canon |
| JCTVC-P0152 | Non-RCE4: Major color table (palette) merge from above and left CU | MediaTek |
| JCTVC-P0153 | Non-RCE4: Major color table (palette) sharing | MediaTek |
| JCTVC-P0160 | Non-RCE4: Palette prediction for palette coding | Samsung |
| JCTVC-P0161 | Non-RCE4: Simplification of major color based palette prediction | Samsung |
| JCTVC-P0179 | Non-RCE4: on palette coding mode in RExt | Sony |
| JCTVC-P0231 | Non-RCE4: Refinement of the palette in RCE4 Test 2 | Qualcomm |
| JCTVC-P0239 | Non-RCE4: Joint proposal of JCTVC-P0231 and JCTVC-P0113: Palette with adaptive rice parameter for run coding | Qualcomm, Canon |
| JCTVC-P0249 | Non-RCE4: A combination of the four-neighbor major color index prediction (JCTVC-P0098) and the transition copy mode (JCTVC-P0115) on top of RCE4 Test1 (JCTVC-P0108) | MediaTek |
| JCTVC-P0253 | Non-RCE4: PU based Color Palette Coding | MediaTek |

# Description of the tested methods

## Test 1

A very simple but efficient histogram based algorithm is used to classify the pixels. In specific, the most significant N peak values in the histogram are selected as major colors. The pixel values that are close to a major color will be quantized to the major color. Other pixels which do not belong to any major color sets are escape pixels, which would also be quantized before coding. For lossless coding, no quantization is used.

By using classification, pixels of a CU can be converted into color indices. After that, the major color number and values are coded. Then, the color indices are coded as follows.

- For each pixel line, a flag is signalled to indicate the coding mode. There are three modes: horizontal mode, vertical mode and normal mode.

  - If the mode is horizontal mode, the whole line shares the same color index. In this case, the color index is transmitted.

  - If the mode is vertical mode, the whole line is the same with the above line. In this case, nothing is transmitted. The current line copies the color indices of the above line.

  - If the mode is normal mode, a flag is signalled for each pixel position to indicate whether it is the same with one of the left and above pixels. If not, the index itself is transmitted.

- If the pixel is escape pixel, the pixel value is transmitted.

## Test 2

The palette CU mode encoding process is as follows:

- Transmission of the palette: An entry-wise prediction scheme is used to encode the current palette based on the palette of the left CU. After that, no predicted entries are transmitted.

- Transmission of pixel values: the pixels in the CU are encoded in a raster scan order using 3 modes.

  - “Run mode”: A palette index is first signaled followed by “palette_run” (M). The following M palette indexes are the same as the signaled.

  - “Copy above mode”: A value “copy_run” (N) is transmitted to indicate that for the following N palette indexes are the same as their above neighbors, respectively.

  - “Pixel mode”: A prediction flag is first transmitted. The flag is 1 indicates prediction residual using reconstructed top neighboring pixel as predictor is transmitted. If this flag is 0, the pixel value is transmitted without prediction.

## Techniques summary in Test 1 and Test 2

| Used techniques | Test 1 (Microsoft) |  | Test 2 (Qualcomm) |  |
| --- | --- | --- | --- | --- |
|  | P0108 | O0182 | P0198 | O0218 |
| Palette/Major color coding | CU level Maximum 15 for each component | CU level Maximum 15 for each component | CU level Maximum 32 | CU level Maximum 32 |
| Palette/Major color prediction | Left CU and above CU | Above CU and/or Left CU | Left CU | Not used |
| Color index coding | Horizontal prediction and vertical prediction | Horizontal prediction and vertical prediction | Horizontal prediction and vertical prediction | Horizontal prediction and vertical prediction |
| Escape pixel coding | No prediction | No prediction | Predicted using above pixel | Not used |
| Context models | 20 | 20 | 20 | 1 |

# Summary of results

The tested methods were implemented on top of HM-12.1+RExt-5.1. The test conditions defined in JCTVC-O1124 [3] are used. Both lossy and lossless results are summarized as follows. The full results can be found in the attached excel files of  the contributions.

Table 1. Lossy coding results

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | Class F | -2.9% | -3.0% | -3.0% | -2.1% | -2.0% | -1.2% | -1.3% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | SC RGB 444 | -8.3% | -10.4% | -12.2% | -7.6% | -9.1% | -6.4% | -7.8% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | SC YUV 444 | -7.1% | -9.4% | -11.2% | -6.2% | -8.2% | -4.6% | -6.2% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | SC(444) GBR Optional | -28.1% | -31.6% | -37.0% | -24.0% | -27.5% | -20.2% | -24.9% |
|  | SC(444) YUV Optional | -16.4% | -22.4% | -29.7% | -14.8% | -19.6% | -13.9% | -17.5% |
|  | Enc. Time[%] | 102% | 102% | 102% | 102% | 102% | 102% | 101% |
|  | Dec. Time[%] | 102% | 101% | 101% | 110% | 109% | 112% | 112% |

|  |  | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 2 | Class F | -0.6% | -1.1% | -1.4% | -0.3% | -0.5% | 0.0% | -0.2% |
|  | Class B | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | SC RGB 444 | -10.0% | -13.0% | -16.3% | -9.3% | -11.4% | -4.2% | -5.4% |
|  | Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -7.0% | -11.1% | -14.7% | -6.8% | -10.3% | -2.9% | -4.4% |
|  | Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.1% |
|  | SC(444) GBR Optional | -26.8% | -30.5% | -37.1% | -21.8% | -24.9% | -11.1% | -15.0% |
|  | SC(444) YUV Optional | -22.0% | -29.4% | -37.2% | -19.3% | -24.3% | -10.2% | -14.0% |
|  | Enc. Time[%] | 118% | 116% | 115% | 107% | 106% | 106% | 105% |
|  | Dec. Time[%] | 99% | 98% | 96% | 100%% | 99% | 101% | 100% |

Table 2. Lossless coding results

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Test 1 | Class F | -0.6% | -1.5% | 0.0% | -0.2% | -0.4% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -12.6% | -36.8% | -0.1% | -10.4% | -36.6% | 0.2% | -9.7% | -34.6% | -0.1% |
|  | Animation RGB 444 | -0.2% | -0.5% | 0.0% | -0.2% | -0.5% | 0.0% | -0.2% | -0.5% | 0.0% |
|  | SC YUV 444 | -8.6% | -26.9% | -0.3% | -7.2% | -26.6% | -0.1% | -6.0% | -23.6% | -0.1% |
|  | Animation YUV 444 | -0.4% | -1.2% | 0.0% | -0.4% | -1.2% | 0.0% | -0.4% | -1.2% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -34.5% | -48.0% | -14.4% | -31.8% | -45.7% | -9.6% | -27.0% | -41.6% | -9.3% |
|  | SC(444) YUV Optional | -27.1% | -40.3% | -12.2% | -24.9% | -39.5% | -6.3% | -22.4% | -37.4% | -5.7% |
|  | Enc. Time[%] |  | 103% |  |  | 102% |  |  | 102% |  |
|  | Dec. Time[%] |  | 94% |  |  | 104% |  |  | 107% |  |

|  |  |  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving | Average | Min | Max | Average | Min | Max | Average | Min | Max |
| Test 2 | Class F | -0.4% | -1.2% | 0.0% | 0.0% | -0.1% | 0.3% | 0.0% | 0.0% | 0.1% |
|  | Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC RGB 444 | -17.8% | -42.5% | -0.3% | -12.0% | -41.8% | -0.2% | -8.2% | -35.0% | -0.1% |
|  | Animation RGB 444 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC YUV 444 | -17.4% | -40.5% | -0.3% | -12.4% | -39.8% | -0.1% | -8.4% | -33.4% | 0.0% |
|  | Animation YUV 444 | -0.8% | -2.5% | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% |
|  | RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SC(444) GBR Optional | -38.9% | -48.5% | -27.1% | -27.8% | -39.0% | -7.0% | -17.3% | -23.8% | -6.2% |
|  | SC(444) YUV Optional | -37.5% | -48.6% | -15.9% | -29.8% | -46.5% | -0.8% | -22.4% | -33.6% | -0.1% |
|  | Enc. Time[%] |  | 123% |  |  | 108% |  |  | 105% |  |
|  | Dec. Time[%] |  | 100% |  |  | 101% |  |  | 102% |  |

# Complexity assessment

| Complexity Analysis | Test 1 (P0108) | Test 2 (P0198) |
| --- | --- | --- |
| Number of operation/Sample for reconstruction (typical case) | 1 LUT (15 x 3) for major color 1 shift and 1 addition for escape pixel | 1 LUT (32) for major color 1 shift and 1 addition for escape pixel |
| Parallelism | Using left copy and above copy | Using above copy Pixel mode reconstruction depends on the above pixel |
| Additional Line Buffer | Line buffer to store top CU palette, size up to 16x3x8xNum_LCU_per_line | No |

# Subjective assessment

| Test Items | Test 1 (P0108) | Test 2 (P0198) |
| --- | --- | --- |
| Tested sequences | SC 444 YUV sequences | Not provided |
| Observed Artifacts | Not observed | Not provided |

# Full results

Table 3. Lossy Results on Test 1 and Test 2

Table 4. Lossless Results on Test 1 and Test 2

# Conclusions

It is recommended to discuss the proposals in RCE 4 and the non-RCE4 proposals and consider their adoption.

# References

- X. Guo, B. Li, J. Xu, Y. Lu, S. Li, F. Wu, “AHG8: Major-color-based screen content coding,” JCTVC-O0182, Geneva, CH, 23 Oct. – 1 Nov. 2013.

- L. Guo, M. Karczewicz, J. Sole and R. Joshi, “Evaluation of Palette Mode Coding on HM-12.0+RExt-4.1”, JCTVC-O0218, Geneva, Switzerland, Oct. 2013.

- L. Guo, X. Guo, A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE4): Palette Coding for Screen Content,” JCTVC-O1124, Geneva, CH, 23 Oct. – 1 Nov. 2013.

- X. Guo, Y. Lu, S. Li, “RCE4: Test 1. Major-color-based screen content coding,” JCTVC-P0108, San José, US, 9–17 Jan. 2014.

- L. Guo, W. Pu, M. Karczewicz, J. Sole, R. Joshi, F. Zou, “RCE4: Results of Test 2 on Palette Mode for Screen Content Coding,” JCTVC-P0198, San José, US, 9–17 Jan. 2014.