| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, USA, 9 – 17 Jan. 2014 | Document: JCTVC-P0041v6 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, USA, 11 – 17 Jan. 2014 | Document: JCT3V-G0031v6 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On picture order count |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Adarsh K. Ramasubramonian Ye-Kui Wang Ying Chen Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA Ming Li Ping Wu ZTE Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 86-25-88014644 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | Qualcomm Incorporated, ZTE |  |  |

_____________________________

# Abstract

This contribution presents an updated design for the signalling and derivation of picture order count in SHVC and MV-HEVC. It is proposed that POC reset be indicated by a two-bit indication, to fully utilize the fact that there would never be POC LSB reset only. Additional a POC LSB is signalled in order to provide better error resilience to the POC derivation process and for support of missing-collocated-picture scenarios. Finally, the MSB value of the picture order count is also signalled for CRA pictures.

The main changes compared to the solution in JCTVC-O0213v4 are as follows:

- In output order conformant decoders, it is proposed to output all earlier pictures in the DPB upon receiving a POC reset picture.

  - It is asserted that by doing this, the problem raised at the 2nd POC conference call, for the solution in JCTVC-O0213v3, about possibility of having erroneous order of output of pictures is addressed.

- Revert the timing of decrement of POC of earlier pictures in the DPB to that of described in v3 of this proposal, that is, POC decrement of earlier pictures in the DPB is done in a layer-specific manner.

  - It is asserted that the combination of outputting all earlier picture in the DPB upon receiving a poc reset picture and decrementing POC of earlier pictures in the DPB only of pictures in the same layer as the current layer addresses the problem raised at the 3rd POC conference call, for the solution in JCTVC-O0213v4, about incorrect POC value decrement in case of down-switching and up-switching and with picture loss.

- Propose to signal POC MSB information in slice header extension when current picture is a CRA or BLA picture.

  - The signalled POC MSB information is used for derivation of POC MSB when current picture is a CRA picture with NoRaslOutputFlag equal to 0 for any conditions and derivation of previous POC MSB for pictures when POC reset is applied at CRA picture.

  - This provides two benefits:

    - The first is to allow correct derivation of POC in some use-cases such as switching down and up, and pseudo-single-loop-decoding where in the base layer only CRA pictures are used for inter-layer prediction and present.

    - The second is to allow correct derivation of POC for trick-mode with CRA pictures, including changing from CRA-based trick-mode to normal playback mode or reduced speed-up ratio e.g adding TemporalId-zero pictures.

The text changes are included in the attachment of this document, in relative to JCT3V-F1004v6.

In v2 of JCTVC-P0041/JCT3V-G0031, the document template/header was corrected, without change marks. The spec text changes and other parts remain unchanged as in v1.

In v3 of JCTVC-P0041/JCT3V-G0031, an example is added in section 3, with change marks. The spec text changes and other parts remain unchanged as in v2.

In v4 of JCTVC-P0041/JCT3V-G0031, the following changes were made, and the text changes are included in the attachment, with changes marks in relative to the attachment in v3 of JCTVC-P0041/JCT3V-G0031 (and the old change marks are also kept, with different user names):

- Added a bitstream constraint to disallow a picture that follows a POC-resetting picture in decoding order to precede, in output order, another picture that precedes the POC-resetting picture in decoding order. This would also address the issue raised at the 5th POC conference call regarding output order of RASL pictures of an IRAP picture and the trailing picures preceding the IRAP picture.

- The semantics of the following SEI messages for which some of the semantics depend on POC values, are updated to ensure that the SEI messages work with the resetting based POC design:

  - pan-scan rectangle SEI message

  - recovery point SEI message

  - progressive refinement segment start SEI message

  - film grain characteristics SEI message

  - tone mapping SEI message

  - frame packing SEI message

  - display orientation SEI messages

  - structure of pictures SEI message

  - region refresh SEI message

- Updated to POC derivation of CRA/BLA pictures to always use the signalled POC MSB, as a bug fix to v3 of JCTVC-P0041/JCT3V-G0031.

For identiciation of a picture in feedback messages, it is suggested that, when operating in the context of an SHVC or MV-HEVC profile, in addition to the POC value, the POC-resetting period ID of the latest decoded picture is also signalled in a feedback message. The encoder can then uniquely identify the previously encoded picture. Upon reception of a feedback message with a POC value and a POC-resetting period ID, when latest encoded picture is in a different POC-resetting period, it would track back to the signalled POC-resetting period and add back the POC delta value decremented for each new POC-resetting period. No spec text change for this aspect is provided.

In v5 of JCTVC-P0041/JCT3V-G0031, some discussions on the approach proposed in v6 of JCTVC-O0275/JCT3V-F0092 are included in section 4.

In v6 of JCTVC-P0041/JCT3V-G0031, some editorial simplifications to the spec text changes were included.

# Introduction

The SHVC and MV-HEVC specifications allow access units that contain both IRAP pictures and non-IRAP pictures. In the previous meeting, it was proposed that the POC reset information be signalled in the slice header in the form of poc_reset_flag, and this flag would be used to reset the POC of the current picture to 0, and also decrement the POC of the pictures in the DPB with the same value of nuh_layer_id as the current picture by a value that is equal to the POC of the current picture before reset. The flag is signalled in the reserved bits of the slice segment header, and signalled not just for the enhancement layer, but also for the base layer.

# Problem Statement

The presence of the POC reset information in the base layer only indicates that the POC derivation process would be different in case of an operating point containing only the base layer, and other operating points. This POC signalling has the following shortcomings:

- When an access unit auA contains an enhancement layer picture that is an IRAP picture with NoRaslOutputFlag equal to 1, and a base layer picture that is non-IRAP (assume two layer bitstream for simplicity), and either the value of the signalled slice_pic_order_cnt_lsb of the EL picture is non-zero, or the derived POC MSB value of the base layer when decoding starts from any preceding IRAP access unit is non-zero, then in order that POC of all the picture in an access unit be aligned, the POC has to be reset at the base layer, and additionally in the enhancement layer when the signalled slice_pic_order_cnt_lsb at the EL picture is non-zero. Under this circumstance, the POC LSB of base layer pictures following auA in decoding order will be assuming the reset value. Consider the example shown in Figure 1, with MaxPicOrderCntLsb equal to 128 for both the base and the enhancement layers. The bitstream has two layers, and the EL picture in AU 241 is an IRAP picture with NoRaslOutputFlag equal to 1. The POC, therefore, is reset for the base layer pictures. In a multi-layer codec, the signalling results in aligned POC values for pictures in an access unit. However, when the bitstream would be decoded by a version 1 HEVC decoder, the POC derived for the BL pictures would be incorrect in their relative ordering with other pictures.

Figure 1: POC reset when an AU contains a non-IRAP BL picture and an IRAP EL picture with NoRaslOutputFlag equal to 1. The numbers in green indicate the POC LSB and MSB after POC is reset at AU 241. The numbers in red indicate that the relative POC numbers are incorrect.

- When the picture containing POC reset information is lost, the derivation of the POC for succeeding pictures in the same layer may be incorrect.

# Proposal

A summary of the various aspects of the proposal are presented below:

- Instead of using two bits, one for POC MSB reset and one for POC LSB reset, a two-bit indication is used, to fully utilize the fact that there would never be only POC LSB reset.

- The two bits, and possibly additional information for support of missing-collocated-picture scenarios as well as for resolving the error resilience issue, are signalled in the slice header extension instead of in the up-to-seven reserved slice header bits.

- A POC resetting period is specified based on the POC resetting period ID. Each non-IRAP picture that belongs to an access unit that contains at least one IRAP picture must be the start of a POC resetting period in the layer containing the non-IRAP picture. In that access unit, each picture would be the start of a POC resetting period in the layer containing the picture. POC resetting and update of POC values of same-layer pictures in the DPB are applied only for the first picture within each POC resetting period.

- For derivation of the delta POC value used for updating the POC values of the same-layer pictures in the DPB as well as for derivation of the POC MSB of the POC value of the current picture, a POC LSB value is conditionally signalled in the slice segment header.

- A loss of a picture at the start of a POC resetting period can be overcome by having one or more same-layer pictures that follow that picture in decoding order and that are assigned the value 3 of the two-bit indication. The same process for support of missing-collocated-picture scenarios apply for resolving the error resilience issue.

- Signal POC MSB value for CRA pictures in order to

  - Derive correct rest the POC values in some use cases

  - To enable trick mode play with CRA pictures

- In output-order conformant decoders, output all pictures in the DPB before a POC resetting period at the beginning of the first access unit of the POC resetting period.

## Some examples

During the 1st, 2nd, and 3rd POC conference calls, several use cases were considered important to be supported. Some of the use-cases and how the proposed solution solves these issues are described in this section.

### Example 1

Figure 2: Example 1

Figure 2 illustrates a bitstream that is received by a decoder. The pictures that are marked with dotted lines are pictures that are not received by the decoder – possibly due to a switch-up and switch-down operation performed on the bitstream by a middle box. The solution described JCTVC-O0213v3 would not work with the above example because the POC reset in the AU that contains the CRA picture would only reset the pictures of the base layer. Then the picture order count of the preceding pictures would not be aligned within an AU, and this could result in erroneous order of output of pictures.

The proposed solution outputs all the pictures in the DPB that are marked as "needed for output" at the beginning of the first access unit of a POC resetting period. As a result even if POC reset AU containing the CRA picture only resets the POC of the BL pictures, it would not cause any erroneous order of output of pictures due to the POC mismatch.

### Example 2

Figure 3: Example 2

Figure 3 illustrates a bitstream that is received by a decoder. The pictures that are marked with dotted lines (without red cross) are pictures that are not received by the decoder – possibly due to a switch-up and switch-down operation performed on the bitstream by a middle box. When the base layer picture at the switch-up point is lost, the solution described JCTVC-O0213v4 would not work because the value needed to decrement the POC value of the base layer pictures during reset would be unknown to the IDR picture. The result would once again be erroneous order of output of pictures.

The proposed solution outputs all the pictures in the DPB that are marked as "needed for output" at the beginning of the first access unit of a POC resetting period. In the above example, this would be initiated at the IDR picture. Since the proposed solution resets the POC only within each layer, the POC would be reset for the base layer only at the subsequent AU, where the picture would have poc_reset_idc equal to 3.

## Signalling MSB values for CRA pictures

Figures 4, 5, and 6 describe why it is beneficial to signal the MSB of the POC for the CRA pictures.

Figure 4: Example 3, where POC MSB is signalled for CRA pictures

Figure 5: In the example in the figure, the POC MSB is needed for CRA pictures in order to decrement the POC of reference pictures in EL that may occur before the switch-down point.

Figure 6: In the example in the figure, POC MSB is not needed for IDR pictures

Due to coding restrictions on IDR pictures, IDR pictures and their associated pictures cannot use pictures preceding the IDR picture, in decoding order, for reference. And since the pictures preceding that POC reset AU are output at the beginning of decoding of that AU and those that belong to the same layer would be removed from the DPB soon, there is no need to reset the POC of those preceding pictures, and hence no need to signal the MSB for the IDR pictures.

Figure 7: Example of bitstream with only CRA pictures, where the signaling of MSB for the CRA pictures would allow the correct decoding of POC values for the CRA pictures.

# Some discussions on the approach proposed in v6 of JCTVC-O0275/JCT3V-F0092

- After an IRAP picture in layerA with NoRaslOutputFlag equal to 1, if there is immediately one or more AUs where there is no picture in layerA but there are pictures in those AUs of another layer, then the picture output process would not work, unless there is a restriction on output order to disallow those pictures to have output order earlier than any picture in an AU that precedes the AU containing the IRAP picture. This is similar to the output order restriction added in P0041v4. The following figure shows an example wherein wrong order of output pictures can happen without such a restriction.

- There is a hidden requirement that slice_pic_order_cnt_lsb must be cross-layer aligned. This should be clearly stated.

- On AU boundary detection, Table 1 of O0275v6 says to use (first_slice_segment_in_pic_flag equal to 1 and nuh_layer_id equal to 0) or (slice_pic_order_cnt_lsb differs in adjacent pictures). However, this is not sufficient, because the base layer picture may be absent or lost in some AUs, and the slice_pic_order_cnt_lsb values can be the same in adjacent pictures – and the requirement of cross-layer alignment of slice_pic_order_cnt_lsb increases the possibility of the same value of slice_pic_order_cnt_lsb in adjacent pictures. The following figure shows an example.

- On POC-related SEI message semantics, Table 1 of O0275v6 says to use POC for the base layer and layer-tree POC for enhancement layers. But what about nested SEI messages that apply to a set of layers containing the base layer and one or more enhancement layers? Also, since LayerTreePicOrderCntVal does not need to be cross-layer aligned, what about nested SEI messages that apply to a set of enhancement layers?

- The output process involving BaseLayerCVSCnt has three problems:

- The value range of BaseLayerCVSCnt of is undefined. So how many bits a decoder needs for that variable?

- When the value of BaseLayerCVSCnt wraps-up, the process would not work.

- The process would not work with CRA pictures with NoRaslOutputFlag equal to 1.

- Why a sign flag is needed for POC MSB cycle and layer-tree POC MSB cycle? They are needed for a current picture in an AUs with IRAPs, which follows, in output order, any previous picture in decoding order, and the MSB cycle is the difference of the MSB values of the current picture and a previous picture in an AU with IRAPs.

- The fact that both POC MSB cycle and layer-tree POC MSB cycle may be signalled, and that may be signalled also for non-IRAP pictures – and that can generate a relatively heavy overhead.

- How does the encoder sets the values of the POC MSB cycle and layer-tree POC MSB cycle?

- The conditions on when the POC MSB cycle, layer-tree POC MSB cycle, and layer_tree_poc_lsb must be signalled are pretty complicated. Please clarify.

  - One aspect we seemed to understand is that, when cross_layer_irap_aligned_flag is equal to 1, layer_tree_poc_lsb and layer_tree_poc_msb_present_flag do not need to be present. Obviously there are also other scenarios when layer_tree_poc_lsb may be absent. When layer_tree_poc_lsb is absent, LayerTreePicOrderCntVal values are not derived. However, LayerTreePicOrderCntVal values is used in the decoding process, SEI message semantics, and so on for enhancement layers. Thus, the method is broken in these scenarios.

  - Another aspect we seemed to understand is that, POC MSB cycle needs to be sent for IDR pictures in independently coded enhancement layers, and probably for IDR pictures in some other situations as well. Why it is needed for IDR pictures? Furthermore, in this case, if the base layer is an IDR picture too, and the signalled POC MSB cycle for the enhancement layer IDR picture is not equal to 0, then the POC values would not be cross-layer aligned.

- The POC anchor picture, prevTid0Pic, can have discardable_flag equal to 1. What if the picture is discarded?

- The definitions of layer tree and layer tree pic order count value are heavily dependent on the reference structure of layers, and moreover there seems to be an assumption that they only lose persistence when NoClrasOutputFlag equal to 1. However, the VPS activation could also occur at an AU that has all IDR pictures, and is complete (all layers present and all layer pictures are IDR). What happens if a new VPS is activated with a different reference structure at an all-IDR access unit – what happens to layer tree definition then?

- When is layer_tree_poc_info_present_flag equal to 1?

- When is layer_tree_poc_msb_present_flag equal to 1? Is it correct to say that it is used to calculate the MSB of the POC of the first IRAP picture of an independent layer, when that layer is not the root layer of its layer tree? If so what happens if you have two independent layers in a layer tree, and the first access unit contains BLA in independent root layer, and non-IRAP in the independent non-root layer?

- NoClrasOutputFlag is only associated with base layer. What does NoClrasOutputFlag for the IRAP picture in an independent root layer mean? Is the assumption that the at the splice point, all the pictures in each independent root layer is present?

- We think there are examples of a switch-up, where the base layer picture at the switch up point is non-IRAP and the EL is CRA, the POC derivation is ambiguous because no one single value of poc_msb_val can be used. This is related to how encoder chooses the values of poc MSB cycle.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).