| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0044 |
| --- | --- |

| Title: | On MinCR |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

The constraint on MinCR is specified to limit the worst case of CABAC decoding process. In the current level/tier definition, the value of MinCR is the same for both Main Tier and High Tier. The current MinCR was defined mainly for Main Tier and larger value is used for higher level. For example, MinCR is equal to 8 for level 5.1 (4K@60p). However this value is too large for high end professional equipments, although recent changes to working text have enabled this value to be scaled by down to ¼. In addition, the implicit minimum sequence compression ratio defined by the maximum bit rate and maximum luma sample rate is asserted to be overly restrictive when using all-intra coding or higher bit depths and chroma formats, to the point where MinCr becomes superfluous. This contribution proposes to reduce the value of MinCR and introduce a new tier to allow increased bit rates for high end professional applications.

# MinCR

The level and tier limits are specified in Table A-1 and A-2 in Annex A as below.

Table A-1 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (1000 or 1100 bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |

Table A-2 – Tier and level limits for the Main and Main 10 profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (CpbBrVclFactor or CpbBrNalFactor bits/s) |  | Min compression ratio MinCr |
| --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |
| 1 | 552 960 | 128 | - | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 |

Main tier is defined mainly for consumer applications. High tier is defined for high end consumer. The bit rate of high tier is higher than that of main tier. However MinCR is the same for both main and high tier.

For large picture, e.g. 4K video, higher value of MinCR is used to reduce the worst case complexity of CABAC. For example, the value of MinCR is 8 for level 5.1 (4K@60p). However MinCR=8 is too high for high end applications. When 4K@60p video is encoded at 160Mbps, this MinCR > 8 is normally occurred for I picture and it is not a rare case.

In addition, MinCR=8 is too high when the target quality is visually lossless. It is hard to achieve visually lossless coding due to this constraint. An HbrFactor has been recently added to the working text that allows this to be scaled.

As well as the per-frame minimum compression ratio defined by MinCR, there also exists an implied minimum compression ratio for the sequence as a whole defined by the maximum bit rate and maximum luma sample rate. In the example given above, if 4K@60p video is to be coded using level 5.1 (the lowest level at which such video can be coded), though the MinCr value is 8, the actual minimum compression ratio is defined as:

Maximum luma sample rate = 534,773,760 samples/second (equal to 4096x2176@60p)

Maximum bit rate (high tier) = 160,000,000 bits/second (main profile CpbBrVclFactor)

Baseband bit rate = 534,773,760 * 1.5 (4:2:0) * 8 (bits/sample) = 6,417,285,120 bits/second

Minimum sequence compression ratio = baseband bit rate / maximum bit rate = 40:1

Though this degree of compression may be acceptable for inter-coded sequences (as the inter pictures can be compressed to a far greater degree than the intra pictures), for all-intra-coded sequences, the MinCR-defined minimum compression ratio becomes purely academic as the sequence must be compressed 40:1 using only intra coding.

Because of this, it will be necessary to modify the maximum bit rate when all-intra coding is in use if the value of MinCR is to be at all relevant. In addition, the above example uses only 4:2:0 8-bit coding. When higher bit depths and more detailed chroma formats are in use, the minimum compression ratio will be even larger. For example, when using 4:4:4 12-bit processing, the minimum compression ratio doubles to 80:1.

Furthermore, for certain profile and level combinations (e.g. HD@30p 4:4:4 12-bit coded using level 4 - MinCr = 4 (MinCrBase) * 0.5 (MinCrScaleFactor) = 2), the Coded Picture Buffer (CPB) (30,000,000 bits at level 4) will not be able to contain a whole coded picture if that picture has compression ratio equal to MinCR. Though there is no actual requirement that a decoder must hold a whole picture at once, it may introduce difficulties in implementation.

There is, however a requirement that the decoder be able to remove pictures from the CPB in real-time. The maximum coded picture size is given as (example: HD@30p 4:4:4 12-bit coded in level 4):

Baseband picture size = 1920 * 1080 * 3 * 12 = 74,649,600 bits

Maximum compressed picture size = baseband size / MinCR = 37,324,800 bits

If the CPB is filled at the maximum bit rate (45,000,000 bits/second for 4:4:4 12-bit at level 4 (high tier)), then the decoder will be receiving only 1.2 pictures/second and will not be able to decode the sequence in real-time. It would be possible to alleviate this problem by coding in a higher level, though even for HD@30p, level 6.1 or higher is required to provide a high enough maximum bit rate.

Therefore, this contribution proposes to reduce the value of MinCR (i.e. at least supporting the recent changes made to the working text) and introduce a new tier for high-end professional applications.

# Proposal

Version 4 of the current working text (JCTVC-O1005_v4) allows MinCR to be halved and maximum bit rate to be doubled via HbrFactor when all-intra coding is in use. Though this modification to MinCR is sufficient, a greater change to the maximum bit rate is needed for use in high-end professional applications.

Since the product development of HEVC had already been started and there are some deployments in market, any changes made should not affect such deployments. There the following options are available.

Option 1:

- Add new “super tier”, selected by a flag in the profile_tier_level() syntax element.

  - When in super tier, CbpBrVclFactor and CbpBrNalFactor are increased and MinCR is reduced.

- Modify base values of CbpBrVclFactor and CbpBrNalFactor for RExt profiles to take maximum bit depth into account as well as chroma format.

- Optionally may affect version 1 profiles (though the flag will always be 0 when coded by a version 1 encoder).

Option 2:

- Add new “super tier”, selected by a flag in the profile_tier_level() syntax element.

  - When in super tier, higher values of MaxBR and MaxCPB are used.

- Adjust MinCR using the method defined in JCTVC-O1005_v4.

- Modify base values of CbpBrVclFactor and CbpBrNalFactor for RExt profiles to take maximum bit depth into account as well as chroma format. (Note that which profiles are adjusted is open to debate.)

- Does not affect version 1 profiles (the flag will always be 0 for version 1).

- Allows the CPB to always store more than one coded picture.

Option 3:

- Modify the MaxBR and MaxCPB values in the high tier.

- Adjust MinCR using the method defined in JCTVC-O1005_v4.

- No additional flags needed.

- Allows the CPB to always store more than one coded picture.

- Affects version 1 profiles.

The text changes for the proposed Option 2 is summarized in the Annex; in the previous version of this document for completeness the Annex also contained details of Options 1 & 3.

# Conclusions

In this contribution, three alternative options to reduce MinCR and increase the maximum bit rate are described; of which Option 2 is the preferred solution and is proposed to be adopted to support visually lossless coding in HEVC.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

## Option 2

Introduce super tier. Use method as in JCTVC-O1005_v4 to modify MinCR, with changes highlighted in yellow.

### Profile, tier and level syntax

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| general_profile_compatibility_flag[ j ] | u(1) |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_max_12bit_constraint_flag | u(1) |
| general_max_10bit_constraint_flag | u(1) |
| general_max_8bit_constraint_flag | u(1) |
| general_max_422chroma_constraint_flag | u(1) |
| general_max_420chroma_constraint_flag | u(1) |
| general_max_monochrome_constraint_flag | u(1) |
| general_intra_constraint_flag | u(1) |
| general_lower_bit_rate_constraint_flag | u(1) |
| general_super_tier_flag | u(1) |
| general_reserved_zero_35bits | u(35) |
| general_level_idc | u(8) |

The value of general_super_tier_flag shall be 0 except when the value of general_tier_flag is 0. The value 1 of general_super_tier_flag when general_tier_flag is 1 is reserved for future use.

[Ed. The following italicized section is taken from various parts of Annex A]

Maximum bit rate is derived as follows:

VCL HRD parameter: Max bit rate = CpbBrVclFactor * MaxBR

NAL HRD parameter: Max bit rate = CpbBrNalFactor * MaxBR

MinCR is derived as follows:

MinCR = MinCrBase * MinCrScaleFactor

HbrFactor is derived as follows:

HbrFactor = 2 − general_lower_bit_rate_constraint_flag

where MaxBR and MinCrBase are defined in Table A-2, CpbBrVclFactor, CpbBrNalFactor,  FormatCapabilityFactor and MinCrScaleFactor are defined in Table A-3.

Table A-1 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (1000 or 1100 bits) |  |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Super tier |  |  |  |
| 1 | 36 864 | 350 | - | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 120 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 200 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 400 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 640 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 960 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 960 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 1 920 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 3 200 000 | 600 | 22 | 20 |

Table A-2 – Tier and level limits for the Monochrome 12, Monochrome 16, Main, Main 10, Main 12,
Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4, Main 4:4:4 10, and Main 4:4:4 12, Main Intra, Main 10 Intra, Main 12 Intra, Main 4:2:2 10 Intra, Main 4:2:2 12 Intra, Main 4:4:4 Intra, Main 4:4:4 10 Intra, and Main 4:4:4 12 Intra profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (CpbBrVclFactor or CpbBrNalFactor bits/s) |  |  | Min compression ratio base MinCrBase |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Super tier |  |
| 1 | 552 960 | 128 | - | - | 2 |
| 2 | 3 686 400 | 1 500 | - | - | 2 |
| 2.1 | 7 372 800 | 3 000 | - | - | 2 |
| 3 | 16 588 800 | 6 000 | - | - | 2 |
| 3.1 | 33 177 600 | 10 000 | - | - | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 120 000 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 200 000 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 400 000 | 6 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 640 000 | 8 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 960 000 | 8 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 960 000 | 8 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 1 920 000 | 8 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 3 200 000 | 6 |

Table A-3 – Specification of CpbBrVclFactor, CpbBrNalFactor, FormatCapabilityFactor, and MinCrScaleFactor

| Profile | CpbBrVclFactor | CpbBrNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Monochrome 12 | 1000 | 1100 | 1.500 | 1.0 |
| Monochrome 16 | 1330 | 1470 | 2.000 | 1.0 |
| Main | 1000 | 1100 | 1.500 | 1.0 |
| Main 10 | 1000 | 1100 | 1.875 | 1.0 |
| Main 12 | 1500 | 1650 | 2.250 | 1.0 |
| Main 4:2:2 10 | 1670 | 1830 | 2.500 | 0.5 |
| Main 4:2:2 12 | 2000 | 2200 | 3.000 | 0.5 |
| Main 4:4:4 | 2000 | 2200 | 3.000 | 0.5 |
| Main 4:4:4 10 | 2500 | 2750 | 3.750 | 0.5 |
| Main 4:4:4 12 | 3000 | 3300 | 4.500 | 0.5 |
| Main Intra | 1000 * HbrFactor | 1100 * HbrFactor | 1.500 | 1.0 ÷ HbrFactor |
| Main 10 Intra | 1250 * HbrFactor | 1375 * HbrFactor | 1.875 | 1.0 ÷ HbrFactor |
| Main 12 Intra | 1500 * HbrFactor | 1650 * HbrFactor | 2.250 | 1.0 ÷ HbrFactor |
| Main 4:2:2 10 Intra | 1670 * HbrFactor | 1830 * HbrFactor | 2.500 | 0.5 ÷ HbrFactor |
| Main 4:2:2 12 Intra | 2000 * HbrFactor | 2200 * HbrFactor | 3.000 | 0.5 ÷ HbrFactor |
| Main 4:4:4 Intra | 2000 * HbrFactor | 2200 * HbrFactor | 3.000 | 0.5 ÷ HbrFactor |
| Main 4:4:4 10 Intra | 2500 * HbrFactor | 2750 * HbrFactor | 3.750 | 0.5 ÷ HbrFactor |
| Main 4:4:4 12 Intra | 3000 * HbrFactor | 3300 * HbrFactor | 4.500 | 0.5 ÷ HbrFactor |

[Ed. the CpbBrVclFactor have been scaled by bit-depth over previous definition in O1005_v4 where 8-bit was assumed. 16-bit profiles would follow the same rules as defined here (P0057,P0058). Note also that which profiles are changed is open to debate and are just indicating a potential conflict.]

[Ed. the following table is illustrative]

Long GOP profiles

| Tier | general_tier_flag | general_super_tier_flag | general_intra_constraint_flag | general_lower_bit_rate_constraint_flag | HbrFactor |
| --- | --- | --- | --- | --- | --- |
| Main | 0 | 0 | 0 or 1 | 1 | 1 |
| High | 1 | 0 | 0 or 1 | 1 | 1 |
| Super | 0 | 1 | 0 or 1 | 1 | 1 |

[Ed. the following table is illustrative]

All Intra profiles

| Tier | general_tier_flag | general_super_tier_flag | general_intra_constraint_flag | general_lower_bit_rate_constraint_flag | HbrFactor |
| --- | --- | --- | --- | --- | --- |
| Main | 0 | 0 | 1 | 0 or 1 | 2 or 1 |
| High | 1 | 0 | 1 | 0 or 1 | 2 or 1 |
| Super | 0 | 1 | 1 | 0 or 1 | 2 or 1 |