| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0050-v2 |
| --- | --- |

| Title: | HLS: SEI message for Knee Function Information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sally Hattori, Ohji Nakagami, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 shinobu.hattori@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a new SEI message to signal knee function information to enable display devices to process decompression or compression of colour samples of the decoded pictures to customize the picture for a particular display environment. The idea was initially proposed in JCTVC-O0064[1]. This contribution made two modifications to the proposal in JCTVC-O0064[1]. 1) The syntax and semantics are enhanced to provide enable flexibility to the characteristics of knee function parameters. 2) Two options on how to integrate the proposed syntax and semantics as SEI messages are provided. The first option is to define a new SEI message dedicated for the knee function information. The second option is to extend the existing tone mapping information SEI message to define knee function information as another model of mapping function.

The version 2 of this contribution includes following changes:

- Change in syntax name from “compression_flag” to “mapping_flag”

- Change in equation from using “x” to “*”.

# Introduction

As introduced in JCTVC-O0064[1], the video cameras typically ameliorate their sensor’s harsh clipping with a knee function which allows compression of the upper portion of the dynamic range. The usage of using a knee function by camera or the video production process is widely being practiced today. The typical compression characteristics of the knee function are shown in Figure 1-1. A knee function composes from slopes (lines) which connects the following three types of points normalized in range of 0.0 to 1.0.

- An origin – (0.0, 0.0)

- A knee point which is composed of an input dynamic range and an output dynamic range – (a, b)

- A point of (1.0, 1.0)

The D range (Dynamic range) in Figure 1-1 represents the peak signal level of the original picture in relative to the nominal luminance level.

Figure 1-1 Example of a knee function and a dynamic range

The purpose of using the knee function has been to fit the dynamic range of the pictures to the display brightness capability which is lower than what can be captured. However, with the recent evolvement of the display brightness, many displays are becoming capable of displaying higher dynamic range with brighter luminance. For these displays, providing parameters to enable decompressing the dynamic range to derive higher dynamic range can be very beneficial to make full use of the display capability.

# Proposals

This contribution proposes to enable signaling of characteristics of knee function with multiple knee points. This enables flexible compression/decompression depending on the corresponding dynamic range. For example, the compression in the range of lower dynamic range and the compression in the range of higher dynamic range may be preferred to be different. Multiple knee points enable to adjust compression by the setting different slopes.

The proposed syntax and semantics of knee function parameters are as shown in Table 2-1.

Table 2-1 Proposed syntax and semantics for knee function parameters

| Syntax | Descriptor | Semantics |
| --- | --- | --- |
| mapping_flag | u(1) | mapping_flag equals to 0 indicates that the knee function is decompression function. mapping_flag equals to 1 indicates that the knee function is compression function. |
| input_d_range | u(32) | input_d_range specifies the peak luminance level for the input picture of the knee function process in relative to the nominal luminance level in units of 0.1%. When the value of input_d_range is 0, the peak luminance level of the input picture is unspecified. |
| input_disp_luminance | u(32) | input_disp_luminance specifies the expected display brightness of peak luminance level for the input picture of the knee function process. The value of input_disp_luminance is in units of candela per square metre. When the value of input_disp_luminance is 0, the expected display brightness of peak luminance level for the input picture is unspecified. |
| output_d_range | u(32) | output_d_range specifies the peak luminance level for the output picture of the knee function process in relative to the nominal luminance level in units of 0.1%. When the value of output_d_range is 0, the peak luminance level of the output picture is unspecified. |
| output_disp_luminance | u(32) | output_disp_luminance specifies the expected display brightness of peak luminance level for the output picture of the knee function process. The value of output_disp_luminance is in units of candela per square metre. When the value of output_disp_luminance is 0, the expected display brightness of peak luminance level for the output picture is unspecified. |
| num_knee_points_minus1 | ue(v) | num_knee_points_minus1 plus 1 specifies the number of knee points. |
| for( i = 0; i <= num_knee_points_minus1; i++ ) { |  |  |
| input_knee_point [ i ] | u(10) | input_knee_point [ i ] specifies the luminance level of i-th knee point for input picture of the knee function process. The luminance level of the knee point for input picture is normalized in range of 0 to 1.0 in units of 0.1%. The value of input_knee_point shall be greater than or equal to 1 and less than or equal to 999. The value of i-th input_knee_point shall be greater than the value of (i-1)-th input_knee_point. |
| output_knee_point [ i ] | u(10) | output_knee_point [ i ] specifies the luminance level of i-th knee for output picture of the knee function process. The luminance level of the knee point for output picture is normalized in range of 0 to 1.0 in units of 0.1%. The value of output_knee_point shall be greater than or equal to 0 and less than or equal to 1000. NOTE – The coordinate of the i-th knee point is defined as (input_knee_point[ i ], output_knee_point[ i ]). |
| } |  |  |

The Figure 2-2 shows example of proposed knee function parameters for the purpose of decompression process of output imaged. The Figure 2-3 shows example of knee function parameters for the purpose of compression process of output images.

Figure 2-2 Example of knee function information for decompression of colour samples

Figure 2-3 Example of knee function information for compression of colour samples

In the contribution, two options to integrate the proposed syntax and semantics for the knee function pramaeters are proposed. The two options are as shown in the following sub-sections.

## Option 1

The first option is to define a new SEI message dedicated for the knee function information. This option provides simpler identification and enables simpler implementation of the SEI message.

The proposed text changes, syntax and semantics of the SEI message are as follows.

### Proposed Text changes

D.2.1 General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 6 ) |  |
| recovery_point( payloadSize ) |  |
| else if( payloadType = = 9 ) |  |
| scene_info( payloadSize ) |  |
| else if( payloadType = = 15 ) |  |
| picture_snapshot( payloadSize ) |  |
| else if( payloadType = = 16 ) |  |
| progressive_refinement_segment_start( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 19 ) |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 23 ) |  |
| tone_mapping_info( payloadSize ) |  |
| else if( payloadType = = 24 ) |  |
| knee_function_info( payloadSize ) |  |
| else if( payloadType = = 45 ) |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 128 ) |  |
| structure_of_pictures_info( payloadSize ) |  |
| else if( payloadType = = 129 ) |  |
| active_parameter_sets( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 131 ) |  |
| temporal_sub_layer_zero_index( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 134 ) |  |
| region_refresh_info( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 132 ) |  |
| decoded_picture_hash( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

D.3.1 General SEI payload semantics

Table D-1 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| Buffering period | - The remainder of the bitstream |
| Picture timing | - The access unit containing the SEI message |
| Pan-scan rectangle | - Specified by the syntax of the SEI message |
| Filler payload | - The access unit containing the SEI message |
| User data registered by Rec. ITU-T T.35 | - Unspecified |
| User data unregistered | - Unspecified |
| Recovery point | - Specified by the syntax of the SEI message |
| Scene information | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains a scene information SEI message |
| Picture snapshot | - The access unit containing the SEI message |
| Progressive refinement segment start | - Specified by the syntax of the SEI message |
| Progressive refinement segment end | - The access unit containing the SEI message |
| Film grain characteristics | - Specified by the syntax of the SEI message |
| Post-filter hint | - The access unit containing the SEI message |
| Tone mapping information | - Specified by the syntax of the SEI message |
| Frame packing arrangement | - Specified by the syntax of the SEI message |
| Display orientation | - Specified by the syntax of the SEI message |
| Structure of pictures information | - The set of access units in the CVS that correspond to entries listed in the SEI message |
| Decoded picture hash | - The access unit containing the SEI message |
| Active parameter sets | - The CVS containing the SEI message |
| Decoding unit information | - The decoding unit containing the SEI message |
| Temporal sub-layer zero index | - The access unit containing the SEI message |
| Scalable nesting | - Depending on the nested SEI messages. Each nested SEI message has the same persistence scope as if the SEI message was not nested |
| Region refresh information | - The set of VCL NAL units within the access unit starting from the VCL NAL unit following the SEI message up to but not including the VCL NAL unit following the next SEI NAL unit containing a region refresh information SEI message (if any) |
| Knee function information | - Specified by the syntax of the SEI message |

It is a requirement of bitstream conformance that when a prefix SEI message with payloadType equal to 17 (progressive refinement segment start) or 22 (post-filter hint) is present in an access unit, a suffix SEI message with the same value of payloadType shall not be present in the same access unit access unit.

Let prevVclNalUnitInAu of an SEI NAL unit or an SEI message be the preceding VCL NAL unit in decoding order, if any, in the same access unit, and nextVclNalUnitInAu of an SEI NAL unit or an SEI message be the next VCL NAL unit in decoding order, if any, in the same access unit. It is a requirement of bitstream conformance that the following restrictions apply:

–	An SEI NAL unit containing an active parameter sets SEI message shall contain only one active parameter sets SEI message and shall not contain any other SEI messages.

–	When an SEI NAL unit containing an active parameter sets SEI message is present in an access unit, it shall be the first SEI NAL unit that follows the prevVclNalUnitInAu of the SEI NAL unit and precedes the nextVclNalUnitInAu of the SEI NAL unit.

–	When an SEI NAL unit contains a non-nested buffering period SEI message, a non-nested picture timing SEI message, or a non-nested decoding unit information SEI message, the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information).

–	When an SEI NAL unit contains a nested buffering period SEI message, a nested picture timing SEI message, or a nested decoding unit information SEI message, the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0 (buffering period), 1 (picture timing), 130 (decoding unit information), or 133 (scalable nesting).

–	When a non-nested buffering period SEI message is present in an access unit, it shall not follow any other SEI message that follows the prevVclNalUnitInAu of the buffering period SEI message and precedes the nextVclNalUnitInAu of the buffering period SEI message, other than an active parameter sets SEI message.

–	When a non-nested picture timing SEI message is present in an access unit, it shall not follow any other SEI message that follows the prevVclNalUnitInAu of the picture timing SEI message and precedes the nextVclNalUnitInAu of the picture timing SEI message, other than an active parameter sets SEI message or a non-nested buffering period SEI message.

–	When a non-nested decoding unit information SEI message is present in an access unit, it shall not follow any other SEI message in the same access unit that follows the prevVclNalUnitInAu of the decoding unit information SEI message and precedes the nextVclNalUnitInAu of the decoding unit information SEI message, other than an active parameter sets SEI message, a non-nested buffering period SEI message, or a non-nested picture timing SEI message.

–	When a nested buffering period SEI message, a nested picture timing SEI message, or a nested decoding unit information SEI message is contained in a scalable nesting SEI message in an access unit, the scalable nesting SEI message shall not follow any other SEI message that follows the prevVclNalUnitInAu of the scalable nesting SEI message and precedes the nextVclNalUnitInAu of the scalable nesting SEI message, other than an active parameter sets SEI message, a non-nested buffering period SEI message, a non-nested picture timing SEI message, a non-nested decoding unit information SEI message, or another scalable nesting SEI message that contains a buffering period SEI message, a picture timing SEI message, or a decoding unit information SEI message.

For a non-nested SEI message, depending on the value of payloadType, the following applies:

–	If payloadType is equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information), the non-nested SEI message applies to the operation point that has OpTid equal to the greatest value of nuh_temporal_id_plus1 among all VCL NAL units in the bitstream, and that has OpLayerIdList containing all values of nuh_layer_id in all VCL units in the bitstream.

–	Otherwise, when payloadType is equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 24, 45, 47, 128, 131, or 134 (i.e. one of the SEI messages that have payloadType not equal to 0, 1, or 130, and that are allowed to be nested SEI messages), the non-nested SEI message applies to the layer for which the VCL NAL units have nuh_layer_id equal to the nuh_layer_id of the SEI NAL unit containing the SEI message.

It is a requirement of bitstream conformance that the following restrictions apply on nesting of SEI messages:

–	A scalable nesting SEI message shall not be nested in a scalable nesting SEI message.

–	An active parameter sets SEI message shall not be nested in a scalable nesting SEI message.

–	When a scalable nesting SEI message contains a buffering period SEI message, a picture timing SEI message, or a decoding unit information SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information).

–	When a non-nested SEI message has payloadType equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 24, 45, 47, 128, 131, or 134 (i.e. one of the SEI messages that have payloadType not equal to 0, 1, or 130, and that are allowed to be nested SEI messages), the SEI NAL unit containing the non-nested SEI message shall have TemporalId equal to the TemporalId of the access unit containing the SEI NAL unit.

It is a requirement of bitstream conformance that the following restrictions apply on the presence of SEI messages between two VCL NAL units of an access unit:

–	When there is a prefix SEI message that has payloadType equal to 0, 1, 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 24, 45, 47, 128, 129, or 131 (i.e. one of the prefix SEI messages that are not user data registered by Rec. ITU-T T.35 SEI message, user data unregistered SEI message, decoding unit information SEI message, scalable nesting SEI message, or region refresh information SEI message) between two VCL NAL units of an access unit in decoding order, there shall be a prefix SEI message of the same type in the same access unit preceding the first VCL NAL unit of the access unit.

–	When there is a suffix SEI message that has payloadType equal to 3 (filler payload), 17 (progressive refinement segment end), 22 (post filter hint), or 132 (decoded picture hash) between two VCL NAL units of an access unit in decoding order, there shall be a suffix SEI message of the same type in the same access unit succeeding the last VCL NAL unit of the access unit.

It is a requirement of bitstream conformance that the following restrictions apply on repetition of SEI messages:

–	For each of the following payloadType values, there shall be less than or equal to 8 identical sei_payload( ) syntax structures within an access unit: 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 24, 45, 47, 128, 129, 131, 132, and 133.

–	There shall be less than or equal to 8 identical sei_payload( ) syntax structures with payloadType equal to 130 within a decoding unit.

–	The number of identical sei_payload( ) syntax structures with payloadType equal to 134 in an access unit shall be less than or equal to the number of slice segments in the access unit.

D.2.2526 Reserved SEI message syntax

D.3.2526 Reserved SEI message semantics

### Proposed new SEI message syntax and semantics

D.2.25 Knee function information SEI message syntax

| knee_function_info ( payloadSize ) { | Descriptor |
| --- | --- |
| knee_function_id | ue(v) |
| knee_function_cancel_flag | u(1) |
| if( ! knee_function_cancel_flag ) { |  |
| knee_function_persistence_flag | u(1) |
| mapping_flag | u(1) |
| input_d_range | u(32) |
| input_disp_luminance | u(32) |
| output_d_range | u(32) |
| output_disp_luminance | u(32) |
| num_knee_points_minus1 | ue(v) |
| for( i = 0; i <= num_knee_points_minus1; i++ ) { |  |
| input_knee_point [ i ] | u(10) |
| output_knee_point [ i ] | u(10) |
| } |  |
| } |  |
| } |  |

D.3.25 Knee function information SEI message semantics

This SEI message provides information to enable decompression or compression of the colour samples of the output decoded pictures by using knee function for customization to particular display environments. The decompression or compression process maps the white level of sample values in the normalized linear RGB colour space. The compression/decompression should be applied to each RGB component produced by colour space conversion of the decoded image accordingly.

A knee function is a function which is composed of slopes which connects three types of points with values normalized in range of 0.0 to 1.0. The three types of points are defined as follows.

- An origin – (0.0, 0.0)

- A knee point of an input dynamic range and an output dynamic range – (xi, yi)

- A point of (1.0, 1.0)

Figure D.10 A Knee Function

knee_function_id contains an identifying number that may be used to identify the purpose of the knee functions. The value of knee_function_id shall be in the range of 0 to 232 − 2, inclusive.

Values of knee_function_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of knee_function_id from 256 to 511, inclusive, and from 231 to 232 – 2, inclusive are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all knee function information SEI messages containing a value of knee_function_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

NOTE 1 – The knee_function_id can be used to support knee function process that are suitable for different display scenarios. For example, different values of knee_function_id may correspond to different display bit depths.

knee_function_cancel_flag equal to 1 indicates that the knee function information SEI message cancels the persistence of any previous knee function information SEI message in output order. knee_function_cancel_flag equal to 0 indicates that knee function information follows.

knee_function _persistence_flag specifies the persistence of the knee function information SEI message.

knee_function _persistence_flag equal to 0 specifies that the knee function information applies to the current decoded picture only.

knee_function _persistence_flag equal to 1 specifies that the knee function information persists in output order until any of the following conditions are true:

–	A new CVS begins.

–	A picture in an access unit containing a knee function information SEI message with the same value of knee_function_id is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ).

mapping_flag equals to 0 indicates that the knee function is decompression function. mapping_flag equals to 1 indicates that the knee function is compression function.

input_d_range specifies the peak luminance level for the input picture of the knee function process in relative to the nominal luminance level in units of 0.1%. When the value of input_d_range is 0, the peak luminance level of the input picture is unspecified.

input_disp_luminance specifies the expected display brightness of peak luminance level for the input picture of the knee function process. The value of input_disp_luminance is in units of candela per square metre. When the value of input_disp_luminance is 0, the expected display brightness of peak luminance level for the input picture is unspecified.

output_d_range specifies the peak luminance level for the output picture of the knee function process in relative to the nominal luminance level in units of 0.1%. When the value of output_d_range is 0, the peak luminance level of the output picture is unspecified.

output_disp_luminance specifies the expected display brightness of peak luminance level for the output picture of the knee function process. The value of output_disp_luminance is in units of candela per square metre. When the value of output_disp_luminance is 0, the expected display brightness of peak luminance level for the output picture is unspecified.

num_knee_points_minus1 plus 1 specifies the number of knee points. num_knee_points_minus1 shall be in the range of 0 to 998, inclusive.

input_knee_point[ i ] specifies the luminance level of i-th knee point for input picture of the knee function process. The luminance level of the knee point for input picture is normalized in range of 0 to 1.0 in units of 0.1%. The value of input_knee_point shall be greater than or equal to 1 and less than or equal to 999. The value of i-th input_knee_point shall be greater than the value of (i-1)-th input_knee_point.

output_knee_point[ i ] specifies the luminance level of i-th knee for output picture of the knee function process. The luminance level of the knee point for output picture is normalized in range of 0 to 1.0 in units of 0.1%. The value of output_knee_point shall be greater than or equal to 0 and less than or equal to 1000.

NOTE 2 – The coordinate of the i-th knee point is defined as (input_knee_point[ i ], output_knee_point[ i ]).

NOTE 3 – The knee function is composed of several lines which starts with the point (0.0,0.0) ends with the point (1.0,1.0) through all knee points in ascending order of i.

NOTE 4 – The input dynamic range and the output dynamic range can be derived with following equations.

(D-26)

(D-27)

  - Option 2

The second option is to extend the existing SEI message, tone mapping information SEI message, to signal the knee function parameters as new model of colour sample mapping function. With this option, there is no need to define a new SEI message.

The proposed text change, syntax and semantics are as follows.

D.2.15 Tone mapping information SEI message syntax

| tone_mapping_info( payloadSize ) { | Descriptor |
| --- | --- |
| tone_map_id | ue(v) |
| tone_map_cancel_flag | u(1) |
| if( !tone_map_cancel_flag ) { |  |
| tone_map_persistence_flag | u(1) |
| coded_data_bit_depth | u(8) |
| target_bit_depth | u(8) |
| tone_map_model_id | ue(v) |
| if( tone_map_model_id = = 0 ) { |  |
| min_value | u(32) |
| max_value | u(32) |
| } else if( tone_map_model_id = = 1 ) { |  |
| sigmoid_midpoint | u(32) |
| sigmoid_width | u(32) |
| } else if( tone_map_model_id = = 2 ) |  |
| for( i = 0; i < ( 1 << target_bit_depth ); i++ ) |  |
| start_of_coded_interval[ i ] | u(v) |
| else if( tone_map_model_id = = 3 ) { |  |
| num_pivots | u(16) |
| for( i = 0; i < num_pivots; i++ ) { |  |
| coded_pivot_value[ i ] | u(v) |
| target_pivot_value[ i ] | u(v) |
| } |  |
| } else if( tone_map_model_id = = 4 ) { |  |
| camera_iso_speed_idc | u(8) |
| if( camera_iso_speed_idc = = EXTENDED_ISO ) |  |
| camera_iso_speed_value | u(32) |
| exposure_index_idc | u(8) |
| if( exposure_index_idc = = EXTENDED_ISO ) |  |
| exposure_index_value | u(32) |
| exposure_compensation_value_sign_flag | u(1) |
| exposure_compensation_value_numerator | u(16) |
| exposure_compensation_value_denom_idc | u(16) |
| ref_screen_luminance_white | u(32) |
| extended_range_white_level | u(32) |
| nominal_black_level_code_value | u(16) |
| nominal_white_level_code_value | u(16) |
| extended_white_level_code_value | u(16) |
| } else if( tone_map_model_id = = 5 ) { |  |
| mapping_flag | u(1) |
| input_d_range | u(32) |
| input_disp_luminance | u(32) |
| output_d_range | u(32) |
| output_disp_luminance | u(32) |
| num_knee_points_minus1 | ue(v) |
| for( i = 0; i <= num_knee_points_minus1; i++ ) { |  |
| input_knee_point [ i ] | u(10) |
| output_knee_point [ i ] | u(10) |
| } |  |
| } |  |
| } |  |
| } |  |

D.3.15 Tone mapping information SEI message semantics

This SEI message provides information to enable remapping of the colour samples of the output decoded pictures for customization to particular display environments. The remapping process maps coded sample values in the RGB colour space (specified in Annex Error: Reference source not found) to target sample values. The mappings are expressed either in the luma or RGB colour space domain, and should be applied to the luma component or to each RGB component produced by colour space conversion of the decoded image accordingly.

tone_map_id contains an identifying number that may be used to identify the purpose of the tone mapping model. The value of tone_map_id shall be in the range of 0 to 232 − 2, inclusive.

Values of tone_map_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of tone_map_id from 256 to 511, inclusive, and from 231 to 232 – 2, inclusive are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all tone mapping information SEI messages containing a value of tone_map_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

NOTE 1 – The tone_map_id can be used to support tone mapping operations that are suitable for different display scenarios. For example, different values of tone_map_id may correspond to different display bit depths.

tone_map_cancel_flag equal to 1 indicates that the tone mapping information SEI message cancels the persistence of any previous tone mapping information SEI message in output order. tone_map_cancel_flag equal to 0 indicates that tone mapping information follows.

tone_map_persistence_flag specifies the persistence of the tone mapping information SEI message.

tone_map_persistence_flag equal to 0 specifies that the tone mapping information applies to the current decoded picture only.

tone_map_persistence_flag equal to 1 specifies that the tone mapping information persists in output order until any of the following conditions are true:

–	A new CVS begins.

–	A picture in an access unit containing a tone mapping information SEI message with the same value of tone_map_id is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ).

coded_data_bit_depth specifies the BitDepthY for interpretation of the luma component of the associated pictures for purposes of interpretation of the tone mapping information SEI message. When tone mapping information SEI messages are present that have coded_data_bit_depth that is not equal to BitDepthY, these refer to the hypothetical result of a transcoding operation performed to convert the coded video to the BitDepthY corresponding to the value of coded_data_bit_depth.

The value of coded_data_bit_depth shall be in the range of 8 to 14, inclusive. Values of coded_data_bit_depth from 0 to 7 and from 15 to 255 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all tone mapping SEI messages that contain a coded_data_bit_depth in the range of 0 to 7, inclusive, or in the range of 15 to 255, inclusive, and bitstreams shall not contain such values.

target_bit_depth specifies the bit depth of the output of the dynamic range mapping function (or tone mapping function) described by the tone mapping information SEI message. The tone mapping function specified with a particular target_bit_depth is suggested to be reasonable for all display bit depths that are less than or equal to the target_bit_depth.

The value of target_bit_depth shall be in the range of 1 to 16, inclusive. Values of target_bit_depth equal to 0 and in the range of 17 to 255, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all tone mapping SEI messages that contain a value of target_bit_depth equal to 0 or in the range of 17 to 255, inclusive, and bitstreams shall not contain such values.

tone_map_model_id specifies the model utilized for mapping the coded data into the target_bit_depth range. Values greater than 45 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore all tone mapping SEI messages that contain a value of tone_map_model_id greater than 45, and bitstreams shall not contain such values.

NOTE 2 – A tone_map_model_id of 0 corresponds to a linear mapping with clipping; a tone_map_model_id of 1 corresponds to a sigmoidal mapping; a tone_map_model_id of 2 corresponds to a user-defined table mapping, and a tone_map_model_id of 3 corresponds to a piece-wise linear mapping, tone_map_model_id of 4 corresponds to luminance dynamic range information, tone_map_model_id of 5 corresponds to knee function decompression or compression.

min_value specifies the RGB sample value that maps to the minimum value in the bit depth indicated by target_bit_depth. It is used in combination with the max_value parameter. All sample values in the decoded picture that are less than or equal to min_value, after conversion to RGB as necessary, are mapped to this minimum value in the target_bit_depth representation.

max_value specifies the RGB sample value that maps to the maximum value in the bit depth indicated by target_bit_depth. It is used in combination with the min_value parameter. All sample values in the decoded picture that are greater than or equal to max_value, after conversion to RGB as necessary, are mapped to this maximum value in the target_bit_depth representation.

When present, max_value shall be greater than or equal to min_value.

sigmoid_midpoint specifies the RGB sample value of the coded data that is mapped to the centre point of the target_bit_depth representation. It is used in combination with the sigmoid_width parameter.

sigmoid_width specifies the distance between two coded data values that approximately correspond to the 5% and 95% values of the target_bit_depth representation, respectively. It is used in combination with the sigmoid_midpoint parameter and is interpreted according to the following function:

(D-1)

where f( i ) denotes the function that maps an RGB sample value i from the coded data to a resulting RGB sample value in the target_bit_depth representation.

start_of_coded_interval[ i ] specifies the beginning point of an interval in the coded data such that all RGB sample values that are greater than or equal to start_of_coded_interval[ i ] and less than start_of_coded_interval[ i + 1 ] are mapped to i in the target bit depth representation. The value of start_of_coded_interval[ 2target_bit_depth ] is equal to 2coded_data_bit_depth. The number of bits used for the representation of the start_of_coded_interval is ( ( coded_data_bit_depth + 7 )  >>  3 )  <<  3.

num_pivots specifies the number of pivot points in the piece-wise linear mapping function without counting the two default end points, (0, 0) and ( 2coded_data_bit_depth − 1, 2target_bit_depth − 1 ).

coded_pivot_value[ i ] specifies the value in the coded_data_bit_depth corresponding to the i-th pivot point. The number of bits used for the representation of the coded_pivot_value is ( ( coded_data_bit_depth + 7 ) >> 3 )  <<  3.

target_pivot_value[ i ] specifies the value in the reference target_bit_depth corresponding to the i-th pivot point. The number of bits used for the representation of the target_pivot_value is ( ( target_bit_depth + 7 )  >>  3 )  <<  3.

camera_iso_speed_idc indicates the camera ISO speed for daylight illumination as specified in ISO 12232, interpreted as specified in Table D-2. When camera_iso_speed_idc indicates EXTENDED_ISO, the ISO speed is indicated by camera_iso_speed_value.

camera_iso_speed_value indicates the camera ISO speed for daylight illumination as specified in ISO 12232 when camera_iso_speed_idc indicates EXTENDED_ISO. The value of camera_iso_speed_value shall not be equal to 0.

exposure_index_idc indicates the exposure index setting of the camera as specified in ISO 12232, interpreted as specified in Table D-2. When exposure_index_idc indicates EXTENDED_ISO, the exposure index is indicated by exposure_index_value.

The values of camera_iso_speed_idc and exposure_index_idc in the range of 31 to 254, inclusive, are reserved for future use by ITU-T | ISO/IEC, and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore tone mapping SEI messages that contain these values.

exposure_index_value indicates the exposure index setting of the camera as specified in ISO 12232 when exposure_index_idc indicates EXTENDED_ISO. The value of exposure_index_value shall not be equal to 0.

Table D-2 – Interpretation of camera_iso_speed_idc and exposure_index_idc

| - camera_iso_speed_idc or exposure_index_idc | - Indicated value |
| --- | --- |
| 0 | - Unspecified |
| 1 | - 10 |
| 2 | - 12 |
| 3 | - 16 |
| 4 | - 20 |
| 5 | - 25 |
| 6 | - 32 |
| 7 | - 40 |
| 8 | - 50 |
| 9 | - 64 |
| 10 | - 80 |
| 11 | - 100 |
| 12 | - 125 |
| 13 | - 160 |
| 14 | - 200 |
| 15 | - 250 |
| 16 | - 320 |
| 17 | - 400 |
| 18 | - 500 |
| 19 | - 640 |
| 20 | - 800 |
| 21 | - 1000 |
| 22 | - 1250 |
| 23 | - 1600 |
| 24 | - 2000 |
| 25 | - 2500 |
| 26 | - 3200 |
| 27 | - 4000 |
| 28 | - 5000 |
| 29 | - 6400 |
| 30 | - 8000 |
| 31..254 | - Reserved |
| 255 | - EXTENDED_ISO |

exposure_compensation_value_sign_flag, when applicable as specified below, specifies the sign of the variable ExposureCompensationValue that indicates the exposure compensation value setting used for the process of image production.

exposure_compensation_value_numerator, when applicable as specified below, specifies the numerator of the variable ExposureCompensationValue that indicates the exposure compensation value setting used for the process of image production.

exposure_compensation_value_denom_idc, when not equal to 0, specifies the denominator of the variable ExposureCompensationValue that indicates the exposure compensation value setting used for the process of image production.

When exposure_compensation_value_denom_idc is present and not equal to 0, the variable ExposureCompensationValue is derived from exposure_compensation_value_sign_flag, exposure_compensation_value_numerator and exposure_compensation_value_denom_idc. exposure_compensation_value_sign_flag equal to 0 indicates that the ExposureCompensationValue is positive. exposure_compensation_value_sign_flag equal to 1 indicates that the ExposureCompensationValue is negative. When ExposureCompensationValue is positive, the image is indicated to have been further sensitized through the process of production, relative to the recommended exposure index of the camera as specified in ISO 12232. When ExposureCompensationValue is negative, the image is indicated to have been further desensitized through the process of production, relative to the recommended exposure index of the camera as specified in ISO 12232.

When exposure_compensation_value_denom_idc is present and not equal to 0, the variable ExposureCompensationValue is derived as follows:

ExposureCompensationValue = ( 1 − 2 * exposure_compensation_value_sign_flag ) *
		exposure_compensation_value_numerator 
		exposure_compensation_value_denom_idc		(D-2)

The value of ExposureCompensationValue is interpreted in units of exposure steps such that an increase of 1 in ExposureCompensationValue corresponds to a doubling of exposure in units of lux-seconds. For example, the exposure compensation value equal to +12 at the production stage may be indicated by setting exposure_compensation_value_sign_flag to 0, exposure_compensation_value_numerator to 1, and exposure_compensation_value_denom_idc to 2.

When exposure_compensation_value_denom_idc is present and equal to 0, the exposure compensation value is indicated as unknown or unspecified.

ref_screen_luminance_white indicates the reference screen brightness setting for the extended white level used for image production process in units of candela per square metre.

extended_range_white_level indicates the luminance dynamic range for extended dynamic-range display of the associated pictures, after conversion to the linear light domain for display, expressed as an integer percentage relative to the nominal white level. The value of extended_range_white_level should be greater than or equal to 100.

nominal_black_level_code_value indicates the luma sample value of the associated decoded pictures to which the nominal black level is assigned. For example, when coded_data_bit_depth is equal to 8, video_full_range_flag is equal to 0, and matrix_coeffs is equal to 1, nominal_black_level_code_value should be equal to 16.

nominal_white_level_code_value indicates the luma sample value of the associated decoded pictures to which the nominal white level is assigned. For example, when coded_data_bit_depth is equal to 8, video_full_range_flag is equal to 0, and matrix_coeffs is equal to 1, nominal_white_level_code_value should be equal to 235. When present, the value of nominal_white_level_code_value shall be greater than nominal_black_level_code_value.

extended_white_level_code_value indicates the luma sample value of the associated decoded pictures to which the white level associated with an extended dynamic range is assigned. When present, the value of extended_white_level_code_value shall be greater than or equal to nominal_white_level_code_value.

mapping_flag equals to 0 indicates that the knee function is decompression function. mapping_flag equals to 1 indicates that the knee function is compression function.

input_d_range specifies the peak luminance level for the input picture of the knee function process in relative to the nominal luminance level in units of 0.1%. When the value of input_d_range is 0, the peak luminance level of the input picture is unspecified.

input_disp_luminance specifies the expected display brightness of peak luminance level for the input picture of the knee function process. The value of input_disp_luminance is in units of candela per square metre. When the value of input_disp_luminance is 0, the expected display brightness of peak luminance level for the input picture is unspecified.

output_d_range specifies the peak luminance level for the output picture of the knee function process in relative to the nominal luminance level in units of 0.1%. When the value of output_d_range is 0, the peak luminance level of the output picture is unspecified.

output_disp_luminance specifies the expected display brightness of peak luminance level for the output picture of the knee function process. The value of output_disp_luminance is in units of candela per square metre. When the value of output_disp_luminance is 0, the expected display brightness of peak luminance level for the output picture is unspecified.

num_knee_points_minus1 plus 1 specifies the number of knee points. num_knee_points_minus1 shall be in the range of 0 to 998, inclusive.

input_knee_point[ i ] specifies the luminance level of i-th knee point for input picture of the knee function process. The luminance level of the knee point for input picture is normalized in range of 0 to 1.0 in units of 0.1%. The value of input_knee_point shall be greater than or equal to 1 and less than or equal to 999. The value of i-th input_knee_point shall be greater than the value of (i-1)-th input_knee_point.

output_knee_point[ i ] specifies the luminance level of i-th knee for output picture of the knee function process. The luminance level of the knee point for output picture is normalized in range of 0 to 1.0 in units of 0.1%. The value of input_knee_point shall be greater than or equal to 0 and less than or equal to 1000.

NOTE 3 – A knee function is a function which is composed of slopes which connects three types of points with values normalized in range of 0.0 to 1.0. The three types of points are defined as follows.

a)	An origin – (0.0, 0.0)

b)	A coordinate of a knee point which is composed of an input_knee_point[ i ] and output_knee_point[ i ] – (input_knee_point[ i ], output_knee_point[ i ]) The knee function is composed of several lines which starts with the point (0.0,0.0) ends with the point (1.0,1.0) through all knee points in ascending order of i. The input dynamic range and the output dynamic range can be dervied with following equations.

(D-19)

(D-20)

c)	A point of (1.0, 1.0)

Figure D.4 A Knee Function

# Reference

- S.Hattori, T.Suzuki, “HLS: SEI message for transfer function information”, JCTVC-N0064, 15th Meeting: Geneva, CH, 23 Oct – 1 Nov. 2013

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).