| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0051 |
| --- | --- |

| Title: | HLS: Extensions to Temporal Motion-constrained tile sets SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sally Hattori, Ohji Nakagami, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 shinobu.hattori@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes an extension to a temporal motion-constrained tile sets SEI message to indicate the level information for a decoder to decode each defined motion-constrained tile set. The proposal provides flexibility for HEVC tile structure to be applied for various applications such as in interactive UHDTV application, dynamic high-quality zoom-in application and interactive on-demand e-learning etc. The idea was proposed in JCTVC-N0117[1] and JCTVC-O0063[2]. The functionality of indicating the level information for each motion-constrained tile set was agreed to be useful with proposal in JCTVC-N0117 and JCTVC-O0063. This contribution further clarifies the specification text on the definitions of bitrate level constraints for motion-constrained tile sets.

# Introduction

As introduced in JCTVC-N0117[1] and JCTVC-O0063[2], applications in which only the partial regions of a picture are delivered to a decoder, such as tiled streaming applications, can be anticipated to be used in future services to make best use of given bandwidth to deliver the content. In order to realize such application, it is necessary that the decoder is capable of distinguishing if the partially delivered data is decodable. Hence, a level indication for such partial regions can be helpful information to determine the capability of decoding the partial region. Figure 1-1 shows an example of tiled streaming application. An example of tiled streaming application in Figure 1-1 shows that each client device may have different decoding capability such that it is useful for the client to be able to select the motion-constrained tile set which it can decode.

Figure 1-1 Example image of Tiled Streaming Application

# Problem Statement

Current HEVC specification[3] defines level constraints in picture based concept. The motion-constrained tile set is partial region of a picture and also can be flexibly set as non-rectangular region or composed of multiple regions which can be apart from each other. Hence, the current level constraints cannot directly apply to motion-constrained tile set. Figure 2-2 shows examples of motion-constrained tile sets. In order to apply the current picture based level constrains to the motion-constrained tile sets, the contribution proposes to define a new concept of region for motion-constrained tile set which enables to apply the picture based level constraints.

Figure 2-2 Examples of Motion-constrained tile sets

# Proposal

The contribution proposes to define a concept of rectangular region composed of a motion-constrained tile set as Tile Set Region. The proposal defines the level constraints to apply for each Tile Set Region. Examples of Tile Set Region are shown in Figure 3-3.

A Tile Set Region is defined as a region with width and height which covers all tiles defined for each motion-constrained tile set. In case the region of motion-constrained tile sets is not in rectanguler shape, the width and height are inferred to be equal to the value of a rectangle which covers the entire tile sets horizontally and vertically.

Figure 3-3 Examples of Tile Set Region

The contribution proposes to enable signaling of level information for each motion-constrained tile set defined in temporal motion-constrained tile sets SEI message with level constraints applied to slice segments within corresponding tile set region.

Current picture based level defines constraints for following four main elements:

- Picture size

- Number of tile columns and rows

- Bitrates

- Minimum Compression Ratio

For each element, the contribution proposes to set the constraints for tile set region as follows.

- Picture size

  - The picture based level constraints for the size of picture with PicSizeInSamplesY.

  - The contribution proposes to define variables for these constraints in tile set based region as follows:

    - mctsSizeInSamplesY[ i ] derived as follows:

      - mctsSizeInSamplesY[ i ] = mctsWidthInSamplesY[ i ] * mctsHeightInSamplesY[ i ]

      - mctsWidthInSamplesY[ i ] is equal to the width of the i-th tile set region, corresponding to the i-th motion constrained tile set in units of luma samples.

      - mctsHeightInSamplesY[ i ] is equal to the height of the i-th tile set region, corresponding to the i-th motion constrained tile set in units of luma samples.

- Number of tile columns and rows

  - The picture based level constraints for number columns and rows with MaxTileCols and MaxTileRows.

  - The contribution proposes to define variables for these constraints in tile set based region as follows:

    - NumTileColumnsInMCTS[ i ] is equal to the number of tile columns in the i-th tile set region, corresponding to the i-th motion constrained tile set.

    - NumTileRowsInMCTS[ i ] is equal to the number of tile rows in the i-th tile set region, corresponding to the i-th motion constrained tile set.

- Bitrate

  - The picture based level constraints for bitrate with MaxLumaSr and MaxBR.

  - The contribution proposes to define variables for these constraints in tile set based region as follows:

    - mctsMaxSliceSegments[ i ] is equal to the number of slice segments in the i-th motion constrained tile set.

    - mctsMaxLumaSr[ i ] is equal to the number of samples per second in the i-th motion constrained tile set.

    - mctsMaxBR[ i ] is equal to the number of bits per second in the i-th motion constrained tile set.

- Minimum Compression Ratio

  - The picture based level constraints for bitrate with MinCr.

  - The contribution proposes to define variables for this constraint in tile set based region as follows:

    - mctsMinCr[ i ] is equal to the minimum compression ratio of the i-th motion constrained tile set.

The proposed extensions to the syntax and semantics are as follows.

D.2.26 Temporal motion-constrained tile sets SEI message syntax

Replace the syntax table with following table:

| temporal_motion _constrained_tile_sets ( payloadSize ) { | Descriptor |
| --- | --- |
| mc_all_tiles_exact_sample_value_match_flag | u(1) |
| each_tile_one_tile_set_flag | u(1) |
| mcts_level_idc_present_flag | u(1) |
| if ( !each_tile_one_tile_set_flag ) { |  |
| num_sets_in_message_minus1 | ue(v) |
| for( i = 0; i <= num_sets_in_message_minus1; i++) { |  |
| mcts_id[ i ] | ue(v) |
| num_tile_rects_in_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_tile_rects_in_set_minus1[ i ]; j++) { |  |
| top_left_tile_index[ i ][ j ] | ue(v) |
| bottom_right_tile_index[ i ][ j ] | ue(v) |
| } |  |
| if ( ! mc_all_tiles_exact_sample_value_match_flag ) |  |
| exact_sample_value_match_flag[ i ] | u(1) |
| if ( mcts_level_idc_present_flag ) |  |
| mcts_level_idc[ i ] | u(8) |
| } |  |
| } else { |  |
| if ( mcts_level_idc_present_flag ) |  |
| max_mcts_level_idc | u(8) |
| } |  |
| } |  |

D.3.26 Temporal motion-constrained tile sets SEI message semantics

add the following paragraph after the semantics of each_tile_one_tile_set_flag:

mcts_level_idc_present_flag equal to 1 indicates that the mcts_level_idc[ i ] or max_mcts_level_idc syntax elements are present. mcts_level_idc_present_flag equal to 0 indicates that the mcts_level_idc[ i ] and max_mcts_level_idc syntax elements are not present.

add the following paragraph after the semantics of exact_sample_value_match_flag[ i ]:

mcts_level_idc[ i ] indicates a level to which the i-th tile set region, corresponding to the i-th motion constrained tile set, conforms. The value of mcts_level_idc[ i ] shall be less than or equal to the value of general_level_idc in the active SPS RBSP.

When not present, the value of mcts_level_idc[ i ] is inferred to be equal to the general_level_idc.

max_mcts_level_idc indicates a level to which all motion constrained tile sets conform. The value of max_mcts_level_idc shall be less than or equal to the value of general_level_idc in the active SPS RBSP.

When not present, the value of max_mcts_level_idc is inferred to be equal to the general_level_idc.

The i-th tile set region is bounding rectangular region of all tiles in the i-th motion-constrained tile set.

–	When mcts_level_idc_present_flag is equal to 0, mctsLevelIdc is inferred to be equal to general_level_idc and Annex A applies to all motion-constrained tile sets.

–	Otherwise (mcts_level_idc_present_flag is equal to 1), the variables mctsLevelIdc, mctsSizeInSamplesY[ i ], mctsWidthInSamplesY[ i ], mctsHeightInSamplesY[ i ], NumTileColumnsInMCTS[ i ], NumTileRowsInMCTS[ i ], mctsMaxSliceSegments[ i ], mctsMaxLumaSr[ i ], mctsMaxBR[ i ] and mctsMinCr[ i ] are derived as follows.

–	When each_tile_one_tile_set_flag is equal to 0,

–	i in the range of 0 to num_sets_in_message_minus1 is equal to i-th motion- constrained tile set

–	The variable mctsLevelIdc for i-th motion constrained tile set is equal to mcts_level_idc[ i ].

–	The variable mctsSizeInSamplesY[ i ] for i-th tile set region is derived as follows:

mctsSizeInSamplesY[ i ] = mctsWidthInSamplesY[ i ] * mctsHeightInSamplesY[ i ]

–	mctsWidthInSamplesY[ i ] is equal to the width of the i-th tile set region, corresponding to the i-th motion constrained tile set in units of luma samples.

–	mctsHeightInSamplesY[ i ] is equal to the height of the i-th tile set region, corresponding to the i-th motion constrained tile set in units of luma samples.

–	NumTileColumnsInMCTS[ i ] is equal to the number of tile columns in the i-th tile set region, corresponding to the i-th motion constrained tile set.

–	NumTileRowsInMCTS[ i ] is equal to the number of tile rows in the i-th tile set region, corresponding to the i-th motion constrained tile set.

–	mctsMaxSliceSegments[ i ] is equal to the number of slice segments in the i-th motion constrained tile set.

–	mctsMaxLumaSr[ i ] is equal to the number of samples per second in the i-th motion constrained tile set.

–	The bitrate of i-th motion consrained tile set is inferred to be equal to CpbBrVclFactor * mctsMaxBR[ i ] for VCL HRD parameters and to be equal to CpbBrNalFactor * mctsMaxBR[ i ] for NAL HRD parameters.

–	mctsMinCr[ i ] is equal to the minimum compression ratio of the i-th motion constrained tile set.

–	Otherwise (each_tile_one_tile_set_flag is equal to 1),

–	i in the range of 0 to (num_tile_columns_minus1+1) * (num_tile_rows_minus1+1) is equal to each tile with TileId equal to i.

–	The variable mctsLevelIdc for all tiles are equal to max_mcts_level_idc.

–	The variable mctsSizeInSamplesY[ i ] for i-th tile is derived as follows:

mctsSizeInSamplesY[ i ] = colWidth[ i ] * rowHeight[ i ] * CtbSizeY * CtbSizeY

–	NumTileColumnsInMCTS[ i ] is equal to 1.

–	NumTileRowsInMCTS[ i ] is equal to 1.

–	mctsMaxSliceSegments [ i ] is equal to the number of slice segments in the i-th tile.

–	mctsMaxLumaSr[ i ] is equal to the number of samples per second in the i-th tile.

–	The bitrate of the i-th tile is inferred to be equal to CpbBrVclFactor * mctsMaxBR[ i ] for VCL HRD parameters and to be equal to CpbBrNalFactor * mctsMaxBR[ i ] for NAL HRD parameters.

–	mctsMinCr[ i ] is equal to the minimum compression ratio of the the i-th tile.

mctsLevelIdc indicates a level to which a motion-constrained tile set conforms, as specified in Annex A with the following additions.

–	The variable PicSizeInSamplesY is replaced with the variable mctsSizeInSamplesY[ i ].

–	The variable MaxTileCols is replaced with the variable NumTileColumnsInMCTS[ i ].

–	The variable MaxTileRows is replaced with the variable NumTileRowsInMCTS[ i ].

–	The variable MaxSliceSegmentsPerPicture is replaced with the variable mctsMaxSliceSegments[ i ].

–	The variable MaxLumaSr is replaced with the variable mctsMaxLumaSr[ i ].

–	The variable MaxBR is replaced with the variable mctsMaxBR[ i ].

–	The variable MinCr is replaced with the variable mctsMinCr[ i ].

Figure D.x Tile Set Region

# Sample of variable derivation process

The variables used to set the level for each motion-constrained tile set can be derived as follows. The process indicated below is one of sample process.

for( i = 0; i <= num_sets_in_message_minus1; i++) {
	mctsLeftCtbPos[i] = PicWidthInCtbsY;
	mctsTopCtbPos[i] = PicHeightInCtbsY;
	mctsRightCtbPos[i] = 0;
	mctsBtmCtbPos[i] = 0;
	
	for ( j = 0; j <= num_tile_rects_in_set_minus1[ i ]; j++) {
		for( ctbPosRs = 0; ctbPosRs < PicSizeInCtbsY; ctbPosRs++ ) {
			tbX = ctbPosRs % PicWidthInCtbsY;
			tbY = ctbPosRs / PicWidthInCtbsY;
			
			if ( TileId[ CtbPosRsToTs[ ctbPosRs ] ] == top_left_tile_index[i][j] ) {
				if ( mctsLeftCtbPos[i] > tbX )
					mctsLeftCtbPos[i] = tbX;
				if ( mctsTopCtbPos[i] > tbY )
					mctsTopCtbPos[i] = tbY;
			}
	
			if ( TileId[ CtbPosRsToTs[ ctbPosRs ] ] == bottom_right_tile_index[i][j] ) {
				if ( mctsRightCtbPos[i] < tbX )
					mctsRightCtbPos[i] = tbX;
				if ( mctsBtmCtbPos[i] < tbY )
					mctsBtmCtbPos[i] = tbY;
			}
		}
	}
	
	mctsWidthInSamplesY[i] = ( mctsRightCtbPos[i] - mctsLeftCtbPos[i] + 1 ) << CtbLog2SizeY
	mctsHeightInSamplesY[i] = ( mctsBtmCtbPos[i] - mctsTopCtbPos[i] + 1 ) << CtbLog2SizeY
	
	mctsTopLeftTileId = TileId[ CtbPosRsToTs [ mctsTopCtbPos[i] * PicWidthInCtbsY + mctsLeftCtbPos[i] ] ]
	mctsTopRightTileId = TileId[ CtbPosRsToTs [ mctsTopCtbPos[i] * PicWidthInCtbsY + mctsRightCtbPos[i] ] ]
	mctsBtmLeftTileId = TileId [ CtbPosRsToTs [ mctsBtmCtbPos[i] * PicWidthInCtbsY + mctsLeftCtbPos[i] ] ]
	
	NumTileColumnsInMCTS[i] = mctsTopRightTileId - mctsTopLeftTileId + 1
	NumTileRowsInMCTS[i] = mctsBtmLeftTileId - mctsTopLeftTileId + 1
	
}

# Reference

- S.Hattori, O.Nakagami, T.Suzuki, “HLS: Extensions to Motion-constrained tile sets SEI message”, JCTVC-N0117, 14th Meeting: Vienna, Austria, AT, July 29 – Aug 2, 2013.

- S.Hattori, O.Nakagami, T.Suzuki, “HLS: Extensions to Temporal Motion-constrained tile sets SEI message”, JCTVC-O0063, 15th Meeting: Geneva, CH, 23 Oct – 1 Nov. 2013.

- D.Flynn, J.Sole, G.Sullivan, T.Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 5”, JCTVC-O1005, 15th Meeting: Geneva, CH, 23 Oct – 1 Nov. 2013.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).