| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0053 |
| --- | --- |

| Title: | RCE3: Subtest C.4 - Padding-based generation of unavailable samples in intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunil Lee Elena Alshina Chanyul Kim | Email: | sunil.lee@samsung.com elena_a.alshina@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this response to RCE3 on intra block copy refinement, the test results of subtest C.4 on the padding-based generation of unavailable samples in intra block copy are reported. In the proposed method, when the current CU to be predicted overlaps the reference CU, the prediction samples in the overlapped region are generated by copying the available samples from either vertical or horizontal direction. The test results show that the coding gain of the proposed method in lossy coding is -0.6% in AI-MT, -0.4% in RA-MT, and -0.3% in LB-MT for SC YUV 444, respectively, and its coding gain in lossless coding is 0.2% in AI, 0.2% in RA, and 0.2% in LB for SC YUV 444, respectively.

# Introduction

At the last meeting in Geneva, a padding-based generation of unavailable samples in intra block copy was proposed [1]. Fig. 1 shows an example of intra block copy in which the current CU to be predicted overlaps the reference CU. In this example, the reference samples in yellow region are not available yet and cannot be used for the prediction. In the current design of intra block copy, the MV referring to the reference CU in Fig. 1 is considered as an invalid MV.

Figure 1. Intra block copy - Invalid MV

In this response to RCE3, a method to generate unavailable samples in intra block copy is proposed. When the reference CU overlaps the current CU to be predicted, the samples in the overlapped region are generated by copying the available samples from either horizontal or vertical direction.

Figure 2. Reference samples of the overlapped region

# Algorithm description

Let MVx, MVy, W, and H be horizontal and vertical components of the current MV in intra block copy, width and height of the current CU, respectively. The proposed method is invoked only when the following condition is satisfied,

( –W < MVx ≤ 0 ) AND ( –H < MVy ≤ 0 ).

When the proposed method is invoked, the reference samples are generated by copying the available samples from either vertical or horizontal direction. Fig. 2 shows the reference samples of the MxN overlapped region in which M = H – | MVy |, N = W – | MVx |, and ri,j is the reference sample at the ith row and jth column. When | MVy | > | MVx |, the reference samples are generated by copying the reference samples of the bottommost row of the above CU,

ri,j=r−1,j   ,0≤i≤(M−1),0≤j≤(N−1).

Otherwise, when | MVy | ≤ | MVx |, the reference samples are generated by copying the reference samples of the rightmost column of the left CU,

ri,j=ri,−1   ,0≤i≤(M−1),0≤j≤(N−1).

The reference sample generation process is more intuitively described in Fig. 3.

Figure 3. Reference samples copied from either (a) left or (b) above CU

# Test results

The coding gain of the proposed method is evaluated using the common test condition of RCE3 [2]. Table 1 and 2 show that the coding gain of the proposed method in lossy and lossless coding configurations, respectively. As shown in Table 1, the coding gain of the proposed method is -0.6% in AI-MT, -0.4% in RA-MT, and -0.3% in LB-MT for SC YUV 444, respectively. Table 2 shows that its coding gain in lossless coding is -0.2% in AI-Main, -0.2% in RA-Main, and -0.2% in LB-Main for SC YUV 444, respectively.

Table 1. Coding gain of the proposed method in lossy coding configuration

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.7% | -0.7% | -0.7% | -0.4% | -0.4% | -0.5% | -0.6% |
| Animation RGB | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| SC YUV 444 | -0.6% | -0.5% | -0.5% | -0.4% | -0.4% | -0.3% | -0.4% |
| Animation YUV | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR | -2.6% | -2.8% | -2.8% | -2.1% | -2.2% | -1.3% | -1.4% |
| SC(444) YUV | -1.5% | -1.5% | -1.5% | -1.4% | -1.6% | -1.3% | -1.2% |
| Enc Time[%] | 101% | 101% | 100% | 100% | 100% | 100% | 100% |
| Dec Time[%] | 100% | 99% | 99% | 99% | 100% | 100% | 99% |

Table 2. Coding gain of the proposed method in lossless coding configuration

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.2% | -0.2% | -0.2% |
| Animation RGB | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.2% | -0.2% | -0.2% |
| Animation YUV | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC(444) GBR | -0.7% | -0.7% | -0.4% |
| SC(444) YUV | -1.0% | -1.1% | -1.0% |
| Enc Time[%] | 101% | 101% | 101% |
| Dec Time[%] | 101% | 99% | 101% |

# Conclusion

In this response to RCE3, a padding-based generation of unavailable samples in intra block copy is proposed. The test results show that the coding gain of the proposed method in lossy coding is -0.6% in AI-MT, -0.4% in RA-MT, and -0.3% in LB-MT for SC YUV 444, respectively, and its coding gain in lossless coding is 0.2% in AI, 0.2% in RA, and 0.2% in LB for SC YUV 444, respectively. It is suggested that the proposed method is adopted into the next version of the HEVC Range Extensions text and reference software.

# Proposed text changes

In 8.4.4

…

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ 0 ] + nCbS − 1, yCb + bvIntra[ 1 ] + nCbS – 1 ) as inputs, the output shall be equal to TRUE.

–	One or both of the following conditions shall be true:

–	bvIntra[ 0 ] + nCbS  <=  0

–	bvIntra[ 1 ] + nCbS  <=  0

…

In 8.4.5.2.7

…

The variable bv representing the block vector for prediction in full-sample units is derived as follows:

bv[ 0 ] = bvIntra[ 0 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubWidthC ) − 1 )	(8-1)

bv[ 1 ] = bvIntra[ 1 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubHeightC ) − 1 )	(8-2)

The reference sample at location ( xTbCmp + x, yTbCmp + y), Ref[xTbCmp + x][ yTbCmp + y], with x, y = 0..nTbS − 1, is specified by

Ref[xTbCmp + x][ yTbCmp + y] = ( Abs( bv[ 1 ] ) > Abs ( bv[ 0 ] ) ) ? Ref[xTbCmp + x][ yTbCmp - 1] : Ref[xTbCmp − 1][ yTbCmp + y]

The (nTbS)x(nTbS) array of predicted samples samples, with x, y = 0..nTbS − 1, is derived as follows:

–	The reference sample location ( xRefCmp, yRefCmp ) is specified by:

( xRefCmp, yRefCmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + y + bv[ 1 ] )	(8-3)

- Each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

# References

- S. Lee, E. Alshina, C. Kim, “AHG5: Extension of intra block copy,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0112, Oct. 2013.

- J. Sole, E. Alshina, D.-K. Kwon, W.-H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1123, Oct. 2013.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).