| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0055r1 |
| --- | --- |

| Title: | RCE3: Subtest B.4 - Sample masking for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Tel: Email: | +358 7180 08000 first.last@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports results of RCE3 Subtest B.4 on sample masking for intra block copy. The technique studied in this test indicates certain region of intra block copy predicted samples within a CU and substitutes all the predicted samples in that area with a single sample value. It is reported that the proposed approach provides objective bitrate reductions of -1.3 %, -1.3 % and -0.5 % for All Intra SC RGB 444, SC YUV 444 and Class F categories, respectively. In non-screen content categories the average of reported bitrate differences is 0.0 %.

# Introduction

The technique tested in this core experiment proposes to signal a mask for the CUs coded with IntraBC mode. The mask is covering a square area extending from one of the borders of the CU and having a width varying from 1 to 3 sample rows or columns. All the sample values in the masked area are substituted with a single sample value obtained from the middle sample of the inner boundary of the mask. The black squares in Figure 1 illustrate locations of the samples used in the substitution process for different masking modes. Figure 2 shows the masking areas and source sample location for the three different mask sizes in the case of mode 2 (mask on the left border of the block) and block size of 8x8.

Figure 1: Examples of intra block copied CUs with different masking modes 1 through 4.

Figure 2: Examples of intra block copied CUs with different masking alternatives for masking mode 2 (mask on the left border of the block). The mask can extend 1, 2 or three sample rows from the border. The black sample location illustrates the sample which value is used to substitute all other sample values in the masked area (gray).

Motivation for the proposed technique was an observation that the current intra block copy operates occasionally inefficiently due to its fixed size. E.g. it can copy significant residue together with the actually desired content. This kind of behaviour is illustrated in Figure 3 below where part of the next letter “a” gets copied together with the letter “c” to the predicted block.

Figure 3: Example of intra block copy operation without intra masking.

Proposed specification text and software implementing the tool is provided as an attachment to this contribution.

# Results of the core experiments

The tables below summarize the results provided by the proposed tool under different test configurations. Next section provides further information about performance of the tool when combined with other tools in RCE3.

Table 1: Results of the lossy tests

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.5% | -0.4% | -0.4% | -0.5% | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -1.3% | -1.2% | -1.2% | -1.3% | -1.2% | -1.2% | -1.3% | -1.1% | -1.2% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -1.3% | -1.1% | -1.0% | -1.2% | -1.1% | -1.0% | -1.2% | -1.1% | -1.1% |
| YCbCr 4:4:4 Animation | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.6% | -1.4% | -1.4% | -1.8% | -1.5% | -1.5% | -1.9% | -1.6% | -1.6% |
| YCbCr 4:4:4 SC (Optional) | -2.1% | -1.8% | -1.7% | -2.3% | -2.0% | -1.9% | -2.6% | -2.2% | -2.2% |
| Enc Time[%] | 100 % |  |  | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 99 % |  |  | 99 % |  |  | 99 % |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.2% | -0.3% | -0.4% | -0.3% | -0.4% |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -1.0% | -0.9% | -0.9% | -1.0% | -0.9% | -0.9% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -1.1% | -0.9% | -1.1% | -1.2% | -1.0% | -1.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -1.2% | -0.9% | -0.9% | -1.5% | -1.0% | -1.1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.9% | -1.6% | -1.5% | -2.1% | -1.7% | -1.9% |  |  |  |
| Enc Time[%] | 99 % |  |  | 99 % |  |  |  |  |  |
| Dec Time[%] | 100 % |  |  | 101 % |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.3% | -0.3% | -0.2% | -0.3% | -0.2% |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -0.9% | -0.7% | -0.8% | -1.0% | -0.7% | -0.8% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.8% | -0.6% | -0.8% | -0.9% | -0.7% | -0.8% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -1.2% | -1.3% | -1.5% | -1.1% | -1.0% | -1.0% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.4% | -1.5% | -1.3% | -1.0% | -1.1% | -1.1% |  |  |  |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |  |  |  |
| Dec Time[%] | 99 % |  |  | 100 % |  |  |  |  |  |

Table 2: Results of the lossless tests

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.7% | -0.7% | -0.5% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.7% | -0.4% | -0.5% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.3% | -1.1% | -1.0% |
| YCbCr 4:4:4 SC (Optional) | -1.9% | -1.8% | -1.7% |
| Enc Time[%] | 101 % | 100 % | 100 % |
| Dec Time[%] | 100 % | 100 % | 100 % |

# Combinations with other RCE3 tools

The tables below summarize the performance of the proposed tool with various other tools in RCE3 under lossy test conditions. Excel result sheets are provided as attachments to this contribution. Note that the tests other than B.4 (stand-alone masking) have been run in a cluster which does not guarantee uniform reporting of execution times.

|  |  | Mandatory |  | Optional |  |  | Gain of RCE3-B.4 (masking) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RCE | Test | RGB SC | YUV SC | RGB SC | YUV SC | Aver. |  |
| B.4 | Mask | -1.3 % | -1.3 % | -1.6 % | -2.1 % | -1.6 % | -1.6 % |
| B.3 | NxN PU | -9.2 % | -8.8 % | -11.2 % | -13.7 % | -10.7 % |  |
| B.3 + B.4 | NxN PU + Mask | -10.1 % | -9.5 % | -12.3 % | -15.3 % | -11.8 % | -1.1 % |
| C.2 | Pad CU hor | -0.4 % | -0.5 % | -1.0 % | -1.4 % | -0.8 % |  |
| C.2 + B.4 | Pad CU hor + Mask | -1.7 % | -1.8 % | -2.6 % | -3.3 % | -2.4 % | -1.5 % |
| C.3 | Pad CTU | -0.3 % | -0.4 % | -0.8 % | -1.3 % | -0.7 % |  |
| C.3 + B.4 | Pad CTU hor + Mask | -1.7 % | -1.8 % | -2.4 % | -3.3 % | -2.3 % | -1.6 % |
| C.4 | Pad CU horver | -0.7 % | -0.6 % | -2.6 % | -1.5 % | -1.3 % |  |
| C.4 + B.4 | Pad CU horver + Mask | -1.9 % | -1.8 % | -4.1 % | -3.4 % | -2.8 % | -1.5 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).