_____________________________

# Abstract

# Proposed text

Coding unit syntax:

| if( intra_block_copy_enabled_flag ) { |  |
| --- | --- |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| intra_bc_mask_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bc_mask_flag[ x0 ][ y0 ] ) { |  |
| intra_bc_mask_mode_minus1[ x0 ][ y0 ] | u(2) |
| intra_bc_mask_size_flag1[ x0 ][ y0 ] | u(1) |
| if( intra_bc_mask_size_flag1[ x0 ][ y0 ] ) |  |
| intra_bc_mask_size_flag2[ x0 ][ y0 ] | u(1) |
| } |  |
| } |  |
| } |  |

Coding unit semantics (add the following):

intra_bc_mask_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in intra block copying mode with sample mask mode maskModeIntraBc larger than zero.  intra_bc_mask_flag [ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded with out a mask and maskModeIntraBc is set to zero.  When not present, the value of intra_bc_mask_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

intra_bc_mask_mode_minus[ x0 ][ y0 ] indicates the applied maskModeIntraBc shall be equal to intra_bc_mask_mode_minus [ x0 ][ y0 ] plus one. When not present, maskModeIntraBc is inferred to be equal to 0.

intra_bc_mask_size_flag1[ x0 ][ y0 ] equal to 0 indicates the width of the masking area for luminance component maskWidthIntraBcLuma is equal to two. intra_bc_mask_size_flag1[ x0 ][ y0 ] equal to 1 indicates that maskWidthIntraBcLuma is defined by intra_bc_mask_size_flag2[ x0 ][ y0 ].

intra_bc_mask_size_flag2[ x0 ][ y0 ] equal to 1 indicates the maskWidthIntraBcLuma is equal to one. intra_bc_mask_size_flag2[ x0 ][ y0 ] equal to 0 indicates maskWidthIntraBcLuma is equal to three.

Specification of general decoding process for coding units coded in intra prediction mode:

### General decoding process for coding units coded in intra prediction mode

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

A variable nCbS is set equal to 1  <<  log2CbSize.

Depending on the values of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for luma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( nCbS * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ), with i, j = 0..nCbS − 1	(8-1)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

- When intra_bc_flag[ xCb ][ yCb ] is equal to 1, the derivation process for block vector components in intra block copying prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and variable log2CbSize as inputs, and the output being bvIntra.

- When intra_bc_flag[ xCb ][ yCb ] is equal to 1, the derivation process for intra block copy masking information as specified in subclause 8.X.X is invoked with the luma location ( xCb, yCb ), variable log2CbSize, and the variable cIdx set equal to 0 as inputs, and the output being mlvIntra and ibcMask.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, the variable mlvIntra, the variable ibcMask, and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to 0, and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

When ChromaArrayType is not equal to 0, the following applies.

The variable log2CbSizeC is set equal to log2CbSize − ( ChromaArrayType  = =  3 ? 0 : 1 ).

Depending on the value of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for chroma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SCb[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = 	pcm_sample_chroma[ ( nCbS / SubWidthC * j ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-2)

SCr[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = pcm_sample_chroma[ ( nCbS / SubWidthC * ( j + nCbS / SubHeightC ) ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-3)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0 or ChromaArrayType is not equal to 3, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input, and the output is the variable IntraPredModeC.

- When intra_bc_flag[ xCb ][ yCb ] is equal to 1, the derivation process for intra block copy masking information as specified in subclause 8.X.X is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), variable log2CbSize, and the variable cIdx set equal to 1  as inputs, and the output being mlvIntra and ibcMask.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, the variable mlvIntra, the variable ibcMask, and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, the variable mlvIntra, the variable ibcMask, and the variable cIdx set equal to 2 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, IntraSplitFlag is equal to 1 and ChromaArrayType is equal to 3), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xBS is set equal to xC + ( nS  >>  1 ) * ( blkIdx % 2 ).

- The variable yBS is set equal to yC + ( nS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xBS, yBS ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xBS, yBS), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to 0, and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xBS, yBS), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to 0, and the variable cIdx set equal to 2 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

Specification of derivation process for intra block copy masking information (NEW SECTION):

### Derivation process for intra block copy masking information

Inputs to this process are:

- a location ( xCb, yCb ) of the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

- a variable cIdx specifying the colour component of the current block

Output of this process is the mask sample location vector mlvIntra and mask array ibcMask.

The variable nCbS is set equal to 1  <<  log2CbSize.

The variables nCbW and nCbH are set as follows:

nCbW = ( cIdx == 0 ) ? nCbS : nCbS / SubWidthC	(8-XX)

nCbH = ( cIdx == 0 ) ? nCbS : nCbS / SubHeightC	(8-XX)

The variable cSc is set as follows:

cSc = ( cIdx == 0 ) ? 1 : ( maskModeIntraBc == 1 || maskModeIntraBc == 3 ) ? SubHeightC : SubWidthC	(8-XX)

The variable maskWidthIntraBc is set equal to maskWidthIntraBcLuma / cSc.

Each value of ibcMask[ xCb + x ][ yCb + y ] with x = 0.. nCbW − 1, y = 0.. nCbH – 1 are set to 0.

If maskModeIntraBc is larger than 0, the following additional steps apply:

–	Variables maskFirstX, maskLastX,  maskFirstY, maskLastY, maskRefX, maskRefY are set as follows:

firstX = maskModeIntraBc == 2 ? nCbW - maskWidthIntraBc: 0;

firstY = maskModeIntraBc == 3 ? nCbH - maskWidthIntraBc: 0;

lastX  = maskModeIntraBc == 4 ? maskWidthIntraBc - 1 : nCbW - 1;

lastY  = maskModeIntraBc == 1 ? maskWidthIntraBc - 1 : nCbH - 1;

–	Variables maskRefX and maskRefY are defined as a function of variables maskModeIntraBc, firstX, firstY, lastX and lastY according to the table below:

| maskModeIntraBc | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| maskRefX | nCbW / 2 | maskFirstX | nCbW / 2 | maskLastX |
| maskRefY | maskLastY | nCbH / 2 | maskFirstY | nCbH / 2 |

–	The components of the mask location vector mlvIntra are set to:

mlvIntra[ 0 ] =  xCb + maskRefX

mlvIntra[ 1 ] =  yCb + maskRefY

- Each value of ibcMask[ xCb + x ][ yCb + y ] with x = maskFirstX..maskLastX, y = maskFirstY.. maskLastY are set to 1.

Specification of intra block copying prediction mode:

##### Specification of intra block copying prediction mode

Inputs to this process are:

–	a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable bvIntra specifying the block copying vector,

–	a variable mlvIntra specifying the mask sample location vector,

–	a variable ibcMask specifying the array of masked samples,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The variable bv representing the block vector for prediction in full-sample units is derived as follows:

bv[ 0 ] = bvIntra[ 0 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubWidthC ) − 1 )	(8-4)

bv[ 1 ] = bvIntra[ 1 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubHeightC ) − 1 )	(8-5)

If maskModeIntraBc is larger than 0, the mask sample location ( xMaskCmp, yMaskCmp ) representing the mask sample location vector in full-sample units is derived as follows:

( xMaskCmp, yMaskCmp ) = ( mlvIntra[ 0 ] + bv[ 0 ], mlvIntra[ 1 ] + bv[ 1 ] )	(8-xx)

The (nTbS)x(nTbS) array of predicted samples samples, with x, y = 0..nTbS − 1, is derived as follows:

–	The reference sample location ( xRefCmp, yRefCmp ) is specified by:

( xRefCmp, yRefCmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + y + bv[ 1 ] )	(8-6)

–	Each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

–	Depending on the value of ibcMask[ xTbCmp  + x ][ yTbCmp  + y ], the following applies:

–	If ibcMask[ xTbCmp  + x ][ yTbCmp  + y ] is equal to 0, sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

–	Otherwise, sample at the location ( xMaskCmp, yMaskCmp ) is assigned to predSamples[ x ][ y ].