| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0057 |
| --- | --- |

| Title: | AHG 5: Profiles for Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
|  | T. Suzuki 2-10-1, Osaki, Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81 50 3750 2740 teruhikos@jp.sony.com |
|  | A. Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | +1 408 352 4715 ali.tabatabai@am.sony.com |
| Source: | Sony Europe Ltd., Sony Corp., and Sony Electronics Inc. |  |  |

_____________________________

# Abstract

Discussion on profiles for RExt has been started, with a set of profiles included in the DAM text. This contribution proposes the following:

- Add all intra coding profiles

- Add profiles support up to 16 bit video

- Profiles for monochrome video (4:0:0)

This contribution encompasses the operating points defined in JCTVC-O1005 (v3), but decreasing the number of proposed profiles defined in (currently unapproved) JCTVC-O1005 (v3).

# Introduction

The initial idea of HEVC RExt profiles was proposed in JCTVC-M0094 and m29109 at Incheon meeting, following which the following profiles for RExt were included in the DAM text.

- Main 12 4:2:0

- Main 10 4:2:2, Main 12 4:2:2

- Main 10 4:4:4, Main 12 4:4:4

At the 15th Meeting, other profiles were briefly discussed, touching again on the need for intra, 4:0:0 and also a 4:4:4 8-bit profile with the inclusion of screen content specific tools. Discussions centered around whether tools that are most effective on screen content should be included in the profiles to be defined at the output of the RExt amendment, with the CfP on screen content coding potentially augmenting the number of profiles in the future. The only decision was that non 4:4:4 profiles would not include any new tools, with the exception of high precision weighted prediction.

This proposal re-iterates and enhances the list of profiles suggested at the Incheon meeting. It is proposed that profiles are designed to support the applications for contents creation and production. All intra coding is used for high end production where already 16bit material is captured and used during the workflow; such 16 bit material has been added to the RExt test set. Therefore, this contribution proposes to include profiles for all intra coding, for bit depth up to 16 bit, and for monochrome video (4:0:0), in the RExt amendment.

# Profiles

It is preferable from an interoperability standpoint rather than defining small number of profiles and letting implementers to choose which tools to support, to define many profiles that cover all the expected use cases and applications. Defining too many profiles would, however, be detrimental.

This contribution proposes to define profiles as shown in [1] – Requested by JCTVC-O0126 & JCTVC-O0364.

| Profile |  | Defined in | Profile IDC | Chroma formats |  |  |  | RExt tools | Ext. precision | Profiles also supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 4:0:0 | 4:2:0 | 4:2:2 | 4:4:4 |  |  |  |
| Main 4:2:0 | Main [8 4:2:0] | V1 | 1 |  |  |  |  |  |  | - |
|  | Main 10 [4:2:0] | V1 | 2 |  |  |  |  |  |  | Main |
|  | Main Still [8 4:2:0] | V1 | 3 |  |  |  |  |  |  | - |
|  | Main 12 Intra 4:2:0 | - | 4 |  |  |  |  |  |  | Main 12 Intra 4:0:0 |
|  | Main 12 4:2:0 | DAM | 4 |  |  |  |  |  |  | Main 10, Main 12 Intra 4:2:0, Main 12 4:0:0 |
| Main 4:2:2 | Main 10 4:2:2 | DAM | 4 |  |  |  |  |  |  | Main 10 |
|  | Main 12 Intra 4:2:2 | - | 4 |  |  |  |  |  |  | Main 12 Intra 4:2:0, Main 12 Intra 4:0:0 |
|  | Main 12 4:2:2 | DAM | 4 |  |  |  |  |  |  | Main 10 4:2:2, Main 12 Intra 4:2:2, Main 12 4:2:0 |
| Main 4:4:4 | Main 8 4:4:4 [1] | - | 4 |  |  |  |  |  |  | Main 8 |
|  | Main 10 4:4:4 | DAM | 4 |  |  |  |  |  |  | Main 8 4:4:4, Main 10 4:2:2 |
|  | Main 12 Intra 4:4:4 | - | 4 |  |  |  |  |  |  | Main 12 Intra 4:2:2 |
|  | Main 12 4:4:4 | DAM | 4 |  |  |  |  |  |  | Main 10 4:4:4, Main 12 Intra 4:4:4, Main 12 4:2:2 |
|  | Main 16 Intra 4:4:4 | - | 4 |  |  |  |  |  |  | Main 12 Intra 4:4:4, Main 16 Intra 4:0:0 |
|  | Main 16 4:4:4 | - | 4 |  |  |  |  |  |  | Main 16 Intra 4:4:4, Main 16 4:0:0, Main 12 4:4:4 |
| Main 4:0:0 | Main 12 Intra 4:0:0 | - | 4 |  |  |  |  |  |  | - |
|  | Main 12 4:0:0 | - | 4 |  |  |  |  |  |  | Main 12 Intra 4:0:0 |
|  | Main 16 Intra 4:0:0 | - | 4 |  |  |  |  |  |  | Main 12 Intra 4:0:0 |
|  | Main 16 4:0:0 | - | 4 |  |  |  |  |  |  | Main 16 Intra 4:0:0, Main 12 4:0:0 |
|  | Only includes RExt tools necessary to support chroma formats and also high precision weighted prediction. |  |  |  |  |  |  |  |  |  |

[1] – Requested by JCTVC-O0126 & JCTVC-O0364

Table 1 : Summary of the proposed profiles for RExt

The points of the above new profiles are:

- Profiles cover the full range up to 16 bits 4:4:4.

- All profiles, with the exception of those in V1, can encode any bit-stream with bit-depth, chroma-format and slice-type (intra/inter) less than or equal to the settings defined for the given profile.

- All profiles are compatible with any other profile whose bit-depth, chroma-format and slice-type (intra/inter) less than or equal to the settings defined for the profile.

- No 14-bit profiles are defined – the extended processing design is already scalable, and therefore 14-bits being encoded under the 16-bit profile will be identical to a 14-bit encoding under a 14-bit profile. Having additional 14-bit profiles only causes decoders to be affected, and it is highly likely that a 14-bit decoder would also be designed to decode 16 bit profiles. The extended precision processing flag would be 0 for all non 16-bit profiles, and is configurable for 16-bit profiles.

- The 12-bit profiles are intended for consumer 12-bit, i.e. mostly likely encoded with positive QPs. For 12-bit high end content creation applications that require the extended precision processing capability, the main 16 profiles would be used, noting again that the extended precision system is bit-depth scalable.

- 4:0:0 12-bit and 16-bit profiles only – again, others seem to be unnecessary, given that the hardware cost is significantly less.

- 16-bit for 4:4:4 and 4:0:0 – the 4:0:0 could be for non-video sensors, such as in medical. Other specific 16-bit profiles for different chroma formats would be unnecessary at this bit depth, noting that the Main 16 4:4:4 profile would also be able to code 4:2:2 and 4:2:0 at 16-bit.

- Intra-only 12-bit and 16-bit profiles only defined because the intra codecs are significantly smaller than inter codecs and the cost in hardware between 8, 10 and 12-bit coding is similar, since the only significant differences will be during the intra prediction (and associated line buffers).

Intra only profiles are defined by limiting all pictures with the following restrictions:

  - nal_unit_type is equal to IDR_N_LP, CRA_NUT or BLA_N_LP

  - No reordering is allowed

  - No short term ref pic sets and no long term pic are allowed

  - Allow to send in-loop filter parameters, but performing in-loop filter is not mandatory

- The RExt tools indicated in the table may or may not include the current full set of tools present in the latest text and software. Such decisions are not part of this profile proposal and should be discussed separately following the profile definitions.

- In comparison with the list of profiles in JCTVC-O1005 (v3), the above table includes:

  - Main 16 Intra 4:0:0

  - Main 12 Intra 4:0:0

  - Main 16 Intra 4:4:4

  - Main 16 4:4:4

whereas the unapproved document JCTVC-O1005 (v3) includes

  - Main [8] Intra [4:2:0]		(Would use Main 12 Intra 4:2:0 in this proposal)

  - Main 10 Intra [4:2:0] 		(Would use Main 12 Intra 4:2:0 in this proposal)

  - Main 10 Intra 4:2:2		(Would use Main 12 Intra 4:2:2 in this proposal)

  - Main [8] Intra 4:4:4		(Would use Main 12 Intra 4:4:4 in this proposal)

  - Main 10 Intra 4:4:4		(Would use Main 12 Intra 4:4:4 in this proposal)

both this proposal and JCTVC-O1005 (v3) include:

  - Main 12 Intra 4:2:0

  - Main 12 Intra 4:2:2

  - Main 12 Intra 4:4:4

with the reduced number of low bit depth intra profiles being defined, since these should be significantly simpler systems than a full inter decoder (an encoder is not affected by the reduced number of intra profiles).

# Syntax

To support new profiles, no new profile_idc value is used. It is signalled by constraint_xxxx_flag. The syntax and semantics changes are summarized in Annex and the attached draft specification.

# Conclusion

This contribution proposes the following:

- Add all intra coding profiles

- Add profiles support up to 16 bit video

- Profiles for mono-chrome video (4:0:0)

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Syntax and semantics

The syntax and semantics changes to support new profiles are summarized in this annex.

### 7.3.3 Profile, tier and level syntax

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| general_profile_compatibility_flag[ j ] | u(1) |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_intra_constraint_flag | u(1) |
| general_max_10bit_constraint_flag | u(1) |
| general_max_12bit_constraint_flag | u(1) |
| general_max_16bit_constraint_flag | u(1) |
| general_reserved_zero_2bits | u(2) |
| general_max_400chroma_constraint_flag | u(1) |
| general_max_420chroma_constraint_flag | u(1) |
| general_max_422chroma_constraint_flag | u(1) |
| general_reserved_zero_35bits | u(35) |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| } |  |
| if( maxNumSubLayersMinus1 > 0 ) |  |
| for( i = maxNumSubLayersMinus1; i < 8; i++ ) |  |
| reserved_zero_2bits[ i ] | u(2) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| if( sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_progressive_source_flag[ i ] | u(1) |
| sub_layer_interlaced_source_flag[ i ] | u(1) |
| sub_layer_non_packed_constraint_flag[ i ] | u(1) |
| sub_layer_frame_only_constraint_flag[ i ] | u(1) |
| sub_layer_intra_constraint_flag | u(1) |
| sub_layer_max_10bit_constraint_flag | u(1) |
| sub_layer_max_12bit_constraint_flag | u(1) |
| sub_layer_max_16bit_constraint_flag | u(1) |
| sub_layer_reserved_zero_2bits | u(2) |
| sub_layer_max_400chroma_constraint_flag | u(1) |
| sub_layer_max_420chroma_constraint_flag | u(1) |
| sub_layer_max_422chroma_constraint_flag | u(1) |
| sub_layer_reserved_zero_35bits[ i ] | u(35) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

### 7.4.4 Profile, tier and level semantics

general_intra_constraint_flag equal to 1 specifies that all pictures have nal_unit_type equal to IDR_N_LP (or CRA_NUT or BLA_N_LP). When general_profile_idc is not equal to 4 and general_profile_compatibility_flag[ 4 ] is not equal to 1, general_intra_constraint_flag shall be equal to 0.

[AJT]

I see for both 4:2:2 & 4:4:4 we use the same profile index – is it OK (just a question)?

[END]

general_max_16bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_‌constraint_flag, general_max_420chroma_constraint_flag and general_max_400chroma_constraint_flag, when general_profile_idc is equal to 4 or general_profile_compatibility_flag[ 4 ] is equal to 1, have semantics specified in Annex Error: Reference source not found. When general_profile_idc is not equal to 4 and general_profile_compatibility_flag[ 4 ] is not equal to 1, general_max_16bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag and general_max_400chroma_constraint_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. When general_profile_idc is not equal to 4 and general_profile_compatibility_flag[ 4 ] is not equal to 1, the value 1 for each syntax element general_max_16bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag and general_max_400chroma_constraint_flag is reserved for future use by ITU-T | ISO/IEC, and decoders shall ignore the values of these syntax elements.

general_reserved_zero_2bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for general_reserved_zero_2bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of general_reserved_zero_2bits.

[AJT]

What is the role of this reserved syntax?

[END]

general_reserved_zero_35bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for general_reserved_zero_35bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of general_reserved_zero_35bits.

[AJT]

What is the role of this reserved syntax?

[END]

sub_layer_intra_constraint_flag equal to 1 specifies that all pictures have nal_unit_type equal to IDR_N_LP (or CRA_NUT or BLA_N_LP). When sub_layer_profile_idc[i] is not equal to 4 and sub_layer_profile_compatibility_flag[ 4 ][ j ] is not equal to 1, sub_layer_intra_constraint_flag shall be equal to 0.

The semantics of the syntax elements sub_layer_profile_space[ i ], sub_layer_tier_flag[ i ], sub_layer_profile_idc[ i ], sub_layer_profile_compatibility_flag[ i ][ j ], sub_layer_progressive_source_flag[ i ], sub_layer_interlaced_source_flag[ i ], sub_layer_non_packed_constraint_flag[ i ], sub_layer_frame_only_constraint_flag[ i ], sub_layer_max_16bit_constraint_flag, sub_layer_max_12bit_constraint_flag, sub_layer_max_10bit_‌constraint_flag, sub_layer_max_422chroma_constraint_flag, sub_layer_max_420chroma_constraint_flag, sub_layer_max_400chroma_constraint_flag, sub_layer_reserved_zero_35bits[ i ], and sub_layer_level_idc[ i ] are the same as the syntax elements general_profile_space, general_tier_flag, general_profile_idc, general_profile_compatibility_flag[ j ], general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag, general_frame_only_constraint_flag, general_max_16bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag and general_max_400chroma_constraint_flag, general_reserved_zero_35bits, and general_level_idc, respectively, but apply to the representation of the sub-layer with TemporalId equal to i.

When not present, the value of sub_layer_tier_flag[ i ] is inferred to be equal to 0.

NOTE 1 – It is possible that sub_layer_tier_flag[ i ] is not present and sub_layer_level_idc[ i ] is present. In this case, a default value of sub_layer_tier_flag[ i ] is needed for interpretation of sub_layer_level_idc[ i ].

A.3.5 Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 and High 4:4:4 16 profiles

Bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 and High 4:4:4 16 profiles shall obey the following constraints:

- In bitstreams conforming to the Main 12, Main 4:2:2 10 or Main 4:2:2 12  profile, SPSs shall have chroma_format_idc constrained as follows:

–	In bitstreams conforming to the Main 12 profile, SPSs shall have chroma_format_idc equal to 0 or 1 only.

–	In bitstreams conforming to the Main 4:2:2 10 or Main 4:2:2 12 profile, SPSs shall have chroma_format_idc equal to 0, 1 or 2 only.

- SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 constrained as follows:

–	In bitstreams conforming to the Main 4:2:2 10 or Main 4:4:4 10 profile, SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

–	In bitstreams conforming to the Main 12, Main 4:2:2 12, or Main 4:4:4 12 profile, SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 in the range of 0 to 4, inclusive.

- In bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 and High 4:4:4 16 profiles,

- SPSs shall have separate_colour_plane_flag, when present, equal to 0 only.

- SPSs shall have extended_precision_processing_flag, when present, equal to 0 only.

- CtbLog2SizeY shall be in the range of 4 to 6, inclusive.

- When a PPS has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a PPS has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

- general_level_idc shall not be equal to 255 (which indicates level 8.5).

- transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, intra_block_copy_enabled_flag, residual_dpcm_intra_enabled_flag and residual_dpcm_inter_enabled_flag shall be equal to <TO BE DECIDED AT THE NEXT MEETING>

- In bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 and Main 4:4:4 12 profiles, extended_precision_processing_flag shall be equal to 0.

- In bitstreams conforming to the High 4:0:0 16 and High 4:4:4 16 profiles, extended_precision_processing_flag is equal to 0 or 1.

- The level constraints specified for the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 or High 4:4:4 16 profile in in subclause Error: Reference source not found, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12 , High 4:0:0 16 or High 4:4:4 16 profile is indicated by general_profile_idc being equal to 4 or general_profile_compatibility_flag[ 4 ] being equal to 1 with the following additional indications as applicable:

- Conformance of the bitstream to the Main 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 1, general_max_420chroma_constraint_flag equal to 1 and general_max_400chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:2:2 10 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 1, general_max_422chroma_constraint_flag equal to 1, general_max_420chroma_constraint_flag equal to 1 and general_max_400chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:2:2 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 1, general_max_420chroma_constraint_flag equal to 0 and general_max_400chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:4:4 10 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 1, general_max_422chroma_constraint_flag equal to 0, general_max_420chroma_constraint_flag equal to 0 and general_max_400chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:4:4 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 0, general_max_420chroma_constraint_flag equal to 0 and general_max_400chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the High 4:0:0 16 profile is indicated by general_max_16bit_constraint_flag equal to 1, general_max_12bit_constraint_flag equal to 0, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 0, general_max_420chroma_constraint_flag equal to 0 and general_max_400chroma_constraint_flag equal to 1.

- Conformance of the bitstream to the High 4:4:4 16 profile is indicated by general_max_16bit_constraint_flag equal to 1, general_max_12bit_constraint_flag equal to 0, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 0, general_max_420chroma_constraint_flag equal to 0 and general_max_400chroma_constraint_flag equal to 0.

All other combinations of general_max_16bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag and general_max_400chroma_constraint_flag equal to 0 with general_profile_idc equal to 4 or general_profile_compatibility_flag[ 4 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. Such combinations shall not be present in bitstreams conforming to this specification. However, decoders conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 or High 4:4:4 16 profile shall allow other combinations as specified below in this subclause to occur in the bitstream.

Decoders conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16 or High 4:4:4 16 profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which all of the following conditions apply:

- One or both of the following conditions apply:

–	general_profile_idc is equal to 1 or 2, general_profile_compatibility_flag[ 1 ] is equal to 1, general_profile_compatibility_flag[ 2 ] is equal to 1.

–	general_profile_idc is equal to 4 or general_profile_compatibility_flag[ 4 ] is equal to 1, and any of the following conditions apply:

–	The decoder conforms to the Main 12 profile, general_max_12bit_constraint_flag is equal to 1, and general_max_420chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:2:2 10 profile, general_max_10bit_constraint_flag is equal to 1, and general_max_422chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:2:2 12 profile, general_max_12bit_constraint_flag is equal to 1 or general_max_10bit_constraint_flag is equal to 1, and general_max_422chroma_constraint_flag is equal to 1 or general_max_420chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:4:4 10 profile and general_max_10bit_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:4:4 12 profile and general_max_12bit_constraint_flag is equal to 1 or general_max_12bit_constraint_flag is equal to 1.

–	The decoder conforms to the High 4:0:0 16 profile and general_max_400chroma_constraint_flag is equal to 1.

- general_level_idc is not equal to 255 and represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.

A.3.6 Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra, High 4:0:0 16 Intra and High 4:4:4 16 Intra profiles

Bitstreams conforming to the Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra, High 4:0:0 16 Intra and High 4:4:4 16 Intra profiles shall obey the following constraints:

- In bitstreams conforming to the Main 12 Intra, all constraints specified in subclause  for the Main 12 profile shall be obeyed.

- In bitstreams conforming to the Main 4:2:2 12 Intra, all constraints specified in subclause  for the Main 4:2:2 12 profile shall be obeyed.

- In bitstreams conforming to the Main 4:4:4 12 Intra, all constraints specified in subclause  for the Main 4:4:4 12 profile shall be obeyed.

- In bitstreams conforming to the High 4:0:0 16 Intra, all constraints specified in subclause  for the High 4:0:0 16 profile shall be obeyed.

- In bitstreams conforming to the High 4:4:4 16 Intra, all constraints specified in subclause  for the High 4:4:4 16 profile shall be obeyed.

- general_intra_constraint_flag shall be equal to 1.

- All pictures shall be IDR picture with nal_unit_type equal to IDR_N_LP.

- When vps_sub_layer_ordering_info_present_flag is equal to 1, vps_max_num_reorder_pics shall be equal to 0.

- When sps_sub_layer_ordering_info_present_flag is equal to 1, sps_max_num_reorder_pics shall be equal to 0.

- Sequence parameter sets shall have num_short_term_ref_pic_sets equal to 0.

- Sequence parameter sets shall have long_term_ref_pics_present_flag equal to 0.

- Picture timing SEI messages, whether present in the bitstream (by non-VCL NAL units) or conveyed equivalently by other means not specified in this Recommendation | International Standard, shall have pic_dpb_output_delay equal to 0.

The operation of the in-loop filter process specified in subclause Error: Reference source not found is not required for decoder conformance to the Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra, High 4:0:0 16 Intra and High 4:4:4 16 Intra profiles.

NOTE – The in-loop filter process specified in subclause Error: Reference source not found or some similar post-processing filter should be performed, although this is not a requirement for decoder conformance to the Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra and High 4:4:4 16 Intra profiles. The syntax elements sent by an encoder for control of the in-loop filter process specified in subclause Error: Reference source not found are considered only as advisory information for decoders conformance to the Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra and High 4:4:4 16 Intra profiles. However, the application of the deblocking filter process specified in subclause Error: Reference source not found is required for decoder conformance to the Main 12, Main 4:2:2 12, Main 4:4:4 12 and High 4:4:4 16 profiles when decoding bitstreams that conform to the Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra and High 4:4:4 16 Intra profiles.

A.4.2 Profile-specific level limits for the Main, Main 10, Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16, High 4:4:4 16, Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra, High 4:0:0 16 Intra and High 4:4:4 16 Intra profiles

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

Bitstreams conforming to the Main, Main 10, Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, High 4:0:0 16, High 4:4:4 16, Main 12 Intra, Main 4:2:2 12 Intra, Main 4:4:4 12 Intra, High 4:0:0 16 Intra and High 4:4:4 16 Intra profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

Table A-2 – Specification of CpbBrVclFactor, CpbBrNalFactor, FormatCapabilityFactor, and MinCrScaleFactor

| Profile | CpbBrVclFactor | CpbBrNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main | 1000 | 1100 | 1.500 | 1.0 |
| Main 10 | 1000 | 1100 | 1.875 | 1.0 |
| Main 12 | 1000 | 1100 | 2.250 | 1.0 |
| Main 4:2:2:10 | 1250 | 1375 | 2.500 | 0.5 |
| Main 4:2:2 12 | 1250 | 1375 | 3.000 | 0.5 |
| Main 4:4:4 10 | 1500 | 1650 | 3.750 | 0.5 |
| Main 4:4:4 12 | 1500 | 1650 | 4.500 | 0.5 |
| High 4:0:0 16 | 2000 | 2200 | 4.500 | 0.5 |
| High 4:4:4 16 | 2000 | 2200 | 4.500 | 0.5 |
| Main 12 Intra | 4000 | 4400 | 2.250 | 1.0 |
| Main 4:2:2 12 Intra | 4000 | 4400 | 3.000 | 0.5 |
| Main 4:4:4 12 Intra | 4000 | 4400 | 3.750 | 0.5 |
| High 4:0:0 16 Intra | 4000 | 4400 | 4.5 | 0.5 |
| High 4:4:4 16 Intra | 4000 | 4400 | 4.5 | 0.5 |