| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0058 |
| --- | --- |

| Title: | AHG 5 and 18: Request for 16 bit Profiles for Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
|  | T. Suzuki 2-10-1, Osaki, Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81 50 3750 2740 teruhikos@jp.sony.com |
|  | A. Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | +1 408 352 4715 ali.tabatabai@am.sony.com |
| Source: | Sony Europe Ltd., Sony Corp., and Sony Electronics Inc. |  |  |

_____________________________

# Proposal

This document proposes to define 16 bit profiles as part of Amd1 of HEVC. The following applications would make use of these profiles:

- 4:4:4 format for High End Content Creation, High End Production & Medical

- 4:0:0 format for Medical

All intra coding is used for high end production where already 16 bit material is captured and used during the workflow; such 16 bit material has been added to the RExt test set. Therefore, this contribution proposes to also include profiles for all intra coding.

| Profile |  |  | Profile IDC | Chroma formats |  |  |  | RExt tools | Ext. precision |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 4:0:0 | 4:2:0 | 4:2:2 | 4:4:4 |  |  |  |
| 4:4:4 |  | 16 Intra 4:4:4 | 4 |  |  |  |  |  |  |  |
|  |  | 16 4:4:4 | 4 |  |  |  |  |  |  |  |
| 4:0:0 |  | 16 Intra 4:0:0 | 4 |  |  |  |  |  |  |  |
|  |  | 16 4:0:0 | 4 |  |  |  |  |  |  |  |
|  | Only includes RExt tools necessary to support chroma formats and also high precision weighted prediction. |  |  |  |  |  |  |  |  |  |

Table 1 : Summary of the proposed 16 bit profiles for RExt

The points of the above new profiles are:

- No 14-bit profiles are defined – the extended processing design is already scalable, and therefore 14-bits being encoded under the 16-bit profile will be identical to a 14-bit encoding under a 14-bit profile.

- For 12-bit high end content creation applications that require the extended precision processing capability, the 16 bit profiles would be used, noting again that the extended precision system is bit-depth scalable.

- 16-bit for 4:4:4 and 4:0:0 – the 4:0:0 would be for non-video sensors, such as in medical. Other specific 16-bit profiles for different chroma formats would be unnecessary at this bit depth, noting that the16 bit 4:4:4 profile would also be able to code 4:2:2 at 16-bit.

- The RExt tools indicated in the table may or may not include the current full set of tools present in the latest text and software. Such decisions are not part of this profile proposal and should be discussed separately following the profile definitions.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).