| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0059 |
| --- | --- |

| Title: | Unifying HM and RExt Inter-Prediction Search |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd. |  |  |

_____________________________

# Abstract

This contribution examines the encoder inter search algorithms used in RExt and HM. At the previous JCTVC meeting, the cross-component decorrelation tool was adopted, along with a modified inter search algorithm. This has led to the RExt model having two different searching algorithms: one that is compatible with HM, and one that is not.

It is proposed that RExt and HM are harmonised, either by applying a patch to HM or by using the HM-compatible search within RExt.

Presented BD rate changes using the HM common test conditions indicate that the modified search within the HM patch has no significant overall effect; BD rate changes using the AHG5 RExt common test conditions indicate that the modified search algorithm is up to 0.3% better for RGB inter simulations than the HM-compatible search; BD rate changes using the AHG8 RExt lossless test conditions indicate that the modified search algorithm is 4.0% worse for RGB inter simulations than the HM-compatible search; other test scenarios show negligible change.

# Introduction

At the previous meeting, the cross-component decorrelation tool was adopted, which used a modified search algorithm that removed HM-compatible behaviour. Maintaining HM-compatible behaviour is a necessary feature in the RExt model, as it allows the model to be verified when patching in the latest changes made to the HM model. The software coordinators therefore attempted to produce an HM-compatible search, although time was limited, and in the end the search algorithm developed by the proponents was specified to be used for all Range Extension test conditions; the HM-compatible search was to only be used for HM test conditions.

The search functions are large, and having such parts of the software duplicated is not ideal.

It is proposed that either HM is patched to become compatible with the modified RExt search algorithm, or the modified RExt search algorithm is removed.

## Search background

In the current HM12.1 software encoder, the inter residual estimation function xEstimateResidualQT() resets the entropy coder at certain points in the estimation process to the state it was in at the start of the function. The resets are positioned such that chroma TUs “run together” (i.e. the entropy coder is not reset between chroma channels), but are separate from luma TUs.

In the RExt software, the corresponding function loops over all components and applies conditional entropy coder resets in order to match HM. Since the resets are applied differently when testing transform-skip, this has required that the RExt code loop over components three times to maintain backwards-compatibility.

There has been a comment in the RExt software that a tidy up of the function would be useful, although as it would break backwards compatibility, this was never acted upon, until the cross-component decorrelation tool adoption.

With the addition of new tools in RExt that directly affect inter residual estimation, such as cross-component decorrelation, it is becoming increasingly difficult to optimally (i.e. with each channel having full information about the choices made for previous channels) search all possible tool options while maintaining backwards-compatibility for HM test conditions.

This complexity is the likely reason for the change in the search algorithm in the cross-component decorrelation proposal, although this change has made the search non-backwards compatible with HM. By adding a small number of extra entropy coder resets to the HM version of xEstimateResidualQT(), HM can be made compatible with the modified RExt search algorithm. In this modified RExt search algorithm, only one loop over components is required, allowing optimal tool mode estimation and therefore reducing losses due to poor encoder-side decision making.

This modification does not include any saving/loading of CABAC states, which was suggested in the AHG7 release e-mail of RExt5.0 to the reflector.

A patch to HM12.1 is included with this contribution that applies the required changes.

# Schemes from the Perspective of Range Extensions Software

## HM12.1-Compatible RExt Encoder Inter Search

For each component

the entropy coder is reset (NOT restored)

the block is transformed & quantised

the block is entropy-code-estimated, yielding the number of bits

For each component

the entropy coder is reset (NOT restored)

if the absolute sum of quantised coefficients is non-zero,

the block is dequantised and inverse-transformed

a cost is derived for this case (using the number of bits derived from the

first component loop)

a cost is derived for the case where all coefficients are 0

the costs are compared, and the lowest selected.

else

the cost is only derived for the coefficients being all 0

For each component

For each tested mode of transform-skip [and cross-component decorrelation]

(where at least one optional tool is enabled)

the entropy coder is restored & reset if the component isn't Cr

the block is transform-skipped and quantised

the block is entropy-code-estimated

a cost is derived for this case

the cost is compared against the previous best, and the lowest is selected

The entropy coder is restored (i.e. 'load') and reset.

Final encode using the decisions made.

## Modified RExt Encoder Inter Search

For each component

For each tested mode of transform-skip [and cross-component decorrelation]

the entropy coder is restored to the initial state & reset

the block is transformed & quantised

the block is entropy-code-estimated

if the absolute sum of quantised coefficients is non-zero,

the block is dequantised and inverse-transformed

else

the inverse-transformed block is all 0s.

a cost is derived.

If tested mode is where all optional tools are disabled,

and the absolute sum of quantised coefficients is non-zero,

a cost is also derived for the case of the coefficients

being 0.

the best cost is selected.

The entropy coder is restored (i.e. 'load') and reset.

Final encode using the decisions made.

# Results

The patched version of HM12.1 (to harmonise HM with the modified inter search in RExt) has been compared to HM12.1 as tagged under standard HM inter test conditions. The results are shown in Table 1, and indicate that there no significant change observed.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8 bit) | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8 bit) |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | -0.5% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.2% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.3% |
| Class E | -0.1% | 0.0% | -0.1% | 0.0% | -0.5% | 0.3% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | 0.4% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8 bit) |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.2% | 0.0% | 0.0% | -0.2% | 0.0% |
| Class D | 0.0% | -0.4% | -0.2% | 0.0% | -0.1% | -0.1% |
| Class E | 0.0% | 0.4% | 0.0% | 0.1% | 0.0% | 0.3% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class F | 0.0% | 0.1% | 0.5% | 0.0% | -0.5% | -0.7% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 1 - Effect of modified inter search under HM common test conditions

The HM12.1_RExt5.1 has been tested with the HM-compatible search method (‘tested’) relative to the modified RExt5.1 search method (‘anchor’). These indicate that the anchor (modified search method) is better, but only for random access and inter, where there is an average BD-rate change of 0.3% for inter RGB 4:4:4 results (and negligible elsewhere).

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.2% | 0.1% | 0.0% | 0.2% | 0.1% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.5% | 0.3% | 0.4% | 0.4% | 0.2% | 0.3% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.3% |  |  |  |
| YCbCr 4:2:2 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.2% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 2 – HM12.1_RExt5.1 with HM-style search vs HM12.1_RExt5.1, using AHG5 CTC

Using AHG8 lossless test conditions the differences indicate that actually the modified RExt inter search algorithm used in the anchor is detrimental to the compression efficiency. The table below again shows the results for the HM-compatible search method (‘tested’) relative to the modified RExt5.1 search method (‘anchor’), giving an average BD-rate change of -4.0% for inter RGB 4:4:4 results, or -0.7% over all non-optional results. i.e. the RExt HM-compatible search is better than the anchor’s modified RExt inter search.

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -1.5% | -2.9% | -4.4% |
| RGB 4:4:4 Animation | 0.0% | -3.9% | -4.6% |
| YCbCr 4:4:4 SC | -0.5% | 0.4% | 0.1% |
| YCbCr 4:4:4 Animation | 0.0% | 1.6% | 2.0% |
| RangeExt | 0.0% | -0.3% | -0.3% |
| RGB 4:4:4 SC (Optional) | -5.9% | -7.9% | -7.3% |
| YCbCr 4:4:4 SC (Optional) | -1.2% | -1.3% | -1.0% |
| Enc Time[%] | 100% | 99% | 99% |
| Dec Time[%] | 100% | 99% | 99% |

Table 3 - HM12.1_RExt5.1 with HM-style search vs HM12.1_RExt5.1, using AHG8 lossless CTC

# Conclusion

Following the previous adoptions, the Range Extension model now contains two inter search algorithms: one that is compatible with HM, and one that is not.

It is proposed that RExt and HM are harmonised, either by applying a patch to HM or by using the HM-compatible search within RExt (and removing the modified search).

BD rate changes using the HM common test conditions indicate that a modified search has no significiant overall effect; BD rate changes using the AHG5 RExt common test conditions indicate that the modified search algorithm is up to 0.3% better for RGB inter simulations (and negligible otherwise); BD rate changes using the AHG8 RExt lossless test conditions indicate that the modified search algorithm is 4.0% worse for RGB inter simulations (or 0.7% over all non-optional results).

The modified search algorithm is significantly easier to understand and integrate new ideas, although could be further improved to save and load the running-best CABAC state, where it would be expected that further improvements in compression efficiency would be observed.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).