| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0060 |
| --- | --- |

| Title: | RCE1: Results of tests B1, B2 and B3a. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd. |  |  |

_____________________________

# Abstract

This document presents the results of subtests B1, B2 and B3a of RCE1.

# Introduction

RCE1 has been set up to examine schemes for improving throughput in the CABAC core, with the target application being high bit rate systems, including, but not limited to, high bit depths. This document covers three of the subtests in the RCE:

SubTest B1: Always align to 256 prior to any sign data.

SubTest B2: Always align to 256 or 384 prior to any sign data.

SubTest B3a: Align to 256 prior to any sign data only if there is some escape data present in the 4x4 sub-block.

The alignment schemes have previously been described in M0178, N0190 and O0046, where the latter also included the conditional alignment of SubTest B3a being applied to the alignment used in SubTest B2.

# Results

The results for the sub-tests are presented below.

When calculating the worst case scenarios, 16-bit processing is assumed, which places a maximum magnitude of 222 for coefficients in the bit-stream. When the Golomb Rice parameter is 0, this magnitude can be represented using an escape code of length 46 [3 (Golomb Rice prefix) + 21 (Exp. Golomb prefix) + 1 (separator) + 21 (Exp. Golomb suffix)]. As the Golomb Rice parameter increases during the processing of coefficients, the Exp. Golomb prefix is reduced by the same amount.

## SubTest B1 (Align to 256)

### AHG5 test conditions

#### BD-rate changes

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 2.2% | 1.9% | 2.0% | 1.9% | 1.7% | 1.8% | 1.6% | 1.4% | 1.5% |
| YCbCr 4:4:4 | 2.1% | 1.8% | 1.9% | 2.0% | 1.8% | 1.9% | 1.8% | 1.6% | 1.8% |
| YCbCr 4:2:2 | 2.1% | 2.1% | 2.2% | 1.9% | 1.8% | 1.9% | 1.7% | 1.5% | 1.6% |
| Overall | 2.1% | 2.0% | 2.0% | 1.9% | 1.8% | 1.9% | 1.7% | 1.5% | 1.6% |
|  | 2.1% | 2.0% | 2.0% | 1.9% | 1.7% | 1.9% | 1.7% | 1.5% | 1.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 1.9% | 1.9% | 2.4% | 1.9% | 1.9% | 2.4% |  |  |  |
| YCbCr 4:4:4 | 1.8% | 1.6% | 2.2% | 1.9% | 1.8% | 2.4% |  |  |  |
| YCbCr 4:2:2 | 1.8% | 2.1% | 2.2% | 1.7% | 2.1% | 2.4% |  |  |  |
| Overall | 1.9% | 1.9% | 2.2% | 1.8% | 1.9% | 2.4% |  |  |  |
|  | 1.8% | 2.0% | 2.3% | 1.9% | 1.8% | 2.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 1.9% | 1.9% | 2.6% | 2.1% | 1.9% | 2.6% |  |  |  |
| YCbCr 4:4:4 | 1.7% | 2.0% | 2.8% | 1.9% | 2.1% | 3.1% |  |  |  |
| YCbCr 4:2:2 | 1.6% | 2.6% | 3.0% | 1.6% | 2.4% | 3.5% |  |  |  |
| Overall | 1.8% | 2.2% | 2.8% | 1.9% | 2.2% | 3.0% |  |  |  |
|  | 1.7% | 2.2% | 2.8% | 1.9% | 2.3% | 3.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

#### Bin counts per sample

|  | Reference |  |  |  | Tested |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Context Bins | Bypass Bins | Raw Bins | Total Bins | Context Bins | Bypass Bins | Raw Bins | Total Bins |
| AI-MT | 0.29 | 0.13 | 0.00 | 0.42 | 0.30 | 0.02 | 0.11 | 0.43 |
| AI-HT | 0.51 | 0.27 | 0.00 | 0.78 | 0.53 | 0.02 | 0.24 | 0.80 |
| AI-SHT | 0.80 | 0.52 | 0.00 | 1.32 | 0.83 | 0.03 | 0.49 | 1.35 |
| RA-MT | 0.07 | 0.02 | 0.00 | 0.09 | 0.07 | 0.00 | 0.02 | 0.09 |
| RA-HT | 0.18 | 0.07 | 0.00 | 0.25 | 0.19 | 0.01 | 0.06 | 0.26 |
| LB-MT | 0.09 | 0.02 | 0.00 | 0.11 | 0.09 | 0.00 | 0.02 | 0.12 |
| LB-HT | 0.23 | 0.08 | 0.00 | 0.30 | 0.23 | 0.01 | 0.07 | 0.31 |

### AHG8 lossless test conditions

#### BD-rate changes

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 1.1% | 1.4% | 1.6% |
| Class B | 0.8% | 1.0% | 1.0% |
| RGB 4:4:4 SC | 1.0% | 1.1% | 1.1% |
| RGB 4:4:4 Animation | 1.0% | 1.3% | 1.3% |
| YCbCr 4:4:4 SC | 1.1% | 1.2% | 1.4% |
| YCbCr 4:4:4 Animation | 1.1% | 1.5% | 1.5% |
| RangeExt | 0.5% | 0.6% | 0.6% |
| RGB 4:4:4 SC (Optional) | 1.3% | 1.3% | 1.3% |
| YCbCr 4:4:4 SC (Optional) | 1.2% | 1.3% | 1.3% |
| Enc Time[%] | 100% | 100% | 100% |
| Dec Time[%] | 97% | 98% | 98% |

#### Bin counts per sample

|  | Reference |  |  |  | Tested |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Context Bins | Bypass Bins | Raw Bins | Total Bins | Context Bins | Bypass Bins | Raw Bins | Total Bins |
| AI | 0.95 | 1.36 | 0.00 | 2.31 | 0.98 | 0.03 | 1.33 | 2.34 |
| RA | 0.74 | 0.94 | 0.00 | 1.68 | 0.76 | 0.02 | 0.93 | 1.71 |
| LB | 0.73 | 0.93 | 0.00 | 1.67 | 0.76 | 0.02 | 0.92 | 1.70 |

### AHG18 test conditions

#### BD-rate changes

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) | Intra 12-bit (internal bit depth) | Lowdelay 16-bit (internal bit depth) | Lowdelay 12-bit (internal bit depth) |
|  |  |  | Y/G | Y/G | Y/G | Y/G |
| SVT |  |  | 0.2% | 0.5% | 0.2% | 0.6% |
| RExt RGB |  |  | 0.3% | 0.7% | 0.4% | 0.9% |
| HDR |  |  | 0.6% | 1.3% | 0.6% | 1.3% |
| 4K from HDR |  |  | 0.3% | 0.7% |  |  |
| HDR 4:0:0 Medical |  |  | 0.7% | 1.6% | 0.8% | 1.9% |
| 4:0:0 Medical |  |  | 0.5% | 1.0% | 0.6% | 1.0% |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0.5% | 1.2% | 0.6% | 1.7% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.7% | 1.7% | 1.0% | 1.6% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 1.2% | 2.1% | 1.7% | 0.8% |
| All Classes |  |  | 0.6% | 1.2% | 0.7% | 1.2% |
| Encoding Time % |  |  | 101% | 101% | 101% | 100% |
| Decoding Time % |  |  | 90% | 95% | 93% | 97% |

#### Bin counts per sample

|  | Reference |  |  |  | Tested |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Context Bins | Bypass Bins | Raw Bins | Total Bins | Context Bins | Bypass Bins | Raw Bins | Total Bins |
| Intra 16-bit | 1.70 | 6.51 | 0.00 | 8.20 | 1.76 | 0.03 | 6.47 | 8.26 |
| Intra 12-bit | 1.20 | 2.53 | 0.00 | 3.72 | 1.24 | 0.03 | 2.50 | 3.77 |
| Lowdelay 16-bit | 1.57 | 4.95 | 0.00 | 6.52 | 1.62 | 0.02 | 4.93 | 6.57 |
| Lowdelay 12-bit | 0.91 | 1.57 | 0.00 | 2.47 | 0.94 | 0.01 | 1.55 | 2.50 |

### Coefficient-group worst case analysis

The following analysis examines the worst case encodings for coefficient data per coefficient group.

|  | Base |  | SubTest B1 |  |
| --- | --- | --- | --- | --- |
| GolombRiceGroupAdaptation | Enabled (4x4 TU) | Disabled | Enabled (4x4 TU) | Disabled |
| Total number of context bins | 25 =16+8+1 |  | 25 =16+8+1 |  |
| Total number of bypass bins with CABAC | 647 = (1+46) + (1+39)*15 | 698 = (1+46) + (1+45) + (1+44) + (1+43) + (1+42)*12 | 0 | 0 |
| Total number of bypass bins with CABAC (Raw) | 0 | 0 | 647 | 698 |

## SubTest B2 (Align to 256, 384)

### AHG5 test conditions

#### rate changes

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.3% | 1.2% | 1.2% | 1.1% | 1.0% | 1.1% | 0.9% | 0.9% | 0.9% |
| YCbCr 4:4:4 | 1.3% | 1.1% | 1.2% | 1.2% | 1.1% | 1.2% | 1.1% | 1.0% | 1.0% |
| YCbCr 4:2:2 | 1.3% | 1.2% | 1.3% | 1.2% | 1.1% | 1.1% | 1.0% | 0.9% | 0.9% |
| Overall | 1.3% | 1.2% | 1.2% | 1.2% | 1.1% | 1.1% | 1.0% | 0.9% | 1.0% |
|  | 1.3% | 1.2% | 1.2% | 1.2% | 1.0% | 1.1% | 1.0% | 0.9% | 1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  | 98% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 1.2% | 1.2% | 1.4% | 1.2% | 1.1% | 1.4% |  |  |  |
| YCbCr 4:4:4 | 1.2% | 1.1% | 1.1% | 1.2% | 1.1% | 1.3% |  |  |  |
| YCbCr 4:2:2 | 1.1% | 1.3% | 1.3% | 1.0% | 1.3% | 1.4% |  |  |  |
| Overall | 1.2% | 1.2% | 1.3% | 1.1% | 1.2% | 1.4% |  |  |  |
|  | 1.1% | 1.3% | 1.3% | 1.2% | 0.9% | 1.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 1.1% | 1.2% | 1.5% | 1.2% | 1.1% | 1.5% |  |  |  |
| YCbCr 4:4:4 | 1.1% | 1.2% | 1.6% | 1.2% | 1.2% | 1.8% |  |  |  |
| YCbCr 4:2:2 | 1.0% | 1.4% | 1.8% | 1.0% | 1.5% | 2.0% |  |  |  |
| Overall | 1.1% | 1.3% | 1.6% | 1.1% | 1.3% | 1.8% |  |  |  |
|  | 1.1% | 1.2% | 1.6% | 1.2% | 1.5% | 1.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

#### Bin counts per sample

|  | Reference |  |  |  | Tested |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Context Bins | Bypass Bins | Raw Bins | Total Bins | Context Bins | Bypass Bins | Raw Bins | Total Bins |
| AI-MT | 0.29 | 0.13 | 0.00 | 0.42 | 0.30 | 0.02 | 0.11 | 0.43 |
| AI-HT | 0.51 | 0.27 | 0.00 | 0.78 | 0.53 | 0.02 | 0.24 | 0.80 |
| AI-SHT | 0.80 | 0.52 | 0.00 | 1.32 | 0.83 | 0.03 | 0.48 | 1.35 |
| RA-MT | 0.07 | 0.02 | 0.00 | 0.09 | 0.07 | 0.00 | 0.02 | 0.10 |
| RA-HT | 0.18 | 0.07 | 0.00 | 0.25 | 0.19 | 0.01 | 0.06 | 0.26 |
| LB-MT | 0.09 | 0.02 | 0.00 | 0.11 | 0.09 | 0.00 | 0.02 | 0.12 |
| LB-HT | 0.23 | 0.08 | 0.00 | 0.30 | 0.23 | 0.01 | 0.07 | 0.31 |

### AHG8 lossless test conditions

#### BD-rate changes

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.6% | 0.9% | 1.0% |
| Class B | 0.5% | 0.6% | 0.6% |
| RGB 4:4:4 SC | 0.6% | 0.7% | 0.7% |
| RGB 4:4:4 Animation | 0.6% | 0.8% | 0.8% |
| YCbCr 4:4:4 SC | 0.7% | 0.7% | 0.8% |
| YCbCr 4:4:4 Animation | 0.6% | 0.9% | 0.9% |
| RangeExt | 0.3% | 0.3% | 0.3% |
| RGB 4:4:4 SC (Optional) | 0.7% | 0.7% | 0.8% |
| YCbCr 4:4:4 SC (Optional) | 0.7% | 0.3% | 0.7% |
| Enc Time[%] | 100% | 100% | 100% |
| Dec Time[%] | 97% | 98% | 98% |

#### Bin counts per sample

|  | Reference |  |  |  | Tested |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Context Bins | Bypass Bins | Raw Bins | Total Bins | Context Bins | Bypass Bins | Raw Bins | Total Bins |
| AI | 0.95 | 1.36 | 0.00 | 2.31 | 0.98 | 0.03 | 1.32 | 2.33 |
| RA | 0.74 | 0.94 | 0.00 | 1.68 | 0.76 | 0.02 | 0.92 | 1.70 |
| LB | 0.73 | 0.93 | 0.00 | 1.67 | 0.76 | 0.02 | 0.91 | 1.69 |

### AHG18 test conditions

#### BD-rate changes

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) | Intra 12-bit (internal bit depth) | Lowdelay 16-bit (internal bit depth) | Lowdelay 12-bit (internal bit depth) |
|  |  |  | Y/G | Y/G | Y/G | Y/G |
| SVT |  |  | 0.2% | 0.3% | 0.2% | 0.3% |
| RExt RGB |  |  | 0.2% | 0.4% | 0.2% | 0.5% |
| HDR |  |  | 0.3% | 0.8% | 0.4% | 0.8% |
| 4K from HDR |  |  | 0.2% | 0.4% |  |  |
| HDR 4:0:0 Medical |  |  | 0.4% | 1.0% | 0.5% | 1.1% |
| 4:0:0 Medical |  |  | 0.3% | 0.6% | 0.3% | 0.6% |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0.3% | 0.7% | 0.4% | 1.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.4% | 1.0% | 0.6% | 0.9% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.7% | 1.2% | 1.0% | 0.4% |
| All Classes |  |  | 0.3% | 0.7% | 0.4% | 0.7% |
| Encoding Time % |  |  | 101% | 100% | 101% | 100% |
| Decoding Time % |  |  | 89% | 95% | 92% | 97% |

#### Bin counts per sample

|  | Reference |  |  |  | Tested |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Context Bins | Bypass Bins | Raw Bins | Total Bins | Context Bins | Bypass Bins | Raw Bins | Total Bins |
| Intra 16-bit | 1.70 | 6.51 | 0.00 | 8.20 | 1.76 | 0.03 | 6.46 | 8.25 |
| Intra 12-bit | 1.20 | 2.53 | 0.00 | 3.72 | 1.24 | 0.03 | 2.49 | 3.76 |
| Lowdelay 16-bit | 1.57 | 4.95 | 0.00 | 6.52 | 1.63 | 0.02 | 4.92 | 6.56 |
| Lowdelay 12-bit | 0.91 | 1.57 | 0.00 | 2.47 | 0.94 | 0.01 | 1.54 | 2.50 |

### Coefficient-group worst case analysis

The following analysis examines the worst case encodings for coefficient data per coefficient group.

|  | Base |  | SubTest B1 |  |
| --- | --- | --- | --- | --- |
| GolombRiceGroupAdaptation | Enabled (4x4 TU) | Disabled | Enabled (4x4 TU) | Disabled |
| Total number of context bins | 25 |  | 25 |  |
| Total number of bypass bins with CABAC | 647 | 698 | 0 | 0 |
| Total number of bypass bins with CABAC (Raw) | 0 | 0 | 647 | 698 |

## SubTest B3a (Conditionally align to 256)

### AHG5 test conditions

#### BD-rate changes

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.9% | 0.8% | 0.9% | 1.0% | 0.9% | 0.9% | 1.0% | 0.9% | 0.9% |
| YCbCr 4:4:4 | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.8% | 0.9% |
| YCbCr 4:2:2 | 0.9% | 0.9% | 0.9% | 1.0% | 0.9% | 0.9% | 1.0% | 0.9% | 0.9% |
| Overall | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% |
|  | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% | 0.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.6% | 0.6% | 0.6% | 0.7% | 0.7% | 0.7% |  |  |  |
| YCbCr 4:4:4 | 0.7% | 0.7% | 0.6% | 0.7% | 0.7% | 0.6% |  |  |  |
| YCbCr 4:2:2 | 0.8% | 0.7% | 0.6% | 0.8% | 0.7% | 0.6% |  |  |  |
| Overall | 0.7% | 0.7% | 0.6% | 0.7% | 0.7% | 0.6% |  |  |  |
|  | 0.7% | 0.7% | 0.6% | 0.7% | 0.4% | 0.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.4% | 0.5% | 0.4% | 0.6% | 0.6% | 0.6% |  |  |  |
| YCbCr 4:4:4 | 0.6% | 0.7% | 0.6% | 0.5% | 0.6% | 0.6% |  |  |  |
| YCbCr 4:2:2 | 0.6% | 0.5% | 0.6% | 0.6% | 0.5% | 0.6% |  |  |  |
| Overall | 0.5% | 0.6% | 0.5% | 0.6% | 0.6% | 0.6% |  |  |  |
|  | 0.5% | 0.2% | 0.5% | 0.6% | 0.6% | 0.6% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

#### Bin counts per sample

|  | Reference |  |  |  | Tested |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Context Bins | Bypass Bins | Raw Bins | Total Bins | Context Bins | Bypass Bins | Raw Bins | Total Bins |
| AI-MT | 0.29 | 0.13 | 0.00 | 0.42 | 0.30 | 0.04 | 0.09 | 0.43 |
| AI-HT | 0.51 | 0.27 | 0.00 | 0.78 | 0.52 | 0.05 | 0.21 | 0.79 |
| AI-SHT | 0.80 | 0.52 | 0.00 | 1.32 | 0.82 | 0.07 | 0.45 | 1.34 |
| RA-MT | 0.07 | 0.02 | 0.00 | 0.09 | 0.07 | 0.01 | 0.01 | 0.09 |
| RA-HT | 0.18 | 0.07 | 0.00 | 0.25 | 0.19 | 0.02 | 0.05 | 0.26 |
| LB-MT | 0.09 | 0.02 | 0.00 | 0.11 | 0.09 | 0.01 | 0.01 | 0.11 |
| LB-HT | 0.23 | 0.08 | 0.00 | 0.30 | 0.23 | 0.02 | 0.05 | 0.31 |

### AHG8 lossless test conditions

#### BD-rate changes

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.7% | 0.7% | 0.6% |
| Class B | 0.8% | 0.9% | 0.9% |
| RGB 4:4:4 SC | 0.8% | 0.8% | 0.8% |
| RGB 4:4:4 Animation | 0.9% | 0.9% | 0.9% |
| YCbCr 4:4:4 SC | 0.8% | 0.7% | 0.8% |
| YCbCr 4:4:4 Animation | 0.7% | 0.7% | 0.7% |
| RangeExt | 0.5% | 0.6% | 0.6% |
| RGB 4:4:4 SC (Optional) | 0.8% | 0.8% | 0.8% |
| YCbCr 4:4:4 SC (Optional) | 1.0% | 1.0% | 0.9% |
| Enc Time[%] | 100% | 100% | 100% |
| Dec Time[%] | 97% | 98% | 98% |

#### Bin counts per sample

|  | Reference |  |  |  | Tested |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Context Bins | Bypass Bins | Raw Bins | Total Bins | Context Bins | Bypass Bins | Raw Bins | Total Bins |
| AI | 0.95 | 1.36 | 0.00 | 2.31 | 0.98 | 0.05 | 1.31 | 2.34 |
| RA | 0.74 | 0.94 | 0.00 | 1.68 | 0.76 | 0.04 | 0.90 | 1.70 |
| LB | 0.73 | 0.93 | 0.00 | 1.67 | 0.76 | 0.04 | 0.90 | 1.69 |

### AHG18 test conditions

#### BD-rate changes

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) | Intra 12-bit (internal bit depth) | Lowdelay 16-bit (internal bit depth) | Lowdelay 12-bit (internal bit depth) |
|  |  |  | Y/G | Y/G | Y/G | Y/G |
| SVT |  |  | 0.2% | 0.5% | 0.2% | 0.6% |
| RExt RGB |  |  | 0.3% | 0.7% | 0.4% | 0.8% |
| HDR |  |  | 0.5% | 0.9% | 0.6% | 0.8% |
| 4K from HDR |  |  | 0.3% | 0.7% |  |  |
| HDR 4:0:0 Medical |  |  | 0.6% | 0.9% | 0.7% | 0.9% |
| 4:0:0 Medical |  |  | 0.5% | 0.7% | 0.6% | 0.7% |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0.5% | 0.9% | 0.6% | 0.8% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.7% | 0.9% | 0.9% | 0.4% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.9% | 0.8% | 0.8% | 0.0% |
| All Classes |  |  | 0.5% | 0.8% | 0.6% | 0.6% |
| Encoding Time % |  |  | 100% | 100% | 100% | 100% |
| Decoding Time % |  |  | 90% | 96% | 93% | 98% |

#### Bin counts per sample

|  | Reference |  |  |  | Tested |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Context Bins | Bypass Bins | Raw Bins | Total Bins | Context Bins | Bypass Bins | Raw Bins | Total Bins |
| Intra 16-bit | 1.70 | 6.51 | 0.00 | 8.20 | 1.75 | 0.04 | 6.47 | 8.26 |
| Intra 12-bit | 1.20 | 2.53 | 0.00 | 3.72 | 1.23 | 0.05 | 2.48 | 3.76 |
| Lowdelay 16-bit | 1.57 | 4.95 | 0.00 | 6.52 | 1.62 | 0.03 | 4.92 | 6.57 |
| Lowdelay 12-bit | 0.91 | 1.57 | 0.00 | 2.47 | 0.94 | 0.03 | 1.53 | 2.50 |

### Coefficient-group worst case analysis

The following analysis examines the worst case encodings for coefficient data per coefficient group. The cases where a sub-group is not aligned and where a sub-group is aligned are listed separately.

|  | Base |  | SubTest B1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| GolombRiceGroupAdaptation | Enabled (4x4 TU) | Disabled | Enabled (4x4 TU) |  | Disabled |  |
| Total number of context bins | 25 |  | 25 |  |  |  |
| Total number of bypass bins with CABAC | 647 | 698 | 16 | 0 | 16 | 0 |
| Total number of bypass bins with CABAC (Raw) | 0 | 0 | 0 | 647 | 0 | 698 |

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).