| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0061 |
| --- | --- |

| Title: | AHG18: Worst-Case Escape Code Length Mitigation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd. |  |  |

_____________________________

# Abstract

This contribution identifies the worst-case code length that may occur when coding an escape code for a coefficient as 46 and asserts that that may be too heavy a burden on a decoder. The contribution proposes an alternative scheme to the exponential-Golomb part of escape coding that claims to reduce the worst-case escape code length to 32, which is the same as the worst-case escape code length in HEVC version 1. All-tier BD-rate changes of no more than 0.1% are reported for AHG18 common test conditions, as defined in RCE1.

# Introduction

In the current HEVC standard, escape codes (any part of a coefficient’s magnitude not described by a CABAC-coded syntax element) are coded using a two-part Golomb-Rice/exponential-Golomb method. When coding 16-bit video using extended precision, this method results in a maximum escape code length of 46 bits, which could be a considerable burden to decode for a single coefficient.

By modifying the mapping between the prefix and suffix lengths in the exponential-Golomb part of the escape coding scheme, the worst case can be reduced to 32 bits without significant adverse effect on coding efficiency or run time.

# Proposed Scheme

When coding 16-bit video using extended precision, the dynamic range of the transform and quantiser and therefore of coefficients entering the entropy coder (maxTrDynamicRange) is equal to 16 + 6 = 22. This results in a maximum coefficient magnitude of 222 = 4194304. When adjusted by subtraction of 1 for the significance map entry and 3 for the Golomb-Rice code (the Rice parameter rParam is 0 in the worst case), the maximum value that can be coded using exponential-Golomb coding is 4194300.

It can be shown that, to code this value using exponential-Golomb in HEVC, a 21-bit suffix (and therefore a 21-bit prefix) is required in addition to a one-bit separator and three extra prefix bits to signal that exponential-Golomb coding is in use. This results in a worst-case total code length of 46.

However, rather than simply map prefix length onto suffix length 1:1, an alternative scheme can be used when extended-precision processing is enabled that limits the number of prefix bits required to signal the worst case. Such a mapping is presented in Table 1:

Table 1 - Proposed mapping of suffix lengths to prefix lengths for extended-precision processing

Using this scheme, the fine-grained coding intervals for small values are maintained, while requiring no more than 7 prefix bits (rather than 21) to signal the largest possible suffix. In addition, since the maximum prefix length is known, the prefix can be coded using truncated-unary coding – omitting the separator if the maximum prefix length is reached.

The worst case escape code length is hence reduced to 7 (prefix) + 22 (maximum suffix) + 3 (not-a-Golomb-Rice-code bits) = 32, allowing any escape code to fit into a single 32-bit integer. This maximum escape code length is the same as that in HEVC version 1 (see JCTVC-J0142), which would have to be supported by a Range Extensions codec.

# Results

The proposed scheme has been simulated under the AHG18 common test conditions, as defined in RCE1. The results are presented in Table 2:

Table 2 - BD-rate changes and coding times vs. HM12.1_RExt5.1 anchor under AHG18 test conditions, as defined in RCE1

# Conclusion

An alternative scheme to exponential-Golomb for escape coding has been presented that reduces the worst case escape code length from 46 to 32, bringing the maximum length in line with that of HEVC version 1. All-tier BD-rate changes of no more than 0.1% are observed for common test conditions.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Working Draft Text

(Based on JCTVC-O1005)

- 9.3.3.X	Table-based binarization process

Input to this process is a request for a table-based binarization for a syntax element with unsigned value synVal, the Rice parameter riceParam and a flag isChroma indicating whether or not the current syntax element relates to a chroma coefficient.

Output of this process is the table-based binarization of the syntax element synVal.

The variable maxTrDynamicRange is derived as follows:

–	If isChroma is equal to 0,

maxTrDynamicRange = extended_precision_processing_flag ? Max( 15, BitDepthY + 6 ) : 15

–	Otherwise,

maxTrDynamicRange = extended_precision_processing_flag ? Max( 15, BitDepthC + 6 ) : 15

The variables suffixLength[x] and minimumUncodeableValue[x] are derived from table 9-XX:

Table 9-XX – Definition of the values suffixLength[x] and minimumUncodeableValue[x]

| x | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| suffixLength[x] | 0 | 1 | 2 | 3 | 4 | 6 | 10 | maxTrDynamicRange - riceParam |
| minimumUncodeableValue[x] | 1 | 3 | 7 | 15 | 31 | 95 | 1119 | - |

The bin string of the table-based binarization process of a syntax element synVal is specified as follows, where each call of the function put( X ), with X being equal to 0 or 1, adds the binary value X at the end of the bin string:

suffix = synVal >> riceParam
prefixLength = 0

while( (prefixLength < 7) && (suffix >= minimumUncodeableValue[prefixLength]) )
{
  prefixLength++
  put(1)
}

put((prefixLength == 7) ? 1 : 0)

if (prefixLength > 0)
{
  synVal –= minimumUncodeableValue[prefixLength – 1] << riceParam
}

totalSuffixLength = suffixLength[prefixLength] + riceParam
while ((totalSuffixLength – –) > 0)
{
  put((synVal >> totalSuffixLength) & 1)
}

#### 9.3.3.9	Binarization process for coeff_abs_level_remaining

…

The variable cMax is derived from cRiceParam as:

cMax = 43  <<  cRiceParam

The binarization of the syntax element coeff_abs_level_remaining[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of cu_qp_delta_abs, prefixVal, is derived as follows:

prefixVal = Min( cMax, coeff_abs_level_remaining[ n ] )

- The prefix bin string is specified by invoking the TR binarization process as specified in subclause 9.3.3.2 for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 43 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of cu_qp_delta_abs, suffixVal, is derived as follows:

suffixVal = coeff_abs_level_remaining[ n ] – cMax

- When extended_precision_processing_flag is equal to 1, the suffix bin string is specified by invoking the table-based binarization process as specified in subclause 9.3.3.X for suffixVal with the variable isChroma set equal to 1 if cIdx is greater than 0 and 0 otherwise.

- Otherwise (extended_precision_processing_flag is equal to 0), the suffix bin string is specified by invoking the EGk binarization process as specified in subclause 9.3.3.3 for suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.

The minimum negative value is -222 = -4194304; the maximum positive value is 222 – 1 = 4194303