| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0065 |
| --- | --- |

| Title: | Guided transcoding using auxiliary pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Yashas Rai, Thomas Rusert, Rickard Sjöberg, Jonatan Samuelsson Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

At the Geneva meeting it was shown in JCTVC-O0127 how to enable guided transcoding using SHVC. Many people found the approach interesting but there were concerns on low-level changes at this stage of the process. This time it is presented how to enable guided transcoding for both spatial and SNR scalability without any normative low-level changes using auxiliary pictures. With this approach the high-fidelity is represented in the base layer and the side information for generation of another resolution/fidelity is represented as an auxiliary picture. The results show gains compared to simulcast coding of 21.5%/26.2% for all intra, 15.1%/18.9%/13.2% for random access and 13.9%/17.6%/11.7% for low delay 2x/1.5x/SNR scalability for the common conditions. In comparison to traditional transcoding (re-encoding to another resolution/fidelity) guided transcoding significantly reduce the transcoding time to be comparable with decoding time and gives less loss in BDR performance compared to single layer coding. The only modification to the specification is to add a new auxiliary picture type AUX_TRANSCODING and an informative note on how to use a primary and auxiliary picture to enable guided transcoding.

# Background

Transcoding is an important feature to enable different resolutions/bit rates from a high-fidelity video bitstream. Transcoding is typically a complex process and gives losses in quality compared to direct coding to the desired resolution/bitrate. At the Geneva meeting it was proposed to significantly reduce transcoding complexity and also improve the coding efficiency by suggesting normative changes to SHVC [2]. This time we propose non-normative changes to SHVC still enabling systems to have the option to use guided transcoding but without giving any additional burden on mainstream SHVC decoders.

# Proposal

To significantly reduce transcoding complexity and also improve the coding efficiency we propose to enable SHVC to carry side information using auxiliary pictures that can be used to guide a transcoder. It is proposed to define a new auxiliary picture type to notify a decoder that the auxiliary layers contain side information that can be used for guided transcoding to a lower fidelity/resolution. By guided transcoding is meant that a transcoder does not need to do time consuming motion and mode selection but instead can be guided by the side information. The side information contains all mode and motion information but lacks the appropriate transform coefficients. A guided transcoder thus only needs to generate the transform coefficients and can then re-encode the side information together with the corrected transform coefficients to generate a transcoded bitstream. To obtain the transform coefficients it use the reconstructed primary picture, if needed down-sampled to the resolution of the auxiliary picture, as the “original” picture and apply a forward transform (unless transform skip) and scaling on the difference between the reconstructed primary picture and auxiliary picture predictions. The transform coefficients need only to be derived for blocks in the auxiliary picture that have non-zero CBF.

# Text modifications

The changes to the text [1]is high-lighted in yellow below.

AuxId[ lId ] equal to 0 specifies the layer with nuh_layer_id equal to lId does not contain auxiliary pictures. AuxId[ lId ] greater than 0 specifies the type of auxiliary pictures in layer with nuh_layer_id equal to lId as specified in Error: Reference source not found.

Table Error: Reference source not found-2 – Mapping of AuxId to the type of auxiliary pictures

| AuxId | Name of AuxId | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3 | AUX_TRANSCODING | Side information intended for guided transcoding to another fidelity/resolution |
| 4-127 |  | Reserved |
| 128-143 |  | Unspecified |
| 144-255 |  | Reserved |

AuxId[ lId ] shall be in the range of 0 to 3, inclusive, or 128 to 143, inclusive, for bitstreams conforming to this version of this Specification. Although the value of AuxId[ lId ] shall be in the range of 0 to 3, inclusive, or 128 to 143, inclusive, in this version of this Specification, decoders shall allow values of AuxId[ lId ] in the range of 0 to 255, inclusive.

For an auxiliary picture with nuh_layer_id equal to nuhLayerIdA, an associated primary picture, if any, is the picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ nuhLayerIdA ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

It is a requirement of bitstream conformance that there shall be an associated primary picture for each auxiliary picture with AuxId[ nuh_layer_id ] equal to AUX_ALPHA or AUX_TRANSCODING.

NOTE 1 – It is not required that each auxiliary picture of each auxiliary picture type has an associated primary picture. For example, a layer with AuxId[ nuh_layer_id ] equal to AUX_DEPTH may represent a viewpoint of a range sensing camera, while the layers containing primary pictures may represent conventional cameras.

..

It is a requirement of bitstream conformance that AuxId[ RefLayerId[ nuhLayerIdA ][ j ] ] for any values of nuhLayerIdA and j shall be equal to AuxId[ nuhLayerIdA ], when AuxId[ nuhLayerIdA ] is in the range of 0 to 2, inclusive.

NOTE 2 – In other words, no prediction takes place between layers with a different value of AuxId, when AuxId is in the range of 0 to 2, inclusive.

NOTE 6 – When AuxId[nuh_layer_id] is equal to AUX_TRANSCODING a guided transcoder is supposed to generate transcoded transform coefficients to be used instead of the TransCoeffLevel parsed in the auxiliary picture before Scaling and transformation in H.8.6. Transform coefficients can be obtained by application of the transpose of the defined transform matrix in 8-277 (omitted if the  transform_skip_flag is active) and forward scaling on the pixel value difference between the reconstructed pixel values of the primary picture, possibly down-sampled, and the prediction obtained as in H.8.4 and H.8.5. Down-sampling can be performed using the re-sampling approach in H.8.1.4 but using the primary picture as the picture to re-sample and as output deriving those pixel values needed to derive the pixel value difference when generating TransCoefLevel. Instead of filter coefficients defined in Table H-1 down-sampling filter coefficients is preferably used.

# Results

In Table 2 the BDR performance for guided transcoding versus simulcast for common conditions is shown and in Table 3 guided transcoded and traditional transcoding is compared with single layer coding of the low resolution/fidelity.

Table 1 Guided transcoding compared to simulcast using HM-12.0

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -22,9% | -22,9% | -22,9% |  |  |  |
| Class B | -20,9% | -21,1% | -21,1% | -26,2% | -26,4% | -26,5% |
| Overall (Test vs Ref) | -21,5% | -21,6% | -21,6% | -26,2% | -26,4% | -26,5% |
| Overall (Test vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Ref vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| EL only (Test vs Ref) | -69,2% | -66,5% | -66,2% | -66,1% | -63,9% | -64,0% |
| Enc Time[%] | 57,8% |  |  | 55,3% |  |  |
| Dec Time[%] | 92,1% |  |  | 91,7% |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -15,8% | -16,0% | -15,9% |  |  |  |
| Class B | -14,8% | -15,0% | -15,1% | -18,9% | -19,2% | -19,3% |
| Overall (Test vs Ref) | -15,1% | -15,3% | -15,3% | -18,9% | -19,2% | -19,3% |
| Overall (Test vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Ref vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| EL only (Test vs Ref) | -47,4% | -38,0% | -38,1% | -44,5% | -36,0% | -36,6% |
| Enc Time[%] | 66,7% |  |  | 59,4% |  |  |
| Dec Time[%] | 110,6% |  |  | 101,1% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -14,0% | -14,5% | -14,4% |  |  |  |
| Class B | -13,8% | -14,2% | -14,3% | -17,6% | -18,1% | -18,2% |
| Overall (Test vs Ref) | -13,9% | -14,3% | -14,4% | -17,6% | -18,1% | -18,2% |
| Overall (Test vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Ref vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| EL only (Test vs Ref) | -42,0% | -33,5% | -33,3% | -39,1% | -30,6% | -31,1% |
| Enc Time[%] | 68,2% |  |  | 57,7% |  |  |
| Dec Time[%] | 110,9% |  |  | 103,7% |  |  |
|  | RA HEVC SNR |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -12,2% | -12,3% | -12,2% | -10,3% | -10,5% | -10,4% |
| Class B | -13,6% | -13,7% | -13,8% | -12,3% | -12,5% | -12,5% |
| Overall (Test vs Ref) | -13,2% | -13,3% | -13,3% | -11,7% | -11,9% | -11,9% |
| Overall (Test vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Ref vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| EL only (Test vs Ref) | -38,3% | -32,9% | -32,8% | -34,9% | -29,7% | -29,3% |
| Enc Time[%] | 59,1% |  |  | 60,2% |  |  |
| Dec Time[%] | 97,2% |  |  | 101,4% |  |  |

Table 2 Performance for guided transcoding and traditional transcoding versus HM-12.0 for the low resolution/fideility.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Proposal | 11,8% | 24,2% | 25,2% | 17,8% | 27,7% | 28,0% |
| Enc Time[%] | 5,0% |  |  | 3,9% |  |  |
| Dec Time[%] | 88,1% |  |  | 95,0% |  |  |
| Traditional transcoding | 8,9% | 29,4% | 29,4% | 15,9% | 30,8% | 30,9% |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Proposal | 10,9% | 34,0% | 33,8% | 16,1% | 37,0% | 36,1% |
| Enc Time[%] | 1,2% |  |  | 0,8% |  |  |
| Dec Time[%] | 98,8% |  |  | 96,9% |  |  |
| Traditional transcoding | 10,5% | 45,3% | 44,0% | 19,8% | 47,2% | 46,5% |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Proposal | 10,9% | 31,2% | 31,3% | 17,1% | 37,2% | 36,2% |
| Enc Time[%] | 0,8% |  |  | 0,5% |  |  |
| Dec Time[%] | 94,6% |  |  | 97,1% |  |  |
| Traditional transcoding | 13,0% | 45,7% | 45,4% | 24,1% | 53,2% | 54,2% |
|  | RA HEVC SNR |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V |
| Proposal | 10,3% | 23,2% | 24,1% | 9,0% | 21,3% | 22,0% |
| Enc Time[%] | 0,5% |  |  | 0,4% |  |  |
| Dec Time[%] | 99,9% |  |  | 103,4% |  |  |
| Traditional transcoding | 12,2% | 27,4% | 27,9% | 13,0% | 28,0% | 29,9% |

# Conclusions

We have shown that auxiliary pictures can be used to enable guided transcoding with significantly less computational complexity compared to traditional transcoding and also with less loss in BDR compared to single layer coding.

We propose that this new functionality is adopted to the SHVC draft.

# References

- “High efficiency video coding (HEVC) scalable extension draft 4”,  JCTVC-O1008_v3, Geneva, November, 2013

- K.Andersson, T. Russert, R. Sjöberg, J. Samuelsson, “Transcoder-friendly scalable coding”, JCTVC-O0127, Geneva, November, 2013

- V. Seregin, Y. He, “Common SHM test conditions and software reference configurations”, JCTVC-O1009, Geneva, November, 2013

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).