| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0066 |
| --- | --- |

| Title: | AHG5: Luma-chroma prediction for different bitdepth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony corporation |  |  |

_____________________________

# Abstract

It is proposed to modify the luma-chroma prediction process when BitDepthY and BitDepthC are different.

Two solutions are studied; 1) to scale the luma residual to align the chroma bitdepth, 2) to prohibit the luma-chroma prediction when the luma and chroma bitdepth are different.

The proposals are tested on HM12.1+RExt-5.1 software. The source code for the first solution is attached in the contribution. It is reported that BDrate differences for the first solution with 12bit luma and 8bit chroma setting for YUV444 test set are -1.4/-5.7/-5.8, -0.6/-7.1/-6.1, and -0.4/-4.6/-3.9 for Y/U/V in AI, RA and LB main-tier, respectively. Likewise, it is reported that the BDrate with 8bit luma and 12bit chroma setting are -1.3/-3.6/-4.4, -0.6/-5.3/-5.2, and -0.5/-4.2/-4.5, respectively.

From the experimental results, it is pointed that the current process is not optimal when the bitdepth are different. Thus, it is recommended to fix the issue with the first solution.

# Background

It is pointed that the current residual modification process for transform blocks using luma-chroma prediction in [1][2] has an issue when BitDepthY and BitDepthC are different. Since the process does not consider the bitdepth difference between luma and chroma, coding performance is not optimal in that case.

# Proposal

To fix the issue, two alternative solutions are proposed.

## Solution1

It is proposed to scale the luma residual to align the chroma bitdepth by right or left shift operation.

It is asserted that the proposed change does not increase the complexity significantly since the current luma-chroma prediction process is conducted by right and left shift operations and the proposed change is a modification of the shift operation.

Proposed text change is provided as yellow highlight on top of JCTVC-O1005_v2.

### 8.6.6 Residual modification process for transform blocks using luma-chroma prediction

This process is only invoked when ChromaArrayType is equal to 3.

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block,

–	an (nTbS)x(nTbS) array of luma residual samples rY with elements rY[ x ][ y ],

–	an (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

Output of this process is the modified (nTbS)x(nTbS) array r of residual samples.

The (nTbS)x(nTbS) array of residual samples r with  x = 0..nTbS − 1, y = 0..nTbS − 1  is modified as follows:

If BitDepthY is larger than or equal to BitDepthC,

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] * ( rY[ x ][ y ] >> (BitDepthY − BitDepthC) ) ) >> 3

Otherwise ( BitDepthY is less than BitDepthC),

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] * (rY[ x ][ y ] << (BitDepthC − BitDepthY) ) ) >> 3

## Solution2

Alternatively, it is proposed to prohibit the luma-chroma prediction as a bitstream restriction when the bitdepth are different. Proposed text change is provided as yellow highlight on top of JCTVC-O1005_v2.

luma_chroma_prediction_enabled_flag equal to 1 specifies that log2_res_scale_abs_plus1 and res_scale_sign_flag may be present in the transform unit syntax for pictures referring to the PPS. luma_chroma_prediction_enabled_flag equal to 0 specifies that log2_res_scale_abs_plus1 and res_scale_sign_flag are not present for pictures referring to the PPS. When not present, the value of luma_chroma_prediction_enabled_flag is inferred to be equal to 0. When ChromaArrayType is not equal to 3, it is a requirement of bitstream conformance that the value of luma_chroma_prediction_enabled_flag shall be equal to 0. When BitDepthY is not equal to BitDepthC, it is a requirement of bitstream conformance that the value of luma_chroma_prediction_enabled_flag shall be equal to 0.

# Experimental result

Coding performance of the proposals is studied.

Original HM-12.1+RExt-5.1 software is used as anchor for comparison. It is noted that the anchor software works with unequal luma and chroma InternalBitdepth setting even when CrossComponentDecorrelation is set equal to 1.

Proposed change for Solution1 is implemented on top of the anchor software. The source code is attached in the contribution.

For Solution2, it is not necessary to change the code since input configuration parameter CrossComponentDecorrelation supports the functionality.

Table 1 shows the anchor and four tested conditions. Other settings are same as RExt CTC [3]. YUV444 sequence set is studied.

In order to study the scaling effect in Solution1, the original and modified software are compared in Test1 and Test2 with the identical configuration setting.

In order to study the performance of the current process, enabling and disabling of CrossComponentDecorrelation are compared in Test3 and Test4 using the original software for both anchor and tested.

It is noted that SAO is disabled in Test2 and Test4 due to the original software problem. The encoder crashes if InternalBitDepth < InternalBitDepthC and SAO is enabled. This is reported as Ticket #1219 in the JCT-VC trac as a separate issue.

Table2 to 5 show the experimental results. It is noted that the runtime may not be accurate due to the simulation environment.

Table 1 Test conditions

|  | CrossComponent-Decorrelation | InternalBitDepth | InternalBitDepthC | SAO | Software | Corresponding proposal |
| --- | --- | --- | --- | --- | --- | --- |
| Anchor | 1 | Same as TestX | Same as TestX | Same as TestX | RExt5.1 | - |
| Test1 | 1 | 12 | 8 | 1 | RExt5.1+change | Solution1 |
| Test2 | 1 | 8 | 12 | 0 | RExt5.1+change | Solution1 |
| Test3 | 0 | 12 | 8 | 1 | RExt5.1 | Solution2 |
| Test4 | 0 | 8 | 12 | 0 | RExt5.1 | Solution2 |

## Test1 result (Soltion1 when Luma bitdepth is larger than Chroma bitdepth)

Table 2 YUV444 result for Test1

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.7% | -1.6% | -2.9% | -0.4% | -0.8% | -2.3% | -0.4% | -0.6% | -1.4% |
| Kimono1_1920x1080_24_10bit_444.yuv | -1.7% | -2.2% | -2.1% | -0.9% | -0.9% | -1.0% | -0.3% | -0.7% | -0.3% |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -1.1% | -14.7% | -3.6% | -1.4% | -6.3% | -2.4% | -0.6% | -1.6% | -1.2% |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -1.5% | -7.4% | -17.4% | -2.3% | -6.5% | -15.0% | -3.0% | -5.0% | -10.6% |
| VenueVu_1920x1080_30_10bit_444.yuv | -2.9% | -7.2% | -5.0% | -3.1% | -5.9% | -4.5% | -2.9% | -4.3% | -3.5% |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -1.2% | -1.8% | -6.1% | -1.5% | -1.8% | -7.4% | -2.2% | -2.3% | -3.2% |
| CrowdRun_1920x1080_50_10bit_444.yuv | -1.0% | -5.2% | -3.6% | -1.5% | -2.9% | -2.8% | -1.3% | -1.8% | -1.9% |
| Overall | -1.4% | -5.7% | -5.8% | -1.6% | -3.6% | -5.1% | -1.5% | -2.3% | -3.2% |
| Enc Time[%] | 110% |  |  | 110% |  |  | 109% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 97% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | 0.0% | -1.9% | -3.7% | 0.0% | -0.8% | -3.4% |  |  |  |
| Kimono1_1920x1080_24_10bit_444.yuv | -0.8% | -5.2% | -1.7% | -0.7% | -1.3% | -1.2% |  |  |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -0.4% | -18.9% | -3.3% | -0.5% | -13.1% | -2.3% |  |  |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.5% | -8.5% | -18.5% | -0.8% | -8.1% | -17.3% |  |  |  |
| VenueVu_1920x1080_30_10bit_444.yuv | -1.5% | -7.2% | -4.3% | -1.8% | -6.8% | -4.1% |  |  |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -0.8% | -1.4% | -7.9% | -1.6% | -0.3% | -10.1% |  |  |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | -0.1% | -6.2% | -3.3% | -0.4% | -3.1% | -2.8% |  |  |  |
| Overall | -0.6% | -7.1% | -6.1% | -0.8% | -4.8% | -5.9% |  |  |  |
| Enc Time[%] | 106% |  |  | 109% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | 0.0% | -1.2% | -2.4% | 0.0% | -0.4% | -2.4% |  |  |  |
| Kimono1_1920x1080_24_10bit_444.yuv | -0.6% | -4.0% | -1.3% | -0.7% | -1.1% | -1.0% |  |  |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 0.0% | -13.6% | -1.5% | -0.2% | -8.5% | -1.2% |  |  |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.2% | -3.0% | -9.4% | -0.3% | -2.8% | -9.6% |  |  |  |
| VenueVu_1920x1080_30_10bit_444.yuv | -0.9% | -6.4% | -3.0% | -1.2% | -5.7% | -3.0% |  |  |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -0.8% | -0.8% | -8.1% | -1.5% | -0.6% | -11.4% |  |  |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | 0.0% | -3.3% | -2.0% | -0.3% | -1.9% | -1.7% |  |  |  |
| Overall | -0.4% | -4.6% | -3.9% | -0.6% | -3.0% | -4.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |  |  |  |

## Test2 result (Solution1 when Chroma bitdepth is higher than Luma bitdepth)

Table 3 YUV444 result for Test2

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.7% | -1.4% | -1.9% | -0.5% | -0.8% | -1.5% | -0.5% | -0.6% | -1.0% |
| Kimono1_1920x1080_24_10bit_444.yuv | -1.5% | -1.7% | -1.7% | -0.9% | -0.7% | -0.9% | -0.4% | -0.5% | -0.3% |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -1.2% | -9.0% | -3.0% | -1.3% | -4.0% | -1.9% | -0.7% | -1.2% | -0.6% |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -1.2% | -2.3% | -12.4% | -1.7% | -2.8% | -11.2% | -2.0% | -3.0% | -9.3% |
| VenueVu_1920x1080_30_10bit_444.yuv | -2.5% | -5.9% | -4.1% | -2.7% | -5.2% | -3.9% | -2.8% | -4.4% | -3.4% |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -1.2% | -1.5% | -5.4% | -1.3% | -1.3% | -7.5% | -2.2% | -2.0% | -4.2% |
| CrowdRun_1920x1080_50_10bit_444.yuv | -0.8% | -3.6% | -2.2% | -1.1% | -2.0% | -1.8% | -1.0% | -1.2% | -1.3% |
| Overall | -1.3% | -3.6% | -4.4% | -1.4% | -2.4% | -4.1% | -1.4% | -1.9% | -2.9% |
| Enc Time[%] | 98% |  |  | 96% |  |  | 95% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.2% | -1.5% | -2.4% | -0.2% | -0.7% | -2.5% |  |  |  |
| Kimono1_1920x1080_24_10bit_444.yuv | -0.9% | -4.9% | -1.6% | -0.8% | -1.2% | -1.3% |  |  |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -0.5% | -12.6% | -3.5% | -0.6% | -10.5% | -2.8% |  |  |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.4% | -4.3% | -15.5% | -0.6% | -5.1% | -15.3% |  |  |  |
| VenueVu_1920x1080_30_10bit_444.yuv | -1.5% | -6.5% | -3.8% | -1.7% | -6.4% | -3.8% |  |  |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -0.8% | -1.8% | -7.4% | -1.1% | -0.9% | -10.7% |  |  |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | -0.2% | -5.3% | -2.4% | -0.5% | -2.8% | -2.3% |  |  |  |
| Overall | -0.6% | -5.3% | -5.2% | -0.8% | -4.0% | -5.5% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.1% | -1.4% | -2.2% | -0.2% | -0.7% | -2.5% |  |  |  |
| Kimono1_1920x1080_24_10bit_444.yuv | -0.9% | -4.3% | -1.6% | -1.1% | -1.1% | -1.3% |  |  |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 0.0% | -10.0% | -1.9% | -0.3% | -8.1% | -2.2% |  |  |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.1% | -3.4% | -11.4% | -0.2% | -3.6% | -12.1% |  |  |  |
| VenueVu_1920x1080_30_10bit_444.yuv | -0.9% | -6.4% | -3.2% | -1.3% | -6.1% | -3.2% |  |  |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -1.4% | -0.7% | -9.1% | -1.5% | -0.3% | -12.8% |  |  |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | -0.1% | -3.3% | -1.7% | -0.4% | -1.9% | -1.6% |  |  |  |
| Overall | -0.5% | -4.2% | -4.5% | -0.7% | -3.1% | -5.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |  |  |  |

## Test3 result (Soltion2 when Luma bitdepth is larger than Chroma bitdepth)

Table 4 YUV444 result for Test3

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.1% | -0.1% | 0.2% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_444.yuv | -0.1% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | -0.2% |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -0.2% | 0.0% | 0.2% | -0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| VenueVu_1920x1080_30_10bit_444.yuv | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | -0.2% | 0.0% | 0.1% | -0.2% |
| CrowdRun_1920x1080_50_10bit_444.yuv | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% |
| Overall | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 97% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  | 96% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.1% | -0.2% | 0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| Kimono1_1920x1080_24_10bit_444.yuv | -0.2% | 0.1% | 0.1% | -0.1% | 0.1% | 0.1% |  |  |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | 0.2% |  |  |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.2% | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% |  |  |  |
| VenueVu_1920x1080_30_10bit_444.yuv | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |  |  |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |  |  |  |
| Overall | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 103% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.3% | -0.1% | -0.2% | -0.1% | 0.0% | 0.1% |  |  |  |
| Kimono1_1920x1080_24_10bit_444.yuv | -0.3% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |  |  |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -0.2% | 0.5% | 0.0% | -0.2% | -0.1% | -0.3% |  |  |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% |  |  |  |
| VenueVu_1920x1080_30_10bit_444.yuv | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% |  |  |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -1.0% | 0.0% | 0.0% | -0.6% | 0.0% | -0.1% |  |  |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Overall | -0.3% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 102% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 105% |  |  |  |  |  |

## Test4 result (Soltion2 when Chroma bitdepth is larger than Luma bitdepth)

Table 5 YUV444 result for Test4

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.2% | 0.2% | 1.7% | -0.1% | 0.1% | 1.6% | 0.0% | 0.2% | 1.1% |
| Kimono1_1920x1080_24_10bit_444.yuv | 0.3% | 1.2% | 1.1% | 0.3% | 0.5% | 0.6% | 0.2% | 0.5% | 0.1% |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -0.2% | 10.8% | 2.7% | 0.3% | 5.5% | 3.2% | 0.3% | 2.0% | 3.0% |
| EBURainFruits_1920x1080_50_10bit_444.yuv | 0.2% | 7.0% | 8.0% | 0.7% | 5.5% | 7.5% | 1.3% | 3.8% | 6.3% |
| VenueVu_1920x1080_30_10bit_444.yuv | 0.4% | 2.1% | 1.7% | 0.5% | 2.1% | 1.7% | 0.6% | 2.0% | 1.5% |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -0.2% | 0.4% | 2.5% | 0.2% | 0.7% | 1.9% | 0.4% | 0.8% | 0.4% |
| CrowdRun_1920x1080_50_10bit_444.yuv | 0.2% | 2.0% | 2.0% | 0.4% | 1.3% | 1.5% | 0.3% | 1.0% | 1.1% |
| Overall | 0.1% | 3.4% | 2.8% | 0.3% | 2.2% | 2.6% | 0.4% | 1.5% | 1.9% |
| Enc Time[%] | 88% |  |  | 87% |  |  | 85% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.3% | 0.5% | 2.2% | -0.3% | 0.1% | 2.1% |  |  |  |
| Kimono1_1920x1080_24_10bit_444.yuv | -0.1% | 2.3% | 0.8% | 0.0% | 0.6% | 0.5% |  |  |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -0.4% | 14.3% | 2.4% | -0.1% | 12.6% | 3.0% |  |  |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.1% | 6.5% | 5.7% | 0.0% | 5.5% | 5.3% |  |  |  |
| VenueVu_1920x1080_30_10bit_444.yuv | 0.0% | 1.4% | 1.0% | 0.2% | 1.6% | 1.1% |  |  |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -0.6% | 0.7% | 3.1% | 0.0% | 0.6% | 3.4% |  |  |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | -0.2% | 1.7% | 1.4% | -0.1% | 0.8% | 1.0% |  |  |  |
| Overall | -0.2% | 3.9% | 2.4% | -0.1% | 3.1% | 2.3% |  |  |  |
| Enc Time[%] | 91% |  |  | 92% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.1% | 0.1% | 1.1% | -0.1% | 0.0% | 1.1% |  |  |  |
| Kimono1_1920x1080_24_10bit_444.yuv | -0.3% | 1.2% | 0.2% | -0.2% | 0.3% | 0.2% |  |  |  |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -0.5% | 10.9% | 1.4% | -0.3% | 8.3% | 1.2% |  |  |  |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.2% | 1.0% | 0.9% | -0.2% | 0.6% | 0.8% |  |  |  |
| VenueVu_1920x1080_30_10bit_444.yuv | -0.2% | 0.8% | 0.2% | -0.1% | 0.9% | 0.3% |  |  |  |
| BirdsInCage_1920x1080_60_10bit_444.yuv | -1.0% | 0.1% | 2.3% | -0.1% | 0.2% | 2.6% |  |  |  |
| CrowdRun_1920x1080_50_10bit_444.yuv | -0.2% | 0.4% | 0.5% | -0.1% | 0.2% | 0.4% |  |  |  |
| Overall | -0.4% | 2.1% | 0.9% | -0.2% | 1.5% | 0.9% |  |  |  |
| Enc Time[%] | 95% |  |  | 93% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |

## Discussion

From Test3 and Test4 results, it is reported that the current luma-chroma prediction process is not optimal when luma and chroma bitdepth are different because expected coding performance stated in [1] is not observed. It is noted that negative values in Table4 and 5 shows the overhead of the current process because CCD is enabled in the anchor. Some positive values in Table5 imply that the large ResScaleVal value (e.g.8 or -8) works as a scaling factor which is corresponding to the bitdepth difference equal to 4 (=C12-Y8).

From Test1 and Test2 results, it is confirmed that proposed scaling scheme enables the luma-chroma prediction process to correctly work in these settings.

# Conclusion

From the results in Section3, it is encouraged to modify the current luma-chroma residual prediction process.

In terms of the spec generality, proponents believe Solution1 is a straightforward approach to fix the issue.

# Reference

- JCTVC-O0202 “RCE1: Descriptions and Results for Experiments 1, 2, 3, and 4”

- JCTVC-O1005 “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 5”

- JCTVC-O1006 “Common test conditions and software reference configurations for HEVC range extensions”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).