| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0067 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0045 |
| --- | --- |

| Title: | MV-HEVC/SHEVC HLS: Comments on POC alignment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming Li Ping Wu Guoqiang SHANG Yutang XIE No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | +86-25-88014644 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

Proposed is a design for signalling and deriving picture order count (POC) in SHEVC and MV-HEVC for POC alignment. In enhancement layer (EL) slice headers, when nuh_layer_id is greater than 0 and POC alignment flag is set to 1, the value of the most significant bit (MSB) for POC calculation is explicitly signalled, and the value of least significant bit (LSB) is conveyed by slice_pic_order_cnt_lsb. Therefore, the proposed design only introduces additional bits to slice headers of enhancement layer (EL) pictures without any changes to base layer, and makes the POC values for both BL and EL pictures unique and static. The encoder sets the MSB and LSB values in EL slice headers so that the decoded POC value of the EL picture is equal to the POC value of the existing or hypothetically existing base layer (BL) picture in the same access unit (AU), which also facilitates the POC alignment for hybrid scalability cases. Furthermore, as full POC is signalled, this design can also be applied to the pictures, which do not need POC alignment, to improve error resilience performance for cases of possible picture loss.

# Introduction

To support layer-wise decoding, cross-layer POC alignment was adopted into MV-HEVC and SHEVC to make the POC values of all the pictures aligned in the access unit containing at least one IRAP picture with NoRaslOutputFlag equal to 1. Based on the discussions on POC alignment, a simplified method is proposed. In EL slice headers, when nuh_layer_id is greater than 0 and POC alignment flag is set to 1, the value of the MSB for POC calculation is explicitly signalled, and the value of LSB is conveyed by slice_pic_order_cnt_lsb. It is asserted that this feature brings the following advantages to the proposed method:

- The POC derivation process for the pictures where POC alignment is performed has no dependence on any previous information from BL and ELs.

- Only one set of POC is maintained for BL and EL pictures, and the POC value for a picture is unique and static.

- The proposed method does not present restrictions on coding structures.

- All the added bits are leaded by the condition of if(nal_unit_type > 0), and no reserved bits are inserted into BL bit-stream or byte-based extension part of EL slice header.

- The decoder can always get the exact POC value of an EL picture before parsing RPS rather than waiting until finishing the slice header extension for some EL pictures.

- No complicated and special MSB derivation process is needed for the pictures where POC alignment is performed, and the decoder does not need to trace various MSBs of the reference layers for possible coming POC alignment operations.

- From the perspective of POC derivation, the proposed method is robust to picture loss.

- The proposed method can be used to do “POC recovery” for any EL picture to improve the error resilience performance, and the said recovery operations are identical to those of ordinary POC alignment.

- The proposed method can be used for hybrid scalability.

# Proposed syntax and semantics

The modified text based on JCTVC-O1008 is as follow.

- In slice segment header:

- Table 1.  Modified slice segment header

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| i = 0 |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| discardable_flag | u(1) |
| } |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| cross_layer_bla_flag | u(1) |
| } |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| poc_reset_flag | u(1) |
| } |  |
| for( i = 1; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if ( nuh_layer_id > 0 ) { |  |
| poc_alignment_flag | u(1) |
| if ( poc_alignment_flag ) { |  |
| set_poc_msb_to_zero_flag | u(1) |
| if ( ! set_poc_msb_to_zero_flag ) { |  |
| bit_length_aligned_poc_msb_minus1 | ue(v) |
| aligned_poc_msb_value | u(v) |
| } |  |
| } |  |
| } |  |
| if( nuh_layer_id > 0 ¦ ¦ ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| …… |  |
| } |  |

poc_alignment_flag equal to 1 specifies that set_poc_msb_to_zero_flag is present in the bit-stream. poc_alignment_flag equal to 0 specifies that set_poc_msb_to_zero_flag is not present in the bit-stream. When not present, the value of poc_alignment_flag is inferred to be 0.

set_poc_msb_to_zero_flag equal to 1 specifies that the value of PicOrderCntMsb in the layer with nuh_layer_id is set equal to 0. set_poc_msb_to_zero_flag equal to 0 specifies that the value of PicOrderCntMsb in the layer with nuh_layer_id may or may not be equal to 0. When not present, the value of set_poc_msb_to_zero_flag is inferred to be 0.

bit_length_aligned_poc_msb_minus1 plus 1 specifies the number of bits used to representing aligned_poc_msb_value.

aligned_poc_msb_value is used in the derivation of the value of PicOrderCntMsb when set_poc_msb_to_zero_flag equal to 1. The number of bits used to representing aligned_poc_msb_value is equal to the value of (bit_length_aligned_poc_msb_minus1 + 1). PicOrderCntMsb is set equal to (aligned_poc_msb_value << (log2_max_pic_order_cnt_lsb_minus4 + 4) ).

- In slice decoding process:

- F.8.3.1  Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in merge mode and motion vector prediction, and for decoder conformance checking (see subclause C.5).

Each coded picture is associated with a picture order count variable, denoted as PicOrderCntVal.

- If the picture is a picture with poc_alignment_flag equal to 1, PicOrderCntVal is derived as follow.

- The value of PicOrderCntMsb is derived as follow.

  - If set_poc_msb_to_zero_flag is equal to 1, PicOrderCntMsb is set equal to 0.

  - Otherwise, PicOrderCntMsb is set equal to (aligned_poc_msb_value << (log2_max_pic_order_cnt_lsb_minus4 + 4) ).

- PicOrderCntVal is set equal to (PicOrderCntMsb + slice_pic_order_cnt_lsb).

- Otherwise (poc_alignment_flag equal to 0), when the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows:

- Let prevTid0Pic be the previous picture in decoding order that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture, and let prevPicOrderCnt be equal to PicOrderCntVal of prevTid0Pic.

- The variable prevPicOrderCntLsb is set equal to prevPicOrderCnt & ( MaxPicOrderCntLsb − 1 ).

- The variable prevPicOrderCntMsb is set equal to prevPicOrderCnt − prevPicOrderCntLsb.

- The variable PicOrderCntMsb of the current picture is derived as follows:

- If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, PicOrderCntMsb is set equal to 0.

- Otherwise, PicOrderCntMsb is derived as follows:

if( ( slice_pic_order_cnt_lsb < prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − slice_pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(Error: Reference source not found-1)
else if( (slice_pic_order_cnt_lsb > prevPicOrderCntLsb )  &&
		( (slice_pic_order_cnt_lsb − prevPicOrderCntLsb ) > ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as follows:

PicOrderCntVal = PicOrderCntMsb + slice_pic_order_cnt_lsb	(Error: Reference source not found-2)

NOTE 1 – All IDR pictures with nuh_layer_id equal to 0 will have PicOrderCntVal equal to 0 since slice_pic_order_cnt_lsb is inferred to be 0 for IDR pictures with nuh_layer_id equal to 0 and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

NOTE 2 – The IDR pictures with nuh_layer_id greater than 0 may have PicOrderCntVal not equal to 0.

When poc_reset_flag is equal to 1, the following steps apply in the order listed:

- The PicOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture is decremented by PicOrderCntVal.

- PicOrderCntVal is set equal to 0.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive. In one CVS, the PicOrderCntVal values for any two coded pictures in the same layer shall not be the same.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(Error: Reference source not found-3)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(Error: Reference source not found-4)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that are not in the range of −215 to 215 − 1, inclusive.

NOTE 2 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

# Conclusions

This document presents a simple design for signalling and deriving POC in SHEVC and MV-HEVC for POC alignment. In EL slice headers, when nuh_layer_id is greater than 0 and POC alignment flag is set to 1, the value of the MSB for POC calculation is explicitly signalled, and the value of LSB is conveyed by slice_pic_order_cnt_lsb. It is recommended to adopt this method into SHEVC and MV-HEVC.

Furthermore, it is suggested introducing some high-level flags to perform ON/OFF control for POC alignment operations in parameter set(s).

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).