| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0070 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0087 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On video parameter set extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Video parameter set (VPS) is extended in parallel for supporting a particular extension for each profile. The existing syntax elements in the current VPS extension are present in the vps_multilayer_common_extension, while the syntax elements for 3D-HEVC are presented in the vps_3d_extension. Each extension of VPS can be independently included or excluded in video_parameter_set_rbsp, similarly to SPS extension.

# Parallel extension of VPS

Even if the current VPS extension have been design for the general multi-layered extension of HEVC by containing common syntax elements, some HEVC extensions may need particular syntax elements other than the common ones, e.g. 3D-HEVC. Therefore, parallel extension of VPS is proposed similarly to SPS extension to support a particular extension for each profile. For parallel extension, vps_extension_type_flag[ i ] is introduced, which indicates whether syntax elements related to the i-th VPS extension are present in the video_parameter_set_rbsp. The existing syntax elements in the current VPS extension are present in the vps_multilayer_common_extension, when vps_extension_type_flag[ 1 ] is equal to 1. Whereas, the syntax elements for 3D-HEVC are presented in the vps_3d_extension when vps_extension_type_flag[ 2 ] is equal to 1. Considering combinations of multiple scalability dimensions having particular syntax elements, parallel extension is more efficient than serial extension.

# Proposed specification text

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) { |  |
| for ( i = 0; i < 8; i++ ) |  |
| vps_extension_type_flag[ i ] | u(1) |
| if( vps_extension_type_flag[ 1 ] ) |  |
| vps_multilayer_common_extension( ) |  |
| if( vps_extension_type_flag[ 2 ] ) |  |
| vps_3d_extension( ) |  |
| if( vps_extension_type_flag[ 7 ] ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_extension_flag equal to 1 specifies that vps_extension_type_flag[ i ] for i in the range of 0 to 7, inclusive are present in the SPS RBSP syntax structure. vps_extension_flag equal to 0 specifies that vps_extension_flag[ i ] for i in the range of 0 to 7, inclusive are not present in the vPS RBSP syntax structure.

vps_extension_type_flag[ i ] shall be equal to 0,for i equal to 0 and in the range of 2 to 6, inclusive, in bitstreams conforming to this version of this Specification. The value of 1 for vps_extension_type_flag[ i ],for i equal to 0 and in the range of 2 to 6, inclusive, is reserved for future use by ITU-T | ISO/IEC. vps_extension_type_flag[ 1 ] equal to 1 specifies that the vps_multilayer_extension syntax structure is present. vps_extension_type_flag[ 1 ] equal to 0 specifies that the vps_multilayer_extension syntax structure is not present. vps_extension_type_flag[ 7 ] equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the VPS RBSP syntax structure. vps_extension_type_flag[ 7 ] shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for vps_extension_type_flag[ 7 ] is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all vps_extension_data_flag syntax elements that follow the value 1 for vps_extension_type_flag[ 7 ] in an VPS NAL unit.

| vps_multilayer_common_extension ( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) |  |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| view_id_len | u(4) |
| if( view_id_len > 0 ) |  |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| vps_sub_layers_max_minus1_present_flag | u(1) |
| if( vps_sub_layers_max_minus1_present_flag) |  |
| for( i = 0; i <= MaxLayersMinus1; i++ ) |  |
| sub_layers_vps_max_minus1[ i ] | u(3) |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i <MaxLayersMinus1; i++ ) |  |
| for( j = i + 1; j <= MaxLayersMinus1; j++ ) |  |
| if( direct_dependency_flag[ j ][ i ] ) |  |
| max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| NumOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_idc | u(2) |
| for( i = 1; i <numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| if( vps_max_layers_minus1> 0 ) |  |
| alt_output_layer_flag | u(1) |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(8) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0) |  |
| vps_rep_format_idx[ i ] | u(8) |
| max_one_active_ref_layer_flag | u(1) |
| for( i = 1; i<= MaxLayersMinus1; i++ ) |  |
| if( NumDirectRefLayers[ layer_id_in_nuh[ i ] ] = = 0 ) |  |
| poc_lsb_not_present_flag[ i ] | u(1) |
| cross_layer_phase_alignment_flag | u(1) |
| dpb_size( ) |  |
| direct_dep_type_len_minus2 | ue(v) |
| default_direct_dependency_flag | u(1) |
| if( default_direct_dependency_flag ) |  |
| default_direct_dependency_type | u(v) |
| else { |  |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| } |  |
| if( vps_vui_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_vui_alignment_bit_equal_to_one | u(1) |
| vps_vui( ) |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).