| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0073-r1 |
| --- | --- |

| Title: | RCE1: Results of subtest A |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qti.qualcomm.com |
| Source: | Qualcomm, Inc. |  |  |

_____________________________

# Abstract

This document reports the experimental results for subtest A of HEVC Range Extensions core experiment 1, as described in document JCTVC-O1121. The switch from CABAC context coding to bypass coding at different points in the transform coefficient coding process is tested. The criterion for the switch is the Golomb Rice parameter at the end of the previous 4×4 subblock. It is reported that methods A1 and A2 show virtually no degradation in BD-rate performance compared to the anchor under AHG18 and AHG5 test conditions. It is asserted that methods A1 and A2 reduce the number of context-coded bins for a 4×4 subblock by 18% and 50%, respectively, in the worst-case. It is reported that for AHG18 test conditions and internal bit-depth of 16 bits, on average, methods A1 and A2 reduce the number of context-coded bins for a 4×4 subblock by 22% and 50%, respectively.

# Introduction

For high bit-depth video sequences, CABAC throughput can be a concern even at resolutions of 1080P or higher. HEVC v1 already incorporates techniques to increase the CABAC throughput by shifting some of the CABAC context-coded bins (coeff_abs_level_greater1 and coeff_abs_level_greater2 flags) to bypass. Subtest A of HEVC Range Extensions core experiment 1 (JCTVC-O1121) proposes shifting more context-coded bins to bypass coding. The shift from context-coding to bypass coding for the current 4×4 subblock is based on the Golomb Rice parameter at the end of the previous 4×4 subblock. Three different subtests (A1, A2 and A3) explored a switch to bypass coding at different stages in the coefficient coding process. A fourth subtest explored coding all the syntax elements in the bypass mode.

## Subtest A1

In this subtest the coding of coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for a 4×4 subblock if the Golomb Rice parameter at the end of the previous 4×4 subblock is greater than or equal to a threshold (4). The coeff_abs_level_remaining syntax element is adjusted to account for the fact that coeff_abs_level_greater1 and coeff_abs_level_greater2 flags are not coded.

## Subtest A2

In this subtest the coding of coded_sub_block_flag, significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for a 4×4 subblock if the Golomb Rice parameter at the end of the previous 4×4 subblock is greater than or equal to a threshold (4). The coeff_abs_level_remaining syntax element is adjusted to account for the fact that significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags are not coded.

In this case the coding pass for coeff_sign_flag comes after the coding pass for the coeff_abs_level_remaining syntax element.

## Subtest A3

In this subtest the coding of coded_sub_block_flag, significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for an entire transform block if the Golomb Rice parameter at the end of the previous transform block is greater than or equal to a threshold (4). The coeff_abs_level_remaining syntax element is adjusted to account for the fact that significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags are not coded.In this case the coding pass for coeff_sign_flag comes after the coding pass for the coeff_abs_level_remaining syntax element. Furthermore, the last significant coefficient position is coded in bypass mode as (blockSizeX – LastSignificantCoeffX) and (blockSizeY – LastSignificantCoeffY) using existing binarization.

## Subtest A4

In this subtest, all the transform blocks are coded as described in subtest A3 with a threshold equal to 0. Furthermore, rest of the syntax elements are also coded in bypass mode.

# Simulation results

The proposal is implemented on top of HM12.1_Rext5.1. Simulations are performed under AHG18, AHG5 and AHG8 (lossless) test conditions. The performance is compared to the anchor in terms of BD-rate savings.

## AHG18 results

Table 1: BD-rate results for subtest A1 under AHG18 test conditions (threshold 4)

Table 2: BD-rate results for subtest A2 under AHG18 test conditions (threshold 4)

Table 3: BD-rate results for subtest A3 under AHG18 test conditions (threshold 4)

Table 4: BD-rate results for subtest A4 under AHG18 test conditions

## AHG5 results

Table 5: BD-rate results for subtest A1 under AHG5 test conditions

Table 6: BD-rate results for subtest A2 under AHG5 test conditions

Table 7: BD-rate results for subtest A3 under AHG5 test conditions

Table 8: BD-rate results for subtest A4 under AHG5 test conditions

## AHG8 Lossless results

Table 9: BD-rate results for subtest A1 under AHG8 lossless test conditions

Table 10: BD-rate results for subtest A2 under AHG8 lossless test conditions

Table 11: BD-rate results for subtest A3 under AHG8 lossless test conditions

Table 12: BD-rate results for subtest A4 under AHG8 lossless test conditions

# Throughput

## Bin counts per sample per component

Table 13: Average bin count for subtest A1 under AHG18 test conditions

Table 14: Average bin count for subtest A2 under AHG18 test conditions

Table 15: Average bin count for subtest A3 under AHG18 test conditions

Table 16: Average bin count for subtest A4 under AHG18 test conditions

## Throughput analysis

Main advantage of the proposed methods is that they reduce the number of context-coded bins and move them to bypass subject to a criterion. Even though both context-coded and bypass bins use CABAC engine, bypass mode is much simpler as there is no state update. On some software architectures, division instruction can be used to decode multiple bypass bins. Although for different architectures may have varying advantages for decoding bypass bins over context-coded bins, it is reasonable to say that the throughput bypass bins may be 5X or higher compared to the context-coded bins.

The previous subsection has provided average bin counts for the proposals. Here we present the analysis of the worst case. Since the criterion for switching to bypass mode depends on the Golomb Rice parameter at the end of the last 4×4 subblock, the worst case complexity is calculated over 2 consecutive 4×4 subblocks. If the Golomb Rice parameter at the end of the first 4×4 subblock is less than 4, the number of total bins is much lower than the worst case. So we will concentrate on the case when the first block achieves the worst case complexity. In this case, the complexities for subtests A1 and A2 are as specified in Tables 17 and 18. Since subtests A3 and A4 have much worse BD-rate performance, we have not included them in the analysis.

For context-coded bins, in the worst case, 16 significance flags, 8 coeff_abs_level_greater1 flags and 1 coeff_abs_level_greater2 flag are possible. For the bypass bins, it is assumed that GolomRiceGroupAdaptation is enabled.

|  | Base | 1st 4×4 subblock | 2nd 4×4 subblock | Average |
| --- | --- | --- | --- | --- |
| Total number of context bins | 16+8+1 = 25 | 25 | 16 | 20.5 |
| Total number of bypass bins with CABAC | (1+46) + (1+39)*15 = 647 | 647 | 647 | 647 |

Table 17: Analysis of worst-case bin count for subtest A1

|  | Base | 1st 4×4 subblock | 2nd 4×4 subblock | average |
| --- | --- | --- | --- | --- |
| Total number of context bins | 16+8+1 = 26 | 25 | 0 | 12.5 |
| Total number of bypass bins with CABAC | (1+46) + (1+39)*15 = 647 | 647 | 647 | 647 |

Table 18: Analysis of worst-case bin count for subtest A2

# Conclusions

Four different methods for increasing the throughput of transform coefficient coding are presented. They represent different trade-offs between throughput and BD-rate performance.

Method A1 has virtually identical BD-rate performance compared to the anchor for AHG18 and AHG5 test conditions and small loss (in the range of 0.0% to 0.3%) for mandatory sequence classes under AHG8 lossless test conditions. It reduces the number of context-coded bins in the worst-case for a 4×4 subblock by 18%. On average, the reduction in context-coded bins is 22% for internal bit-depth of 16 bits.

Method A2 has small BD-rate loss (average 0.1%) compared to the anchor for AHG18 and AHG5 test conditions. However, under AHG8 lossless test conditions, large losses are observed for screen content sequences. Method A2 reduces the number of context-coded bins in the worst-case for a 4×4 subblock by 50%. On average, the reduction in context-coded bins is nearly 60% for internal bit-depth of 16 bits.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).